3GPP TSG-RAN WG2 Meeting #132	R2-2508279

Dallas, USA, 17 – 21 November, 2025

Agenda Item:	9.7.2

Source:	Huawei, HiSilicon

Title:	Further discussion on voice over NB-IoT NTN via GSO

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, RAN2 discussion focused on how to reply the LSs to SA2 and SA4 for voice over NB-IoT NTN, and reached some agreements as follows [1]:

Agreements regarding the reply LS to SA2:

1.	Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

2.	Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

3.	Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

4.	Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Agreements regarding the reply LS to SA4:

1.	RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

2.	For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size

3.	For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size

4.	RAN2 will provide further feedback on the expected average RoHC header size

5.	We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

In this contribution, we provide further insights from RAN2 perspective about the support of voice over NB-IoT NTN via GSO.

# 2	Discussion

2.1	Down-selection between CP and UP solution

In the last meeting, RAN2 indicated in the LS to SA2 that saying that the majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

In the meantime, during the SA2#171 meeting, SA2 reached an interim conclusion on using UP solution to transport the voice packets over NB-IoT via GSO [2].

-------------Excerpted from S2-2509293-------------

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Interim conclusions for Key Issue #1

Editor's note 1:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Based on the situation in SA2 and RAN2, it is proposed that RAN2 make a recommendation on selecting UP solution to transmit voice data packets to RAN Plenary.

Proposal 1: RAN2 recommends to select the UP solution for voice data and signalling packets transmission.

In this context, voice data packets are transmitted via UM DRB. For voice signaling packets, there is still different opinions on using DRB or SRB. However, it is straightforward to use AM DRB to transmit voice signaling packets under the UP-solution architecture. SRB should only be used for NAS data transfer in CP solution. Since RAN2 confirms there is no issue to support more than 2 DRBs, there is no strong reason to use SRB for voice signaling packets.

Proposal 2: In UP solution, the voice data packets are transmitted via UM DRB while the voice signalling packets are transmitted via AM DRB.

2.2 Size of RoHC header

RAN2 received the LS on the RAN simulation assumptions for ULBC from SA4 as follows [3]:

# To SA2 and RAN2:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

Regarding this LS, RAN2 agreed in the last meeting to provide further feedback on the expected average RoHC header size. Although this is out of RAN2 scope, some discussion can happen in RAN2 to see whether some common understanding can be achieved.

ROHC is specified in IETF. So, we can try to refer to the IETF specs. According to clause 5.7 in RFC3095, we have the following formats for different kinds of compressed header, e.g., in different ROHC modes and different ROHC status:

| In subsequent sections, the position of the large CID in the diagrams is indicated using this notation: +===+===+===+===+===+===+===+===+ |  |  |
| --- | --- | --- |
| R-0 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 0 0 ¦ SN ¦ +===+===+===+===+===+===+===+===+ | R-0-CRC 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 0 1 ¦ SN ¦ +===+===+===+===+===+===+===+===+ ¦SN ¦ CRC ¦ +---+---+---+---+---+---+---+---+ | UO-0 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 0 ¦ SN ¦ CRC ¦ +===+===+===+===+===+===+===+===+ |
| R-1 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦ SN ¦ +===+===+===+===+===+===+===+===+ ¦ M ¦ X ¦ TS ¦ +---+---+---+---+---+---+---+---+ | R-1-ID 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦ SN ¦ +===+===+===+===+===+===+===+===+ ¦ M ¦ X ¦T=0¦ IP-ID ¦ +---+---+---+---+---+---+---+---+ | R-1-TS 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦ SN ¦ +===+===+===+===+===+===+===+===+ ¦ M ¦ X ¦T=1¦ TS ¦ +---+---+---+---+---+---+---+---+ |
| UO-1 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦ TS ¦ +===+===+===+===+===+===+===+===+ ¦ M ¦ SN ¦ CRC ¦ +---+---+---+---+---+---+---+---+ | UO-1-ID 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦T=0¦ IP-ID ¦ +===+===+===+===+===+===+===+===+ ¦ X ¦ SN ¦ CRC ¦ +---+---+---+---+---+---+---+---+ | UO-1-TS 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 0 ¦T=1¦ TS ¦ +===+===+===+===+===+===+===+===+ ¦ M ¦ SN ¦ CRC ¦ +---+---+---+---+---+---+---+---+ |
| UOR-2 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 1 0 ¦ TS ¦ +===+===+===+===+===+===+===+===+ ¦TS ¦ M ¦ SN ¦ +---+---+---+---+---+---+---+---+ ¦ X ¦ CRC ¦ +---+---+---+---+---+---+---+---+ | UOR-2-ID 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 1 0 ¦ IP-ID ¦ +===+===+===+===+===+===+===+===+ ¦T=0¦ M ¦ SN ¦ +---+---+---+---+---+---+---+---+ ¦ X ¦ CRC ¦ +---+---+---+---+---+---+---+---+ | UOR-2-TS 0 1 2 3 4 5 6 7 +---+---+---+---+---+---+---+---+ ¦ 1 1 0 ¦ TS ¦ +===+===+===+===+===+===+===+===+ ¦T=1¦ M ¦ SN ¦ +---+---+---+---+---+---+---+---+ ¦ X ¦ CRC ¦ +---+---+---+---+---+---+---+---+ |

If we assume the CID is typically 1 Byte, we can see from the above figure that the ROHC header varies among 2 Bytes, 3 Bytes and 4 Bytes. For initial state where the header is not compressed, since it is predictable, NW can use dynamic scheduling. So, we may only need to consider to use SPS for the packets with 2 Bytes, 3 Bytes and 3 Bytes ROHC header. Besides, since RAN1 implies in their LS [4] to SA2 that in line-of-sight condition, ROHC status change is negligible and other cases may not happen very often, from RAN2 perspective, we can assume that in most cases, the ROHC header can be as small as 2 Bytes, and occasionally it may change to 3 Bytes or 4 Bytes.

Proposal 3: RAN2 assumes in most cases, the ROHC header can be as small as 2 Bytes, and occasionally it may change to 3 Bytes or 4 Bytes.

2.3 Scheduling of variable-size packets

During the last meeting, RAN2 replied to SA2 that RAN2 will further study and discuss the details of the scheduling mechanism to handle the voice packets of different sizes. At the same time, there were concerns that we may introduce potentially large resource waste or longer schedule delay.

Downlink scheduling

For DL, since NW can detect the packet size change before making scheduling decisions, it can be up to NW implementation on using dynamic scheduling or a new SPS if the packet size changes. This doesn’t introduce extra resource waste and scheduling delay.

Proposal 4: RAN2 confirms that for downlink NW can handle scheduling of variable-size packets by implementation.

Uplink scheduling

For UL, RAN2 needs to discuss how to schedule the variable-size packets. Since resource is limited in NB-IoT case due to the narrow band and propagation delay is long due to GEO case, RAN2 should avoid using a scheduling mechanism with these drawbacks for UL.

Proposal 5: For UL, RAN2 strives to design a scheduling mechanism to handle the variable-size packets considering to minimize the resource waste and scheduling delay.

Based on this principle, it is not recommended to use a single SPS whose resource size is overly large to accommodate the largest possible packet or to use multiple simultaneous active SPS due to large resource waste. It is also not recommended to use dynamic scheduling due to the long propagation delay of the dynamically signaling.

Preconfiguring multiple SPS but ensuring only one to be activated at a time is a good direction to go. In this case, NW can still allocate the resource of the non-active SPS for other use before it is activated.

Proposal 6: For UL, RAN2 considers to preconfigure multiple SPS but ensuring only one to be activated at a time.

# 3	Conclusion

This contribution discusses the voice over NB-IoT NTN via GEO and we have the following proposals:

Down-selection between CP and UP solution

Proposal 1: RAN2 recommends to select the UP solution for voice data and signalling packets transmission.

Proposal 2: In UP solution, the voice data packets are transmitted via UM DRB while the voice signalling packets are transmitted via AM DRB.

Size of RoHC header

Proposal 3: RAN2 assumes in most cases, the ROHC header can be as small as 2 Bytes, and occasionally it may change to 3 Bytes or 4 Bytes.

Scheduling of variable-size packets

Proposal 4: RAN2 confirms that for downlink NW can handle scheduling of variable-size packets by implementation.

Proposal 5: For UL, RAN2 strives to design a scheduling mechanism to handle the variable-size packets considering to minimize the resource waste and scheduling delay.

Proposal 6: For UL, RAN2 considers to preconfigure multiple SPS but ensuring only one to be activated at a time.

# 4	References

- R2-2507703, Report from Break-out session on NES, NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 131bis.

- S2-2509293, Interim conclusions on KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, SA2

- S4-251584, LS on the RAN simulation assumptions for ULBC, SA4.

- R1-2508096, Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC