3GPP TSG-RAN WG2 Meeting #132	R2-2508284

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on harmonized design for TN and NTN

Document for: Discussion and Decision

1	Introduction

6GR intends to enable new business opportunities, reduce total cost of ownership (TCO) and improve user experience. In the updated RAN study item [1], the following objective was agreed:

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration

NTN topic in the context of 6GR was discussed during RAN2#131bis meeting and the following guidance was made [2]:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we analyse NTN characteristics and scenarios, and discuss how to support harmonized design for TN and NTN from RAN2 point of view.

2	Discussion

2.1	Critical NTN Characteristics

Large cell size

While TN deployments typically assume small cell coverage (e.g., inter-site distance <7.5km in rural areas), NTN can support significantly larger coverage areas for outdoor UEs. For example, in 5G NTN, a single beam footprint may span approximately 50km in diameter, with each cell potentially comprising one or multiple beams.

NTN can therefore serve as supplemental coverage to TN in outdoor scenarios due to its large coverage area. However, the significantly larger cell size (orders of magnitude beyond TN) poses new challenges that should be considered during the 6G system design. Specially, the cell management procedures such as system information delivery, becomes less efficient due to the large cell size.

Long propagation delay

While enabling wide coverage, NTN introduces substantial propagation delays for service that has impact on user experience. In the following Table 2.1-1, we summarize the long propagation delays introduced by NTN access, taking GEO and LEO (orbit altitude 600km) as examples. According to the table, the delay in the NTN communication can be up to tens or hundreds of milliseconds depending on the scenarios and needs to be considered in the 6G design.

Table 2.1-1 NTN scenarios versus delay constraints (cited from TR 38.821[3])

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

Frequent and multi-UE simultaneous mobility

In 5G NTN, three cell models were specified including earth fixed cell, quasi-earth fixed cell and earth-moving cell. Usually, the earth fixed cell is more like cells deployed in TN, where the UE mobility drives handovers/reselections. In the quasi-earth fixed cell and earth moving cell, the mobility is caused by both satellite (e.g., LEO) mobility and UE mobility. LEO satellites' high velocity (e.g., the relative speed of satellite with respect to earth is 7.56 km/s for LEO with 600km altitude) causes large Doppler shifts and frequent change of the serving satellite, e.g., the average serving duration of satellite for a UE is around 3 mins with altitude 600km (calculated with the trace in Figure 2.1-1). What's worse, many UEs may perform handover to the same satellite almost at the same time, especially in the quasi-earth fixed cell case. This may cause signalling storm and increase collision probability. To avoid the degradation of user experience, the frequent and multi-UE simultaneous mobility should be considered in the 6G design.

Figure 2.1-1 example of frequent mobility in LEO scenario

Unobvious near-far effect

The unobvious near-far effect has been identified in 5G as one of the key issues to be addressed for NTN mobility. Unlike TN, where signal quality typically degrades noticeably from the cell centre to the cell edge, NTN exhibits minimal signal variation across the coverage area. As a result, RSRP-based mobility management becomes less effective. This characteristic should be taken into account in the 6G system design.

Proposal 1: RAN2 considers the following NTN characteristics: large cell size, long propagation delay, frequent and multi-UE simultaneous mobility, unobvious near-far effect, in 6G design for NTN communication.

2.2	Dense or Sparse Constellation

The LEO satellite provides moving coverage, which requires multiple satellites to be launched to the same orbital plane to ensure continuous coverage. A satellite operator may plan to deploy a large LEO constellation achieves a dense coverage. However, as the satellite deployment is both time consuming and costly, completing a full LEO constellation is a long-term process that may take several years.

During the early deployment stage, the coverage of the LEO constellation may not ensure continuous service in the target areas, resulting in discontinuous coverage and potential impact on early NTN network operation. 6G is expected to provide an acceptable user experience during such early operational phases, for example, by utilizing wide-area coverage provided by GEO satellites to complement the sparse LEO deployment via NTN mobility. As the LEO constellation’s deployment evolves from sparse to dense, both deployment stages should be taken into account in 6G system design.

Proposal 2: RAN2 studies whether there is any impact on NTN-NTN mobility considering dense or sparse constellation.

2.3	Functionalities for Harmonized Design for TN and NTN

In the 6GR design, we aim at a harmonized design for TN and NTN. A harmonized radio interface design shall strive to achieve the maximum commonality between TN and NTN, and consider only essential NTN specific features. To better understand the potential 6GR features, we classify the NTN related functionalities into the following three categories, as shown in figure 2.2-1.

- TN/NTN common design (no specific consideration for NTN)

For this category, TN and NTN will share the same design. Discussions can be conducted in the common agendas.

- TN/NTN design with common framework (with only necessary adaptation for NTN)

For this category, necessary adaptation is needed for NTN compared to the design for TN. It can be discussed in the common agendas taking the agreed NTN characteristics and scenarios into account.

- NTN specific design

For this category, the functionalities try to address the NTN specific requirements and issues which does not apply for TN. It is more appropriate to be discussed in the NTN agenda.

Harmonized 6GR air interface design for TN & NTN

NTN-specific

design

Common framework for TN & NTN

Common design for TN & NTN

Figure 2.3-1: Harmonized 6GR air interface design for TN and NTN

Proposal 3: Functionalities for NTN are classified into three categories:

- TN/NTN common design: no specific consideration for NTN is required

- TN/NTN design with common framework: a common design framework is used, with only necessary adaptation for NTN

- NTN specific design: dedicated functionalities designed specifically for NTN operation.

2.3.1 TN/NTN common design

To have a harmonized design for TN and NTN, from RAN2 perspective, the basic design should be common, e.g., the L2 protocol stacks to support communication, the basic scheduling mechanisms (e.g., LCP for UL transmission), and the basic mobility procedure to enable the UE to switch the serving cell. More common functionalities can be included for this category if identified.

For now, we propose that the following functionalities are classified into the common design category:

Proposal 4: RAN2 studies at least the following functionalities as TN/NTN common design: L2 protocol stacks, basic scheduling, basic RRM-based mobility procedure, etc.

2.3.2 TN/NTN design with common framework

In the 5G design, NTN was not a Day-1 feature, and the TN basic designs were already stable when NTN was introduced in later releases, making it difficult to adapt fundamental features (e.g., SSB periodicity for initial access) to NTN characteristics and posing negative impacts on NR NTN commercialization. Building on lessons from 5G NTN designs, the issues associated with the NTN characteristics and scenarios should be addressed in the 6G Day-1 design.

In the following, we list some identified functionalities within the common framework for TN and NTN.

In 5G, GNSS function of UE and the ephemeris information in system information are essential for NTN initial access, which is not the case for TN design. In 6G, RAN1 already agreed to study GNSS-less/resilient/based operation [4]. RAN1 also agreed to studying initial access for TN and NTN (including cell search and SSB periodicity), the necessary RAN2 impacts can be discussed after further progress is made in RAN1.

Similarly, the system information acquisition procedure can follow the common framework. However, the system information deemed essential may differ between TN and NTN communications, necessitating the introduction of essential adaptations for NTN. The detailed adaptation for NTN can be further studied in the agenda items where the corresponding functionalities are discussed for TN.

The basic paging design, which enables UEs to remain reachable for services, is recommended to adopt a unified framework applicable to both TN and NTN. However, due to the limited RF capability and power constraints of handheld UEs, establishing NTN communication successfully may not always be feasible without user assistance (e.g., user intervention to achieve UE-to-satellite alignment). This scenario can occur, for instance, when the UE is stored in a pocket or bag. To minimize unnecessary user disturbance, it is advisable to consider a mechanism that allows the user to decide whether to initiate communication in response to paging.

Proposal 5: RAN2 studies at least the following functionalities as a part of TN/NTN design within a common framework: initial access (with or without ephemeris/GNSS), basic system information design, basic paging design.

2.3.3 NTN specific design

In the following, several NTN specific functionalities which we think are essential are identified.

For instance, NTN specific mobility should be studied to address unique NTN characteristics and scenarios. This includes location/time/angle-based mobility management to mitigate the unobvious near-far effect and satellite switching with resync to handle the frequent and multi-UE simultaneous mobility occurrence.

In addition, efficient design approaches for NTN should be explored to reduce power consumption and enhance user experience. For example, beam-level system information may be considered to optimize the system information delivery and address the challenges introduced by large cell sizes.

Therefore, we have following proposal:

Proposal 6: RAN2 studies at least the following functionalities as part of NTN specific design: NTN specific mobility and beam-level system information.

3	Conclusion

This contribution includes the following observations and proposals:

Critical NTN Characteristics and Scenarios

Proposal 1: RAN2 considers the following NTN characteristics: large cell size, long propagation delay, frequent and multi-UE simultaneous mobility, unobvious near-far effect, in 6G design for NTN communication.

Proposal 2: RAN2 studies whether there is any impact on NTN-NTN mobility considering dense or sparse constellation.

Functionalities for Harmonized Design for TN and NTN

Proposal 3: Functionalities for NTN are classified into three categories:

- TN/NTN common design: no specific consideration for NTN is required

- TN/NTN design with common framework: a common design framework is used, with only necessary adaptation for NTN

- NTN specific design: dedicated functionalities designed specifically for NTN operation.

Proposal 4: RAN2 studies at least the following functionalities as TN/NTN common design: L2 protocol stacks, basic scheduling, basic RRM-based mobility procedure, etc.

Proposal 5: RAN2 studies at least the following functionalities as a part of TN/NTN design within a common framework: initial access (with or without ephemeris/GNSS), basic system information design, basic paging design.

Proposal 6: RAN2 studies at least the following functionalities as part of NTN specific design: NTN specific mobility and beam-level system information.

4	References

- RP-251881, “NEW SID: STUDY ON 6G RADIO”, 3GPP TSG RAN MEETING #108, PRAGUE, CZECH REPUBLIC, JUNE 9-13, 2025.

- R2-2508002	, RAN2#131bis Meeting Report, MCC

- TR 38.821, Solutions for NR to support Non-Terrestrial Networks (NTN)

- R1-2508301, Report of RAN1#122bis meeting, MCC