3GPP TSG-RAN WG2 Meeting #132	R2-2508288

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	Remaining issues for AIoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The document from 3GPP TSG-RAN WG2 Meeting #132 discusses open issues and proposals related to Ambient IoT (AIoT) in Release 19, focusing on scenarios where no NAS response is provided, the need for a new indication to signal no NAS data availability, considerations on authentication-related delayed NAS responses, and addressing denial of service attack concerns linked to paging responses within the current MAC specifications.

# 2	Discussion

## 2.1	Issue 3-7, 6-2, No NAS response

During RAN2#131bis it was agreed that RAN2 assumes the case where AIoT NAS does not provide data at all. Additionally, RAN2 determined that the device would be able to be aware of such cases, and there thus was a need for and opportunity to make the reader aware of such cases.

Agreements 1. RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. 2. For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). 3. For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

The main arguments against the report in general is that

- the device may not be aware whether data will be available or not since the data unavailability can be due to lack of power, broken sensor, no data, or other cases

- The indication may need to be futureproof for other cases, and thus may become more bits than necessary.

Optimally, since the devices are very simple, they should reply with the 0 SDU and in the end, it must be up to AIoT-F or reader whether to continue polling the device.

During online discussion, some companies wanted to repurpose the two bits available i.e. SDU length and MDI, whereas others wanted to have a new indication.

The SDU and MDI fields are defined as follows:

-	More Data Indication (MDI): This field indicates whether there are more upper layer data to be sent from the device (when set to 1) or not (when set to 0). This length of this field is 1 bit.

-	SDU Length: This field indicates the length of the Data SDU field in the unit of byte. The length of this field is 7 bits.

Each of the two fields has a distinct usage both in the description as well as the procedural text, and we believe that it would break the lean design of 38.391 if these are repurposed to indicate a semi-unrelated issue. Furthermore, extending the usage to other cases would make the spec messy. Thus, we propose that for explicit determination of whether the reader can expect NAS data or not, a new indication is needed. It is also important to note, that it make sense to have this indication a few bits worth of length.

Proposal 1: [3-7] Introduce a new indicator for no NAS available, allowing the device to indicate no data is expected. FFS on the length.

A new open issue based on the discussion from 3-7 is whether other the no data available case should be applied to more general delayed NAS cases, e.g., inventory response message. This issue was raised due to the additional time the device should use to perform the authentication mandated by SA3 at least for CFA, where no intermediate procedure exists i.e. to transmit the random number. The scenario may be valid, but we see that the limited expected energy consumption from a device which is already approaching what is physically possible, CFA will either work or fail continuously for all devices. As of such, we see RAN2 either needs to treat the CFA authentication failure as the primary case, or not at all.

Observation 1: AIoT transmission from devices should be rather deterministic in processing time.

Proposal 2: [6-2] RAN2 to first discuss whether the authentication request is seen as a corner case or not before deciding on whether to apply the no NAS data available solution here.

Observation 2: In case the delayed NAS data case is valid, and more than a corner case, significant D2R overhead will happen due to many empty messages indicating that the devices is processing.

Proposal 3a: [6-2] In case RAN2 agrees that the delayed NAS response for inventory i.e. due to authentication is not seen as a corner case, RAN2 should find another solution than delayed NAS response to avoid D2R resource wastage.

Proposal 3b: [6-2] In case RAN2 agrees that the delayed NAS response for inventory i.e. due to authentication is seen as a corner case, RAN2 assume this can be handled by NW implementation.

## 2.2	6-1 on CT1’s concern for Denial of service attacks

In C1-256624 CT1 mentions concerns of RAN2s agreement that “a Rel-19 device always responds to the new service indicated by the received paging message applicable for that device” as they see the issue that any rogue device will be able to broadcast a paging with no paging ID and thus interrupt the paging session.

We understand that the issue was discussed in RAN2 while discussing the end of service request, and that RAN2 made the agreement well known of the impact. CT1 asks whether it is possible to delay the response to the new paging until after the current paging has been processed. Our understanding of the MAC spec is, that this is not possible and a spec amendment for such procedures may not be trivial, as it will bring up the question of; which “next paging should be considered in case multiple new pagings comes during a session”? Essentially, in case RAN2 accepts the device to delay response, how many pagings should be stored, and how will the device know when the session has ended?

Observation 3: Enabling a device to delay reply to a paging autonomously to avoid service interruption may cause a race condition, or requirement of several buffered AIoT pagings to deny other denial of service attacks.

Observation 4: The best solution to address CT1’s concern is the end-of-service indication.

Proposal 4: [6-1] RAN2 to reply to CT1 that it is not possible with the current MAC spec to reply later to a paging and thus avoiding attackers to do denial of service attacks.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: [3-7] Introduce a new indicator for no NAS available, allowing the device to indicate no data is expected. FFS on the length.

Observation 1: AIoT transmission from devices should be rather deterministic in processing time.

Proposal 2: [6-2] RAN2 to first discuss whether the authentication request is seen as a corner case or not before deciding on whether to apply the no NAS data available solution here.

Observation 2: In case the delayed NAS data case is valid, and more than a corner case, significant D2R overhead will happen due to many empty messages indicating that the devices is processing.

Proposal 3a: [6-2] In case RAN2 agrees that the delayed NAS response for inventory i.e. due to authentication is not seen as a corner case, RAN2 should find another solution than delayed NAS response to avoid D2R resource wastage.

Proposal 3b: [6-2] In case RAN2 agrees that the delayed NAS response for inventory i.e. due to authentication is seen as a corner case, RAN2 assume this can be handled by NW implementation.

Observation 3: Enabling a device to delay reply to a paging autonomously to avoid service interruption may cause a race condition, or requirement of several buffered AIoT pagings to deny other denial of service attacks.

Observation 4: The best solution to address CT1’s concern is the end-of-service indication.

Proposal 4: [6-1] RAN2 to reply to CT1 that it is not possible with the current MAC spec to reply later to a paging and thus avoiding attackers to do denial of service attacks.