3GPP TSG-RAN WG2 Meeting #132	R2-2508289

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on Downlink coverage enhancements

WID/SID:	NR_NTN_Ph3-Core  - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc address’ the postponed issues on Downlink coverage enhancement from RAN2#131bis.

# 2	Discussion

## 2.1	[A200] Reference location association with smtc1

One postponed issue from RAN2#131bis was RIL A200 on whether the Rel-15 SMTC in SIB2 should be able to be associated with a reference location in refLocList-r19. This was based on the assumption that the serving cell is the one mandated to provide the largest periodicity and thus allowing a reference location to be associated would provide the network flexibility to i.e. have the largest periodicity on another cell, compared to the serving cell.

The original description and proposed changes are:

[Description]: The current assumption from CR rapporteur is SMTC in SIB2 is associated with reference location in SIB19, and it is used for serving cell measurement. We think it is not always the case. Note that when NW provides smtc in SIB2, it should associate with the largest SSB periodicity of serving and neighbor cell. For example, if serving cell has a SSB periodicity of 160ms, and neighbor cell has a SSB periodicity of 20ms, the smtc in SIB2 has to be with 160ms periodicity. That is, it is actually for neighbor cell measurement. And smtc5list can provide another periodicity of 20ms for serving cell measurement. Therefore, we would like to clarify that there should be an explicit reference location associated with legacy smtc configured in SIB2. [Proposed Change]: Allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

We note that SMTC is the primary measurement timing configuration. It is used by both DL-CE capable UEs, that have support for the refLocList-r19 and smtc5list, and by legacy UEs, which cannot read the refLocList-r19. According to the RAN2 design in Rel-17, the periodicity of all SMTCs in smtc4list is the same as the one defined in smtc1. A UE that is not capable of DL-CE, may use only the information in smtc1 and in smtc4list to measure the cells in a given frequency layer. If the serving cell SMTC is defined in smtc5list, due to having a periodicity different than the one in smtc1, a legacy UE will be configured with the SMTC to be used in serving cell measurements. Therefore, the periodicity defined in smtc1 should be the one used for the serving cell measurements.

Observation 1: Both a legacy UE, and a UE not supporting the configuration of smtc5 nor refLocList-r19, relies only on SMTC and smtc4 for the measurement configuration.

Considering the reasoning above, we do not agree with issue A200, and do not support the association of smtc with a position in the refLocList-r19. The SMTC configuration should be the one used for the serving cell measurements.

Proposal 1: [A200] To not support issue A200.

## 2.2	[H251] Neighbour cell propagation delay for SMTC5

Another open issue, also considering SIB2, is whether the field description should be modified to ensure that UE does not need to consider neighbour cell propagation delay when smtc5list is configured.

The original description and proposed changes are:

[Description]: It was agreed to have 7 SMTCs altogether on a single frequency. Serving cell does not require a reference location, and in this case the legacy smtc is used for the serving cell measurement. However, this understanding is a bit different from legacy releases because smtc is now changed to a cell-specific SMTC rather than a frequency-specific SMTC. Also, the field description of smtc related to SMTC adjustment based on PDD needs to revised so that UE does not need to consider neighbour cell propagation delay. smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell and smtc5list is not configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. If the field is broadcast by an NTN cell and smtc5list is configured, smtc is for serving cell measurements and the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference equals to 0 ms.

In this issue, we agree with the comment of the rapporteur. In our view, the cells that are measured with smtc are those that have the same propagation delay and periodicity as the serving cell. So some neighbour cell may be measured with the same smtc as the serving cell. The text in the specification applies for both cases, when smtc5list is configured or not configured, and the change is not needed.

Proposal 2: To not support issue H251.

## 2.3	Prohibit timer of UEAssistanceInformation for reporting closest reference locations

One postponed issue raised in RAN2#131bis was to introduce a prohibit timer to the UE assistance information report for closest reference locations in order to avoid extensive reporting. Although the issue is relevant, as the implementation of the Rel-19 defines a procedure to transmit an updated report if the UE determines the location changes. However, we do believe that the UE does not obtain it’s location without need, and thus it may be that the ping-pong may not happen.

Observation 2: A UE should only obtain a new location from GNSS if needed i.e. to update timing advance etc. thus ping-pong report of closest reference locations may not be an issue.

Proposal 3: A prohibit timer for UE assistance information for reporting reference location report is not needed.

## 2.4 	Inter-frequency measurements

In the past RAN2 meeting, it has been agreed that the closest reference location information can be used to assist the SMTC and measurement gap configuration.

[H250]: Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisted SMTC and measurement gap configuration” (can come back to this after the discussion on inter-frequency support) We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized. We introduce a new (per-UE) capability with signalling for the support in RRC connected of 2 periodicities in the inter-frequency case

From the discussion in the previous RAN2 meeting R2-2507763, the following scenarios were listed:

- Scenario 1: the adjacent cells in the inter-frequency layers are non-overlapping with the cells in the intra-frequency layer.

- Scenario 2: the adjacent cells in the inter-frequency layers overlapping (with no spatial offset) with the cells in the intra-frequency layer.

- Scenario 3: the adjacent cells in the inter-frequency layers overlapping (with a spatial offset) with the cells in the intra-frequency layer.

- Scenario 4: the adjacent cells in the inter-frequency are smaller than the cells in the intra-frequency layer, and overlap with the cells in the intra-frequency layer.

Observation 3: RAN2 solution for the SMTC selection for inter-frequency measurements should consider the different scenarios regarding the overlap of the cells in the inter-frequency layers with the cells in the intra-frequency layer, prioritizing scenarios 1 to 3.

Consider the example below in Figure 1, in which two UEs would provide the same reference location report in the inter-frequency layer, but are closer to different neighbor cells. In the figure, the dashed line represents the intra-frequency cells and the colors represent the inter-frequency cells. The numbers represent the SMTCs configuration for the inter-frequency neighbor cell measurements.

UE1 is closer to the cells: 1, 4, 2 and 3, in descending order. And UE2 is closer to cells 1, 2, 3 and 4. Both would provide the same bitmap to the gNB. Additionally, in NTN, in RRC_CONNECTED, a UE can be configured with up to 4 SMTCs per frequency layer and 2 measurement gap configurations if parallelMeasurementGap-r17 is indicated.

Observation 4: The number of measurement gaps that can be configured per frequency layer (up to 2) is smaller than the number of SMTCs (up to 4).

The indication of 4 SMTCs per frequency layer may not provide enough information for the gNB for the configuration of the measurement gaps. Consider the example in Figure 2, in which the UE is configured with 4 SMTCs for the neighbour cell measurements, but only 2 measurement gaps can be configured. By receiving the bit map with the indication of SMTCs 1, 2, 3 and 4, the gNB may configure MG1 and MG3 for both UEs, when UE2 would benefit from the configuration of MG1 and MG2 instead. The indication of which neighbor cells are closer to the UE will not help the gNB in configuring the best measurement gap for inter-frequency measurements.

Therefore, we propose that for inter-frequency measurements, the number of reported locations in the reference location report is limited to the number of measurement gaps that can be configured to a specific UE.

Proposal 4: The number of reference locations to be reported by a UE for inter-frequency scenario is limited by the number of SMTCs that can be measured at the UE with 1 or 2 measurement gaps, depending on the UE capability.

Figure 1 – Example of location report provided by UE1 and UE2

Figure 2 - Example of SMTC configuration and Measurement Gaps (MG)

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Both a legacy UE, and a UE not supporting the configuration of smtc5 nor refLocList-r19, relies only on SMTC and smtc4 for the measurement configuration.

Proposal 1: [A200] To not support issue A200.

Proposal 2: [H251]To not support issue H251.

Observation 2: A UE should only obtain a new location from GNSS if needed i.e. to update timing advance etc. thus ping-pong report of closest reference locations may not be an issue.

Proposal 3: A prohibit timer for UE assistance information for reporting reference location report is not needed.

Observation 3: RAN2 solution for the SMTC selection for inter-frequency measurements should consider the different scenarios regarding the overlap of the cells in the inter-frequency layers with the cells in the intra-frequency layer, prioritizing scenarios 1 to 3.

Observation 4: The number of measurement gaps that can be configured per frequency layer (up to 2) is smaller than the number of SMTCs (up to 4).

Proposal 4: The number of reference locations to be reported by a UE for inter-frequency scenario is limited by the number of SMTCs that can be measured at the UE with 1 or 2 measurement gaps, depending on the UE capability.