3GPP TSG-RAN WG2 Meeting #132	R2-2508290

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on MBS

WID/SID:	NR_NTN_Ph3-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#131bis the following issues were not yet resolved for MBS, but are discussed here:

- R2-2506868

- R2-2507634

# 2	Discussion

## 2.1	C003 Service area information in SIB and USD

C003 address the issue that in some cases, the service area provided via SIB or USD may not be completely aligned, e.g., due to the different formats with one being a circle and the other a polygon, or not timely update frequency of the MCCH. C003 propose to define how UE determines the service area if the Target Service Area in the USD or the ISA in SIB27 are not totally aligned

We agree that there is a valid case where the target service area in the USD and the ISA in SIB27 are not completely aligned, due to different formats of the area definition. As the ISA is SIB27 is the area that is referred to in the context of MCCH, it makes sense to prefer the ISA information in SIB27. However if this is an high impact change, we believe that the UE behavior should be specified, rather than in a note.

Proposal 1: [C003] Prioritize the ISA information in SIB27

Proposal 2: [C003] The UE behavior upon mismatch between SIB and USD should be added in procedural text.

## 2.2	Service continuity for MBS

Due to satellite mobility, cells are frequently changing, which is an inherent characteristic of (non-GSO) NTN deployments. If a broadcast service is provided via NTN, it is reasonable to assume that the next satellite/cell replacing the current one will provide similar service covering the same area. Hence, a UE changing cell due to satellite mobility (and not due to UE mobility) should be able to continue the same service.

Observation 1: It can be assumed that a satellite cell replacing the previous one will provide the same service.

Under this assumption, it would be very inefficient to update and re-acquire the same information upon every satellite switch. Therefore, it is reasonable to assume that for mobility due to satellite movement, the MCCH content stays the same. This should be the baseline assumption when deploying MBS from satellites and thus there is also no need of notifying the UE about this.

Proposal 3: RAN2 baseline assumption is that the MTCH configuration stays the same over a geographic area.

If this is the case, the UE would not re-acquire MCCH upon cell change due to satellite movement. So, there is also no benefit adding future neighbor cell information in the MCCH.

Proposal 4: There is no need to regularly add future neighbor cell information.

# 3	Conclusion

Proposal 1: [C003] Prioritize the ISA information in SIB27

Proposal 2: [C003] The UE behavior upon mismatch between SIB and USD should be added in procedural text.

Observation 1: It can be assumed that a satellite cell replacing the previous one will provide the same service.

Proposal 3: RAN2 baseline assumption is that the MTCH configuration stays the same over a geographic area.

Proposal 4: There is no need to regularly add future neighbor cell information.