3GPP TSG-RAN WG2 Meeting #132	R2-2508299

Dallas, US, November 17th – 21st, 2025

Agenda Item:	8.17

Source:	OPPO

Title:	Discussion on SPS collision in IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In this contribution, we discuss SPS collision issue in IoT NTN TDD mode and present our view.

Discussion

Regarding ULSPS in IoT NTN TDD mode, RAN2 made the following agreement in RAN2#129bis meeting.

Agreements in RAN2#129bis:

1.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

There may be a case that both SPS occasions for UE1 and UE2 which are determined based on the SPS formula are located in non-U NB-IoT subframes within the same 90ms TDD period. With this postponement, the SPS occasion for both UE1 and UE2 would be postponed the next first valid UL subframe, leading to SPS collision issue, and both UE1 and UE2 cannot be aware of the UL SPS collision.

Figure 1 SPS collision issue in IoT NTN TDDmode

- There may be SPS collision between different UEs, and UEs cannot be aware of the UL SPS collision.

In the last RAN2 meeting, this issue was discussed. Many companies thought it is a valid issue, but has not reached consensus on the solution. In general, the following potential solutions are on the table:

- Option 1: Introduce new SPS periodicities as multiple of 90ms

- Option 2: UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for SPS

The SPS occasion formular captured in MAC spec is as below.

After a Semi-Persistent Scheduling uplink grant is configured, the MAC entity shall: - if twoIntervalsConfig is enabled by upper layer: - set the Subframe_Offset according to Table 7.4-1. - else: - set Subframe_Offset to 0. - consider sequentially that the Nth grant occurs in the TTI for which: - subframe SPS is used: - (10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * semiPersistSchedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240. - slot or subslot SPS is used: - (10 * SFN * sTTI_Number_Per_Subframe + subframe * sTTI_Number_Per_Subframe + sTTI_number) = [(10 * SFNstart time * sTTI_Number_Per_Subframe + subframestart time * sTTI_Number_Per_Subframe + sTTIStartTimeUl) + N * semiPersistSchedIntervalUL-sTTI+ Subframe_Offset * (N modulo 2) * sTTI_Number_Per_Subframe] modulo (10240 * sTTI_Number_Per_Subframe). Where SFNstart time, subframestart time and sTTIStartTimeUl are the SFN, subframe and sTTI_number, respectively, at the time the configured uplink grant were (re-)initialised.

Based on the above highlighted part, SPS occasions are sequentially calculated based on SFNstart time, subframestart time and SPS period. For IoT NTN TDD mode, if we introduce new SPS periodicities as multiple of 90ms, and if it can be always up to network to ensure the 1st SPS occasion corresponding to SFNstart time and subframestart time is located in U NB-IoT subframes, all the SPS occasion based on the formula will be located in U NB-IoT subframes, thus there is no need to postpone, and no SPS collision issue. Otherwise, if we only introduce new SPS periodicities, the SPS collision issue may still exist if the 1st SPS occasion corresponding to SFNstart time and subframestart time is located in non-U NB-IoT subframes. According to RAN1 spec, SFNstart time and subframestart time are determined by k0 included in PDCCH indicating SPS activation. Currently, only a limit discrete values are defined for k0, it seems difficult to ensure the subframe corresponding to SFNstart time and subframestart time to be in U NB-IoT subframe. Note that the scheduling delay issue, i.e., very limited k0 is suitable for scheduling NPUSCH Format 1 in IoT NTN TDD mode, was discussed in RAN1#122bis, and some companies wants to optimize for this, but no conclusion was made. So in our view, option 1 cannot solve the SPS occasion issue. Besides, if SPS can be applied to traffic in the feature, e.g., with the support of IMS voice in IoT NTN in Rel-20, and when it is used in IoT NTN TDD mode, the SPS periodicity should match with the traffic packet arrival interval, thus use of 90ms periodicity is not suitable.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of - n+k0+Koffset DL subframe for FDD, - k0 NB-IoT UL subframes following the end of n+8 subframe for TDD, a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where - subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and …… - value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD Table 16.5.1-1: for DCI format N0 for FDD. 0; 8. 1; 16. 2; 32. 3; 64.

- Introducing new SPS periodicities as multiple of 90ms only cannot solve the UL SPS collision issue, since 1st SPS occasion corresponding to SFNstart time and subframestart time may be located in non-U NB-IoT subframe.

For option 2, it can be up to network configuration to avoid this SPS collision, e.g., both UE1 and UE2 can be configured with a subframe id within the 8 U NB-IoT subframes for UL SPS transmission. Since there are also UL SPS occasions for other UEs, e.g., UE3 in figure 2 located within the U NB-IoT subframes, these UL SPS occasions without any postponement may collide with other postponed UL SPS occasions. To resolve the problem, it is needed that for a SPS occasion within the 8 UL NB-IoT subframes, its subframe id also follow the configured subframe id. In other words, the configured subframe id should be applied to all UL SPS transmissions, regardless of the UL SPS occasion overlapping with non-U NB-IoT subframes or not.

In this option, UE determines SPS occasions as follow:

1. UE calculates the SPS occasion e.g., SFN i, subframe j, based on the current formula.

2. if (SFN i, subframe j) is a U NB-IoT subframe, and subframe j is the configured subframe for SPS, then no need to adjust the SPS occasion; else, UE adjusts the SPS occasion to the next configured subframe for SPS which located in U NB-IoT subframe

Figure 2 Avoiding SPS collision by configuring a subframe id for SPS

In NTN TDD mode, UE is configured with a subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

- Conclusion

Based on the discussion we make the follow observation:

- There may be SPS collision between different UEs, and UEs cannot be aware of the UL SPS collision.

- Introducing new SPS periodicities as multiple of 90ms only cannot solve the UL SPS collision issue, since 1st SPS occasion corresponding to SFNstart time and subframestart time may be located in non-U NB-IoT subframe.

And we propose the following:

- In NTN TDD mode, UE is configured with a subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

- Reference

[1] RP-243293, Revised WID on introduction of IoT-NTN TDD mode