3GPP TSG-RAN WG2 Meeting #132	R2-2508304

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on RO type switch and Msg1 repetition number fallback issue

Document for: Discussion and Decision

# 1	Introduction

During and post RAN2#131-bis meeting, the issue of simultaneous fallbacks of RO type and Msg1 repetition number was discussed with no conclusion. In this contribution, we will further discuss this issue.

# 2	Discussion

2.1 Issue on simultaneous fallbacks of RO type and Msg1 repetition number:

In the current MAC spec for SBFD, the UE shall determine whether to select another RO type using the variable PREAMBLE_TRANSMISSION_COUNTER, i.e., determine whether or not the condition PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1 is met. Once this condition is met, the RO type is switched from the first PRACH occasions to the second PRACH occasions or vice versa. After the RO type switching, the UE shall further determine whether to select another Msg1 repetition number also using the same variable PREAMBLE_TRANSMISSION_COUNTER if the Random Access Preamble is transmitted with repetitions, i.e., determine whether or not PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1 is satisfied, as following:

2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

The threshold parameter preambleTransMaxRO-Type can be configured to one value from n1, n2, n4, n6, n8, n10, n20, n50, n100, n200, and the other threshold parameter preambleTransMax-Msg1-Repetition can also be configured to one value within the same value space n1, n2, n4, n6, n8, n10, n20, n50, n100, n200. Considering the limited number for those values especially for the larger values, there could be non-negligible probability both thresholds are configured with the same value. The consequence is that, when the RO fallback condition is met, the Msg1 repetition number fallback condition is also satisfied, i.e., the RO type fallback and Msg1 repetition number fallback conditions are met at the same time.

In this case, according to the above action sequence in the MAC spec, the UE shall first select a RACH resource set with different RO type with a same or next higher Msg1 repetition number, and then immediately (however unnecessarily) select another RACH resource set with further higher Msg1 repetition number.

2.2 The intended behavior:

According to the agreements in RAN2#130 and in RAN2#131, once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. The Msg1 repetition number for the other RO type would be the same or the next higher Msg1 repetition number if the same Msg1 repetition number is not available. It is understood in this case, the UE shall not further increase the Msg1 repetition number immediately after the RO type switch where the same Msg1 repetition number or the next higher Msg1 repetition number has been chosen.

The reason for this understanding is that the transmission quality could be already improved by switching the RO type.

For example, the preambleTransMaxRO-Type-r19 = n4 and preambleTransMax-Msg1-Repetition = n4, and the RACH resource set for (the starting) 1st-RO is associated with Msg1 repetition number 2,4,8, and the RACH resource set for 2nd-RO is associated with Msg1 repetition number 4,8. When the PREAMBLE_TRANSMISSION_COUNTER reaches 5 with Msg1 repetition number is 2, the RO type is switched from 1st-RO to 2nd-RO, the UE shall select RACH resource set for 2nd-RO with Msg1 repetition number 4.

In such case, if the UE further check the condition of Msg1 repetition number fallback, the UE shall immediately select RACH resource set for 2nd-RO with Msg1 repetition number 8 during this RACH attempt, i.e., the UE will skip RACH attempt with Msg1 repetition number 4. This should not be allowed for Msg1 repetition feature, as this is never the intended UE behavior when the RO type switch is designed for RACH in SBFD resource.

Observation 1: During one RACH attempt, if the both conditions for RO type fallback and Msg1 repetition number fallback are met, the UE may skip the intended next higher Msg1 repetition and jump to the highest Msg1 repetition number according to the current MAC spec however this is not the intended behavior.

2.3 Possible solutions:

There was alternative RRC solution other than MAC solution [2] discussed during the post RAN2#131bis email discussion. In the RRC solution, it is suggested to introduce more values for the thresholds such that the "collision" probability could be lowered. However for this RRC solution, more signaling bits are needed to introduce more threshold values thus this should be not preferred as this configuration signaling is the common signaling, for which increasing its overhead should be usually avoided.

On the other hand, even the "collision" chance can be made lower with more threshold values, the issue (of unnecessarily repeating RACH resource set selection) still exists when this "collision" did happen, i.e., the issue has not been fully addressed with the RRC solution.

Observation 2: Introducing more values for the fallback thresholds in RRC spec does not properly solve the issue of simultaneous fallbacks.

Another suggested solution is to let gNB implementation to handle this issue. It would be not suitable to impose this unnecessarily restriction on gNB implementation that gNB implementation has to track two un-correlated behaviors all the time, the RO type switching and the Msg1 repetition number fallback. Also it would be anyway difficult for gNB implementation to handle if the value space is limited.

A viable MAC solution of this issue is to bypass the check for Msg1 repetition number fallback after the UE performs RO type fallback for one RACH attempt (not for the whole RACH procedure though). The UE only checks the condition of Msg1 repetition number fallback during one RACH attempt when the condition of Msg1 repetition number fallback is not met.

The TP for MAC is given as below, note this TP (yet still simple and straightforward) has avoided the problem of TP in [2] and the 'usual' Msg1 repetition number fallback will be permitted even the RO type has been changed from 1st-RO to 2nd-RO or vice versa during the RACH procedure, as long as the two conditions are not met at the same time.

Observation 3: The updated TP (yet still simple and straightforward) has avoided the problem of TP in [2] and the 'usual' Msg1 repetition number fallback will be permitted even the RO type has been changed from 1st-RO to 2nd-RO or vice versa during the RACH procedure, as long as the two conditions are not met at the same time.

3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER is not equal to preambleTransMaxRO-Type plus 1 (if preambleTransMaxRO-Type is applied): 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

Proposal 1: Agree the TP of bypassing the immediate Msg1 repetition number fallback condition check after the UE performs RO type switching.

# 3	Conclusion

Based on the above discussion we have the following proposals for open issues:

Observation 1: During one RACH attempt, if the both conditions for RO type fallback and Msg1 repetition number fallback are met, the UE may skip the intended next higher Msg1 repetition and jump to the highest Msg1 repetition number according to the current MAC spec however this is not the intended behavior.

Observation 2: Introducing more values for the fallback thresholds in RRC spec does not properly solve the issue of simultaneous fallbacks.

Observation 3: The updated TP (yet still simple and straightforward) has avoided the problem of TP in [2] and the 'usual' Msg1 repetition number fallback will be permitted even the RO type has been changed from 1st-RO to 2nd-RO or vice versa during the RACH procedure, as long as the two conditions are not met at the same time.

Proposal 1: Agree the TP of bypassing the immediate Msg1 repetition number fallback condition check after the UE performs RO type switching.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R2-2507003, Discussion on residual issues for MAC spec, Huawei, HiSilicon