3GPP TSG-RAN WG2 Meeting #132	R2-2508310

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	UP functionalities and requirements

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#131bis, as RAN2 initiated 6G discussions, UP functions were briefly reviewed, and it was agreed continuing the discussions:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

The UP functions that have been developed and maintained since LTE are well proven to operate reliably across diverse scenarios/use cases. Therefore, for 6G, a whole re-validation and re-design of each individual function is neither necessary nor practical. That said, considering adds up of UP features/functions through multiple releases, high expectations for 6G, and the intent to accommodate emerging applications and use cases, it would be good to selectively refine certain functions to achieve a more streamlined and efficient support.

In this contribution, we outline the areas identified as candidates for enhancements and describe the corresponding requirements that RAN2 should take into account.

# 2	Discussions

## 2.1	QoS flow to RB mapping

At the previous meeting, questions were raised as to whether SDAP only introduces unnecessary header processing given the limited practical use of Reflective QoS (RQoS).

Reflective QoS can be useful for fast setup of the services like voices and gaming - where the uplink and downlink traffic share similar QoS requirements, but it has not been widely used in practice today. This is because explicit QoS rules are more precise and providing static QoS rule is just sufficient while offering more deterministic management where, e.g., rules are removed explicitly rather than by a timer. However, we note that RQoS is one of SDAP functionalities, and the primary SDAP role remains the QoS flow to DRB (re)mapping, including the use of End-Marker. As long as the RB concept is retained, QoS flow to DRB (re)mapping should continue to be supported in 6G.

Proposal 1: (Re)mapping of QoS flow to RB with the use of End-Marker is retained in 6G while the necessity of Reflective QoS remains in 6G is further discussed.

## 2.2	Packet concatenation above MAC

Hardware-friendly processing has been a key UP design principle since 5G. As reconfirmed in the last meeting, this importance would remain unchanged for 6G:

Agreements on UP

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

While no specific UP function has been identified as problematic to this principle, it is generally understood that additional headers are translated into more processing at both transmitter and receiver.

In 5G NR, each IP packet requires separate security processing and carries its own headers across PDCP, RLC, and MAC, resulting in inefficient data handling and processing burden that scales with increasing data rate. As new services requiring higher data rate emerge – possibly with small inter-arrival times, this linear scaling of processing load can become a bottleneck for energy-efficient, low-latency, and scalable 6G UP designs.

Packet concatenation can address this issue by bundling multiple packets into a single PDU, e.g., in PDCP, so that only one round of processing is needed instead of many. As a starting point, discussion on packet concatenation should focus on scenarios where clear gains are expected - such as for high-rate traffic with relaxed latency requirements, traffic with short inter-arrival times, or cases where concatenation can be implemented without complex buffer handling or adverse effects on pre-processing. RAN2 also needs to consider additional guardrails for concatenation. For instance, concatenation should not impose constraints on lower layers; MAC and RLC should remain free to schedule and retransmit according to their own mechanisms. In addition, concatenation should not weaken the QoS separation between independent traffic.

Proposal 2: For scenarios with clear gains (e.g. high-rate, short inter-arrival, or relaxed latency traffics) investigate packet concatenation in 6GR that minimises operational constraints on lower layers and preserve QoS/prioritisation separation.

## 2.3	L2 retransmission

At the last meeting, RAN2 only briefly touched L2 retransmission, and it was agreed to continue studying how ARQ can be better with HARQ:

Agreements on UP

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

HARQ provides rapid retransmission via immediate physical layer feedback, e.g., via PUCCH or NDI value, whereas ARQ retransmission is a slower and timer-based mechanism that is only initiated after multiple HARQ retransmission has failed or in case of NACK to ACK error. Consequently, the ARQ latency inherently includes the latency of multiple failed HARQ retransmission, plus RLC Status Report and retransmission procedure.

RLC retransmission in coordination with HARQ can make the RLC retransmission faster, which has two main advantages: latency can be reduced, while such reduction of the RLC RTT also has positive impacts on the L2 buffering requirements for a given data rate. On the other hand, these advantages do come with some challenges. Too early RLC retransmissions triggered while HARQ is still running may result in unnecessary retransmissions, hence in a loss of throughput and spectral efficiency. Also, since the HARQ transmission result is only per TB, such HARQ-assisted RLC retransmission would, by nature, require retransmission of all RLC AMD PDUs contained in the failed TB, and would impose a constraint on MAC to track which logical channels were multiplexed into the TB.

Furthermore, the HARQ imperfection, e.g., the network gives up further HARQ retransmission or HARQ NACK-to-ACK errors, also needs to be considered, since in that case the HARQ transmission is effectively stopped, e.g., HARQ buffer can be flushed, and retransmission fully relies on RLC ARQ.

Proposal 3: investigate HARQ-assisted ARQ in 6GR, which avoids premature RLC retransmission, provides a reasonable trade-off between RLC RTT reduction and memory requirements, and accounts for HARQ imperfections.

## 2.4	QoS and LCP

As discussed in R2-2507157 [1], the envisioned 6G QoS framework will need more intelligence to cope with future traffic diversity and scenarios in the coming decade. One key RAN capability for this is uplink-related QoS adjustment - i.e., how to ensure that the QoS requirements of a given QoS flow or a group of flows are met, in other words how to control the dynamic mapping of QoS flows onto uplink TBs.

We have identified five essential requirements for the control of uplink QoS flow(s):

1.	A QoS flow must have the possibility to go through first no matter what (e.g. for SRB);

NOTE:	SRBs are not QoS flows but let’s assume they are for the sake of reasoning.

2.	The RAN must be able to configure the UE in order to guarantee a bit rate (e.g. GBR for voice);

3.	The RAN must be able to configure the UE in order to avoid starving low priority traffic (e.g. using PBR);

4.	The priority of a QoS flow must be allowed to vary according to the looming possibility of discard (e.g. as for XR in Rel-19);

5.	The RAN must be able to configure the UE in order to limit a bit rate (e.g. MFBR).

The LCP framework of 5G in Rel-19 fulfils the first four requirements but not the fifth one (at least not without artificial segregation of resources and mapping restrictions on those). This is something that must be enhanced in 6G to efficiently support e.g. Slicing, QoS control and also make SCONE relevant.

Proposal 4: The QoS framework of 6G is based on that of 5G, with a control of uplink QoS flows that shall allow absolute prioritisation, guarantee a bit rate, avoid starvation, adjust the priority based on the looming possibility of discard, and enforce bit rate limits, while being observable.

## 2.5	Random Access design principles

In 5G, RA partitioning has been done for several features in different releases, e.g. Coverage Enhancement (CE), RedCap, SDT, Slicing, etc, on top of beams and preamble group A/B, which results in fragmentation of the RA resources as well as increased specification and implementation complexity. For 6G, we should aim to minimize the RA resource fragmentation to only do it for the features that RAN1 concluded as absolutely needed (e.g. CE) that requires distinguishing at random access response step, but not for the ones that is enough to be indicated in MSG3 or later (e.g. SDT, Slicing).

Besides, RA procedure design initially relied only on the 4-step RA procedure, and the 2-step RA procedure was introduced in a later release to reduce the latency of the sequential steps of RAP-RAR-Msg3-Msg4. While 2-step RA procedure enables faster access with possibility to fallback to 4-step, it also introduces complexity due to, for instance, RA configuration, RA type selection, RA resource selection, and multiple response formats.  Additionally, in the context of Rel-19 NTN, CB-MSG3 was introduced without preamble transmission with the assumption of TA compensation at the UE side.

In our view, for 6G, depending on RAN1 decides which types of the request are to be supported, the goal in RAN2 should be to design a unified RA procedure that integrates all supported types and supports the device type with minimum capability, which would allow handling of all cases consistently, thereby simplifying the design and avoid resource fragmentation.

For RA PUSCH payload size, e.g. MSG3/MSGA/CB-PUSCH for NTN depending on which ones are supported, similar to the agreement made for SIB1 size, RAN2 should study if any issue and what would be the desired content/size for RA request payload size from RAN2 point of view. While the final agreement it should be jointly made by RAN1 and RAN2 taking into account the link budget for UL coverage and our desired messages size.

Proposal 5: Design a unified Random Access procedure which minimizes the need of RA partitioning, and study, together with RAN1, possible issues with RA PUSCH payload size.

## 2.6	UP latency and Scheduling information - SR/BSR/DSR

In the reply LS from RAN to ITU-R WP5D RP-251825 [2], 1ms and 4ms were decided as for UP latency requirements:

| - User Plane Latency | IC: 4ms HRLLC: 1ms |
| --- | --- |

In the previous RAN2 meeting, it was agreed we should aim to reduce radio and end-to-end latency:

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

Since it is already possible to achieve 1ms requirement with 1ms periodicity CG as introduced for URLLC for NR, it is unclear what would be the aim here on top. We understood it would be impossible to configure 1ms periodicity CG to all the UEs, but such requirement is not for all UEs either. Whether CG capacity is a concern and whether CB-PUSCH would perform better in terms of latency should be for RAN1 to evaluate.

Regardless of the CB-PUSCH discussion, we believe the dedicated SR from NR should be considered as baseline for all the UEs, from both delay requirement point of view and resource efficiency point of view as it would be impossible to use CB-PUSCH for all the UEs even if it were to be supported as a replacement of SR since it would increase the collision rate and be counterproductive.

For detailed scheduling information, buffer status and delay information should be supported to provide enough information to the NW for better scheduling decisions. More details can be found in R2-2508127 [3].

Proposal 6: For scheduling information, dedicated SR to request UL resource as well as BSR/DSR like mechanism to report buffer status and delay information are to be supported for 6G, regardless of whether CB-PUSCH would be introduced.

## 2.7	Energy efficiency

Independent design of network energy saving (NES) and UE energy saving (ES) mechanisms may limit the targeted benefits from both. In some cases, UE energy saving mechanisms may increase power consumption on the network side. Similarly, designing NES operations without taking the UE ES into account may lead to UE energy consumption issues. In previous 5G NR studies, the alignment between network and UE energy-saving functionalities in different RRC states has been deemed beneficial. For 6G, the network energy saving impact on UE energy consumption and vice versa need to be evaluated and taken into account when designing 6G NES operations and UE power saving mechanisms. Energy efficiency is considered one of the important aspects for the design of 6G networks with the following agreement made in the previous RAN2 meeting:

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

From NW point of view, the NR Rel-18/19 NES WI mainly focused reduce the transmission of always one signalling by introducing Cell DTX/DRX, OD-SIB1, OD-SSB, paging clustering etc. which are nice features but might be difficult to be used in practise with legacy UEs to be considered. It would be a good chance to consider such directions from day 1 for 6G.

For UE power saving, most of the features NR features are focusing on reducing the PDCCH monitoring and RRM measurement activities, e.g. with DRX, PDCCH skipping, SSSG switching, BWP switching, LP-WUS, RRM relaxation etc. Such design principle should be kept for 6G as well.

Proposal 7: From day one RAN2 should design 6G energy saving mechanisms considering joint network and UE optimizations, aim at reducing transmissions of always-on signals for network energy saving and minimizing monitoring/measurement power consumption from the UE side without performance degradation.

# 3	Conclusion

This document discussed UP functionalities and requirements with the following proposals proposed:

Proposal 1: (Re)mapping of QoS flow to RB with the use of End-Marker is retained in 6G while the necessity of Reflective QoS remains in 6G is further discussed.

Proposal 2: For scenarios with clear gains (e.g. high-rate, short inter-arrival, or relaxed latency traffics) investigate packet concatenation in 6GR that minimises operational constraints on lower layers and preserve QoS/prioritisation separation.

Proposal 3: investigate HARQ-assisted ARQ in 6GR, which avoids premature RLC retransmission, provides a reasonable trade-off between RLC RTT reduction and memory requirements, and accounts for HARQ imperfections.

Proposal 4: The QoS framework of 6G is based on that of 5G, with a control of uplink QoS flows that shall allow absolute prioritisation, guarantee a bit rate, avoid starvation, adjust the priority based on the looming possibility of discard, and enforce bit rate limits, while being observable.

Proposal 5: Design a unified Random Access procedure which minimizes the need of RA partitioning, and study, together with RAN1, possible issues with RA PUSCH payload size.

Proposal 6: For scheduling information, dedicated SR to request UL resource as well as BSR/DSR like mechanism to report buffer status and delay information are to be supported for 6G, regardless of whether CB-PUSCH would be introduced.

Proposal 7: From day one 3GPP should design 6G energy saving mechanisms considering joint network and UE optimizations, aim at reducing transmissions of always-on signals for network energy saving and minimizing monitoring/measurement power consumption from the UE side without performance degradation.

# Reference

[1] R2-2507157, 6G UP design, Nokia, Nokia Shanghai Bell

[2] RP-251825, Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

[3] R2-2508127, On scheduling, retransmissions and uplink scheduling information, Nokia, Nokia Shanghai Bell