3GPP TSG-RAN WG2 Meeting #132																			  R2-2508318

Dallas, US, Nov 17 – 21, 2025

Source: 	      ZTE Corporation, Sanechips

Title:                 Design principles for 6G

Agenda item:   10.2.3

Document for: Discussion and Decision

# Introduction

This contribution highlights high-level principles, guidelines specific to RAN2 and discusses specific aspects of these for various deployment scenarios and architectures for 6G. The contribution also highlights aspects where there are dependencies with the work going on in other WGs.

# General design principles

For 6G the overall protocol structure will strongly depend on the migration decision in plenary (including the decision on inter-RAT and 6G-6G DC) and the decision on CU/DU split architecture in RAN3. The timeline for this discussion in plenary is as follows.

Figure 1: 6G study timeline for RAN2

The check point for CU/DU split and the check point for DC will have significant implications on overall design decisions in RAN2. These checkpoints only come towards the later part of the study (see Figure 1). In the meanwhile, RAN2 protocol design should proceed and we have to design a system that is optimal based on the current assumptions on the table. RAN2 agreements in the meanwhile should ensure an optimal protocol design for standalone operation whilst not precluding optimal operation in either CU/DU split architecture or DC (including MR-DC) as there will be limited time left (i.e. only 3 quarters) in case we have to do complete redesign of the system for optimal support of these features. In other words, the RAN2 design should enable an optimal protocol stack for operation without DC and without CU/DU split, whilst ensuring forward compatibility with both CU/DU split and DC.

Observation 1: Optimal support of CU/DU split and DC (including inter-RAT DC for migration) will have substantial impact on RAN2 protocol design

Proposal 1:	Interim RAN2 decisions on overall protocol design (before the check points for DC and CU/DU split) should ensure an optimized operation for stand-alone mode whilst ensuring forward compatibility with optimal support for both DC (MR-DC / 6G-DC) and CU/DU split.

In 5G, for the same or similar purpose, various features have been introduced, which dramatically increases the spec complexity and leads to market fragmentation. For example, for mobility enhancement, several mobility methods have been specified in 5G, e.g. L3 HO, CHO, (S)CPAC, LTM, CLTM, etc. Both CHO and CLTM are introduced for robustness improvement. Similarly, LTM was introduced for latency reduction, however, in late release, RACH-less mechanism is also supported for L3 HO for latency reduction. Another example is about multi-carrier handling (e.g. SUL and CA), Initially, SUL was introduced to enhance uplink coverage, while CA primarily aimed to improve resource utilization and throughput. Both technologies are based on the assumption of a common MAC scheduler for multi-carrier operations. However, as technology has evolved, the boundaries between CA and SUL have become increasingly blurred, with both technologies seeking enhancements in coverage and throughput. This overlap increases the risk of standard and market fragmentation, leading to unnecessary complexity. Hence, to achieve a streamlined design and minimize complexity in both standardization and implementation, in 6G, RAN2 should firstly study the different requirements, and for the same or similar requirements, a unified solution and framework should be pursued.

Proposal 2:	Unique functionality should be developed for satisfying a given set of similar 6GR requirements by avoiding defining multiple solutions to satisfy requirements which are similar (e.g., mobility and multi-carrier operation).

# Use cases and scenarios

# Support of existing 5G use cases and scenarios in 6G

When 5G study was initiated, the focus was mainly on identifying the key components of the RAN protocol architecture that are needed to support the requirements for the key usage scenarios including eMBB, URLLC and mMTC. With 6G, in addition to the extension of the above scenarios, there are 3 new day1 scenarios that need to be considered: Ubiquitous connectivity (TN/NTN integration), AI and Sensing. This is depicted in Figure 1 below.

Figure 2: 6G usage scenarios

Then, with respect to the functionality, focus of RAN2 study should be aligned with the usage scenarios highlighted in Figure 2. Specifically, where the functionality is currently there in 5G (especially from Rel-15), the aim of the study should be to reuse the functionality whilst making it streamlined, eliminating multiple options and simplifying the implementations. On the other hand, new functionality will be needed to support new day 1 services such as those which are needed for supporting the Ubiquitous Connectivity, AI and Sensing.

Going into the details, the high-level usage scenarios above may be mapped into various sub features which need to be studied in RAN2 as day 1 features.

- NTN: Aim to integrate in a harmonized way from day1 into the protocol stack without creating exceptions for NTN

- HRLLC: Including support for TSN (largely reusing the 5G framework), i.e. no need to spend too much time during the study item on this.

- SON/MDT: Support should be possible from day1 and mechanisms from 5G can be largely reused (i.e. not much to study during the study item)

- Slicing: Should be supported from day1 but we should wait for the framework from SA2 to be clarified first to understand the detailed design in RAN2

- Relays/repeaters and RIS: Reuse the framework developed in 5G and study any specific enhancements needed to support RIS

- Specific use cases for high mobility: UAV, HST.

- AI/ML related use cases

- Sensing scenarios: New study is required but wait for the initial framework from RAN1 to be finalised first.

Proposal 3:	For 6GR, RAN2 study shall cover the use cases including NTN, HRLLC, SON/MDT, Slicing, Relays (including repeaters and RIS), UAV, HST, AI/ML and Sensing scenarios

Proposal 4: 	For the existing use cases in 5G, RAN2 study should identify at an early stage, functionality that can be reused from 5G as a baseline to minimize the additional work in these areas (e.g. for SON/MDT, TSN, Slicing – depending on SA2 output etc)

# New and enhanced use cases and scenarios for 6G

# AI use cases and applications

6G shall enable AI for network and network for AI from day 1.

AI for network

With respect to AI for network, RAN2 should focus on radio related use cases such as AI based mobility/positioning enhancements from day1. In addition, RAN2 design should also natively support other radio related AI/ML enhancements such as CSI feedback enhancement, beam management and positioning enhancements for AI. Whilst the system should be designed to be operable with these enhancements, the RAN2 design shall also ensure that the basic system performance for non-AI mode of operation is also optimal.

Network for AI

With regards to network for AI use cases, as captured in 3GPP TR 22.870, the 6GR should be optimised to support new AI based applications including generative AI and AI-agents. The general protocol architecture should be able to support different operating modes of these AI applications. This includes support for fast QoS changes, fast modality changes between different AI modes including switching between Video, Audio and/or text based communication modes etc. Furthermore, compared to traditional applications, AI based applications also have new requirements on protocol stack, such as error tolerant modes of communication, requirement for bursty and intermittent traffic with varied QoS requirements etc.

AI services

The overall protocol design shall enable different AI services to be deployed on 6G networks. This includes optimal support for full life cycle management of AI models (including training, management, inference, model transfer, model updates etc). Whilst SA2 should take the lead role in AI service and hosting environment and related network architecture decisions, the RAN2 protocol design should be flexible to accommodate AI services to be provided in an optimal way in 6G.

# Sensing use cases

ISAC will be a new day 1 feature in 6G. ISAC can not only assist communication, but can also be provided as a service on itself. In Rel-19 ISAC channel modeling study, the use cases of detection and tracking of UAV, pedestrian, Vehicle, and AGV had been justified, and the corresponding channel modeling had been completed. Therefore, the support of these use cases should be considered in 6G study. It should be noted that Rel-20 5G-A only considers TRP based sensing without UE involvement. When it comes to 6G, the uses cases that involve UE as a sensing transmitter and/or receiver should be supported. Supporting this will have impact to RAN2 protocol design. Moreover, the sensing assisted communication is important to enhance the reliability and system responsiveness of wireless network and should be studied in 6G as well.  RAN2 discussions on detailed sensing related aspects can start once RAN1 design is complete.

Based on the above, the following proposal is made for the new AI and sensing related use cases in 6G:

Proposal 5: 	For the new and enhanced 6G related use cases for AI and sensing, RAN2 study should focus on the following:

  - Enabling AI based enhancements for 6GR including existing and new use cases, whilst ensuring that the system operation is optimal both with and without these AI based enhancements

  - Signalling design to enable optimal support for UE based sensing modes for sensing services and sensing assisted communication once RAN1 initial design is stable

# Protocol architecture

The protocol architecture for 6GR should support RAN functions to support CP and UP data transfer. In addition, RAN functionality should also support NAS data transfer. The overall protocol architecture will depend on the following:

- Overall system architecture (SA2):

  - Overall QoS framework for 6G will have significant impact on the RAN protocol architecture. In particular, we need to understand whether the QoS-flow based architecture will be largely reused or not.

  - Functional split between CN and RAN, specifically the anchor points for AS and NAS transfer and session management framework

  - General framework for AS/NAS interactions

- RAN architecture (RAN3):

  - The main dependency on RAN3 is whether CU/DU split exists in 6G. This will have significant impact on our protocol design both for UP and CP

- Security architecture (SA3):

  - RAN Security requirements both CP and UP will be needed

  - Any new security requirements for 6GR, e.g. security for system information, security for lower layer signalling messages etc need to be clarified

Regardless of the above dependencies, as a minimum, RAN2 protocol design shall still support CP/UP data transfer and shall also support efficient mechanisms to carry NAS signalling messages. Furthermore, the protocol architecture should also support transfer of a wide variety of data including user plane data (IP traffic), control signalling (that terminates in RAN and in CN – NAS) and other data including data for sensing and AI which either terminates in RAN or in CN.

Proposal 6:	The RAN2 protocol architecture for 6GR should support separation of CP/UP data transfer and functionality to transport signalling (Radio signalling and NAS signalling) and data (UP data, data for sensing/AI). The design should allow termination points for both data and signalling to be either in RAN or in CN.

# Diverse device types

The work on support of diverse device types is currently ongoing in both RAN1 and in RAN plenary. The involvement of RAN2 in this overall framework should be to ensure the following:

- UE capability framework for signalling of various device types/categories: This work should start once the RAN1 design for UE categories is clear. Especially we need to understand the basic capability sets for each device type, in terms of MIMO layer support, number of antennas, minimum supported BW etc. So, RAN2 can wait for RAN1/plenary study of these for various device types.

- Signalling to support incremental configuration of features: The RRC signalling framework should support incremental configuration of the complex features. For instance, the UE should be able to access the network using an SRB with basic configuration and this SRB should then be used for configuration of enhanced and complex features once the UE capability is known. In general, RAN2 design is in line with this paradigm. What this means in practice is that the RAN2 aspects of initial access (e.g. RACH procedure, SRB0 operation and default SRB1 configuration etc) should not be designed such that a UE with basic capability can access the system.

Based on the above, whilst RAN2 can wait for RAN1/RAN plenary progress for the overall device type discussion, the signalling design should be ensured to enable incremental configuration of features based on UE capabilities. Thus, the following proposal is made:

Proposal 7:	To support diverse device types, RAN2 signalling design should allow incremental activation of UE features during connection setup, while ensuring that the procedures and signalling required for initial access (e.g., RACH, SRB0, and a minimal SRB1 configuration) remain universally supported by all devices.

# Conclusion

Observation 1: Optimal support of CU/DU split and DC (including inter-RAT DC for migration) will have substantial impact on RAN2 protocol design

Proposal 1:	Interim RAN2 decisions on overall protocol design (before the check points for DC and CU/DU split) should ensure an optimized operation for stand-alone mode whilst ensuring forward compatibility with optimal support for both DC (MR-DC / 6G-DC) and CU/DU split.

Proposal 2:	Unique functionality should be developed for satisfying a given set of similar 6GR requirements by avoiding defining multiple solutions to satisfy requirements which are similar (e.g., mobility and multi-carrier operation).

Proposal 3:	For 6GR, RAN2 study shall cover the use cases including NTN, HRLLC, SON/MDT, Slicing, Relays (including repeaters and RIS), UAV, HST, AI/ML and Sensing scenarios

Proposal 4: 	For the existing use cases in 5G, RAN2 study should identify at an early stage, functionality that can be reused from 5G as a baseline to minimize the additional work in these areas (e.g. for SON/MDT, TSN, Slicing – depending on SA2 output etc)

Proposal 5: 	For the new and enhanced 6G related use cases for AI and sensing, RAN2 study should focus on the following:

  - Enabling AI based enhancements for 6GR including existing and new use cases, whilst ensuring that the system operation is optimal both with and without these AI based enhancements

  - Signalling design to enable optimal support for UE based sensing modes for sensing services and sensing assisted communication once RAN1 initial design is stable

Proposal 6:	The RAN2 protocol architecture for 6GR should support separation of CP/UP data transfer and functionality to transport signalling (Radio signalling and NAS signalling) and data (UP data, data for sensing/AI). The design should allow termination points for both data and signalling to be either in RAN or in CN.

Proposal 7:	To support diverse device types, RAN2 signalling design should allow incremental activation of UE features during connection setup, while ensuring that the procedures and signalling required for initial access (e.g., RACH, SRB0, and a minimal SRB1 configuration) remain universally supported by all devices.

- References