3GPP TSG-RAN WG2 Meeting #132   																							R2-2508320

Dallas, USA, Nov. 17 – 21, 2025

Source: 		 ZTE Corporation, Sanechips

Title:                 6G User plane functionality and requirements

Agenda item:   10.3.1.1

Document for: Discussion and Decision

# Introduction

In this contribution we identify the essential functionality for 6G UP and identify any dependencies for the UP functionality with the functionality that will be studied by other 3GPP WGs/TSGs.

# Day1 UP functionality in 6G

The overall UP protocol stack in 3GPP has evolved over time between various generations and in case of 5G, it looks as shown in Figure 1.

Figure 1: UP protocol architecture structure in NR

The overall functionality in UP is distributed across different protocol layers. Whilst the functionality close to physical layer naturally fits in the lower layers of the UP stack and the functionality related to upper layers fits naturally in the upper layers, the eventual choice of placing different functionality in different layers was a result of a combination of factors including the need to support features such as DC, CU/DU split etc. This also meant that some functionality is duplicated across various layers and the Figure 2 below summarizes this, highlighting the functional redundancy for certain aspects within the protocol stack of NR for UP.

Figure 2: L2 UP functionality in NR

Based on the above, we can may be observed in the 5G UP stack:

The following functionality exists in UP protocol stack in 5G with no cross-layer redundancy across protocol stack ():

- SDAP: QoS handling

- PDCP: Compression (ROHC, EHC and UDC in PDCP) and RB Security (in PDCP), Timer based SDU Discard

- RLC: Segmentation and re-segmentation

- MAC: Multiplexing, Scheduling requests (BSR/SR), Retransmissions with soft combining (HARQ) and LCP

The following functionality exists in UP protocol stack in 5G with some cross-layer redundancy across layers (PDCP and RLC) ():

- PDCP and RLC:

  - Duplicate detection

  - Sequence numbering

  - Packet retransmissions without soft combining (ARQ-like)

As can be seen from above, there is some functional redundancy (mainly between PDCP and RLC protocol layers in 5G). Taking the 5G functionality as the general starting point, the goal for 6G study should be to make the overall protocol architecture as lean as possible. With this in mind, the functionality in 5G protocol layers which exists today without redundancy across protocol layers can be adopted as baseline functionality to be supported in 6G with enhancements for any specific use cases. On top, any functionality that has redundancy across protocol layers need further study for simplification. In the rest of the paper, we propose way forward for the overall UP functionality in 6G. As a general principle, RAN2 should first discuss the functionality that is needed and then, once the functionality is stable, we can discuss the overall protocol layer architecture and how to distribute this functionality across different protocol layers. Based on the above, the following proposal is made:

Proposal 1: 	6GR shall support the following functionality in a higher layer(s) in UP protocol stack:

- Mapping between QoS flows (or sub flows as designed by SA2) to a radio bearer in UL and DL

- Security protection, Sequence numbering of data PDUs, Header and Data compression, and timer-based discard for delayed PDUs

- Duplicate detection and discard

Proposal 2: 	6GR shall support the following functionality in a lower layer(s) in UP protocol stack:

- Segmentation of packets based on physical layer grant size

- Logical channel prioritization and multiplexing, Scheduling requests (SR/BSR like functionality)

- Transmission and retransmission with and without soft combining (HARQ and ARQ)

# UP Functionality that needs further discussion

# Reflective QoS

SDAP layer in 5G enables QoS flow to radio bearer mapping and also allows a fast remapping of QoS flows to a different RB using reflective QoS mechanism via in-band signalling in SDAP header. Whilst reflective QoS mechanism is mainly aimed at fast remapping of QoS flows from one RB to another RB, making this an in-band signalling was unnecessarily complex since this requires the UE to monitor every DL packet for possible remapping, whilst the actual remapping does not happen that often. Instead, an alternative mechanism for switching the QoS flows from one RB to another RB without full RRC reconfiguration could be adopted to replace the current reflective QoS based on inband signalling. One option for this is to use UP control signalling (e.g. a new PDCP control PDU) to enable remapping of an existing QoS flow to a different DRB. This provides enough flexibility for fast remapping of QoS flows for applications that need fast QoS changes whilst avoiding the SDAP overhead and the complexity at the UE to monitor every DL packet header.

Proposal 3: 	RAN2 should study mechanisms to replace the reflective QoS mechanism of 5G with a new QoS flow to DRB remapping mechanism using UP control signalling (e.g. PDCP control PDU)

# New compression function for data collection

5G supports various functions for compression of header and data parts of the UP traffic.

- ROHC is important for real-time multimedia applications where IP header overhead is critical since voice will be supported in 6G, this functionality is needed from day1

- UDC is important for IoT and messaging applications, SIP signalling etc considering massive communication is an important use case, this functionality is needed

- EHC is important for industrial and factory automation use cases where Ethernet is use used for IP traffic and for SIP again this should be supported for 6G use cases

All this functionality exists in 5G UP protocol stack and resides in PDCP layer.

On top of normal UP traffic, large quantities of data are exchanged for applications such as AI/Sensing and data collection applications. For these applications, RAN2 needs to study if any new mechanisms for effective data compression may be needed on top of what exists today and if a need is identified for such a scheme, we can specify an additional compression scheme tailored for AI/Sensing applications in the UP stack. For example, if UP based data collection is endorsed, and JSON or XML based format is considered instead of ASN.1, then enhancement of compression functionality may be required for the data collection and we may need to specify explicit new functionality for this in the UP stack.

Proposal 4:	New day 1 functionality to enable data compression for AI/Sensing related functions should be studied by RAN2

# Additional segmentation function at a higher layer in UP stack

Segmentation is clearly needed when the TB size provided by lower layers is smaller than the upper layer packets size. This segmentation function is typically available at a lower layer in the UP-protocol stack (similar to RLC in 5G). On top of this, for data collection (for AI/Sensing applications), it is unclear whether IP or non-IP based transfer mechanism will be used. If non-IP based data exchange is to be supported then there may be a need for segmentation functionality in the upper layers for large volumes of data. If the volume of the upper layer data packet could exceed what could be supported by the highest layer in the UP-protocol stack (e.g. PDCP), then we need further mechanism to segment the upper layer packets. This functionality is referred to as segmentation in above L2. For data collection, there is RAN level data collection as well as CN level data collection. For RAN data collection mechanism, RAN2 can discuss and decide whether a segmentation function in a higher layer of L2 protocol stack is required.

Proposal 5:	RAN2 should discuss whether an additional segmentation functionality in a higher layer in L2 protocol stack is required to handle large volumes of data packets for RAN data collection.

# Minimizing processing complexity

6GR should support very high data throughputs. This requires the UP-protocol stack to handle large quantities of data in short period of time. One of the key pre-requisites to enable fast data transfer is to minimize the processing requirements of the protocol stack. Specifically, the protocol stack operations such as construction of headers and populating the header fields should not be a bottleneck for achieving full potential for the overall throughput. As a general principle, this means that the headers fields should be static and any last-minute inclusion and changes to the TB contents should be minimized. The transmit side should be able to pre-construct most of the contents of the TB prior to the grant. In 5G these general requirements are followed as a guideline. According to this, one of the key differences between 5G and 4G was the removal of RLC multiplexing. This is something that should also be applied for 6G design.

Proposal 6:	To reduce overall processing delay and simplify the header design for various UP sub-layers, 6GR should support logical channel multiplexing only in MAC sub layer (same as 5G)

# Distribution of UP functions in 6G protocol stack

Based on the functionality proposed above, the overall protocol stack for 6G is proposed to be very similar to 5G with the following distribution of overall functionality across the protocol stack:

- SDAP: QoS flow to RB mapping (no Reflective QoS). Header structure and simplification can be considered based on new supported functionality

- PDCP: Compression (ROHC, EHC and UDC in PDCP) and RB Security (in PDCP), Timer based SDU Discard. Additional compression and segmentation functions for supporting AIML data collection services can be studied further.

- RLC: Segmentation and re-segmentation. No additional sequence numbering for stand-alone operation (see R2-25xxxxx).

- MAC: Multiplexing, Scheduling requests (BSR/SR), Retransmissions with soft combining (HARQ) and LCP. Support of fast SR/BSR mechanisms based on contention-based resources can be studied further.

Based on the above the following UP protocol structure is proposed for 6G:

Figure 3: Overall UP protocol stack for 6G

Based on the above, the following proposal is made:

Proposal 7: 	For 6GR, the following distribution of functions across protocol layers is proposed as baseline:

- SDAP: QoS flow to RB mapping (no Reflective QoS). Header structure and simplification can be considered based on new supported functionality

- PDCP: Compression (ROHC, EHC and UDC in PDCP) and RB Security (in PDCP), Timer based SDU Discard. Additional compression and segmentation functions for supporting AIML data collection services can be studied further.

- RLC: Segmentation and re-segmentation. No additional sequence numbering for stand-alone operation (see R2-25xxxxx).

- MAC: Multiplexing, Scheduling requests (BSR/SR), Retransmissions with soft combining (HARQ) and LCP. Support of fast SR/BSR mechanisms based on contention-based resources can be studied further

# Conclusion

Proposal 1: 	6GR shall support the following functionality in a higher layer(s) in UP protocol stack:

- Mapping between QoS flows (or sub flows as designed by SA2) to a radio bearer in UL and DL

- Security protection, Sequence numbering of data PDUs, Header and Data compression, and timer-based discard for delayed PDUs

- Duplicate detection and discard

Proposal 2: 	6GR shall support the following functionality in a lower layer(s) in UP protocol stack:

- Segmentation of packets based on physical layer grant size

- Logical channel prioritization and multiplexing, Scheduling requests (SR/BSR like functionality)

- Transmission and retransmission with and without soft combining (HARQ and ARQ)

Proposal 3: 	RAN2 should study mechanisms to replace the reflective QoS function of 5G with a new QoS flow to DRB remapping mechanism using UP control signalling (e.g. PDCP control PDU)

Proposal 4:	New day 1 functionality to enable data compression for AI/Sensing related functions should be studied by RAN2

Proposal 5:	RAN2 should discuss whether an additional segmentation functionality in a higher layer in L2 protocol stack is required to handle large volumes of data packets for RAN data collection.

Proposal 6:	To reduce overall processing delay and simplify the header design for various UP sub-layers, 6GR should support logical channel multiplexing only in MAC sub layer (same as 5G)

Proposal 7: 	For 6GR, the following distribution of functions across protocol layers is proposed as baseline:

- SDAP: QoS flow to RB mapping (no Reflective QoS). Header structure and simplification can be considered based on new supported functionality

- PDCP: Compression (ROHC, EHC and UDC in PDCP) and RB Security (in PDCP), Timer based SDU Discard. Additional compression and segmentation functions for supporting AIML data collection services can be studied further.

- RLC: Segmentation and re-segmentation. No additional sequence numbering for stand-alone operation (see R2-25xxxxx).

- MAC: Multiplexing, Scheduling requests (BSR/SR), Retransmissions with soft combining (HARQ) and LCP. Support of fast SR/BSR mechanisms based on contention-based resources can be studied further

# References