3GPP TSG-RAN WG2 Meeting #132	R2-2508335

Dallas, USA, 17th – 21st November 2025

Agenda item:	  10.3.3.1

Source:	  Lenovo

Title:                            6GR Data transfer and AIML

Document for:	  Discussion and Decision

# Introduction

In this paper, we discuss aspects related to data transfer and the AIML framework, targeted for the Study on 6G Radio [1].

# Discussion

Agreements (RAN2 #131bis)

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

## 2.1 Data Transfer

In RAN2 #131bis, initial discussions on 6G – AIML and data transfer have been kicked off. As many contributions indicated a desire for a unified data framework for use cases and services such as AI/ML, sensing, SON/MDT, QoE; it has been agreed to first study and analyze the requirements of these applications and their respective type of data. The following considerations should be taken into account with respect to such a diverse type of data:

- Applicable use cases

- Type of data

- End points for data

- Data size

- Frequency of data reporting

- QoS requirements (latency, priority, etc.)

- Protocol used for data transfer

In legacy systems, the UE can be configured by the network to report measurement results and related information for various purposes, as summarized in the table (Table 1) below.

First, we would like to clarify that the collection and transfer of data discussed here focuses on data with low priority compared to legacy control information, such as L1/L3 measurement reports. Unlike legacy reporting of measurements, these data types do not demand stringent latency requirements. The L1/L3 measurement reports — which do require low latency — are typically transferred via UCI/RRC, as in legacy systems. This approach is straightforward and well-established. For features that require data with a different level of urgency and priority than legacy data, such as data with non-stringent latency requirements for AIML, QoE monitoring, MDT, and SON — separate data collection and transfer frameworks were adopted during the 5G discussions, as each feature was addressed independently. These frameworks are summarized in the table below. However, this fragmented approach has resulted in non-unified UE handling, complicating implementation and maintenance.

Table 1: Legacy UE reports and corresponding framework

| Report type | Signaling Framework | Latency requirement |
| --- | --- | --- |
| L1 measurement report | CSI report via UCI | Stringent |
| L3 measurement report | RRC measurement report via SRB1 | Stringent |
| QoE | RRC MeasurementReportAppLayer via SRB4/SRB5 Can be paused by NW | Relaxed |
| ImmediateMDT | RRC measurement report via SRB1 | Moderate |
| LoggedMDT | UE log, Logged MDT report available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB2 | Relaxed |
| SON | UE log, SON report available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB1 | Moderate |
| NW side data collection (AIML) | UE log, Logged Data available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB6 | Relaxed |

Observation 1: The purpose of data collection (e.g., MDT, QoE, SON, AIML) and, consequently, the corresponding requirements for data transfer differ from those of traditional radio-related control or user plane data.

Overall, the data collection/transfer design can consider two cases corresponding to different data consumers. As shown in Figure 1, the data transfer and data storage can be performed in some cases at the RAN node or at an entity in the CN/OAM. For example, in Case-I, the RAN node collects and utilizes the data for its own usage, e.g., training RAN side AI/ML models or sensing data to support communication. Whereas in Case-II, some other network entities in CN/OAM, e.g., sensing server, UE server, etc, may collect and process the data with or without awareness of RAN. Certain data transfer requirements can be driven by each of these data types individually (e.g., AI/ML, MDT, QoE, sensing, etc.); for example, a low latency requirement for sensing-related data and a non-real-time latency requirement for AI/ML training-related data. At the same time, it is necessary to take into account the data transmission path and the data management entity (responsible for storing and managing the data) for these various types of data. Consequently, an overall consistent and unified framework, if feasible, must be designed in 6G for the management of the data collection-related processes and the data itself.

Figure 1: Cases for the transfer of various types of data

In addition, legacy data collection/transfer design relies significantly on the control plane, often resulting in inefficiencies.  For example, in 5G-A Release 19, several alternatives were brought up for data collection for the purpose of training AI/ML models. For network-side models, the data collection/transfer is mainly performed using the existing RRC signaling framework. Whereas, for UE-side models, RAN2 has not made significant progress, and a potential solution via user plane transfer is under study in SA2. We would like to emphasize that although the RRC-based data transfer for network-side model demands less specification impact, the current design is inefficient for such AI/ML-related data transfer. In particular, considering the legacy InformationRequest and InformationResponse framework, in which every transfer is triggered in response to an individual request message and is restricted by the size limitation of the InformationResponse message. The general data transfer framework for AI/ML should be extended, considering the requirement of large data volume to be logged and transferred with respect to different use cases and inference configurations.

Observation 2: The legacy data collection/transfer via control plane signalling is sometimes inefficient.

In the last meeting, in the scope of data transfer framework, RAN2 has listed examples like AIML, sensing, QoE, SON/MDT, however, it is not clear if the data transfer framework discussion includes cases where real-time UE data report or measurement report is required, such as:

- Inference data for AIML, e.g., AIML based beam management

- Point cloud data for sensing, e.g., point cloud map

Please note that in the past discussion in 5GA, the UE-side data collection and NW-side data collection are only about collecting AIML data for offline training. Similarly, it is certainly not the intention to discuss the real-time CSI report via physical signaling or real-time RRM measurement report via control signaling within the scope of data transfer.

Observation 3: In the past 5GA discussion about UE/NW-side data collection does not include those cases where real-time measurement report is required.

Proposal 1: RAN2 clarifies that the scope of data transfer framework does not include the cases where real-time report (e.g., CSI report, RRM measurement report) is required. Data related to data collection (AI/ML, sensing, SON/MDT, QoE) can be treated separately from control plane and user plane data.

The table below lists the requirements for the applications listed above.

| Aspect | AIML | MDT | SON | QoE | Sensing |
| --- | --- | --- | --- | --- | --- |
| Latency requirement | AIML Training: non-real-time | Logged MDT: non-real time | Non-real time | Non-real time | Training: non-real-time |
|  | AIML monitoring: near-real-time | Immediate MDT: near-real-time |  |  | Real-time sensing |
| GBR | NA | NA | NA | NA | Yes |
| Priority | AIML Training: Low AIML Monitoring: medium | Logged MDT: Low Immediate MDT:medium | Medium | Medium | Low to high |
| Producers | UE, RAN | UE | UE | UE | UE, RAN |
| Consumers | UE, RAN, NF, external/UE servers | OAM/CN | RAN | OAM/CN | UE, RAN, NF, external/sensing servers |
| Size of data | Large | Medium | Small | Small | Medium to large |
| Frequency of data reporting | Low to medium | Low | Low | Low | Medium to high |

The baseline requirements for AIML were agreed in the last meeting, highlighting the data security, privacy, user consent, MNO control and visibility of standardized data, impact on UE battery, processing, and power saving. In our opinion, these baseline requirements are not limited to AIML and can be set as a general principle for any applications or use cases that require data collection. To make progress, it would be beneficial to capture that these requirements apply to all types of data collection, unless specified otherwise.

Proposal 2: The following baseline requirements apply to all applications (AIML, sensing, SON/MDT, QoE), including data collected at the UE-side and the NW-side.

-	The data collected is secured and data integrity and confidentiality for that data is ensured.

-	User data privacy, anonymity and user consent is respected.

-	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose (This includes initiating, terminating, and fully managing data transfer).

-	MNO has full visibility for standardized data.

-	The design is future-proof and extendable.

-	The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

-	UE data collection should minimize impact to user traffic transmission and power saving features

As the consumers of the data could differ for each of these applications, it is essential to understand the impact of these different data termination points. Considering the agreement from the last RAN2 meeting, related to MNO control of the standardized data collection transfer process, and management of data transfer, an obvious question is whether MNO control and management can be useful for cases in which RAN is the data termination point. The data transfer purposes can be categorized based on the two main data termination points, i..e., (1) CN/OAM and (2) RAN. Since MNO control has been listed as one of the core requirements, it can be beneficial to analyze the main pillars that can enable these fundamental requirements for data collection. We think the primary question is how MNO control can be ensured and whether CN involvement is necessary to fulfill these core requirements. If CN/OAM involvement in the data transfer and management of data is always required, then it seems logical to study whether an entity in CN/OAM should be the centralized entity that governs the data transfer process and the collected data.

Proposal 3: RAN2 should study if and how MNO control can be ensured for both UE-side and NW-side data collection, e.g., whether the involvement of the CN/OAM or a centralized entity in CN/OAM can be common for all applications.

The main distinguishing factor for data transfer for a specific application is the latency requirements of the data reporting. Therefore, we think it is important to compare the latency requirements of these applications before considering further factors such as the size of data, frequency of reporting, etc. Additionally, the frequency of data transfer can be down-prioritized as a criterion, since most of the applications require low to medium frequency of data transfer.

Proposal 4: As the first step, RAN2 should prioritize analysing the latency requirements of data transfer for these diverse applications.

## 2.2 AI/ML framework

To enhance the quality of experience, it will be more important than ever in 6G to improve the utilization of resources and enable faster QoS adaptation. AI/ML is one of the key features that must be native to the RAN protocol. Beyond use case-specific AI/ML functionality, 6G-RAN will be required to facilitate a flexible protocol design with AI as an integral component (AI-native) to cater to different device types, such as smartphones, wearables, sensors, IoT devices, AR/VR devices, holographic devices, etc. Given that these devices differ in their individual device capabilities, in essence, their ability to support AI/ML features may largely vary. For example, IoT devices typically have limited processing and storage capacity, while smartphones are equipped with higher computational capabilities. The RAN protocol design from 6G Day-1 should consider flexibility to handle these devices, ranging from no AI capability to full-featured AI devices. For designing an AI-native protocol, the core L2/L3 functions, e.g., MAC functions, such as link adaptation, resource allocation, UE reports, etc, should be studied as adaptive AI functions which can bring benefits for stringent services in terms of their demanding QoS requirements.

- For the application of AI/ML-based features, an essential aspect of the applicability of the AI/ML functionality has been introduced during 5G AI/ML work. The applicability status of an AI/ML functionality or configuration dictates whether the functionality can be applied for inference based on model availability, UE internal conditions, and network-side conditions if available. An applicability reporting framework in Release-19 was introduced to enable information exchange related to the AI/ML functionality/configuration applicability between the UE and the network. The reporting of initial applicability is performed using RRCReconfigurationComplete based on the received RRC configuration, and the subsequent reporting of change in applicability/inapplicability status of a functionality/configuration is signaled using the existing framework using UE Assistance Information (UAI). Since UAI was originally designed for UE preference related to parameters such as preferred RRC state, DRX preference, overheating, etc., which allows UE to optionally provide its preference to the network, if enabled. Given that the applicability/inapplicability of AI/ML-related configuration is crucial for meaningful inference outcomes, therefore, applicability reporting and consecutive network behavior should not be restricted to UE preference. In particular, due to the dynamic nature of AI/ML applicability based on UE internal conditions and/or network-side conditions, UE may encounter a situation where a previously applicable inference configuration becomes inapplicable, which can lead to an inaccurate/garbage inference outcome if not reconfigured/deconfigured by the network. Therefore, it’s important for RAN2 to study the requirements for AI/ML-related signaling and consider enhancements to ensure consistency in the application of AI/ML functionality at the UE and the network side. An AI/ML-specific framework (including applicability reporting framework) should be analyzed to consider the benefits and impacts of UE autonomous behavior on the applicability/inapplicability of AI/ML functionality.

- Proposal 5: RAN2 is suggested to study enhancements for an AI/ML-specific applicability framework (e.g., applicability reporting and updates) to ensure consistency at the UE and the network side, e.g., deactivate inapplicable AIML configuration timely.

Due to the restrictions of inter-vendor collaborations, the scope of AI/ML framework in 5G is limited to some extent to cell-specific models.  Although support through multiple cells and the functioning of AI/ML features during handover and radio link failure (RLF) have been discussed, the outcome does not necessarily ensure an uninterrupted functioning of features enabled by AI/ML and a guaranteed performance accuracy during handover or RLF.  Network-side additional conditions (represented by associated IDs) are required for UE to determine the applicability and to perform useful inference if applicable. Enhancing the validity of associated ID(s) to larger areas can help to broaden the applicability of AI/ML features without interruption. We think it is essential to study consistency in AI/ML performance while the UE is mobile and transitions between different RRC states, and if required, inter-vendor collaboration should be supported for a future-proof solution in terms of applicability of AI/ML features, inference, and data collection.

Proposal 6: RAN2 will study how to ensure a smooth transition and maintain performance accuracy of active AI/ML features during mobility and RLF.

# Conclusion

Based on the discussion above, we propose:

Observation 1: The purpose of data collection (e.g., MDT, QoE, SON, AIML) and, consequently, the corresponding requirements for data transfer differ from those of traditional radio-related control or user plane data.

Observation 2: The legacy data collection/transfer via control plane signalling is sometimes inefficient.

Observation 3: In the past 5GA discussion about UE/NW-side data collection does not include those cases where real-time measurement report is required.

Proposal 1: RAN2 clarifies that the scope of data transfer framework does not include the cases where real-time report (e.g., CSI report, RRM measurement report) is required. Data related to data collection (AI/ML, sensing, SON/MDT, QoE) can be treated separately from control plane and user plane data.

Proposal 2: The following baseline requirements apply to all applications (AIML, sensing, SON/MDT, QoE), including data collected at the UE-side and the NW-side.

-	The data collected is secured and data integrity and confidentiality for that data is ensured.

-	User data privacy, anonymity and user consent is respected.

-	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose (This includes initiating, terminating, and fully managing data transfer).

-	MNO has full visibility for standardized data.

-	The design is future-proof and extendable.

-	The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

-	UE data collection should minimize impact to user traffic transmission and power saving features

Proposal 3: RAN2 should study if and how MNO control can be ensured for both UE-side and NW-side data collection, e.g., whether the involvement of the CN/OAM or a centralized entity in CN/OAM can be common for all applications.

Proposal 4: As the first step, RAN2 should prioritize analysing the latency requirements of data transfer for these diverse applications.

- Proposal 5: RAN2 is suggested to study enhancements for an AI/ML-specific applicability framework (e.g., applicability reporting and updates) to ensure consistency at the UE and the network side, e.g., deactivate inapplicable AIML configuration timely.

- Proposal 6: RAN2 will study how to ensure a smooth transition and maintain performance accuracy of active AI/ML features during mobility and RLF.

# 4   References

- [1] 	  RP-252912, Revised SID: Study on 6G Radio.

- [2]	  3GPP TR 22.870 V0.4.0 (2025-09) Study on 6G Use Cases and Service Requirements.