3GPP TSG-WG2 Meeting #132	R2-2508338

Dallas, USA, November 17 – 21, 2025

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Aspects for Ambient IoT Topology 2

Document for:	Discussion/Decision

# 1	Introduction

The RAN2 objectives for the work item on Ambient IoT Rel-20 is defined as the following in [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify RRC-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

The work is subject to the following constraints:

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

We focus on aspects for Topology 2 including device addressing, handling out of connection and resource allocation in this paper.

# 2	Discussion

RAN2 has made the below agreements at RAN2#131bis

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

## 2.1 Device AS ID handling

For Topology 1, it has been agreed device AS ID is applied at least for Inventory + command case and is assigned by reader. It is up to reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. This principle should be reused for Topology 2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. If the device AS ID is assigned by the gNB, in CBRA case, the UE reader needs to inform the gNB that a random ID is received in Msg1, and gNB assigns a device AS ID and informs it to the UE reader. This requires extra signalling over Uu and unnecessarily prolongs the AIoT procedure. Some one may concern that the same AS ID may be assigned to devices across different UE readers, which may lead to confusion of AS ID among UE readers. However, we don’t think this is a valid concern since we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

- In Topology 1 device AS ID is assigned by reader and it is up to reader to decide whether to reuse the random ID.

- Allocating device AS ID by gNB in Topology 2 will lead to extra signalling overhead over Uu.

- we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

Reuse Rel-19 AS ID allocation mechanism for D2T2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as AS ID.

## Besides, some companies (e.g., [2], [3]) proposed that UE reader should inform device AS ID in RRC message including inventory response to gNB and gNB binds it with the device NGAP id. In our view this is needed in CBRA inventory plus command procedure as when the gNB receives a command request with the device NGAP id, it can map the device NGAP id to the device AS ID and includes it in the RRC message including command request so that the UE reader knows what device AS ID to use when forwarding the command request to the device. In CFA inventory plus command procedure this is not needed as device AS ID is initially sent to the device with command request and it is up to UE reader what device AS ID to use.

UE reader informs AS ID(s) to gNB based on which the gNB assigns NGAP device ID(s).

## 2.2 How to generate transaction ID

In Rel.19 it has been agreed that the transaction ID can be generated by reader based on CN correlation ID and how to generate transaction ID is left to reader implementation. In Topology 2, however, if the transaction ID is generated by UE reader, it could happen different UE readers map the same CN correlation ID to different transaction IDs and/or map different CN correlation IDs to the same transaction ID, unless there are coordination between UE readers which are not supported. For the former case the device will send duplicate responses to subsequent paging unnecessarily, for the latter case the device will regard the later received A-IoT paging message as subsequent paging message and ignore mistakenly if it has successfully responded earlier. To resolve this the gNB could generate the transaction ID and inform it to the UE reader (at least for CBRA) thus avoid the aforementioned problems. For CFA, RAN2 agreed that transaction ID is not included in paging message, therefore there is no need for the gNB to generate the transaction ID and inform it to the UE reader.

- In Topology 1 transaction ID is generated by reader. This leads to duplicate responses to subsequent paging and/or ignoring a paging message mistakenly.

The gNB generates a transaction ID for a paging message and sends to the UE reader.

## 2.3 A-IoT operation when temporary out of connection

The WID states the RRC_CONNECTED reader including RLF and handover cases and radio resources for A-IoT are only valid in the serving cell.  Regardless of if the UE reader has occurred out of connection due to RLF or handover, it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

- it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

Aim for designing a unified way of handling out of connection in terms of A-IoT radio resource allocation for UE reader regardless of RLF or handover.

A-IoT device type 1 behaviour is the same no matter UE reader is in RLF or handover.

For A-IoT operation when temporary out of connection, two options have been studied and captured in TR38.369:

-	Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases):

-	Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

-	Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

In our view option 1 is better because if the UE reader does not recover from out of connection it will not be able to deliver buffered D2R upper layer data to the NW nor command the A-IoT devices. Therefore, it may be a waste that the UE reader schedules A-IoT transmissions upon temporary out of connection. In addition, it may be a risk to cause interference to other UE(s) by the UE reader if the UE reader continues using the radio resources allocated to the UE reader before out of connection occurs, since the UE reader in out of connection may already lose UL sync towards the gNB. We think the ongoing A-IoT session/procedure should be suspended. On the other hand, for D2R transmissions that are already triggered before the suspension, it could be up to the UE reader implementation whether to receive the D2R transmissions.

- TR 38.169 captured two options for handing temporary out of connection:

- Option 1: the UE reader considers the resources as temporarily invalid during the temporary out of connection condition

- Option 2: The UE reader considers that the resources remain valid for a time period

The UE reader stops sending R2D message for ongoing A-IoT session/procedure during the temporary out of connection condition.

- The UE reader stops sending R2D message for ongoing A-IoT session/procedure during the temporary out of connection condition.

The UE reader considers the resources as temporarily invalid during the temporary out of connection condition, i.e., Option 1 in TR 38.169.

Besides, a suspended A-IoT session/procedure can be resumed after the connection is restored. However, the UE should not perform this autonomously using the previous configured resources as the resources may be given to another UE which is unknown to the UE reader. The UE reader should resume the A-IoT session/procedure after receiving a RRCReconfiguration message from the gNB which instructs the UE to resume using previously configured or new resources. Alternatively the UE may inform the gNB that it wants to recover a suspended A-IoT session/procedure, the gNB may accept this and instruct to the UE to continue the A-IoT session/procedure using previously configured or new resources. The suspended A-IoT session/procedure should be terminated in case the connection cannot be restored.  In addition, we don’t think the inter-gNB handover or RLF recovery is feasible in this release, since the targeted scenario is the indoor scenario wherein a UE reader is not expected to handover or recover RRC connection to a different gNB.

For A-IoT operation under temporary out of connection condition, RAN2 only considers intra-gNB scenario.

The UE reader resumes a suspended A-IoT session/procedure after receiving a RRC message instructing the UE reader to resume. FFS whether the UE reader can send a resume request to the gNB.

The UE reader terminates A-IoT session/procedure if the UE reader cannot recover from the temporary out of connection condition.

If the provided resources for continuing the A-IoT session/procedure are different from the resources configured before temporary out of connection, the UE reader can send a R2D signalling to reconfigure the resources to devices. it can be further discussed if the paging message can be applied for reconfiguring the resources to devices in the same paging round.

The UE reader may update radio resources to devices after its connection has been restored. FFS on signalling details.

## 2.4 Enhancements on Uu procedures to enable A-IoT operations

For Topology 2 only RRC-based architecture is considered. How to enhance Uu procedure to incorporate A-IoT operations should start from type of A-IoT information to be delivered over Uu interface. RAN2#131bis agreed that gNB will provide A-IoT information to the UE reader via RRC dedicated signalling based on NGAP A-IoT information received from CN and FFS what information. We think there are two types of A-IoT information, A-IoT upper layer data and A-IoT control information.

- A-IoT upper layer data is information to be terminated at upper layer (A-IoT NAS), including, e.g.,:

  - Inventory request/response, command request/response,

- A-IoT control information is to be consumed by A-IoT AS sublayer, i.e., device and reader, including, e.g.,:

  - Transaction id.

  - Assistance information (originated from CN) for a paging round sent from NG-RAN to UE reader, e.g., approximate number of devices, expected D2R message size, time interval for sending inventory report,  etc.

    - The assistance information can also be used by NG-RAN for resource allocation purpose to UE reader to serve A-IoT devices, or

    - NG-RAN can also modify assistance information, e.g., it can remove some information or modify CN assistance information subject to RAN conditions.

  - A-IoT resource management related information between NG-RAN and UE reader such as resource allocation signaling, resource consumption status and resource rejection from UE reader, resource reconfiguration, RLF and HO control signaling.

- A-IoT information to be exchanged over legacy Uu interface includes A-IoT upper layer data (inventory, command request and reply) and A-IoT control information (for assistance information and resource management).

One related question is how to transfer A-IoT information over Uu between UE reader and gNB. In our view, the A-IoT resource management related information can be exchanged over existing RRC signaling, e.g., reconfiguration/SRB1 as the information is to be exchanged between gNB and UE reader, i.e., AS level information. Whereas a new SRB can be defined to carry A-IoT upper layer data signaling given that this type of data is to be terminated at the device and CN, i.e., transparent to gNB. Similarly, new RRC messages and procedure(s) to be defined.

Use existing SRB for delivery of A-IoT resource management related information in both UL and DL signalling. FFS details (including whether existing RRC messages are reused or not).

Define a new SRB for delivery of A-IoT upper layer signalling. FFS details.

Figure 1: RRC based protocol stack (adopted from SA2 TR23.700-30-020)

SA2 has discussed the RRC based protocol stack as shown in Figure 1 (see also TR23.600-30-020 (clause 7). RAN2 can base the discussion on this protocol stack.

RRC based protocol stack in TR23.700-30-020 (clause 7) is adopted.

In addition, in Release 19, it was agreed that CN sends information to the reader via NGAP which can include assistance information, follow on command indication []. Extending to Topology 2, the indication behaviour needs to be further clarified. There can be at least two options. In one option, either, gNB can define new RRC IEs carrying the NGAP information. Or in another option, the NGAP container can be included in RRC messages. Both options have pros and cons. Option 1 requires more standardization effort and causes overhead as it requires defining of new RRC IEs. However, the UE reader behaviour is simple comparatively, as the decoding behaviour at UE is exactly as other RRC messages decoding (legacy manner). On the other hand, Option 2 is simple from network point of view as it is some kind of extension of NGAP message to UE reader. However, complexity lies at UE side, we may need to equip UE to understand decoding of message with NGAP container.

- In Release 19, CN sends the NGAP information to gNB which contains assistance information, follow on command indication, etc.

RAN2 to down select among the two options for the gNB to forward the NGAP information received from the CN to the UE reader:

- Option 1: Introduce new RRC IEs for information in RRC signalling,

- Option 2: Introduce NGAP containers for information in RRC signalling.

## 2.5 A-IoT resource allocation

In Topology 2 gNB is in charge of the A-IoT resource allocation and the resource shall be allocated to the UE reader via Uu RRC signaling. In TR38.369, two options have been captured for A-IoT resource allocation to UE reader in RRC_CONNECTED:

- Option 1:	UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration.

- Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

In our view, it is enough to support option 1, as a UE reader in RRC connected should be fully controlled by the gNB. If due to some reason (e.g., temporary out of connection) the UE reader is temporarily not controlled by the gNB, the A-IoT procedure should be suspended.

The resource for A-IoT operation is fully controlled by the gNB and remains valid until explicitly released by the gNB. No explicit timer for resource validity is needed.

One related question is how (based on what) a serving gNB can allocate A-IoT resources.

For the radio resources allocation request, following alternatives are studied and captured in the TR38.369:

-	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

-	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

NOTE 2:	The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

in RAN2#131bis it was agreed that both alternatives are supported and FFS what the assistance information may be useful to be provided to gNB.

In our view, given, the allocated resource from UE reader may become scarce, and in such cases, the UE reader able to indicate request for additional resource. Although, resource allocation is primarily provided by gNB, but such mismatch for the need of resource can happen due to multiple following reasons

- No indication of approximate devices in assistance information from CN,

- Poor estimate of approximate devices in assistance information from CN,

- Poor channel conditions between UE reader and devices, thus requiring more resource utilization,

- Witnessing large collision over resources leading to demand of more resources.

- The resource allocated by gNB to UE reader to serve A-IoT devices can become insufficient, e.g., due to mismatch in resource allocation based on no/poor device load estimate indication from CN, or poor channel conditions between UE reader and devices.

Further, the UE reader can indicate in assistance information regarding the request for resources in time or frequency domain or both. Resultant, a one or more parameters can be defined on resource indication in time/frequency domain in RRC message containing assistance information. In a particular example, the UE reader may indicate its preference on the resource allocation refinement (e.g., increase or decrease of the resource allocation given by the gNB)

UE reader can provide assistance information to the gNB indicating its preference on A-IoT resources allocation refinement.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In Topology 1 device AS ID is assigned by reader and it is up to reader to decide whether to reuse the random ID.

Observation 2	Allocating device AS ID by gNB in Topology 2 will lead to extra signalling overhead over Uu.

Observation 3	we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

Observation 4	In Topology 1 transaction ID is generated by reader. This leads to duplicate responses to subsequent paging and/or ignoring a paging message mistakenly.

Observation 5	it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

Observation 6	TR 38.169 captured two options for handing temporary out of connection:

	Option 1: the UE reader considers the resources as temporarily invalid during the temporary out of connection condition

	Option 2: The UE reader considers that the resources remain valid for a time period

Observation 7	The UE reader stops sending R2D message for ongoing A-IoT session/procedure during the temporary out of connection condition.

Observation 8	A-IoT information to be exchanged over legacy Uu interface includes A-IoT upper layer data (inventory, command request and reply) and A-IoT control information (for assistance information and resource management).

Observation 9	In Release 19, CN sends the NGAP information to gNB which contains assistance information, follow on command indication, etc.

Observation 10	The resource allocated by gNB to UE reader to serve A-IoT devices can become insufficient, e.g., due to mismatch in resource allocation based on no/poor device load estimate indication from CN, or poor channel conditions between UE reader and devices.

Proposal 1	Reuse Rel-19 AS ID allocation mechanism for D2T2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as AS ID.

Proposal 2	UE reader informs AS ID(s) to gNB based on which the gNB assigns NGAP device ID(s).

Proposal 3	The gNB generates a transaction ID for a paging message and sends to the UE reader.

Proposal 4	Aim for designing a unified way of handling out of connection in terms of A-IoT radio resource allocation for UE reader regardless of RLF or handover.

Proposal 5	A-IoT device type 1 behaviour is the same no matter UE reader is in RLF or handover.

Proposal 6	The UE reader considers the resources as temporarily invalid during the temporary out of connection condition, i.e., Option 1 in TR 38.169.

Proposal 7	For A-IoT operation under temporary out of connection condition, RAN2 only considers intra-gNB scenario.

Proposal 8	The UE reader resumes a suspended A-IoT session/procedure after receiving a RRC message instructing the UE reader to resume. FFS whether the UE reader can send a resume request to the gNB.

Proposal 9	The UE reader terminates A-IoT session/procedure if the UE reader cannot recover from the temporary out of connection condition.

Proposal 10	The UE reader may update radio resources to devices after its connection has been restored. FFS on signalling details.

Proposal 11	Use existing SRB for delivery of A-IoT resource management related information in both UL and DL signalling. FFS details (including whether existing RRC messages are reused or not).

Proposal 12	Define a new SRB for delivery of A-IoT upper layer signalling. FFS details.

Proposal 13	RRC based protocol stack in TR23.700-30-020 (clause 7) is adopted.

Proposal 14	RAN2 to down select among the two options for the gNB to forward the NGAP information received from the CN to the UE reader:

Option 1: Introduce new RRC IEs for information in RRC signalling,

Option 2: Introduce NGAP containers for information in RRC signalling.

Proposal 15	The resource for A-IoT operation is fully controlled by the gNB and remains valid until explicitly released by the gNB. No explicit timer for resource validity is needed.

Proposal 16	UE reader can provide assistance information to the gNB indicating its preference on A-IoT resources allocation refinement.

# 4	References

RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

R2-2506930, “Discussion on topology 2 for A-IoT”, OPPO, RAN2#131bis, Prague, Czech Republic, October 13-17, 2025

R2-2506943, “Discussion on Topology-2 for Ambient IoT”, CATT, RAN2#131bis, Prague, Czech Republic, October 13-17, 2025