3GPP TSG-WG2 Meeting #132	R2-2508344

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.3.1.3

Source:	Ericsson

Title:	Scheduling, retransmissions and uplink scheduling information

Document for:	Discussion, Decision

# 1	Introduction

The evolution of 5G New Radio (NR) technology has fundamentally transformed the landscape of wireless communication. As 5G deployments mature and move beyond initial commercial implementations, there emerges a critical need to refine and optimize the user plane protocol stack architecture to meet the demanding performance, energy efficiency, and flexibility requirements of next-generation applications.

To address these requirements, the following detailed objectives are relevant for the RAN user plane as part of the 6G RAN study item description [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

…

k) System simplification, including reducing configuration complexity, enabling more efficienCell/UE management, etc.

(3)	 Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

a)	User Plane architecture and protocol design [RAN2]

# 2	Discussion

At RAN2#131bis, RAN2 has made the following agreements on scheduling and retransmission

Agreements on UP

- 1.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

2.  Study need for scheduling enhancements to address the SR/BSR/DSR latency

3.  Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we further discuss aspects on scheduling and retransmission mechanisms including HARQ and L2 ARQ.

## 2.1 	BSR over CB-PUSCH

When trying to improve the end-to-end (E2E) performance, 3GPP tends to focus on enhancing the data rate that the Uu connection offers to a user or to all users in the system. However, we observe in simulation and field data that the E2E performance is often limited by the effective E2E round-trip-time.

For small data transfers and for the initial phase of larger data transfers, the latency impacts the performance. This is visible in applications like web-browsing, video streaming and real-time communications, i.e., for both UL- and DL heavy applications. Most traffic also requires quick turnaround times for DNS enquiry, TCP or QUIC handshakes, and other connection-establishment procedures.

With increasing L1 data rates and decreasing backhaul/internet delays, the main source of latency is the time it takes to get the first grant for a data-burst using SR/BSR. UL data arriving in an empty UL queue triggers a BSR. If an NR UE has no valid UL grant, it transmits an SR on PUCCH to the gNB for requesting resources. The gNB replies with a small grant allowing the UE to send a BSR on PUSCH. After receiving the BSR, the gNB allocates a larger grant for the remaining data.

Figure 1. A typical application and protocol behaviour

The latency introduced by this procedure becomes the dominant delay component when the UE transmits intermittent UL data. It should be noted that this is also the case for DL heavy traffic which, especially during the early phase of the connection, requires fast transport layer feedback (TCP or QUIC). Typical mobile broadband applications must wait for a response or acknowledgement after transmitted messages, as depicted in Figure 1. Thus, the delay associated with the legacy SR/BSR procedure may occur for every UL message, adding to the overall E2E latency.

NR’s SR/BSR procedure is the dominant contributor to the end-to-end delay and limits the end-to-end performance of UL- and DL-heavy applications.

The Rel-18 WI on XR specified a refined BSR format to provide precise, fine-grained buffer information even for larger buffer and included the delay status report (DSR) to assist the network scheduler in prioritizing among UEs. For effective dynamic scheduling, accurate reports are important. However, these enhancements did not aim for decreasing reporting latency. Therefore, uplink buffer information reporting procedures in 6G should strive to reduce latency while maintaining accuracy and providing sufficient information to support optimal scheduling decisions.

NR’s recent enhancements to BSR can improve the BSR granularity but not its delivery time.

In the past frequent PUCCH-SR allocations have been proposed to reduce the uplink latency due to the delay in acquiring the BSR. However, frequent SR allocations pose practical limitations:

- Frequent allocations cost prohibitively many UL resources that could not be used for user data, thus effectively reducing the uplink capacity.

- Given the non-negligible false positive rate of NR’s SR, frequent PUCCH-SR allocations also trigger many unnecessary grants and thereby waste more resource. In some cases, the network may wait to assign UL grants to a UE until the network has received multiple SRs consecutively, in order to mitigate the SR false detection, which may further introduce longer latency to the UE.

- Frequent PUCCH allocations for SR require more frequent multiplexing with other uplink control information (e.g., HARQ and CSI) when PUCCH resources overlap. In 5G, this process becomes complex and increases UE overhead, since the UE must repeatedly check overlapping PUCCH resources for SR and other UCIs.

Frequent SR allocation comes at a high cost, reducing the uplink capacity. At the same time, due to SR false positive, the number of unnecessary grants and wasted resource increases.

Beyond practical limitations, frequent SR allocation would not solve the problem since they reduce only the delay to SR. Upon reception of the SR, the network still needs to schedule UEs for sending the BSR in order to obtain the complete buffer status information including priority and, buffer size. And since an SR does not indicate the actual amount or importance of the pending UL data, it is difficult for the network to prioritize SRs from different users at high load (downlink control channel limitation or PUSCH limitation) leading to long delays for high priority logical channels.

Frequent SR allocations do not eliminate the time to BSR due to the additional step of scheduling a grant for BSR nor does it solve the issue for the network to prioritize between users in high load situations.

While the delivery of a BSR at the beginning of a new UL data burst is latency critical, we observe that the load on the configured SR resources is generally very low. because 1) most UEs in RRC‑connected mode aren’t active continuously, and 2) once the UE’s BSR reaches the network the gNB can schedule the reported burst efficiently by dynamic scheduling. Allocating dedicated resources to individual UEs solely for sending BSRs is therefore inefficient.

NR’s configured grant (CG) allows the network to preconfigure dedicated PUSCH resources for a user, which can reduce downlink control signalling (PDCCH) and improve the uplink latency. Such UE-specific configured grants can be efficient if the inter-arrival time and the amount of UL data as well as the UL channel conditions are known. However, when the inter-arrival times of traffic bursts vary, configured grants increase either the latency (if occasions are sparse) or the signalling overhead. Hence, configuring UE specific CGs to reduce the scheduling latency for general purpose traffic doesn’t appear feasible.

Figure 2: PUxCH resource cost of UE specific SR and CG [2 PRB per UE] compared to the cost of a common PUSCH resource shared by all UEs.

For 6G we have investigated the possibility to allocate a small, but frequently recurring PUSCH on which any UE may send a BSR if it has UL data but no valid UL grant. We refer to this as contention based PUSCH (CB-PUSCH) for BSR.

Figure 2 compares the resource consumption of NR’s SR and CG with a common CB-PUSCH resource. With BSR or SR transmission opportunities in every uplink slot, CB-BSR (i.e., CB-PUSCH for BSR) is more resource efficient than CG already at very low load (2 RRC connected UEs) and it is more resource efficient than SR in every slot when the load is above 25 RRC connections per cell. If the UEs have a transmission opportunity every 5th slots, the observation is similar, i.e., the resource cost for both SR and CG based BSR transmission grows very quickly compared to CB-PUSCH based one.

The proposal is based on the fact that UEs trigger SR seldom because 1) most UEs in RRC‑connected mode aren’t active continuously, and 2) once the UE’s BSR reaches the network the gNB can schedule the reported burst efficiently by dynamic scheduling. A large number of UEs — on the order of hundreds — can be configured with the same resources (for example, one PRB) very frequently (for example, every slot) and contend, when there are no dedicated UL-SCH resources available, by sending any triggered BSRs on the shared resources. To realize this idea, the following functionalities and aspects are required:

- Since the same resource is shared among hundreds of UEs, we require a reliable and low computational cost method to identify the contending UEs. As an example, the UE identity could be directly included in the payload. A successful decoding attempt then reveals the UE identity to the network.

- Allowing hundreds of UEs to share the same resource requires limiting the access to manage the collisions, i.e., to only when BSR is triggered (i.e., an BSR is triggered due to new data arrival while the UE has no grant).

- Collision management can be further improved by allowing UEs that are contending for resources to randomly select different resources or configurations. For example, UEs can randomly select different DMRS sequences to help resolve collisions among UEs, in addition, they can randomly choose pre-allocated resources at each contention opportunity.

The above points jointly are not sufficiently addressed in 5G and needs to be studied for 6G.

The advantages of initial BSR over CB-PUSCH translate directly into measurable gains perceived at application. We have conducted system level simulations for two different cases with an FDD carrier and another case with a midband TDD carrier.

For FDD, we use either SR every slot (0.5ms), every 10th slot (5ms) or every 40th (20ms). In simulations, the case of contention based PUSCH for BSR (denoted as CB-PUSCH) uses 2 PRBs as CB-PUSCH resources. Each PUSCH transmission applies MCS0 to ensure most reliable coverage for UEs. 8 DMRS orthogonal sequences are configured for CB PUSCH resources in each cell. The resulting application-layer performance is seen in Figure 3. For the web page and the small uploads, the CB-PUSCH shows an improved bitrate at all loads. Comparing with the typical MBB configuration of 20 ms, the web bitrate is 26% higher and the upload bitrate is 57% higher. We can also clearly see that SR every slot does not work at higher loads as the SR overhead heavily increases as depicted in Figure 2.

For the uplink video (middle figure), the CB-PUSCH exhibits a lower tail latency than all the SR configurations. The low-load tail latency for SR with 20 ms period is more than double than for the CB case. We see that all the configurations reach a capacity limit for the uplink video before 350 users/cell, which is the service capacity in this scenario and thus not related to which mechanism we use for SR/BSR.

Figure 3 FDD low-band – end user performance

For TDD, the most frequent SR configuration also uses SR in every uplink slot, but only every 5th slot is an uplink slot. The other two configurations have the same periods as for FDD. Naturally, the CB occasions are also limited by the number of uplink slots and thus appear in every 5th slot (i.e. every uplink slot). The CB-PUSCH configuration shown for these simulations uses 5 CB allocations at each opportunity, where each is one PRB wide. The same application performance metrics as for FDD are shown in Figure 4.

Figure 4 TDD [4:1] mid-band – end user performance

Here, we see an even larger relative gain for CB-PUSCH for the web and upload services. Comparing with the typical MBB configuration of 20 ms, the web bitrate is 105% higher and the upload bitrate is 95% higher. The CB-PUSCH improvement for real time video is similar as for FDD.

When combining the application performance for both FDD and TDD, we so far do not see any load point where any of the SR configurations have better performance than CB-PUSCH.

Transmission of BSR on contention based PUSCH provides a significant latency reduction compared to the conventional SR plus BSR based mechanism.

5G does not support the contention based PUSCH solution since it requires further mechanisms for the contention resolution, e.g., DMRS differentiation and UE identification, and restriction rules to access the CB resource.

Study the contention based PUSCH to reduce latency for uplink buffer information reporting.

## 2.2	HARQ interworking with ARQ

In 5G, HARQ and RLC AM together provide fast and efficient transmissions with very high reliability. NR’s HARQ protocol offers fast retransmissions and soft combining under tight scheduler control. However, it has limited reliability, primarily due to errors in DL grants and UL feedback. RLC AM on the other hand, has very high reliability in terms of packet errors, it can also distribute data over multiple carriers. RLC provides also the possibility to differentiate retransmission behaviour on a per bearer basis i.e. for those services that need it. RLC AM is however relatively slow as it operates on its one state variables and timers, independently from underlying HARQ protocol. Furthermore, RLC is configured per radio bearer and according to their QoS requirements, but parameters cannot be adapted to the characteristics of different CCs. This was also observed and studied in NR Rel-19 to enhance XR support.

In 5G, HARQ and RLC ARQ operate independently. HARQ is driven by Layer 1 signaling and feedback on a per‑component carrier basis, enabling fast retransmissions but leaving some residual losses. In contrast, RLC ARQ is timer‑driven and works on a per radio bearer basis, making it slower but highly reliable.

The following agreement was made at RAN2#131bis in terms of HARQ and ARQ interworking:

Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In [4], we propose that 6G UEs send uplink HARQ feedback and CSI reports in the PUSCH payload (MAC Sub-PDUs). If the 6G UE mapped its HARQ feedback into MAC PDUs it would become as reliable as NR’s RLC status reports. Based on the reliable HARQ feedback, the scheduler could decide to abandon a continuously failing downlink HARQ process and to trigger a (possibly re-segmented) RLC retransmission. This could reduce latencies and reduce overhead from otherwise needed frequent RLC status reporting. If HARQ feedback was transmitted on PUCCH, there would be a risk of ACK-to-NACK errors leading to redundant RLC retransmissions leading to inefficiencies. Meanwhile, NACK-to-ACK errors would rely on RLC retransmissions (triggered based on RLC status report) to recover from the errors, which incurs additional latency. For some services, RLC retransmissions triggered based on HARQ NACKs could even make RLC status reporting fully redundant.

In general, RLC status reporting is still useful for efficient handling of the RLC transmitter window using the highest acknowledged RLC sequence numbers (SN). Otherwise, the HARQ layer would be required to keep track of all RLC SDUs mapped into a HARQ process. For HARQ NACK only, the gNB could parse the MAC PDU to derive the RLC bearers and corresponding SNs to provide an RLC NACK for the correct RLC entity and corresponding SNs.

For DL, study the solution which allows the gNB to use HARQ NACKs instead of waiting for an RLC status report to reduce the latency for RLC retransmissions.

For UL transmissions in 5G, the gNB does not provide any explicit HARQ-FB (only for shared spectrum channel access, explicit HARQ-FB can be provided for PUSCH). HARQ NACKs are implicit by a gNB’s UL retransmission grant. If the gNB’s scheduler decides to abandon a continuously failing HARQ process, it would be difficult to immediately trigger an RLC status report since it does not know which RLC SDUs and from which radio bearer are contained in the failed HARQ process. The gNB would need to guess what information to include in the RLC status report. This would be inefficient as it requires extra processing in the gNB and as it could trigger unnecessary RLC retransmissions.

The mapping can only be known at the transmitter side, i.e. at the UE HARQ/RLC layer, but the UE is not aware of the decoding result at the gNB. Instead, the gNB can indicate in the UL grant whether it prefers a HARQ retransmission or rather an RLC retransmission, i.e. for the latter to abandon the current HARQ process and allow for RLC resegmentation and thus enable the selection of a more robust MCS scheme as illustrated in Figure 5. This would enable faster RLC retransmissions than waiting for NACK in the RLC status report.

Figure 5. Network requests RLC UL retransmission via UE’s HARQ transmitter.

For UL, study the solution which allows the gNB to request the UE’s HARQ transmitter to trigger an RLC retransmission.

## 2.3 UE buffer reporting unification

Delay Status Report (DSR) was introduced during 5G XR standardization as a way for the UE to report delay information for the delay critical data in the UE buffer to the network. It was observed that delay information can be used by the network when scheduling and prioritizing between XR users, e.g. in a delay scheduler. Delay-information is useful when handling different services with different delay requirements.

Delay information is one part of the UE buffer status which is generally useful for the network scheduler.

When the DSR was introduced in NR, it was decided that it should be separate from the BSR, since it was expected this would lead to a simpler solution as delay information is different from the normal buffer size information. However, later during XR standardization it was brought up multiple times that there can be coupling between BSR and DSR, and there is complex relationship between them, e.g. if a BSR should be cancelled because of a DSR reporting the same buffer size information. The complexity also stems from the fact that the information in the BSR and DSR both has merits to be delivered. The DSR doesn’t report the complete data situation in the buffer, only the delay reporting part, and has limited trigger conditions since delay information is not necessary to always update. The BSR helps scheduler to have precise knowledge about the changes in buffer size while the DSR provides important information about the changes to remaining time. Thus, both reports were deemed needed in NR, and collisions can occur. In a unified approach, only one report is transmitted but the actual information contained can be dependent on the situational need.

In addition, a single DSR or BSR only conveys one part of the complete UE buffer status information. A unified UE reporting approach would be beneficial to allow the UE to provide complete UE buffer status (including priority information, buffer size and delay) in one report. It may be also beneficial to simplify the UE procedure by monitoring the same set of trigger events and maintaining the same timers/counters.

In NR, BSR and DSR separation has led to unnecessary overhead and complexities because of the dependencies to each other.

For 6G this should be solved by already from the start aligning the reporting frameworks into one and avoid such extra complexities.

RAN2 to study a unified format and trigger mechanism for UE buffer reporting including buffer size and delay information.

## 2.4 Improved UL scheduling

For 6G it is expected that multiple services with different requirements will be run in parallel. The advancement of new services and use cases, such as Immersive communications, etc. , comes with new requirements and traffic characteristics. In many cases this implies tough delay and bitrate requirements. Still there will be a need to support services with more relaxed requirements, such as traditional eMBB traffic. When moreservices with different requirements run in parallel in a UE, this creates a higher demand for precise prioritization between the services. This service differentiation can be enabled by mapping services to different logical channels and then prioritizing between the logical channels.

The 5G solution for intra UE prioritization is the Logical Channel Prioritization (LCP) mechanism. This mechanism controls how much data is selected from each logical channel (i.e. service) when allocating resources to the UL grant. To do the selection the mechanism uses a number of parameters such as the logical channel priority, prioritized bitrate (PBR) and bucket size duration (BSD). These parameters are configured by the network. However, the problem with this framework is the static nature of this configuration. Because of the dynamic nature of the radio link the UL transmission rate varies but the LCP configuration does not. For example, a high PBR may be wanted for a prioritized service to reach high QoE. This is ok while the transmission rate is high since other low priority services will still be served. However, when the rate is low a configured high PBR value for high priority service can make the other low priority services to be starved since the high priority service consumes all the resources. In the current framework, this leads to that a lower than wanted PBR need to be configured for the high priority service and worse QoE for this service when transmission rates are high. In essence with the current framework the LCP parameters need to be configured to avoid starvation rather than optimizing the QoS.

The static nature of the 5G LCP mechanism can lead to starvation and poor QoS.

Thus, in 6G RAN2 should study how to enhance UL scheduling.

RAN2 to study how to enhance UL scheduling.

# 3	Conclusion

In this paper, we have reached these observations:

Observation 1	NR’s SR/BSR procedure is the dominant contributor to the end-to-end delay and limits the end-to-end performance of UL- and DL-heavy applications.

Observation 2	NR’s recent enhancements to BSR can improve the BSR granularity but not its delivery time.

Observation 3	Frequent SR allocation comes at a high cost, reducing the uplink capacity. At the same time, due to SR false positive, the number of unnecessary grants and wasted resource increases.

Observation 4	Frequent SR allocations do not eliminate the time to BSR due to the additional step of scheduling a grant for BSR nor does it solve the issue for the network to prioritize between users in high load situations.

Observation 5	Transmission of BSR on contention based PUSCH provides a significant latency reduction compared to the conventional SR plus BSR based mechanism.

Observation 6	5G does not support the contention based PUSCH solution since it requires further mechanisms for the contention resolution, e.g., DMRS differentiation and UE identification, and restriction rules to access the CB resource.

Observation 7	In 5G, HARQ and RLC ARQ operate independently. HARQ is driven by Layer 1 signaling and feedback on a per‑component carrier basis, enabling fast retransmissions but leaving some residual losses. In contrast, RLC ARQ is timer‑driven and works on a per radio bearer basis, making it slower but highly reliable.

Observation 8	Delay information is one part of the UE buffer status which is generally useful for the network scheduler.

Observation 9	In NR, BSR and DSR separation has led to unnecessary overhead and complexities because of the dependencies to each other.

Observation 10	The static nature of the 5G LCP mechanism can lead to starvation and poor QoS.

We propose:

Proposal 1	Study the contention based PUSCH to reduce latency for uplink buffer information reporting.

Proposal 2	For DL, study the solution which allows the gNB to use HARQ NACKs instead of waiting for an RLC status report to reduce the latency for RLC retransmissions.

Proposal 3	For UL, study the solution which allows the gNB to request the UE’s HARQ transmitter to trigger an RLC retransmission.

Proposal 4	RAN2 to study a unified format and trigger mechanism for UE buffer reporting including buffer size and delay information.

Proposal 5	RAN2 to study how to enhance UL scheduling.

# 4	References

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, Prague, Czech Republic, June 9-13, 2025 [online]

Internet Engineering Task Force (IETF), RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture”, December 2023

R2-2507074, Common User Plane and Control Plane Aspects for 6G, 3GPP RAN2#131bis, Prague, Czech Republic, October 13-17, 2025.

R2-2508615, UCI on L2, Ericsson, 3GPP RAN2#132, Dallas, USA, November 17 – 21, 2025.