3GPP TSG-RAN WG2#132	R2-2508346

Dallas, TX, USA, 17-21 November 2025

Agenda Item:	10.3.2.3

Source: 	MediaTek Inc.

Title:  	System information and access for 6G radio

Document for:	Discussion, decision

# 1	Introduction

This document discusses topics on system information and access in 6G.  We assume that the basic mode of discovering and accessing a cell is carried forward (i.e., SSB => MIB => SIB1 => other SI => camping => access), but the analysis should be generally adaptable if 6G diverges from the 4G/5G baseline in this respect.

# 2	Discussion

## 2.1	System information

### 2.1.1	On-demand SIB1

It is clear from discussion at RAN2#131bis that there is interest in specifying on-demand SIB1 requests for 6G.  The natural question is how a UE newly arrived in a cell (either from another cell or from power-off) determines that the cell is there and finds an appropriate configuration to request SIB1.  The detection of the cell depends on RAN1 decisions (e.g., detecting the SSB might be enough to start the process).

Regarding the request configuration, if SIB1 is not being broadcasted in a cell, the UE may know the PHY parameters of the cell (from SSB and/or MIB), any additional configuration in the MIB, and/or the contents of system information (including SIB1) from another cell or a previous visit to the same cell.  (The OD-SIB1 mechanism in NR depends on the last of these: The UE obtains its OD-SIB1 request configuration from SIB26 of some cell where it previously camped.)  Any of these pieces of information could be leveraged to provide a simple configuration for a SIB1 request.  In order of ascending complexity, we see the following possibilities:

- Specified fixed uplink configuration for requesting SIB1 in a cell

- Small set of configurations for requesting SIB1, with an index provided by SSB parameters or in the MIB

- Simple explicit uplink configuration in MIB (we assume this is not feasible using only SSB)

- More complex uplink configuration available only from other SI in a prior visit to some cell (like NR)

Observation 1: If SIB1 is transmitted on demand, a UE could use any of several mechanisms to determine what uplink configuration to use to request SIB1.

It seems premature to narrow down the options, especially without knowing how much information might be conveyed by PHY or how much space is available in the MIB (or equivalent).

Proposal 1: Study options for requesting SIB1 on demand, including how a newly-arrived UE in a cell can obtain a configuration for requesting SIB1, e.g., via a fixed uplink configuration, an indexed uplink configuration in PHY/MIB, or an explicit uplink configuration signalled in the MIB or in other SI.

As a complement to the mechanism for requesting OD-SIB1, it might be useful to relax the conditions for operating in a cell with a previously stored SIB1 (either from this cell or another).  If SIB1 is valid over an area (except for obviously cell-specific information like cell IDs), the UE could use a neighbour cell’s SIB1 and forgo or delay requesting SIB1 from the serving cell.  Access control, which is often cell-specific, would need to be considered (e.g., the UE is required to receive SIB1 if access control is active in the cell).

Observation 2: Even if SIB1 is valid over an area, some cell-specific information like cell IDs and access control needs to be considered.

Proposal 2: Study mechanisms for allowing the UE to operate in a cell without (or before) receiving SIB1 from the cell, including area validity of SIB1 and considering the need to respect cell-specific access control.

### 2.1.2	SIB1 in RRC_CONNECTED

During RAN2#131bis online discussion, we mentioned 5G pain points related to usage of SIB1 in RRC_CONNECTED. No explicit agreement to study this was reached as the topic was not captured in any of the treated contributions, but further contributions on which aspects of the system information handling to study were invited by the Chair [2]:

Based on the above, we discuss two 5G pain points related to usage of SIB1 in RRC_CONNECTED and provide a proposal to guide the 6G RRC design in the direction where the same pain points would not reappear in 6G.

The first 5G pain point related to SIB1 usage in RRC_CONNECTED is the servingCellConfigCommon carried in SIB1, for which 38.331 clause 5.2.2.4.2 specifies the following:

5.2.2.4.2 Actions upon reception of the SIB1 Upon receiving the SIB1 the UE shall: 1> store the acquired SIB1; < omit unrelated text> 1> if in RRC_CONNECTED while T311 is not running: 2> disregard the frequencyBandList, if received, while in RRC_CONNECTED; 2> forward the cellIdentity to upper layers; 2> forward the trackingAreaCode to upper layers, if included; 2> forward the trackingAreaList to upper layers, if included; 2> forward the received posSIB-MappingInfo to upper layers, if included; 2> apply the configuration included in the servingCellConfigCommon; < omit the rest of the clause >

That is, upon acquisition of SIB1 in RRC_CONNECTED, the UE is required to apply servingCellConfigCommon (excluding frequencyBandList), which contains fundamental parameters of the PCell, such as offset to Point A, SCS specific carrier list, and common parts of initial DL/UL BWP configurations. The issues related to this are following:

Should the UE accept an update of the mentioned fundamental parameters of the PCell "on the fly" upon SIB1 reception (e.g., SI modification), even if the same parameters can be updated by the dedidated RRC signalling only with reconfiguration with sync procedure?

How should the UE combine the current UE configuration with the new configuration given in SIB1, assuming there is an overlap or a collision between them?

Over the years, we have observed IODT/interoperability issues with several infra-vendors about this topic.

It should be noted that RAN2 made the following agreement in RAN2#115-e:

- Fields that are present in ServingCellConfigCommon delivered by dedicated signalling shall have the same value as the corresponding field in SIB1.

However, we understand this agreement to be binding only for a certain point in time (e.g., at the time of handover), i.e., it does not exclude the scenario where the network modifies the SIB1 content later on.

Observation 3: In 5G, servingCellConfigCommon carried in SIB1 has been a source of confusion and IODT/interoperability issues when SIB1 is acquired by the UE in RRC_CONNECTED.

The second 5G pain point related to SIB1 usage in RRC_CONNECTED is that certain parameters required by the UE to operate in RRC_CONNECTED are carried only in SIB1 and not in dedicated RRC signalling. With this, we mean the following parameters:

BCCH configuration and PCCH configuration; required for BCCH change monitoring;

Access barring parameters; required for UAC procedure upon request from NAS (e.g., a new service needs to be established by NAS while in RRC_CONNECTED);

NAS system information; required for MRU (TA update) by NAS.

It should be noted that 38.331 does not specify any UE action for the case where the UE fails to acquire SIB1 in RRC_CONNECTED. However, in practice, the UE cannot operate without the mentioned parameters carried in SIB1, so the UEs may implement proprietary mechanisms, such as trigger RRC connection re-establishment, to recover from the situation where the UE fails to acquire up-to-date SIB1 in RRC_CONNECTED.

Observation 4: In 5G, parameters required by the UE for BCCH monitoring and UAC, as well as NAS system information, are carried only in SIB1 and not in dedicated RRC signalling. In practice, the UE cannot operate properly if the UE fails to acquire up-to-date SIB1 in RRC_CONNECTED.

We can anticipate that also 6G SIB1 will likely contain basic parameters of the PCell (equivalent to servingCellConfigCommon), parameters required for BCCH monitoring, access barring parameters, and NAS system information. For this reason, to prevent the pain points presented above from reappearing in 6G, we propose that the UE does not acquire SIB1 for the operation in RRC_CONNECTED in 6G. This does not exclude acquisition of SIB1 in RRC_CONNECTED for secondary purposes, such as CGI reporting.

Proposal 3: In 6G, the UE does not acquire SIB1 for the operation in RRC_CONNECTED. The necessary parameters for UE operation in RRC_CONNECTED (including UE configuration, access barring parameters, and NAS system information) are sent in dedicated RRC signalling.

### 2.1.3	SI change procedure

With the proliferation of SIBs over multiple releases in earlier systems, a well-known pain point for the UE arises connected to SI modification.  The so-called “short message” only indicates that the system information changed (except for the special case of PWS), and the UE is required to wake up and receive SIB1 to determine which SIBs are affected.  This wake-up procedure is something of a waste of energy for a UE in NR already, but if paging is monitored by a low-power radio in 6G, the impact would be greater, since receiving SIB1 would most likely require activating the main radio—often only to discover that a SIB changed for some service in which the UE has no interest.

Observation 5: The problem of UEs needing to wake up for every SI change may be expected to be worse in 6G, especially if paging is monitored by a low-power radio.

To alleviate this issue, it is desirable to have somewhat finer granularity in the SI change notification.  The network could indicate if the change affects all UEs (fundamental SIBs governing access to the cell), only certain UEs (e.g., SIBs needed in idle/inactive but not in RRC_CONNECTED), or only certain services (list and level of granularity would need to be determined, but the mechanism should be reasonably extensible for future services that may not be of interest to all UEs).  If the “short message on PDCCH” model is maintained, cooperation with RAN1 will be required to understand how many bits may be available for this notification and what the other pressures on those bits are expected to be.

Proposal 4: Study (from RAN2 perspective) an increase in granularity for the SI change notification, with the objective of reducing wake-up events for UEs not interested in the SIB(s) that changed.

### 2.1.4	SI window scheduling scheme

In NR, the scheduling scheme based on SI windows (largely inherited from LTE) became a problem as more SIBs were added and the scheduling proved not to be flexible enough to handle a large number of broadcast SIBs.  The result was a set of requirements with four different scheduling conditions (for schedulingInfoList, schedulingInfoList2, posSchedulingInfoList without offsetToSI-Used, and posSchedulingInfoList with offsetToSI-Used), and 6G should be able to rely on a unified and more extensible scheme.

We understand the SI window was introduced both to provide network flexibility and to allow UE combining of SI message repetitions within the window (the latter is one reason for SI windows not to overlap).  These concerns are still valid for 6G and should be taken into account, suggesting that some form of the window scheme would still be useful.  Enhancements should, however, be considered; some plausible examples include allowing the windows to overlap (which would require some way of supporting combining, perhaps differentiating with different SI-RNTIs), expanding the explicit scheduling mechanism currently used in schedulingInfoList2, and grouping service-related SIBs in common containers that can be scheduled together as currently done with posSIBs.

Observation 6: The drivers for the original SI window design are still valid in 6G, but the complexity of the NR SI scheduling design(s) calls for some enhancements/simplifications.

Proposal 5: Study a baseline approach to SI scheduling in which the window scheme is adapted for greater flexibility, considering as possible enhancements overlapping windows, multiple SI-RNTIs, explicit scheduling of window positions, and grouping of service-specific SIBs.

### 2.1.5	Security for SI

SA3 have previously studied false base station attacks on NR systems, generating some interest in security for system information (e.g., integrity to allow the UE to confirm that it faces a genuine BS).  The NR study was not conclusive, but it seems likely that the same issues will be examined for 6G.  We address this topic in a separate document in [1].

## 2.2	Access

The access procedure in NR comes in several flavours: 2-step, 4-step, and the special case of SDT.  It is not obvious in advance which of these should be carried forward into 6G.  4-step random access seems necessary for use in FR1, but 2-step may be attractive for FR3.  SDT could be quite valuable as a way to exchange short intermittent traffic (e.g., keep-alives) without state transitions.  Thus, it seems reasonable to study all alternatives and determine later if there is reason to downselect.

Proposal 6: Study 2-step and 4-step access procedures as well as SDT, with the potential to downselect for normative work if it is found that some procedures are unnecessary in 6G.

The size of Msg3 is a recurrent issue for RAN2, with different-sized Msg3 formats for different circumstances and coverage conditions.  In general, a larger Msg3 baseline size would be useful if coverage expectations can support it.  Evaluating the coverage depends on RAN1, but RAN2 can focus on what information would be useful to have in Msg3 and attempt to spur RAN1 to find ways to make it fit.

Proposal 7: Study the needed contents of Msg3, targeting a unified access procedure and with the intention of indicating to RAN1 during the study phase what size of Msg3 RAN2 consider desirable.

# 3	Conclusion

This document promulgated the following mix of observations and proposals:

[On-demand SIB1]

Observation 1: If SIB1 is transmitted on demand, a UE could use any of several mechanisms to determine what uplink configuration to use to request SIB1.

Proposal 1: Study options for requesting SIB1 on demand, including how a newly-arrived UE in a cell can obtain a configuration for requesting SIB1, e.g., via a fixed uplink configuration, an indexed uplink configuration in PHY/MIB, or an explicit uplink configuration signalled in the MIB or in other SI.

Observation 2: Even if SIB1 is valid over an area, some cell-specific information like cell IDs and access control needs to be considered.

Proposal 2: Study mechanisms for allowing the UE to operate in a cell without (or before) receiving SIB1 from the cell, including area validity of SIB1 and considering the need to respect cell-specific access control.

[SIB1 in RRC_CONNECTED]

Observation 3: In 5G, servingCellConfigCommon carried in SIB1 has been a source of confusion and IODT/interoperability issues when SIB1 is acquired by the UE in RRC_CONNECTED.

Observation 4: In 5G, parameters required by the UE for BCCH monitoring and UAC, as well as NAS system information, are carried only in SIB1 and not in dedicated RRC signalling. In practice, the UE cannot operate properly if the UE fails to acquire up-to-date SIB1 in RRC_CONNECTED.

Proposal 3: In 6G, the UE does not acquire SIB1 for the operation in RRC_CONNECTED. The necessary parameters for UE operation in RRC_CONNECTED (including UE configuration, access barring parameters, and NAS system information) are sent in dedicated RRC signalling.

[SI change notification]

Observation 5: The problem of UEs needing to wake up for every SI change may be expected to be worse in 6G, especially if paging is monitored by a low-power radio.

Proposal 4: Study (from RAN2 perspective) an increase in granularity for the SI change notification, with the objective of reducing wake-up events for UEs not interested in the SIB(s) that changed.

[SI window scheduling]

Observation 6: The drivers for the original SI window design are still valid in 6G, but the complexity of the NR SI scheduling design(s) calls for some enhancements/simplifications.

Proposal 5: Study a baseline approach to SI scheduling in which the window scheme is adapted for greater flexibility, considering as possible enhancements overlapping windows, multiple SI-RNTIs, explicit scheduling of window positions, and grouping of service-specific SIBs.

[Access]

Proposal 6: Study 2-step and 4-step access procedures as well as SDT, with the potential to downselect for normative work if it is found that some procedures are unnecessary in 6G.

Proposal 7: Study the needed contents of Msg3, targeting a unified access procedure and with the intention of indicating to RAN1 during the study phase what size of Msg3 RAN2 consider desirable.

- References

[1]	R2-2508347, “Security assumptions for system information”, MediaTek Inc., RAN2#132

[2]	R2-25xxxxx: “Report of 3GPP RAN2 WG2 meeting #131bis, Prague, Czech Republic”, ETSI MCC, RAN2#132