3GPP TSG-RAN2 WG2 Meeting #132	R2-2508354

Dallas, USA, 17 – 21 November 2025

Agenda Item:	10.4

Study Item:	FS_6G_Radio

Source: 	Qualcomm Incorporated

Title: 	Views on Mobility and RRM for 6G

Document for:	Discussion/Decision

# Background

This paper provides our views on some of the topics that will be addressed by this agenda item:

Mobility framework and targets, Measurement Framework, Intra-RAT Connected Mobility (e.g. L3, CHO, LTM, RLM/RLF), Cell Selection and Reselection, Inter-RAT.

The following were the agreements from RAN2#131bis:

Study mobility with the following requirements in 6G mobility design:

-    Minimize interruption time and ensure service continuity (i.e., minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-    Robustness of mobility procedures

-    Energy efficiency for both UE and NW

Study aspects related to mobility (e.g., early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

# Discussion

## Mobility framework and targets for 6G

### Early DL/UL synchronization

In 5G, to further reduce mobility interruption time w.r.t L3 HO, mechanisms such as early DL sync and early UL sync for the candidate cells were introduced for LTM. Early DL sync and early UL sync is achieved by UE obtaining the TCI state information and TA information of the LTM candidate cells respectively, prior to the LTM cell switch. This enables the UE to perform fine time tracking, acquiring timing information of the target cell and SSB processing in advance and thereby perform a RACH-less LTM cell switch and reducing the mobility interruption time as shown in Figure 1 below.

Figure 1: Mobility Latency for RACH-less LTM (from Figure G-3 in TS 38.300)

Below we highlight some of the benefits and complexities we see with the current early DL/UL sync mechanism supported in 5G and propose some aspects RAN2 can further consider in the 6G study.

Early DL sync

Benefits: Shorter interruption time by performing early DL sync in advance with the candidate cells

Complexities: If not done properly, network-controlled TCI state activation of candidate cells may result in additional UE measurement/processing overhead: If TCI activation of candidate cells is not selectively/intelligently configured by the gNB, UE might need to monitor the activated SSB/CSI-RS of several “unnecessary” candidate cells, adding burden to UE’s processing capability and measurement modules. This may further compete with serving cell measurement and beam management opportunities, especially in FR2.

Goal for 6G: 6G should study how to enable more selective early DL sync on the most relevant candidate cells without adding burden to UE’s measurement/processing capabilities (e.g., via UE-based activation/deactivation of candidate cell TCI states).

Early UL sync

Benefits: Shorter interruption time by performing early UL sync in advance with the candidate cells

Complexities:

- Network-triggered/controlled early UL sync procedure as in 5G (with limited control to UE):

  - In LTM, source gNB tracks the TA of the candidate cells and maintains the TA validity timer of candidate cells via implementation. In case the source gNB didn’t receive any TA value from a desired candidate cell (e.g., due to failed Msg1) or if the source gNB decides that the received TA value is no longer valid, source gNB needs to retransmit PDCCH order to ask UE to redo the early UL sync. This might lead to repetitive early UL sync procedures and might not be efficient.

  - Even in case of CLTM, this depends on source gNB sending the TA of candidate cells along with an appropriate CLTM TA validity timer. But this CLTM TA validity timer might not be optimally configured, either resulting in unnecessary RACH-based CLTM execution or unnecessary TA updates for the candidate cells

- RACH resource reservation for early UL sync may be “expensive” if not properly: In 5G, each LTM candidate cell reserves a pool of RACH resources (not UE-specific) for performing early UL sync and these RACH resources are informed to the source cell via backhaul signalling. Then it is up to the source cell to determine how to allocate these RACH resources among UEs. When UE sends Msg1 to candidate cell, the candidate cell computes the TA value and forwards the TA value along with RAPID and other necessary information. The source cell identifies the UE based on the received information and then can send the CSC MAC-CE to the UE with the received TA value. This RACH resource reservation might be expensive if not properly e.g., done quite early or done without careful pooling.

- Tight timeline at the UE: UE needs to perform several things upon reception of PDCCH order to sending of PRACH e.g., RO selection, determine pathloss for sending the PRACH, suspend transmission with the serving cell, BWP switch (in case PRACH is to be sent outside the active BWP), RF retune (in case of inter-frequency candidate) and this timeline might be quite tight from UE processing perspective.

- Interruption to serving cell during early sync: Performing early UL sync results in interruption to serving cells before/during/after the PRACH transmission

Goal for 6G: RAN2 should study how to support a more efficient early UL sync procedure w.r.t. the 5G procedure (e.g., via UE-initiated TA acquisition procedure), enabling selective early UL sync on the most relevant candidate cells and without causing much interruption to the serving cell.

Irrespective of some of the complexities highlighted above in regard to the early DL sync and early UL sync procedure in 5G, we think it is a very useful feature to support in order to reduce the mobility interruption, at least in some cases (e.g., when timing is known apriori or via backhaul signaling or when UE’s capabilities allow it to perform early DL sync towards the candidate cells).

When it is not possible to do early UL sync and/or early DL sync, UE can always fall back to perform RACH-based mobility.

We therefore propose the following:

Early DL/UL sync in 5G enables shorter interruption times during LTM cell switch by source cell pro-actively obtaining the TCI state information and TA information of the LTM candidate cells, prior to the LTM cell switch.

Network-triggered early UL sync procedure in 5G may not always be efficient as there may be repeated (e.g., due to TA becoming invalid) and unnecessary (e.g., due towards unnecessary candidate cells) early UL sync procedures. Also, efficient pooling of RACH resources for early UL sync is important for minimizing resource reservation.

Candidate cell TCI activation/deactivation procedure (“early DL sync”) in 5G may result in additional measurement/processing overhead at the UE if not selectively done by the network.

6G mobility framework should support efficient early DL/UL sync procedures to minimize mobility interruption time without significant UE measurement/processing overhead and resource reservation issues at the network. RAN2 to study enhancements w.r.t. early UL/DL sync procedures supported in NR.

### Pre-configuration of candidate cells

In 5G, CHO/LTM/CLTM supports pre-configuring the UE with several candidate cell configurations in advance. Below we highlight some of the benefits and complexities we see with the current pre-configurations supported in 5G.

Benefits:

- Reducing interruption time and increasing robustness via early RRC decoding: Pre-configurations enables early decoding of RRC configurations at the UE and reducing mobility interruption time w.r.t. regular L3 HO. Also providing the candidate cell configurations in advance avoids the need to provide the target cell configuration immediately before the HO, thereby reducing chances of too late HO and increasing robustness to the mobility procedure.

- Lean HO command: If candidate cell configurations are pre-configured, source node does not send the full RRC configuration of the target cell in the HO command (as done via RRCReconfigWithSync in case of L3 HO) and can just the indicate the pre-configured candidate config ID as done via LTM Cell Switch MAC-CE)

- Enables fast recovery in case of RLF/HOF: Pre-configuration of candidate cells helps in “fast recovery” e.g., in case UE encounters an RLF or HOF and selects a cell which is also a pre-configured candidate cell, UE can directly apply the candidate cell configuration and perform fast recovery, in contrast to performing a re-establishment procedure.

Complexities

- Additional UE memory: UE might need to store the candidate cell configurations (e.g., 8 candidate cells) in its memory (sometimes unnecessary configurations e.g., if that candidate cell was not selected as a target cell) and might add to UE memory requirements.

  - Enabling faster recovery and having a lean HO command seems to outweigh the cons for UE to store some candidate cell configurations.

- Resource reservation at candidate cells: The candidate RAN node(s) also need to reserve some resources (e.g., RACH resources in case of RACH-based or CG resources in RACH-less) to enable these pre-configurations.

  - There were some concerns raised in 5G on this resource reservation issue for CHO/LTM/CLTM. But we don’t think this should be a big concern at the network considering the RAN node can modify/release the reserved resources anytime it wants (e.g., if it’s in a resource crunch) and can modify/release the candidate cell configuration at the UE.

- Additional backhaul signalling: From network perspective, the additional impact to support pre-configurations is for source RAN node to do some “early” HO preparation by coordinating with candidate RAN node(s) to obtain the candidate cell configuration(s).

Pre-configuration of candidate cells helps in reducing interruption time and increasing robustness via early RRC decoding at the UE, enables fast recovery in case of RLF/HOF and enables to have a lean HO command.

The complexities w.r.t. reserving resources when multiple candidate cells are pre-configured can be addressed via smart network implementations e.g., by selecting the right set of candidate cells, reserving the resources for a suitable duration and using modify/cancel procedures as needed.

Considering the benefits outweigh the complexities, we propose the following:

6G mobility framework should support pre-configuring a UE with one or more candidate cell configurations to minimize mobility interruption time (e.g., via early RRC decoding of the candidate cell configurations) and to enable fast recovery post RLF/HOF.

### Conditional mobility

In 5G, both CHO and CLTM supports configuring the UE with execution conditions associated to each candidate cell, via conditional events in RRC in case of CHO or conditional events in MAC in case of CLTM. Below we highlight some of the benefits and complexities we see with the conditional mobility and propose some aspects RAN2 can further consider in the 6G study.

Benefits:

- Lower interruption time: Conditional mobility enables lower mobility interruption times w.r.t, network triggered mobility, without the need for UE to send a measurement report, wait for a HO command, process the HO command and then execute the HO/cell switch.

- Higher robustness: Conditional mobility also ensures higher robustness w.r.t. network-triggered mobility e.g., in case radio conditions dynamically change from the time MR is sent to HO command is received. With conditional mobility, UE can execute HO/cell switch at the right time without resulting in Too late HO or HO to wrong cell scenarios.

- Leverages additional knowledge at UE: Besides evaluating the execution conditions provided by the RAN node, UE can leverage additional knowledge (e.g., event/measurement/RLF predictions via AI/ML techniques or other side information such as ongoing service, UE battery/power status) to select a more suitable candidate cell e.g., in case there is more than 1 suitable candidate cell and for the timing of the cell switch (without waiting for the NW to send the HO command).

Complexities:

- Unclear when source gNB should perform data forwarding: In 5G, some concerns which were raised for conditional mobility was that the gNB might not know when the cell switch will be executed and therefore the source gNB wouldn’t know when to start early data forwarding towards the candidate gNB(s) to ensure lossless data delivery.

- Unclear how long to reserve resources at candidate cells: Similar concern as mentioned in section 2.1.2. If the source gNB doesn’t know when the switch will happen, it might have to reserve resources at the candidate gNB(s) for a long/unknown amount of time.

In our view, an intelligent gNB should be able to determine when to perform early data forwarding and how long to reserve resources via implementation e.g., based on predicting the HO execution time or measurement prediction. But if it’s indeed a significant problem and can’t be resolved at the gNB side via implementation, 6G can study whether any enhancements are needed to the conditional mobility framework e.g., UE can send a “bye message” to the source RAN node before the conditional mobility execution with some useful information (e.g., future timepoint of conditional mobility execution).

Conditional mobility (e.g., CHO/CLTM in 5G) can help in lowering interruption time, increasing robustness and enables the usage of local knowledge at the UE for efficient mobility execution. There were some concerns that CHO implementations in 5G didn’t show the promised benefits because of source gNB not being able to perform efficient resource reservatio and early data forwarding to candidate gNB(s).

6G mobility framework should support conditional mobility (i.e., cell switch execution by the UE based on the satisfaction of certain event triggers). For efficient data forwarding and resource reservation, RAN2 can further study potential enhancements (e.g., UE sending a “bye message” to source RAN node) to support conditional mobility in 6G.

### Early CSI acquisition

In 5G, R19 LTM enabled mechanisms to perform early CSI acquisition towards LTM candidate cells e.g., by providing the CSI-RS resource/report configuration of candidate LTM cells to the UE in advance, along with the candidate cell configurations. UE then can measure CSI-RS from the configured candidate LTM cells before the cell switch and can report CSI measurement results to the target cell via an UCI over PUSCH upon cell switch. This enables the target gNB to quickly know the DL channel conditions and schedule accordingly e.g., can schedule aggressively immediately post cell switch to ensure there is no throughput degradation and high throughput can be ensured if necessary.

Also, if we support early CSI acquisition for SCells, this can ensure that a UE in CA also leverages the benefits and doesn’t have to see throughput degradation in SCell post the cell switch.

Although early CSI acquisition of candidate cells ensures minimal throughput degradation, this comes at the expense of UE needing to measure several candidate cells and adds to the burden to UE processing/measurement complexity.

In 5G although the CSI is measured early, it is not reported early and can only be reported with the 1st UL Tx in the target cell. RAN2 can further study mechanisms for early CSI reporting as well (e.g., to the source cell which can forward it to the target cell via backhaul signaling).

For 6G, RAN2 should study how to support early CSI acquisition without high complexities added to the UE e.g., by enabling more selective early CSI acquisition towards the most relevant candidate cells, early CSI reporting etc.

In 5G, early CSI acqusition of LTM candidate cells enables UE to report CSI measurements to the target cell immediately post the LTM Cell switch to ensure minimal throughput degradation post the cell switch. Early CSI reporting of LTM candidate cells is not supported in 5G.

In order to minimize/avoid throughput degradation after mobility, 6G should support UE to perform efficient early CSI acquisition/reporting for the candidate cells (i.e., without adding much burden to UE’s measurement/processing capabilities).

## Measurement framework for 6G

### Requirements for 6G Measurements

We think several aspects of the 5G measurement framework can be incorporated in the 6G measurement framework.

For example, 5G measurement framework supports measurement reporting based on evaluation of beam quality (for L1 measurement reporting) and cell quality (for L3 measurement reporting). Both cell level measurements and beam level measurements can be reported by the UE.

UE can report SSB-based measurements and CSI-RS based measurements, for both intra-frequency and inter-frequency. Both gap-assisted measurements and gap-less (non-gap-assisted) measurements are supported. Both intra-RAT and inter-RAT measurements are supported.

There are different types of filtering supported in 5G (e.g., L1 filtering, L3 Beam filtering, L3 filtering for cell quality). Some types of filtering (e.g., L1 filtering) are up to UE implementation whereas other types of filtering (e.g., L3 Beam filtering and L3 filtering for cell quality) are based on RRC configured parameters.

Enhancements to cell quality derivation

In 5G, for L3 based measurement, the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are “linearly” averaged to derive the cell quality. Cell quality derivation is based on NW configured parameters such as nrofSS-BlocksToAverage and absThreshSS-BlocksConsolidation, which strictly dictates how UE should derive cell quality.

However, using a linear average and a single threshold for determining the good beams is not always optimal. A UE may prefer to use some specific beams based on other internal criteria. For example, based on UE location and mobility, some beams may be more optimal and may have more “weight”.

We therefore propose RAN2 to study more UE flexible mechanisms to perform beam consolidation, cell quality derivation and in determining the “good” beams.

6G Measurement framework should support cell/beam level measurements, measurements for serving/non-serving cells, different RS types (e.g,. wide/narrow beams), different purposes (e.g., mobility, RLM, BM), intra-RAT/inter-RAT, gap-assisted/non-gap-assisted, periodic/event triggered measurement reporting. RAN2 should study more flexible mechanisms (compared to NR) to perform cell quality derivation at the UE.

### Unified measurement framework

In original releases of NR (Rel-17 or earlier), L1 CSI measurements and L3 measurements had different objectives and therefore had an independent measurement resource/report framework.

L1 CSI measurements were used for obtaining the channel quality measurements of the serving cell for the purpose of DL data reception, BM, etc. L3 measurements were used for obtaining the channel quality measurements of the serving and neighbour cell for the purpose of mobility.

In 5G RRC, SSB and CSI-RS resource configurations were defined independently in two different places for L1 and L3 measurement resources, even though this resulted in some duplicate configurations in RRC signalling. For LTM, a new dedicated IE for LTM CSI report and resource was however specified in R18/R19 due to lack of neighbouring cell support in L1 CSI report framework.

To avoid such duplicate measurement resource configurations, we propose 6G to strive to define unified measurement resource configuration structure for different types of measurements (e.g., L1 CSI measurement, L3 measurement),

Further, we also propose RAN2 to study that a unified measurement report configuration structure can be supported for 6G, wherein both serving cell CSI measurement (e.g., L1 CSI measurement) and serving/neighboring cell measurement for mobility (e.g., L3 measurement) are defined under a single report configuration structure.

Such a unified measurement framework would enable a simplified RRC configuration that can be applicable to both L1 CSI measurement and L3 measurement. This can also avoid duplication of CSI-RS/SSB configurations across L1 and L3 measurements, as was the case in 5G. This can also enable a scalable and forward-compatible measurement configuration, e.g., if we have to support early neighbouring cell CSI measurements.

5G measurement framework is quite fragmented and supports different measurement resource and reporting framework for L1/L3 measurements, serving/neighboring cells, SSB vs. CSI-RS.

6G should strive to support a unified measurement configuration framework e.g., via unification of measurement resource configuration structure (for L1/L3 measurements, for serving/neighbor cells/beams, for different types of RS) and unification of measurement report configuration structure.

## Inter-RAT mobility between 6G and 5G

In RAN#109, an initial set of architecture and migration requirements was agreed in RP-252870, including the following requirement for inter-RAT mobility:

The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

Inter-RAT mobility between 5G and 4G was specified via UE obtaining an RRC message with the target RAT configuration. The UE performs RACH in the target cell belonging to a different RAT and then applies the target RAT’s configuration after the inter-RAT mobility is successfully completed. There were no enhancements specified in 5G to optimize the interruption time or robustness for inter-RAT mobility between 5G and 4G. For example, features that were later introduced for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) were never extended to inter-RAT mobility.

Several features supported for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) are not supported for inter-RAT mobility between 5G and 4G

Inter-RAT mobility between 4G and 5G was kept simple because 4G doesn’t support many of the mobility features which were introduced in NR (e.g., LTM, early UL/DL sync). Even features such as CHO were introduced for LTE quite late, only after introducing them in NR. So, inter-RAT mobility between 4G and 5G was left untouched. Also, several of the mobility features in NR were introduced over the releases and not on day-1, thereby the motivation to change inter-RAT mobility was not significant.

Unlike the above, 5G and 6G might end up sharing a lot of common mobility features (e.g., conditional mobility, RACH-less mobility). And 6G “unified” mobility may support most of the useful features from day-1 itself.

Hence, for the inter-RAT mobility between 6G and 5G, we propose RAN2 to study whether features that will be supported by intra-6G unified mobility framework (e.g., RACH-less inter-RAT mobility, inter-RAT conditional mobility) can be supported for inter-RAT mobility between 6G and 5G as well, thereby improving the robustness and interruption time for inter-RAT mobility as well. Hence, the following is proposed:

RAN2 to study whether Inter-RAT mobility between 6G and 5G can leverage the same features supported by intra-6G mobility framework e.g., early DL/UL sync, pre-configurations, conditional mobility, early CSI acquisition etc.

# Summary

Early DL/UL synchronization

Observation 1:	Early DL/UL sync in 5G enables shorter interruption times during LTM cell switch by source cell pro-actively obtaining the TCI state information and TA information of the LTM candidate cells, prior to the LTM cell switch.

Observation 2:	Network-triggered early UL sync procedure in 5G may not always be efficient as there may be repeated (e.g., due to TA becoming invalid) and unnecessary (e.g., due towards unnecessary candidate cells) early UL sync procedures. Also, efficient pooling of RACH resources for early UL sync is important for minimizing resource reservation.

Observation 3:	Candidate cell TCI activation/deactivation procedure (“early DL sync”) in 5G may result in additional measurement/processing overhead at the UE if not selectively done by the network.

Proposal 1:	6G mobility framework should support efficient early DL/UL sync procedures to minimize mobility interruption time without significant UE measurement/processing overhead and resource reservation issues at the network. RAN2 to study enhancements w.r.t. early UL/DL sync procedures supported in NR.

Pre-configuration of candidate cells

Observation 4:	Pre-configuration of candidate cells helps in reducing interruption time and increasing robustness via early RRC decoding at the UE, enables fast recovery in case of RLF/HOF and enables to have a lean HO command.

Observation 5:	The complexities w.r.t. reserving resources when multiple candidate cells are pre-configured can be addressed via smart network implementations e.g., by selecting the right set of candidate cells, reserving the resources for a suitable duration and using modify/cancel procedures as needed.

Proposal 2:	6G mobility framework should support pre-configuring a UE with one or more candidate cell configurations to minimize mobility interruption time (e.g., via early RRC decoding of the candidate cell configurations) and to enable fast recovery post RLF/HOF.

Conditional mobility

Observation 6:	Conditional mobility (e.g., CHO/CLTM in 5G) can help in lowering interruption time, increasing robustness and enables the usage of local knowledge at the UE for efficient mobility execution. There were some concerns that CHO implementations in 5G didn’t show the promised benefits because of source gNB not being able to perform efficient resource reservation and early data forwarding to candidate gNB(s).

Proposal 3:	6G mobility framework should support conditional mobility (i.e., cell switch execution by the UE based on the satisfaction of certain event triggers). For efficient data forwarding and resource reservation, RAN2 can further study potential enhancements (e.g., UE sending a “bye message” to source RAN node) to support conditional mobility in 6G.

Early CSI acquisition

Observation 7:	In 5G, early CSI acqusition of LTM candidate cells enables UE to report CSI measurements to the target cell immediately post the LTM Cell switch to ensure minimal throughput degradation post the cell switch. Early CSI reporting of LTM candidate cells is not supported in 5G.

Proposal 4:	In order to minimize/avoid throughput degradation after mobility, 6G should support UE to perform efficient early CSI acquisition/reporting for the candidate cells (i.e., without adding much burden to UE’s measurement/processing capabilities).

Measurement framework for 6G

Proposal 5:	6G Measurement framework should support cell/beam level measurements, measurements for serving/non-serving cells, different RS types (e.g,. wide/narrow beams), different purposes (e.g., mobility, RLM, BM), intra-RAT/inter-RAT, gap-assisted/non-gap-assisted, periodic/event triggered measurement reporting. RAN2 should study more flexible mechanisms (compared to NR) to perform cell quality derivation at the UE.

Observation 8:	5G measurement framework is quite fragmented and supports different measurement resource and reporting framework for L1/L3 measurements, serving/neighboring cells, SSB vs. CSI-RS.

Proposal 6:	6G should strive to support a unified measurement configuration framework e.g., via unification of measurement resource configuration structure (for L1/L3 measurements, for serving/neighbor cells/beams, for different types of RS) and unification of measurement report configuration structure.

Inter-RAT mobility between 6G and 5G

Observation 9:	Several features supported for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) are not supported for inter-RAT mobility between 5G and 4G

Proposal 7:	RAN2 to study whether Inter-RAT mobility between 6G and 5G can leverage the same features supported by intra-6G mobility framework e.g., early DL/UL sync, pre-configurations, conditional mobility, early CSI acquisition etc.