3GPP TSG-RAN WG2 Meeting #132	R2-2508362

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.6.2

Source:	Nokia

Title:	Miscellaneous Release 19 LTM corrections

WID/SID:	NR_Mob_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are describing few Release 19 LTM issues which in our opinion should be concluded before 3GPP freezes the specification. We propose corrections to CLTM and reiterate the issue with semi-persistent CSI-RS resources.

# 2	Discussion

## 2.1	PUCCH resources to be used by UE in case of overlapping SR resources

In RAN2#131bis the following was agreed MAC open issue regarding the PUCCH resource to be used by UE in case of overlap of priority among the SRs:

- (MAC-O02) The SR resource for (truncated) MR MAC CE has the same priority as the SR resource for BFR MAC CE. - (MAC-O02) If the SR resource for (truncated) MR MAC CE overlaps with SR resource for BFR MAC CE (for an SCell or a BFD-RS set), it’s up to UE implementation which SR resource to be selected. Update the Note in the current MAC specification.

Based on the above agreement on MAC-O02 from RAN2#131bis, the SR resource for L1 measurement report has the same priority as BFR MAC CE, which includes both cases, SR for SCell beam failure recovery, and SR for beam failure recovery of a BFD-RS set. The “NOTE 6” in clause 5.4.4 in TS 38.321 explains the UE behaviour in case of overlapping PUCCH resources for pending SR for SCell beam failure recovery, and SR for beam failure recovery of a BFD-RS set.

With the addition of the scenario of PUCCH resource overlap with the existing scenarios of overlapping, potentially there are three scenarios to be considered. The three scenarios are that of the existing SCell beam failure recovery and beam failure recovery of a BFD-RS set, and the new scenario for the L1 measurement report.

The existing NOTE 6, before the modifications done as part of R2-2507728 from R2#131bis meeting MAC CR for mobility covers the overlapping scenario of SCell beam failure recovery and beam failure recovery of a BFD-RS set. With the addition of L1 measurement reporting, the following scenarios are possible for overlap of PUCCH resource for pending SR.

Scenario 1: Overlapping PUCCH resource for pending SR for SCell beam failure recovery, PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, and PUCCH resource for pending SR for event triggered L1 measurement report. (all three possible scenarios)

Scenario 2: Overlapping PUCCH resource for pending SR for SCell beam failure recovery, and PUCCH resource for pending SR for beam failure recovery of a BFD-RS set. (Existing and already addressed scenario in TS 38.321, clause 5.4.4, NOTE 6)

Scenario 3: Overlapping PUCCH resource for pending SR for SCell beam failure recovery, and PUCCH resource for pending SR for event triggered L1 measurement report.

Scenario 4: Overlapping PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, and PUCCH resource for pending SR for event triggered L1 measurement report.

The proposed changes to “NOTE 6” as part of R2-2507728 from R2-131bis meeting addresses only the scenario 1 described above. The scenarios 2, 3, and 4 have not been addressed. It is also essential to specify the behaviour of the UE when there is an overlap of PUCCH resources for pending SR between any two of the three possible combinations.

Proposal 1: Clarify the behaviour of the UE when there is an overlap of PUCCH resources for pending SR between the different possible combinations. Adopt the Text Proposal provided in Appendix A.

## 2.2		L3 based C-LTM Corrections [RIL N142]

When two measurement ID are configured as execution condition for conditional mobility these measurement IDs should have same measurement object. The field description of execution condition for CHO include the restriction that both Measurement-ID should map to same Meas-object. However some configurations of the meas-ID can be different across these measurement ID. This information is captured as informative text for CHO as below in 5.3.5.13.4.

C-LTM also reuses the same configuration for L3 based C-LTM. This information note is also applicable for the meas-ID configured for C-LTM.

Observation 1: Information text related to 2 measurement-ID and its configuration for CHO is also applicable for C-LTM.

Proposal 2: Introduce informative text in section 5.3.5.18.8 on selection of candidate cell for execution similar to the one in CHO execution section.

In case of CHO, when the conditional event is evaluated based on L3 measurements it is possible that more than one candidate cell meets the execution condition. In such case how the UE select one of the candidate cell is left to UE implementation. However NOTE is added to provide example on the aspects the UE may consider for selection of candidate.  This situation is also applicable for C-LTM when the condition is met for more than one candidate.

Observation 2: For CHO execution the specification include information text on how the select the candidate cell for execution when more than one candidate cell meets the execution condition. This information text is also applicable for C-LTM.

Proposal 3: Introduce informative text in section 5.3.5.18.6 on selection of candidate cell for execution similar to the one in CHO execution section.

## 2.3	CLTM execution conditions [RIL N141]

In the current RRC specification, the serving cell execution conditions are defined as follows:

-- ASN1START -- TAG-LTM-CONFIG-START LTM-Config-r18 ::= SEQUENCE {[RIL]: O001, MOB,SLRelay ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} [RIL]: V400,MOB OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ..., [[ ltm-ServingCellNoSecurityChangeID-r19 LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need N ltm-ServingCellExecutionCondition-r19 SetupRelease {LTM-ExecutionConditionList-r19} [RIL]: H150, MOB OPTIONAL -- Need M ]] } -- TAG-LTM-CONFIG-STOP -- ASN1STOP

Moreover, procedurally, we capture that whenever the serving cell execution conditions are updated from the NW, UE restarts the evaluation for all the CLTM candidates:

1> if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup: 2> for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition: 3> if the UE is performing LTM cell switch conditions evaluation based on L1 measurements: 4> request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations; 3> if the UE is performing LTM cell switch conditions evaluation based on L3 measurements: 4> stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8; 3> if l3-Conditions is included within ltm-ServingCellExecutionCondition: 4> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ServingCellExecutionCondition; 3> else if l1-Conditions is included within ltm-ServingCellExecutionCondition: 4> request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition; 1> else (ltm-ServingCellExecutionCondition set to release): 2> release the ltm-ServingCellExecutionCondition.

Observation 3: Whenever the serving cell execution conditions are updated from the NW, UE restarts the evaluation for all the CLTM candidates (even the ones whose execution conditions were not modified).

Current specification text seems to mandate that whenever serving cell updates the CLTM execution conditions for the UE, UE restarts evaluation for all cells. For instance, assume that UE is doing evaluation for cells C1 – C5. Further assume that NW updates CLTM execution conditions for C1. Under the current specification text, UE would need to restart the evaluation for cells C2 – C5, even though NW didn’t change anything for them. Moreover, this would incur increased over the air signalling. Such approach of specifying the CLTM is different from other conditional mobility procedures such as CHO, S-CPAC, wherein delta signalling for updating the conditional execution conditions is supported.

Proposal 4: Similar to CHO and S-CPAC, delta signalling for update of CLTM execution conditions is supported, i.e., UE restarts the TTT only for the cells for which the CLTM execution conditions are modified. Adapt the TP in Appendix C into TS 38.331.

## 2.4 	On the deactivation of target cell’s SP CSI-RSs used for L1-RSRP measurements (MAC-N02)

In our TDoc submitted to RAN2#131bis (R2-2507462) we have underlined a potential discrepancy in the previous decisions on when the UE deactivates SP CSI-RS resources after the cell switch. However, this issue was not handled online in October 2025 as it was suggested RAN1 is still discussing the subject and might notify RAN2 via LS. This has not happened and is not likely to occur. Thus, we would like to reiterate the problem below and ask RAN2 to resolve it.

For the deactivation of SP CSI-RSs of candidate cells that were activated by the source cell, the following agreements were made:

- At RAN2#130: The UE deactivates SP CSI-RS resources of candidate cells (other than the target cell) after cell switch. FFS for the target cell. - At RAN2#131: The SP CSI-RS resources of the target cell are deactivated after CSI reporting in the first UL transmission. How this is captured in MAC is up to the MAC CR rapporteur.

The agreement text provided above appears to cover both the CSI-RSs used for L1-RSRP measurements and early CSI acquisition. However, it is important to note that RAN1 has specified two distinct features based on SP CSI-RSs:

- L1-RSRP measurements (RRM measurements):

  - These measurements are performed using one or two-port CSI-RSs (i.e., CSI-RSs used for beam management).

  - Measurements are reported to the serving cell to support LTM procedures (e.g., early DL/UL synchronization, cell switch, and/or target beam selection for cell switch).

  - Reporting is triggered by configurations provided by the serving cell (via CSI-MeasConfig of ServingCellConfig). Each candidate cell is associated with its own reporting configurations in its respective candidate cell configuration.

  - After a cell switch, the UE is expected to use measurement RSs associated with the reporting configuration included in the candidate cell configuration of the newly selected target cell.

  - The new serving cell may activate SP CSI-RS resources associated with its own reporting configurations.

  - There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

- Early CSI acquisition:

  - The UE performs measurements using multi-port CSI-RSs (configured separately from the CSI-RSs used for L1-RSRP measurements) and acquires CSI (e.g., CQI, PMI, RI, CRI).

  - Such measurements are directly reported to the target cell after reception of the cell switch command.

  - The LTM CSI report configurations for CSI acquisition for a candidate cell are configured separately in the respective LTM Candidate IE of each candidate cell.

  - For these measurements, the UE sends a CSI report to the target cell after receiving the cell switch command.

As per currently available RAN2 agreements, the deactivation of SP CSI-RSs associated with the target cell applies to CSI-RSs configured for CSI acquisition, since a CSI report, based on SP CSI-RSs activated by the previous source cell, is sent to the target cell. However, for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell, no report is sent to the target cell. Without further explicit clarification, this implies that SP CSI-RSs of the target cell used for L1-RSRP measurements may not be deactivated after a cell switch, leading to ambiguity.

Observation 4: There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

Observation 5: Without additional explicit clarification made in 3GPP, SP CSI-RSs for the target cell activated by the previous source cell for L1-RSRP measurement will not be deactivated, since the RAN2#131 agreement on deactivation (quoted above) was explicitly linked to the CSI reporting after the cell switch.

Proposal 5: Add a clarification that for L1-RSRP measurements, the UE deactivates SP CSI-RS resources of the target cell after cell switch.

# 3	Conclusion

This document has made the following observations:

Observation 1: Information text related to 2 measurement-ID and its configuration for CHO is also applicable for C-LTM.

Observation 2: For CHO execution the specification include information text on how the select the candidate cell for execution when more than one candidate cell meets the execution condition. This information text is also applicable for C-LTM.

Observation 3: Whenever the serving cell execution conditions are updated from the NW, UE restarts the evaluation for all the CLTM candidates (even the ones whose execution conditions were not modified).

Observation 4: There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

Observation 5: Without additional explicit clarification made in 3GPP, SP CSI-RSs for the target cell activated by the previous source cell for L1-RSRP measurement will not be deactivated, since the RAN2#131 agreement on deactivation (quoted above) was explicitly linked to the CSI reporting after the cell switch.

And proposed the following:

Proposal 1: Clarify the behaviour of the UE when there is an overlap of PUCCH resources for pending SR between the different possible combinations. Adopt the Text Proposal provided in Appendix A.

Proposal 2: Introduce informative text in section 5.3.5.18.8 on selection of candidate cell for execution similar to the one in CHO execution section.

Proposal 3: Introduce informative text in section 5.3.5.18.6 on selection of candidate cell for execution similar to the one in CHO execution section.

Proposal 4: Similar to CHO and S-CPAC, delta signalling for update of CLTM execution conditions is supported, i.e., UE restarts the TTT only for the cells for which the CLTM execution conditions are modified. Adapt the TP in Appendix C into TS 38.331.

Proposal 5: Add a clarification that for L1-RSRP measurements, the UE deactivates SP CSI-RS resources of the target cell after cell switch.

# Appendix A: TP for Proposals in 2.1

### 5.4.4	Scheduling Request

… (text omitted)

NOTE 6:	When the MAC entity has overlapping PUCCH resource for pending SR for SCell beam failure recovery,  PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, and PUCCH resource for pending SR for event triggered L1 measurement report for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set or PUCCH resource for event triggered L1 measurement report. When the MAC entity has overlapping PUCCH resource for pending SR for SCell beam failure recovery, and PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set. When the MAC entity has overlapping PUCCH resource for pending SR for SCell beam failure recovery, and PUCCH resource for pending SR for event triggered L1 measurement report, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for pending SR for event triggered L1 measurement report. When the MAC entity has overlapping PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, and PUCCH resource for pending SR for event triggered L1 measurement report, it is up to UE implementation to select PUCCH resource for pending SR for beam failure recovery of a BFD-RS set or PUCCH resource for pending SR for event triggered L1 measurement report.

# Appendix B: TP for Proposals in 2.2

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2>	if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

NOTE:	If more than one LTM candidate configuration has triggered this procedure, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution

##### 5.3.5.18.8	LTM cell switch conditions evalution based on L3 measurements

… (text omitted)

1>	if event(s) associated with all measId(s) for an ltm-CandidateId within the LTM-ExecutionConditionList IE are fulfilled:

2> inform lower layers that an event based on L3 measurements to perform an LTM cell switch procedure is fulfilled;

2>	perform the LTM cell switch procedure for the LTM candidate configuration associated to the ltm-CandidateId according to the actions specified in 5.3.5.18.6.

NOTE 1:	Up to 2 MeasId can be configured for each l3-conditions. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

#### 6.3.2	Radio resource control information elements–

#### LTM-ExecutionConditionList

The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions.

LTM-ExecutionConditionList information element

-- ASN1START

-- TAG-LTM-EXECUTIONCONDITIONLIST-START

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::=     SEQUENCE {

ltm-CandidateId-r19                LTM-CandidateId-r18,

executionCondition-r19             CHOICE {

l1-Conditions-r19                  LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                  SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

...

}

-- TAG-LTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

LTM-ExecutionConditionList field descriptions

l3-conditions If 2 meas-IDs are configured for L3 execution condition the network ensures that both refers to same measObject.

# Appendix C: TP for Proposals in 2.3

In 6.3.2 Radio resource control information elements:-- ASN1START -- TAG-LTM-CONFIG-START -- (Text ommited) ..., [[ ltm-ServingCellNoSecurityChangeID-r19 LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need N ltm-ServingCellExecutionConditionToAddModList-r19 ltm-ServingCellExecutionConditionToAddModList-r19 OPTIONAL, –-Need N ltm-ServingCellExecutionConditionToReleaseList-r19 ltm-ServingCellExecutionConditionToReleaseList-r19 OPTIONAL –-Need N ]] } ltm-ServingCellExecutionConditionToReleaseList-r19 ::= SEQUENCE (SIZE (1.. maxNrofLTM-Configs-r18)) OF Ltm-CandidateID-r18 -- TAG-LTM-CONFIG-STOP -- ASN1STOP

Define the ToAddModList as:

#### –	ltm-ServingCellExecutionConditionToAddModList

The IE ltm-ServingCellExecutionConditionToAddModList variable concerns a list of conditional LTM execution conditions to add or modify. For each entry, it contains the corresponding LTMCondExecutionConditionId and the associated fields.

ltm-ServingCellExecutionConditionToAddModList

-- ASN1START

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-START

ltm-ServingCellExecutionConditionToAddModList-r19 ::=     SEQUENCE (SIZE (1.. maxNrofLTM-Configs-r18)) OF ltm-ServingCellExecutionConditionToAddMod-r19

ltm-ServingCellExecutionConditionToAddMod-r19 ::=     SEQUENCE {

ltm-CandidateID-r19                   Ltm-CandidateID-r18,

executionCondition-r19             CHOICE {

l1-Conditions-r19                  LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                  SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                       OPTIONAL,   -- Need R

...

}

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

In 7.4 UE variables capture the following:

#### –	VarLTM-ExecutionConditionList

The UE variable VarLTM-ExecutionConditionList is used to store the LTM execution conditions for MCG LTM currently used by the UE.

VarLTM-ExecutionConditionList UE variable

-- ASN1START

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-START

VarLTM-ExecutionConditionList-r19 ::=     SEQUENCE {

ltm-ExecutionConditionList-r19            ServingCellExecutionConditionToAddModList-r19                         OPTIONAL

}

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

In 5.3.5.18.3 LTM candidate configuration addition/modification:

| ##### 5.3.5.18.1 LTM configuration […] 1> reconfigure the UE according to all other fields of the received LTM-Config IE; 1> if the received LTM-Config includes the field ltm-ServingCellExecutionConditionToReleaseList: 2> perform LTM cell switch execution conditions removal as specified in 5.3.5.18.1a 1> if the received LTM-Config includes the field ltm-ServingCellExecutionConditionToAddModList: 2> perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1b |  |  |
| --- | --- | --- |

Moreover, capture the following:

| ##### 5.3.5.18.1a LTM cell switch execution conditions removal The UE shall: 1> For each LTMCondExecutionConditionId value included in ltm-ServingCellExecutionConditionToReleaseList that is part of the current UE conditional LTM execution conditions in VarLTM-ExecutionConditionList: 2> request lower layers to stop any LTM cell switch conditions evaluation based on L1 measurement for the LTM candidate configurations; 2> stop LTM cell switch conditions evaluation based on L3 measurements for the configured LTM candidates as specified in 5.3.5.18.8; ##### 2> remove the entry with the matching LTMCondExecutionConditionId from the VarLTM-ExecutionConditionList; ##### 5.3.5.18.1b LTM cell switch execution conditions addition or modification For each LTMCondExecutionConditionId received in the ltm-ServingCellExecutionConditionToAddModList IE the UE shall: - if this procedure is triggered by LTM cell switch execution as specified in 5.3.5.18.6 and if ltm-ExecutionCondition is configured in the LTM-Candidate to which LTM cell switch is performed: 2> clear VarLTM-ExecutionConditionList; 2> request lower layers to stop any LTM cell switch conditions evaluation based on L1 measurement for the LTM candidate configurations; 2> stop LTM cell switch conditions evaluation based on L3 measurements for the configured LTM candidates as specified in 5.3.5.18.8; - else if an entry with a matching LTMCondExecutionConditionId exists in the ltm-ServingCellExecutionConditionToAddModList within the VarLTM-ExecutionConditionList; ##### 2> if the entry in ltm-ServingCellExecutionConditionToAddModList includes an ltm-ExecutionCondition; 3> Replace ltm-ExecutionCondition within the VarLTM-ExecutionConditionList with the value received for this LTMCondExecutionConditionId; 1> else: 2> Add a new entry for this LTMCondExecutionConditionId within the VarLTM-ExecutionConditionList; - Perform conditional LTM evaluation as specified in 5.3.5.18.1c; ##### 5.3.5.18.1c LTM cell switch execution conditions evaluation The UE shall: 1> For each LTMCondExecutionConditionId within the VarLTM-ExecutionConditionList; 2> if l3-Conditions is included in the LTM-ExecutionCondition; 3> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition; - else if l1-Conditions is included in the LTM-ExecutionCondition: 3> request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition. |  |  |
| --- | --- | --- |

In 5.3.5.18.6	LTM cell switch execution:

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

[...]

1>	perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a.