3GPP TSG RAN WG2 Meeting #132                                                           R2-2508365

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	9.2.2

Source:	Ofinno

Title:	RAN2 impacts to support D2T2 for DT and DO-DTT traffic

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 impacts to support D2T2 for DT/DO-DTT traffic as captured in Rel-20 WID [1].

Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- The following are not supported: Coordination btwn UEs operating as UE readers; New Uu protocol layer(s); Simultaneous (time overlapping) operation of Uu and A-IoT at intermediate UE in the same band

…

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

The following are the agreements from RAN2#131bis:

1.   gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.  gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3.  Reader schedules the AIoT interface transmissions within the resources allocated by the gNB

# Discussion

The following figure, from Rel-18 TR 38.848 [2], provides an overview of Topology 2 with intermediate node (i.e., UE Reader). Rel-20 A-IoT limited the scope to enable UE Reader operation when in connected mode.

It is also important to keep in mind that Rel-20 WID [1] clearly captures that the A-IoT radio resources (allocated by the gNB to the UE reader) are only valid within the serving cell and that time overlapping operation of the UE Reader over Uu and A-IoT would not be supported.

Topology 2 refers to a A-IoT device having bidirectional communication with a base station via an intermediate node (intermediate UE acting as a Reader, i.e. UE Reader) while in RRC_CONNECTED.

A-IoT radio resources (configured by the gNB to the UE reader) are only valid within the serving cell and there should not be time overlapping for the UE Reader when operating over Uu and A-IoT interfaces.

## A-IoT radio resources

TR 38.769 [3] captured the following on the A-IoT radio resources used to enable Topology 2:

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control. The radio resource validity across multiple cells is not supported in this release.

For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase):

-	Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface:

- Option 1:	UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or

- Option 2: 	UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

-	Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases):

- Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

- Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

NOTE 1:	The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.)

RAN2 should continue their Rel-19 work on identified impacts on this to enable Topology 2.

- To discuss whether the A-IoT radio resources (configured by the gNB to the UE reader) could be (re)configured via RRC reconfiguration (including for handover), resume and/or re-establishment.

- To discuss when/how the A-IoT radio resources become invalid or released, considering option 1) explicit network release, and/or option 2) implicit release (e.g., releasing after a period of time).

- To discuss whether/how to handle A-IoT radio resources when a UE reader is temporary out of coverage (e.g., RLF and handover), considering option 1) temporarily invalid and/or option 2) temporarily valid (i.e., valid for a period of time after being out of coverage and invalid afterwards).

TR 38.769 [3] also captured the following on how the network (BS) gets information of the A-IoT radio resources used to enable Topology 2:

For the radio resources allocation request, following alternatives are studied:

-	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

-	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

NOTE 2:	The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

Both alternatives 1 and 2 can co-exist as RAN2 concluded; if agreed, the enabling of alternative 2 should be driven by other WGs (SA2/RAN3) or leave it up to network implementation (assuming the required information is already known/visible to the gNB) while alternative 1 should be driven (studied) by RAN2. If RAN2 confirms the need for a UE reader to request A-IoT radio resources, RAN2 would need to discuss how this SR like may be done e.g., whether A-IoT specific/differentiation may be needed in the RB/LCH level.

- To allow a UE reader requests A-IoT radio resources to the gNB (including both new or update of stored configuration) and wait for SA2/RAN3 input on which A-IoT CN information is provided.

## Uu impact due to A-IoT operation

RAN2 needs to discuss how the A-IoT traffic is treated from the UE reader to the gNB. For example, which radio bearer are (or can be used) for A-IoT traffic, and how the UE reader maps the received A-IoT traffic into the Uu interface (e.g., should this be a 1:1, N:1 or N:M mapping).

- To discuss how the A-IoT traffic is handled over the Uu interface (e.g. which radio bearers are to be used and how the UE reader maps the received A-IoT traffic to be sent over the Uu interface).

## Parallel service and multi-reader

During Rel-19 A-IoT, RAN2 limited the operation to a single service/reader at the time considering the limited TUs and the target use case. As Rel-20 work is starting (and there is still time), we suggest for RAN2 to re-discuss aiming to define an A-IoT functionality which can be commercially competitive with other technologies used (e.g., RFID). It is also important to keep in mind that Rel-19 only focus in indoor / private network while Rel-20 broader the scope e.g. outdoor, active A-IoT devices.

- To discuss whether parallel service operation for one device is considered within Rel-20 scope.

- To discuss whether simultaneous/inter-leaved operation of multiple devices is considered within Rel-20 scope.

- To discuss whether multiple-reader operation is considered within Rel-20 scope (even though coordination between UE readers is not).

## Energy related Assistance Information

RAN2 should confirm from the beginning of the Rel-20 on the intention to enable a mechanism for A-IoT devices to provide energy related assistance information to reader/network considering that this had a large support during Rel-19 discussion and was not considered due to the lack of time.

Energy Status Indication

R2-2506048	Ambient IoT data transmission and energy status indication	MediaTek Inc., Vodafone, Nokia, Samsung, NTT DOCOMO INC., InterDigital, T-Mobile USA	discussion	Rel-19	Ambient_IoT_Solutions-Core	R2-2506046

Proposal 1: Specify a mechanism for the device to indicate to the reader if the device can complete a requested command procedure when considering its energy status and radio conditions (taking into account both R2D and D2R directions, and with no requirement on the reader to take any specific action on the indication).

- Can consider it for Rel-20

- To enable a mechanism for a Rel-20 A-IoT device to provide energy related assistance information to the network.

# Conclusion

The observations captured are the following:

Observation 1.	Topology 2 refers to a A-IoT device having bidirectional communication with a base station via an intermediate node (intermediate UE acting as a Reader, i.e. UE Reader) while in RRC_CONNECTED.

Observation 2.	A-IoT radio resources (configured by the gNB to the UE reader) are only valid within the serving cell and there should not be time overlapping for the UE Reader when operating over Uu and A-IoT interfaces.

The proposals captured are the following:

Proposal 1.	To discuss whether the A-IoT radio resources (configured by the gNB to the UE reader) could be (re)configured via RRC reconfiguration (including for handover), resume and/or re-establishment.

Proposal 2.	To discuss when/how the A-IoT radio resources become invalid or released, considering option 1) explicit network release, and/or option 2) implicit release (e.g., releasing after a period of time).

Proposal 3.	To discuss whether/how to handle A-IoT radio resources when a UE reader is temporary out of coverage (e.g., RLF and handover), considering option 1) temporarily invalid and/or option 2) temporarily valid (i.e., valid for a period of time after being out of coverage and invalid afterwards).

Proposal 4.	To allow a UE reader requests A-IoT radio resources to the gNB (including both new or update of stored configuration) and wait for SA2/RAN3 input on which A-IoT CN information is provided.

Proposal 5.	To discuss how the A-IoT traffic is handled over the Uu interface (e.g. which radio bearers are to be used and how the UE reader maps the received A-IoT traffic to be sent over the Uu interface).

Proposal 6.	To discuss whether parallel service operation for one device is considered within Rel-20 scope.

Proposal 7.	To discuss whether simultaneous/inter-leaved operation of multiple devices is considered within Rel-20 scope.

Proposal 8.	To discuss whether multiple-reader operation is considered within Rel-20 scope (even though coordination between UE readers is not).

Proposal 9.	To enable a mechanism for a Rel-20 A-IoT device to provide energy related assistance information to the network.

# Reference

- RP-251885, New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2, Rel-20, June 2025.

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Rel-18.

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Rel-19.

- SP-250834, SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2, Rel-20.

- 3GPP TR 23.700-30, .