# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508374
Dallas, USA, Nov. 17-21, 2025

Agenda item:	9.2.2

Source:	Apple

Title:	Discussion on Topology 2

WID/SID:	Ambient_IoT_Solutions_Ph2 – Release 20

Document for:	Discussion and Decision

# 1 Introduction

RAN2#131bis [1] has reached the following agreements for Topology 2 design,

RAN2#131bis agreements [1]: 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this paper, we continue to discuss the operation of the UE reader in Topology 2.

# 2 Discussion

## 2.2 Validity of A-IoT resource allocated to UE reader

During RAN2#131bis meeting [1], the validity issue has been briefly discussed and there was a debate on whether a timer is needed to indicate how long the resource of A-IoT air interface allocated by gNB is valid. This is related to the two options listed in TR 38.769 [2] which has been studied in Rel-19:

Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface:

-	Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or

-	Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

Note that for one-shot A-IoT resource allocation, even if supported, there is no validity issue. Our understanding is that the validity issue is mainly based on the assumption that gNB allocates semi-static resources: e.g., continues resources (contagious) and periodic resources (non-contagious) to a UE reader. While it would be rare for gNB to allocate continuous contagious resources all to the same UE reader, it may be possible if gNB only manages a single UE reader.

Anyway, we think NW has all the means to reconfigure the resource allocation if it wants ending a prior resource allocation (e.g., by release a resource set index in an RRC siganling) to an RRC_CONNECTED UE. So there seems no need to add a validity timer in this case. Therefore, we propose to adopt the Option 1.

Proposal 1 	For A-IoT semi-static resource allocation for RRC_CONNECTED UE reader, Option 1 (“UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly”) is used.

Then, regarding the resource validity issue for the “temporary out of connection scenarios”, there are also two options given from Rel-19 Study [2].

Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases):

-	Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

-	Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

For this scenario, we think it is logical to have a time constraint of the resource validity. Also, the UE reader in those cases may already in the border area of the cell or even moves out of coverage, so it may have difficulty to receive dedicated RRC message from the gNB. But it is not proper to allow the A-IoT resource to be valid for an indefinitely long time. Hence, the timer-based Option 2 is more appropriate.

Proposal 2 	For A-IoT resource allocation for temporary out of connection scenarios, Option 2 (“resources remain valid for a time period”) is used.

Given the above considerations, an RRC_CONNECTED UE reader may get the allocated resource associated with timer (Option 2) and/or resource allocated w/o timer (Option 1). Whether those resource can be configured in the same signaling can be further discussed later, but the Option 2 timer is supposed to be used only when temporary out-of-connection event occurs.

Proposal 3 	For A-IoT resource allocation with a timer-based validity for temporary out of connection scenarios, UE starts the timer only when Uu RLF is detected or HO command is received.

## 2.2 Resource Allocation Granularity

## The resource allocated by gNB is supposed to be only used by a single UE reader if it is given in a dedicated RRC signalling to this UE reader. It is NW responsibly to guarantee that the resource allocated to a UE reader is orthogonal to the resources allocated to other UE reader(s). What’s more, the resource can be considered as a general-purpose allocation, which means the UE reader can use it for communicate with any A-IoT device, for any A-IoT transactions. It would be too restrictive to have NW to allocate A-IOT resource to a UE reader for a certain transaction, or for a particular A-IoT device. If gNB provides every detail about how an A-IOT resource should be used for what purpose, then UE reader becomes simply a puppet of the “virtual gNB reader“ and has no autonomy at all. Such a design would make the UE reader more like a repeater and will incur a lot of Uu signaling overhead. Therefore, we propose to not over-optimize the Uu resource allocation procedure for finer granularity. Anyway, the gNB can always adjust the configured resource to UE reader at any time if it wants to increase/decrease the amount of resource allocated. We just do not see the benefits for NW to set unnecessary restrictions of resource usage, which would increase the complexity of UE reader.

Proposal 4 	gNB allocates dedicated A-IOT resource for UE reader to use, without providing finer granularity to limit the usage of A-IoT resource to a particular A-IoT transaction or a particular A-IoT device.

For R2D resource to be used by UE reader, Sidelink mode 1 RA can be simply reused for A-IoT enabled UE. For instance, DCI-based and RRC-based CG can be considered, and DCI-based dynamic grant can also be supported. Those allocated A-IoT grants are supposed only used by a particular UE reader, and not to be shared with other UE readers.

Proposal 5 	Sidelink Mode 1 resource allocation method can be used as a baseline to design NW-allocated resource control for R2D resource allocation to A-IoT enabled UE.

## 2.3 Assistance information from UE reader

In RAN2#131bis meeting, it has been agreed that

UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.

It is agreed that the resources used for A-IoT air interface are supposed to be configured by the serving gNB of UE reader. To make this configuration effective and accurate, some assistance information from UE reader needs to be shared with A-IoT enabled gNB. First, the UE reader need report the expected A-IoT traffic amount, or the corresponding A-IoT radio resource needed in both R2D and D2R directions to the gNB, so that the gNB can dimension the resource allocation correspondingly. Secondly, the UE reader also has measurements of the A-IoT air interface signals, which could be beneficial to gNB for resource allocation.

Proposal 6 	UE reader reports the A-IoT traffic amount (or corresponding resource amount) and A-IoT air interface measurements as assistance information to its serving gNB.

Then, regarding how the assistance information is sent from UE reader to its serving gNB, we think both RRC message and MAC signalling can be considered. For instance, BSR-like procedure may be needed for solicit A-IoT grants. This is because the A-IoT traffic initiated by a CN A-IoT function may be transparent to gNB and serving gNB is not aware of actual traffic situation in the UE reader, in general. This is similar to the Sidelink BSR design [4] where the grants of another radio interface are solicited by a BSR procedure.

Proposal 7	Both RRC signalling and MAC CEs can be considered to carry assistance information from UE reader to gNB.

# 3 Conclusion

In this contribution, we discuss the Topology 2 support for Ambient IoT, and have the following proposals:

Proposal 1 	For A-IoT semi-static resource allocation for RRC_CONNECTED UE reader, Option 1 (“UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly”) is used.

Proposal 2 	For A-IoT resource allocation for temporary out of connection scenarios, Option 2 (“resources remain valid for a time period”) is used.

Proposal 3 	For A-IoT resource allocation with a timer-based validity for temporary out of connection scenarios, UE starts the timer only when Uu RLF is detected or HO command is received.

Proposal 4 	gNB allocates dedicated A-IOT resource for UE reader to use, without providing finer granularity to limit the usage of A-IoT resource to a particular A-IoT transaction or a particular A-IoT device.

Proposal 5 	Sidelink Mode 1 resource allocation method can be used as a baseline to design NW-allocated resource control for R2D resource allocation to A-IoT enabled UE.

Proposal 6 	UE reader reports the A-IoT traffic amount (or corresponding resource amount) and A-IoT air interface measurements as assistance information to its serving gNB.

Proposal 7	Both RRC signalling and MAC CEs can be considered to carry assistance information from UE reader to gNB.

# 4 References

[1] Chairman’s Notes, R2 131bis meeting in Prague, Czech Republic

[2] TR38.769 v19.0.0 Study on solutions for Ambient IoT (Internet of Things) in NR