3GPP TSG-RAN WG2 Meeting #132	R2-2508381

Dallas, TX, USA, 17 – 21 November 2025

Agenda item:	10.3.1.1

Source:	Apple

Title:	Views on 6G User Plane: Functionalities and Processing

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

During RAN2 #131bis, we have reached the following agreements on 6G User Plane [1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This paper aims to provide our views on the which 5G NR functionalities for the L2 protocols should be supported by 6G user plane, and what enhancements or modifications could be introduced in 6G in order to make the user plane protocol more processing friendly, as agreed in RAN2 #131bis.

# Discussions

## Fundamental L2 User Plane Functionalities for 6GR

We have tabulated our views on which L2 functionalities of 5G/5G-A should be retained in 6GR:

Table 1 An analysis of 5G/5G-A L2 functionalities and their need for 6GR

| 5G NR L2 Functionality | 5G Protocol Layer | Need in 6GR | Comments |
| --- | --- | --- | --- |
| QoS flow-to-DRB Mapping | SDAP | Yes |  |
| Reflective QoS | SDAP | No | This has not been widely used in 5G, and better approach can be considered in 6G to adapt radio treatment for different QoS flows. |
| CP/UP Data Transfer | PDCP | Yes |  |
| Maintenance of sequence number | PDCP/RLC | Yes |  |
| Header Compression | PDCP | Yes |  |
| Data Compression | PDCP | Yes |  |
| Ciphering and Deciphering | PDCP | Yes |  |
| Integrity Protection/verification | PDCP | Yes |  |
| Packet Discarding | PDCP | Yes |  |
| Split Bearer and routing | PDCP | FFS | RAN2 needs to identify the need of splitting a radio bearer into multiple paths if DC is not supported in 6GR. |
| Duplication | PDCP | FFS | This has not been widely used in 5G, but RAN2 can study if this is essential for certain 6G use cases. |
| L2 ARQ | RLC (AM) | Yes |  |
| Unacknowledgement Mode | RLC (UM) | Yes |  |
| Random Access | MAC | Yes |  |
| TB (de-)Multiplexing/Assembly | MAC | Yes |  |
| HARQ | MAC | Yes |  |
| Radio Resource handling | MAC | Yes |  |
| MAC CE Handling | MAC | Yes |  |
| Scheduling information feedback: Buffer Status Report (BSR) | MAC | Yes |  |
| Scheduling information feedback: Delay Status Report (DSR) | MAC | FFS | The benefits of DSR have not been proven in the field, and calculation of delay reporting data volume is quite complex. |
| Logical Channel Priority handling | MAC | Yes |  |
| DRX | MAC | Yes |  |

As shown above, we think Reflective QoS could be replaced by some other more efficient mechanisms if there is a need to change DRB mapping for a QoS flow more dynamically. It is worth noting that, the Reflective QoS mechanism requires the UE to monitor each downlink packet to generate the corresponding mapping rule, which can be quite complicated for UE implementation. Furthermore, it is not widely deployed in the field. With this in mind, we think Reflective QoS should not be inherent in 6GR.

On the other hand, currently this is not affirmative whether we will continue to have dual-connectivity (DC) in 6GR, so we are not sure if split bearer should be supported. Having said that, even if DC is not supported eventually, there could be some other use cases where multiple legs are needed within one DRB (e.g. for mechanisms related to service-awareness). Thus, we think RAN2 can further study if split bearer architecture should be retained.

For PDCP duplication, we understood it was mainly introduced in 5G to support URLLC. Nevertheless, such feature was also not widely used in the field. However, considering that we have agreed 6GR should aim to reduce both radio and end-to-end latency, perhaps packet duplication is still a useful feature for many 6G use cases. We think RAN2 can further discuss if this should be considered as an essential 6G Day-1 feature.

The delay status reporting (DSR) has been introduced and enhanced in Rel-18/Rel-19 XR work items to enable delay-aware scheduling for UL traffic that would be discarded if not transmitted on time. While we acknowledge this is important to have more timely scheduling, we have also noted that the data volume calculation procedures for Rel-19 DSR can be quite complicated. Additionally, we think this is more crucial to achieve timely scheduling immediately upon arrival of the traffic, rather than waiting until it becomes “delay-critical” (i.e. when the remaining time of which falls below a threshold). Considering that DSR has only been specified in the last couple of releases, we think some more study is needed by RAN2 to confirm that DSR is a feature that we would like to have for 6G Day-1.

Proposal 1: 6GR does not need to support Reflective QoS. RAN2 can study other more efficient mechanisms to enable DRB remapping if there is a need.

Proposal 2: RAN2 can further study the need of split bearer architecture and packet re-routing.

Proposal 3: RAN2 can further study the need of PDCP duplication in 6GR.

Proposal 4: RAN2 can further study 6GR needs to support DSR from Day-1, or if more efficient mechanism should be supported to enable timely scheduling.

## Processing Overhead Minimization

During RAN2 #131bis, we have agreed that 6GR should be designed to be more hardware friendly. In our views, the processing overhead could be improved in the L2 protocol stack by:

- PDCP SDU concatenation

- L2 Header optimization

- Sequence number unification

### PDCP SDU Concatenation

Currently in 5G NR, the relationship between PDCP SDU and PDCP PDU is one-to-one mapping. For UL the UE requires to perform security handling and header addition to each of the incoming PDCP SDU, which is obviously inefficient from UE processing perspective, especially when many packets with small payload size arrive within a short period of time. This results in high header processing overhead, as header is added to each SDU at multiple L2 sublayers. Furthermore, the CPU load can surge when the UE needs to process many small packets at the same time. Thus, we think RAN2 should study the possibility of mapping multiple SDUs to one PDU via packet concatenation, which can significantly decrease the UE burden of processing. An illustration of the PDCP SDU concatenation is shown in Figure 1 (Note that this figure assumes all L2 protocol layers are still retained in 6GR, which may not be the case depending on the future discussions).

Figure 1 An illustration of PDCP SDU concatenation

Proposal 5: RAN2 should consider supporting PDCP SDU concatenation in 6GR to reduce processing and signalling overhead.

### L2 Header Optimization

On the other hand, 5G NR has provided flexibility of data structure through many different packet header formats and associated options, that are optimized for different requirements. It is questionable whether 6G should support so many different options of L2 headers, considering that packet header processing can be costly for implementation of both UE and network. If we assume 6GR can offer higher data rates (spectral efficiency), transmission of a few extra bits could become more affordable. With this in mind, it is beneficial to have a more unified header structure in 6G by reducing the options of e.g. sequence number (SN) length, segment offset in RLC, logical channel ID in MAC etc.

Proposal 6: RAN2 should study how L2 header can be optimized to streamline 6GR protocol processing.

### Sequence Number (SN) Unification

Currently, PDCP and RLC maintains different sequence number space, even though in many cases they are the same. We see some benefits of removing such redundancy. In particular, since PDCP SN and RLC SN may be added to PDCP header and RLC header respectively, this is inefficient as the two SN are added to the header. Therefore, we suggest that RAN2 can further study if 6GR can be optimized by only have one sequence number per packet that is commonly used across different protocol layers.

Proposal 7: RAN2 should study if sequence number (SN) for a packet could be unified across in PDCP and RLC layers.

# Conclusions

This contribution has examined the functionalities that are required in 6G user plane architecture and how the processing overhead can be optimized. We have the following proposals:

Proposal 1: 6GR does not need to support Reflective QoS. RAN2 can study other more efficient mechanisms to enable DRB remapping if there is a need.

Proposal 2: RAN2 can further study the need of split bearer architecture and packet re-routing.

Proposal 3: RAN2 can further study the need of PDCP duplication in 6GR.

Proposal 4: RAN2 can further study 6GR needs to support DSR from Day-1, or if more efficient mechanism should be supported to enable timely scheduling.

Proposal 5: RAN2 should consider supporting PDCP SDU concatenation in 6GR to reduce processing and signalling overhead.

Proposal 6: RAN2 should study how L2 header can be optimized to streamline 6GR protocol processing.

Proposal 7: RAN2 should study if sequence number (SN) for a packet could be unified across in PDCP and RLC layers.

# References

[1] RAN2 #131bis Chair’s Notes, October 2025.