3GPP TSG-RAN WG2#132											   R2-2508391

Dallas, USA, Nov. 17th - 21st

Agenda Item:	9.2.2

Source: 	NEC

Title:	        Discussion on Topology 2 for Ambient IoT

Document for:	 Discussion

# Introduction

RAN2#131bis meeting achieved following agreements:

1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signalling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. 3. The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

# Discussion

## 2.1 Applicability of Rel-19 TP1 AIoT radio interface solution to Rel-20 TP2

In Rel-19, the AS protocol stack for AIoT radio interface between the AIoT device and the Base Station AIoT reader contains AIoT MAC layer and AIoT physical layer as shown below in Figure-1. The AS layer control information and data are handled by AIoT MAC layer and AIoT physical layer. For AIoT radio interface, there is no differentiation between the control plane and the user plane.

In Rel-20, Ambient IoT radio interface is between the AIoT device and an intermediate UE reader.

Figure-1: AS protocol stack for AIoT

Rel-19 AS protocol stack over Ambient-IoT radio interface applies to Rel-20 Ambient-IoT radio interface between the AIoT device and an intermediate UE reader.

## 2.2 UE Reader Selection

WID clearly states that only UE in RRC_CONNECTED state is available for AIoT services. Based on that, RAN2 need to discuss whether RRC_ INACTIVE/IDLE UE can be chosen as the UE reader. If the RRC_ INACTIVE/IDLE UE can be chosen as the UE reader, a paging procedure may be specified to wake up the UE to enter RRC_CONNECTED state.

RAN2 to clarify whether RRC_ INACTIVE/IDLE UE can be chosen as the UE reader, or only RRC_CONNECTED state UE can be chosen as the UE reader.

In Rel-19, AIoTF leads the reader selection. However, radio-related factors may need gNB to participate in UE Reader selection, allowing gNB to filter candidates that match AIoT service requirements. UE Capability or other radio characteristics may be used for UE Reader Selection: UEs able to be readers must report capabilities to gNB during access. For some selection scenarios, they also need to provide e.g., Uu link quality, mobility state to assist gNB evaluation.

If gNB is involved in UE reader selection, consider UE capability and some radio characteristics, e.g., Uu link quality, mobility state, to assist gNB’s evaluation.

## 2.3 Resource allocation to UE Reader

Topology 2 adopts an RRC-based architecture, which requires gNB to manage the resources of UE readers through dedicated RRC signaling. The "per UE reader" resource granularity enables precise binding resources to individual UE readers. This allows gNBs to adjust resource parameters according to the specific needs of each UE reader, while avoiding the management collision that would arise from shared resources.

Furthermore, UE readers are often mobile, and their locations may change. In such cases, shared resources may cause interference due to overlapping coverage. By contrast, exclusive resources allocated to each UE reader can inherently isolate their transmissions, ensuring reliable AIoT communication without the need for additional coordination mechanisms.

Taking “per UE reader” as the baseline of resource allocation granularity.

Sharing resource across UE Readers is not supported in Rel-20, and resource overlap between different UE Readers must be avoided through gNB scheduling.

RAN2#131-bis meeting agreed:

The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB.

In our understanding, such assistance information related to AIoT transmission is provided from the UE reader to the gNB as the AIoT procedure progresses at the UE reader, with the main motivation being to request AIoT resource allocation adjustment. Therefore, the discussion of assistance information is closely linked to resource validation, which needs to be further discussed.

Following the Rel-19 principle, A-IoT resources are defined in both the time and frequency domains. In the time domain, there are mainly three alternatives as described below:

- Alternative 1: Periodic A-IoT resource allocation (similar to CG): for example, resources can be defined or configured with parameters such as periodicity, duration, and start time offset.

- Alternative 2: Continuous A-IoT resource allocation with explicit start and release indications controlled by the gNB.

- Alternative 3: One-shot A-IoT resource allocation with explicit start indication controlled by the gNB and an optional duration.

Since the gNB may not have sufficient knowledge to configure the periodicity and duration of the A-IoT resource allocation, and the A-IoT operation is expected to be completed following the initiation of resource allocation by the gNB, with the actual completion time depending on the UE reader and the number of A-IoT devices within its coverage, we believe that at least Alternative 2 needs to be supported in Rel-20.  Furthermore, Alternative 2 is more effective in ensuring the timing coordination between the reader and device, resulting in a smaller impact on the R19 A-IoT procedure.

Support continuous A-IoT resource allocation with explicit start and release indication controlled by the gNB in Rel-20.

If Proposal 6 is agreed upon, the assistance information related to A-IoT resource adjustment in the time domain would be the A-IoT operation completion indication sent from the UE reader to the gNB, which would be helpful for the gNB to determine when to release the resource allocation. On the other hand, assistance information for frequency resource adjustment would also be beneficial. This is because frequency resources are primarily used for A-IoT D2R transmission, and only the UE reader is aware of the results of the A-IoT D2R transmission.

RAN2 to confirm that the A-IoT operation completion indication and assistance information regarding frequency resource adjustment from the UE reader to the gNB would be beneficial.

## 2.4 AIoT service request forwarding over Uu

RAN2#131-bis meeting agreed that:

gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information.

In Rel-19, NGAP AIoT information received from CN are mainly included in Inventory Request Transfer IE (included in NVENTORY REQUEST Message) and Command Request Transfer IE (included in COMMAND REQUEST Message). Details of these two IEs are shown below:

- Inventory Request Transfer IE: provides the inventory request related information from the AIOTF to the NG-RAN node.

    - A-IoT Correlation Identifier: uniquely identifies an A-IoT session within an AIOTF.

    - A-IoT Device Identification Requested: represents the Requested Identity of the A-IoT device(s), e.g., single device, group device or all devices

    - Requested Service Area Information: includes the Requested Service Area Information for the A-IoT inventory, e.g., requested reader list, requested AIoT area list

    - Inventory Assistance Information: includes the assistance information for the A-IoT Inventory Service, e.g., expected D2R Message Size, approximate Number of Target A-IoT Devices, etc.

    - Follow on Command Indication: indicates that there is command(s) to be transmitted.

- Command Request Transfer: provides the command request related information from the AIOTF to the NG-RAN node.

  - A-IoT Correlation Identifier: the same as above

  - RAN A-IoT Device NGAP ID: uniquely identifies the A-IoT Device over the NG interface within the NG-RAN node within a A-IoT Session

  - A-IoT NAS PDU

  - Command Assistance Information: includes the assistance information for the A-IoT Command Service, e.g., estimate of Expected D2R Message Size

For the UE reader to properly and accurately generate AIoT MAC messages, at least the following information from the CN needs to be forwarded:

For inventory services: A-IoT Device Identification Requested, inventory assistance information, follow on command indication;

For command services: A-IoT NAS PDU, Command Assistance Information.

RAN2 to confirm that at least following information from the CN needs to be forwarded:
For inventory services: A-IoT Device Identification Requested, inventory assistance information, follow on command indication;
For command services: A-IoT NAS PDU, Command Assistance Information.

Regarding the remaining information from the CN, we further discuss each item individually:

A-IoT Correlation Identifier: Following the Rel-19 principle, an identifier may be needed to uniquely identify an A-IoT session within a gNB/AIOTF. However, it is still unclear whether the A-IoT correlation identifier received from the CN can be reused for this purpose. An identifier assigned or notified by the gNB to the corresponding UE reader could also be effective. Therefore, we suggest confirming the need for an identifier that uniquely identifies an A-IoT session within a gNB/AIOTF, and further discussing the details of this identifier, such as whether to reuse the A-IoT correlation identifier received from the CN or to allow the gNB to generate the identifier.

RAN2 to confirm that to differentiate A-IoT sessions, an identifier uniquely identifies an A-IoT session within a gNB/AIOTF is needed for the immediate UE reader. FFS details of the identifier.

Requested Service Area Information: If reader selection is performed correctly by the CN and/or gNB, there does not seem to be a strong motivation to forward the requested service area information to the UE reader from the RAN2 perspective. Since this information is mainly being discussed by RAN3, we suggest revisiting this topic after RAN3 has made further progress.

RAN A-IoT Device NGAP ID: The RAN A-IoT Device NGAP ID is allocated by NG-RAN to uniquely identify an NG-RAN node's A-IoT device context over the NG interface within the NG-RAN node for a specific A-IoT session. For devices using follow-on command services, the RAN A-IoT Device NGAP ID is transmitted from the NG-RAN to the AIOTF via the Inventory Report Transfer IE. When the AIOTF receives a RAN A-IoT Device NGAP ID, it stores the identifier for the duration of the A-IoT session. If the AIOTF requests a command service for a given device, the RAN A-IoT Device NGAP ID is sent from the AIOTF to the NG-RAN to identify the device.

In Rel-19, the identifier used to identify a specific device over the A-IoT interface is the AS ID, which is also allocated by the gNB reader: in MSG 2 for CBRA, and in the first R2D upper-layer data transfer message for CFA. Therefore, Rel-19 gNB readers are aware of the association between the RAN A-IoT Device NGAP ID and the AS ID, and there are no issues.

However, for topology 2, if the following assumptions hold:
(1) The AS ID is allocated by the UE reader; and
(2) The RAN A-IoT Device NGAP ID is allocated by an NG-RAN node,
then the method of associating the AS ID and the RAN A-IoT Device NGAP ID becomes a potential issue, since these identifiers are allocated at different nodes, and so far, no information exchange is supported between the two nodes.

Whether the RAN A-IoT Device NGAP ID needs to be forwarded to UE readers depends on how the AS ID and the RAN A-IoT Device NGAP ID are associated. For example, if the UE reader is responsible for associating the AS ID and RAN A-IoT Device NGAP ID, then the RAN A-IoT Device NGAP ID may need to be forwarded to the UE readers. Alternatively, if the NG-RAN node is responsible for associating the AS ID and RAN A-IoT Device NGAP ID, then the AS ID allocated at the UE reader needs to be reported to the NG-RAN node in a suitable manner.

Whether the RAN A-IoT Device NGAP-ID is needed to be forwarded to the UE readers depends on how to associate the AS ID and the RAN A-IoT Device NGAP-ID.

RAN2 to further discuss which node (either NG-RAN node or UE reader) to associate the AS ID and the RAN A-IoT Device NGAP-ID and maintain the association.

## 2.5 Data forwarding over Uu for AIoT Device

In R19, the NG-RAN may perform AIoT service operation result aggregation with a specific Correlation ID, based on the aggregation assistance information received from the AIOTF for a service operation request [1].

For Topology 2, two issues arise:

- When D2R data segmentation is performed, delivering these data segments over the Uu interface may result in frequent signaling exchanges, which could be highly inefficient. One solution is that, if data segmentation occurs for D2R traffic of an AIoT device, the intermediate UE may assemble all data segments belonging to the same upper-layer data transfer for the Ambient IoT device.

- For AIoT service operations involving multiple AIoT devices, there is the issue of how the UE reader should forward A-IoT D2R/R2D data. In such cases, the intermediate UE may receive AIoT service operation results from different Ambient IoT devices and perform aggregation of these results.

RAN2 to discuss whether the UE reader supports data aggregation for the same Ambient IoT device when data segmentation occurs for D2R traffic of the AIoT device.

RAN2 to discuss whether the UE reader supports data aggregation for different Ambient IoT devices.

# Conclusion

Observation 1:	Whether the RAN A-IoT Device NGAP-ID is needed to be forwarded to the UE readers depends on how to associate the AS ID and the RAN A-IoT Device NGAP-ID.

Proposal 1:	Rel-19 AS protocol stack over Ambient-IoT radio interface applies to Rel-20 Ambient-IoT radio interface between the AIoT device and an intermediate UE reader.

Proposal 2:	RAN2 to clarify whether RRC_ INACTIVE/IDLE UE can be chosen as the UE reader, or only RRC_CONNECTED state UE can be chosen as the UE reader.

Proposal 3:	If gNB is involved in UE reader selection, consider UE capability and some radio characteristics, e.g., Uu link quality, mobility state, to assist gNB’s evaluation.

Proposal 4:	Taking “per UE reader” as the baseline of resource allocation granularity.

Proposal 5:	Sharing resource across UE Readers is not supported in Rel-20, and resource overlap between different UE Readers must be avoided through gNB scheduling.

Proposal 6:	Support continuous A-IoT resource allocation with explicit start and release indication controlled by the gNB in Rel-20.

Proposal 7:	RAN2 to confirm that the A-IoT operation completion indication and assistance information regarding frequency resource adjustment from the UE reader to the gNB would be beneficial.

Proposal 8:	RAN2 to confirm that at least following information from the CN needs to be forwarded: For inventory services: A-IoT Device Identification Requested, inventory assistance information, follow on command indication; For command services: A-IoT NAS PDU, Command Assistance Information.

Proposal 9:	RAN2 to confirm that to differentiate A-IoT sessions, an identifier uniquely identifies an A-IoT session within a gNB/AIOTF is needed for the immediate UE reader. FFS details of the identifier.

Proposal 10:	RAN2 to further discuss which node (either NG-RAN node or UE reader) to associate the AS ID and the RAN A-IoT Device NGAP-ID and maintain the association.

Proposal 11:	RAN2 to discuss whether the UE reader supports data aggregation for the same Ambient IoT device when data segmentation occurs for D2R traffic of the AIoT device.

Proposal 12:	RAN2 to discuss whether the UE reader supports data aggregation for different Ambient IoT devices.

# References

[1] 3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things.