3GPP TSG-RAN WG2 Meeting #132	R2-2508392

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.4

Source:		LG Electronics Inc.

Title:		Consideration on mobility aspects for 6G

Document for:	Discussion

1.	Introduction

In the provious meeting RAN2#131bis, the followings were agreed:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this paper, we present our view on 6GR connected mobility, RLM/RLF, idle/inactive mobility, and NTN mobility.

2.	Discussion

Mobility in RRC_CONNECTED

Design principle of 6GR mobility franework

Various mobility optimization funtionalities have been introduced in 5G, such as provision of candidate configurations, L1/L3 measurement and reporting, fast/dynamic cell switch command, RACH-less cell switch, subsequent cell switch, conditional cell switch, L2 reset skipping, early CSI aquisition. Some functionalities are introduced for reducing mobility latency (e.g., RACH-less cell switch), and some are introduced for improve mobility robustness (e.g., conditional cell switch).

However, the use cases of the funtionalities are limited due to the diverged mobility schemes (e.g., L3HO, LTM, CHO). For example, RACH-less L3 HO is only applicable for the case where the UE uses the same timing advance value (i.e., NTA value) at the target cell as in the source cell or timing advance value (i.e., NTA value) of 0. Early synchronization procedure, L2 reset skipping, and lower layer cell switch command are applicable for LTM but not for L3HO. Subsequent cell switch is only available for LTM and SN mobility (i.e., CPAC). These fragemented uses of mobility funtionalities may result in less promising gain.

Observation 1.	Each of NR mobility functionalities (e.g., RACH-less cell switch) were introduced with its own benefits. But, the eventual mobility performance optimization gain seems less promising with the fragmented optimizations for the diverged mobility schemes.

In 6GR, mobility with low latency/interruption and high robutness will remain crucial to support, e.g., immersive services during mobility. Hence, it is desired that mobility optimization funtionalities should be supported from 6G day-1 and all the funtionalities should be applicable for the mobility core function taken as a basis for 6G mobility framework to be supported.

LTM introduced in 5G Rel-18 has been receiving a big attention thanks to its low mobility interruption/latency. LTM is further enhanced in Rel-19 and Rel-20. LTM is designed to work well in CU-DU split architecture from a network architecture perspective. In our view, this functional split in network architecture can be supported in 6G because of its economic benefits. In addition, NR LTM supports most mobility functionalities described above. Therefore, NR LTM could be a good starting point for designing 6GR mobility framework. In Rel-20 NR mobility WI, it will be specified the following functionalities: 1) LTM SCell activation improvements; and 2) dynamic L1 measurement and reporting configuration change, where those functionalities can also be considered for 6GR mobility framework.

Proposal 1.	6GR mobility framework supports all NR mobility functionalities from 6G day-1. NR LTM including Rel-20 NR mobility enhancements can be a starting point.

In the rest of this section, we provide the detailed view on unified measurement and reporting framework, unified cell switch execution, and further study on mobility functionalities, where those constitute 6GR mobility framework.

Unified measurement and reporting framework

In terms of RRM procedure, LTM taken as baseline for 6GR mobility implies that the measurement reporting procedure would be MAC-centric, as opposed to RRC-centric legacy L3HO. In LTM, measurement report and cell switch command are transmitted as MAC Control Element (MAC CE). While the MAC-centric mobility procedure in 5G LTM is beneficial in reducing processing and transmission delay, it has a drawback of less flexible nature of MAC CEs, compared to a full flexibility offered by RRC. Considering that measurement report procedure serves various purposes such as serving cell management for CA and mobility and network performance optimization (SONMDT), etc., less flexibility of MAC CE based measurement reporting procedure may restrict applicability of the measurement reporting. Then, if a unified RRM procedure is pursued in 6G to support both beam-level measurement reporting and cell-level measurement reporting, the appropriate approach between MAC-centric approach and RRC-centric approach should be studied.

Further, in 5G, the reference signals pool for RRC-centric measurement reporting (i.e., L3 MR) and that for MAC-centric measurement reporting (i.e., L2 MR) are separately provided to the UE. Similarly, the configurations for RRC-centric and MAC-centric measurement reporting are also separately provided. The separate configuration for RRC-centric and MAC-centric approaches can result in signaling redundancy because the same reference signals can be provided for both RRC-centric and MAC-centric measurement reporting. In addition, redundant measurement reporting for a same cell triggered by both MAC-centric and RRC-centric approaches may happen. Therefore, single signaling structure for both MAC-centric and RRC-centric measurement reporting should be studied.

Observation 2.	The separate configuration for L3 MR and L2 MR can result in signaling redundancy because the same reference signals can be provided for L3 MR and L2 MR.

Basically, beam-level measurements are fast, less averaged measurements providing quick feedback for beam management and LTM, while cell-level measurements use a longer-term view of channel conditions with L3 filtering to smooth fluctuations, preventing unnecessary handovers and reducing overhead for RRC signaling-based mobility events. From a mobility perspective, beam-level measurement provides a better radio quality during/after mobility execution but may lead to frequent mobility (i.e., ping-pong), as opposed to cell-level measurement based mobility. In our view, mixture of beam-level and cell-level measurement can improve mobility performance. For example, joint beam-level and cell-level measurements not only prevent frequent measurement reporting but also provide robust results compared to beam-level measurement based measurement reporting.

Observation 3.	Joint beam-level and cell-level measurement can improve mobility performance by leveraging the strengths of each approach.

In 5G NR, the number of L1 measurement reference signals (RS) configured for a UE has increased to enable beam-level cell changes via LTM, consequently increasing the UE’s L1 measurement overhead. To address this issue, NR Rel-20 will introduce a network-controlled L1 measurement activation/deactivation technique, with standardization scheduled to commence at the next meeting. We believe that the L1 measurement activation/deactivation technique provides energy efficiency for both the UE and the network by reducing the measurement burden on the UE and the measurement RS transmission burden on the network. So, it would be good to introduce the L1 measurement activation/deactivation technique starting from 6G day-1.

However, the L1 measurement activation/deactivation technique under consideration in NR Rel-20 is based on network-based control scenarios and traditional NR measuremen/reporting framework. For 6G mobility, the following aspects may require further consideration, e.g.: 1) UE-based L1 measurement activation/deactivation; 2) smart L1 measurement activation/deactivation based on new 6GR measurement framework considering both beam-level and cell-level measurement; and so on. UE-based L1 measurement activation/deactivation not only reduces Uu signaling for L1 measurement activation/deactivation but also enables robust L1 measurement activation/deactivation by allowing the UE to perform L1 measurement activation/deactivation based on highly dynamic beam-level measurement results. Furthermore, it allows the UE or network to determine the appropriate activation/deactivation timing by simultaneously considering both beam-level and cell-level measurement results.

Observation 4.	For L1 measurement activation/deactivation in 6GR mobility, the followings can be further studied for robust and smart activation/deactivation: 1) UE-based L1 measurement activation/deactivation; 2) smart L1 measurement activation/deactivation based on new 6GR measurement framework considering both beam-level and cell-level measurement; etc.

Based on Observations 2, 3, and 4, we propose:

Proposal 2.	Study unified measurement and reporting framework including:

- single RRM configuration for reducing signaling redundancy;

- RRM procedure supporting beam-level, cell-level, and joint beam- and cell-level measurements;

- minimizing energy consumption of UE and NW by measurement/reporting overhead reduction (e.g., dynamic (de)activation of measurement/reporting, UE-based (de)activation of measurement/reporting).

Unified cell switch execution

There are various cell switch commands (CSC) for triggering 5G NR cell switch execution such as L3 CSC (i.e., reconfiguration with sync), L2 CSC (i.e., LTM CSC MAC CE), and UE-based cell switch execution (i.e., conditional mobility), where the signaling structure for each CSC is defined independent on the signaling structure for the other CSCs. For example, L3 CSC is included in the target/candidate configuration, while L2 CSC is separately transmitted after providing the candidate configuration. Such fragmented cell switch execution procedure may result in inefficiency of UE and network behaviours.

For example, CSCs provided at the L2 and L3 layers respectively can cause race conditions. Assume that an L3 CSC may be triggered first, but due to the network structure, an L2 CSC triggered later may be delivered to the UE first. In this case, the UE may perform a cell switch based on the L2 CSC, causing the L3 CSC to be lost and not delivered to the UE. Then, the gNB may retransmit the L3 CSC, but the target configuration associated with the L3 CSC may be outdated due to e.g. cell group change from L2 triggered cell switch. Because of invalid configuration, finally, the UE initiates RRC connection re-establishment procedure. This can eventually result in significant interruption.

Observation 5.	Fragmented cell switch execution procedure may result in significant interruption due to e.g. RRC connection re-establishment followed by racing between L2 cell switch command (CSC) and L3 CSC.

In NR, CHO was introduced to improve mobility robustness, but CHO has not yet been widely commercialized. The primary reason is that the network has to reserve radio resources for handover for the UE, which does not know when handover execution will occur. As the number of CHO candidate cells increases, the network burden for radio resource reservation grows. In addition, the inability to fully control the UE after providing CHO configurations may be viewed as problematic from the operator’s perspective. Above all, in NR networks mainly deployed with FR1, the robustness of the conventional L3HO may not be significantly compromised.

Observation 6.	Network burden to reserve candidate radio resource for conditional mobility and network’s inability to fully control of UE mobility are obstacles to NR CHO commercialization.

Despite these drawbacks, CHO may offer advantages in high-frequency communication environments such as FR2 or FR3. Since FR2 or FR3 wireless channels can exhibit more abrupt changes in radio performance compared to FR1 wireless channels, conventional MR-based handover may fail to guarantee mobility robustness. Of course, since FR2 has not been primarily deployed in 5G NR, there may be questions about the necessity of such a high-frequency communication environment for 6GR. Based on 6GR SID [1], sensing is one of the important topics of 6GR study, i.e., especially in terms of integration with communication services. In academic fields, research on integrated sensing and communication (ISAC) systems utilizing reconfigurable intelligent surface (RIS) is actively underway. According to one expert, RISs are most attractive in mmWave bands [2]. RIS technology aligns well with the characteristics and challenges of mmWave communications. Its ability to redirect signals, enhance beamforming, and extend coverage in a cost-effective manner makes it a promising candidate for future mmWave deployments—especially as 6G networks evolve. Hence, high robust mobility procedure will be essential to support 6GR sensing systems.

Observation 7.	High robust mobility procedure (e.g., conditional cell switch) will be essential to support 6G sensing systems.

To address the issues (i.e., Observation 5) coming from fragmented cell switch execution procedure, we think it is necessary to introduce an unified cell switch execution procedure for 6GR mobility. For example, a single CSC for all cell switch execution types (e.g., LTM, L3HO, and CHO) can be used. In our view, NR LTM CSC MAC CE can be a baseline for 6GR CSC. Further, RAN2 can also study the followings for 6GR conditional cell switch to tackle the problem described in Observation 6, e.g.: 1) how to minimize radio resource waste due to reservation for candidate configurations; 2) controllable conditional mobility for the operator; and so on. For example, if a dynamic CSC indicating conditional mobility is used, the network can start reserving the radio resources for conditional mobility at an appropriate time through signaling between the source node and the target node. By doing this, the time required for radio resource reservation for conditional cell switch can be minimized, thereby reducing radio resource waste.

Concludingly, we propose:

Proposal 3.	Study unified cell switch execution procedure including:

- single cell switch command for all cell switch execution types (i.e., RACH-less or RACH-based, conditional or non-conditional);

- minimizing radio resource waste due to candidate radio resource reservation for conditional cell switch (e.g., dynamic control of conditional evaluation);

- how to provide network controllability of conditional cell switch

Further study on mobility functionalities

In 5G NR, candidate configurations can be provided as a single reference configuration and one or more candidate cell configurations for a specific mobility (e.g., LTM). The reference configuration is generated by the CU based on the information provided by the candidate CUs and the candidate DUs associated with LTM candidate cells. Since the complete configuration of a candidate cell is generated by applying the candidate cell configuration on top of the reference configuration, each candidate cell configuration does not need to include the information included in the reference configuration. This implies the generation of reference configuration may require a complex implementation, e.g., multiple handshaking between the CUs and the DUs to optimize the reference configuration and candidate cell configurations. If a single reference configuration is forced to 6GR mobility, such a complicated way of generating reference configuration could become another obstacle to market adoption. Further, it is likely that the commonality of candidate cell configurations between different CUs is very small because of, e.g., different capabilities of CUs involved in inter-CU LTM configuration. Consequently, the gain in signaling efficiency achieved by introducing reference configuration may be reduced. Based on above observations, multiple reference configurations can be introduced to enjoy the benefit in signaling efficiency from using reference configuration while avoiding complicated network signaling.

Observation 8.	Single reference based candidate configuration could make issues such as

- too complicated joint signaling for the entire CU and DU involved in cell switch preparation may be required; and

- marginal gain is expected due to less commonality of candidate configurations between different CUs.

As described before, various mobility optimization functionalities have been introduced in 5G, but the use cases of the functionalities are limited due to the diverged mobility schemes. In order to maximize the gain of mobility functionalities in 6GR, we think it is essential to expand the use cases of mobility functionalities such as RACH-less cell switch, early UL/DL synchronization, L2 reset skipping, etc.

In NR LTM, only configured grant (CG) based RACH-less CLTM is specified. Regarding dynamic grant (DG) based RACH-less CLTM, there was strong preference among multiple companies to reduce network burden and waste caused by radio resource reservation for conditional mobility. However, it was excluded from the standardization scope due to limitations in the NR procedures at the time.

Since CG-based conditional cell switch requires pre-allocating the CG for the UE in each candidate cell, CG-based RACH-less conditional cell switch may cause operational burdens and radio resource waste in the network. To resolve the issue, 6GR mobility should support DG-based RACH-less conditional cell switch in addition to CG-based one. In recent NR mobility discussions, during RACH-less cell switch, the UE can transmit a scheduling request (SR) before the cell switch completion because it has the target cell’s uplink synchronization information (i.e., TA). Based on the SR transmission during cell switch, the network can start allocating CG for the UE or the network can indicate DG to the UE, which can result in efficient radio resource utilization. We believe now there is no blocking issue to introduce DG-based RACH-less cell switch for 6GR mobility.

Observation 9.	DG-based RACH-less cell switch can alleviate the network burden caused by radio resource reservation for CG-based RACH-less cell switch.

For early synchronization in 5G NR, RA-based TA acqusition and UE-based TA measurement are specified for uplink synchronization, and candidate TCI state activation is introduced for downlink synchronization.

In RA-based TA acquisition, the network may trigger UE to perform random access to a candidate cell. If UE performs random access to a candidate cell, network obtains the TA value for the UE toward the candidate cell and may informs the TA value to the UE to trigger RACH-less mobility to the candidate cell. For a case where UE acquires the uplink timing of multiple candidate cells via RA-based TA acquisition, UE should sequentially transmit the corresponding RA preamble to each candidate cell. Since UE needs to tune its radio to the candidate cell, the UE cannot communicate with the serving cell during RA based TA acquisition, which can result in serving cell interruption. As the number of candidate cells that need to obtain TA increases, serving cell interruption increases. Further, from the UE perspective, transmitting RA preamble multiple times to multiple candidate cells increases power consumption and reduces energy efficiency.

In UE-based TA measurement, the identifier indicating UE-based TA measurement included in a candidate cell configuration is a single integer. This means that the serving cell and all the candidate cells are grouped orthogonally for UE-based TA measurement. Orthogonal grouping of serving/candidate cells for UE-based TA measurement reduces the opportunity for UE-based TA measurement, which may increase the frequency of use of RA-based TA acquisition. Consequently, network overhead and UE interruption due to RA-based TA aquisition may increase.

Based on above, we think it is crucial to study energy efficient early UL synchronization for 6GR mobility. Energy efficiency can be achieved by, e.g., expanding the opportunity of UE-based TA measurement, acquiring multiple candidate TAs through minimal RA preamble transmission, and so on.

Observation 10.	In NR, RA-based TA acquisition can result in significant UE energy consumption when RA-based TA acquisition is frequently triggered by network. Furthermore, the lack of opportunities of UE-based TA measurement may trigger RA-based TA acquisition more frequently, potentially worsening UE energy consumption.

In NR LTM, skipping RLC/PDCP re-establishment was introduced to provide service continuity by reducing mobility interruption due to L2 reset. In addition, HARQ continuation was also discussed to further reduce interruption, but it was excluded from the scope of NR LTM. This is because there was no critical use case (e.g., time sensitive application) for further reduction of mobility interruption, and consequently it was concluded that it was sufficient for service continuity to skip RLC/PDCP re-establishment at that time (i.e., Rel-18). For 6GR, however, immersive service such as XR is a key use case that requires robust service continuity. In our view, service continuity can be improved by further reducing L2 layer interruption during cell switch, e.g., HARQ continuation.

Observation 11.	Service continuity can be improved by further reducing L2 layer interruption during cell switch.

In conclusion, we propose:

Proposal 4.	Study mobility functionalities with the following aspects:

- minimizing signaling redundancy of candidate configurations (e.g., multiple reference configurations)

- minimizing UE overhead due to early TA acquisition (e.g., expanding opportunities of UE-based TA measurements, multi-TA acquisition based on minimal UL transmission).

- further reducing L2 layer interruption during cell switch (e.g., HARQ continuation);

RLM/RLF

In NR, when a PCell experiences a Radio Link Failure in single cell or carrier aggregation scenarios, the UE shall perform RRC re-establishment to recover the RRC connection, which leads to significant service interruption. In dual connectivity, however, the UE can report a PCell failure via the secondary cell group, enabling recovery without RRC re-establishment. This mechanism reduces service interruption and improves overall reliability.

Observation 12.	Significant service interruption can occur due to RRC re-establishment based recovery from PCell failure in NR without dual connectivity.

For 6G, we propose extending this principle to carrier aggregation scenario where dual connectivity is not configured. Specifically, when a PCell RLF occurs, the UE could use an SCell to notify the network of the failure and receive recovery command without performing RRC re-establishment. To support this, the reliability of the SCell must be ensured, for example by applying the radio link monitoring on the SCell, similar to the monitoring of SCG link in NR dual connectivity. With this mechanism, the network can rely on the SCell-based failure report and provide reliable recovery commands.

This enhancement would enable fast PCell failure recovery in carrier aggregation scenarios, minimize service interruption compared to conventional RRC re-establishment, and improve user experience with higher reliability in multi-carrier deployments.

Proposal 5	Study PCell RLF recovery without interruption, e.g. without RRC re-establishment, in 6G carrier aggregation where the UE can utilize SCell to report PCell RLF and perform RLM.

Mobility in RRC_IDLE/INACTIVE

In LTE and NR, basic cell reselection is strictly based on frequency priorities and radio quality. UE needs to select a cell on a highest ranked cell on a frequency of a highest priority. This reselection rule is simple but has several limitations. For example, UEs are concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion. Dedicated priority may relax such concentration somehow, but its applicability for RRC_IDLE/INACTIVE is limited due to necessity of dedicated configuration. To distribute UEs across frequencies of multiple frequencies, a simple distribution mechanism can be incorporated into cell reselection rule to be developed in 6G. MCLD (multi-carrier load distribution) introduced in LTE can be considered as a starting point. Another limitation arises in a scenario when different cells/frequencies may offer different set of services (or slices) and UE’s preferred services (or slices) are different across UEs. To better address this scenario, NR introduces a slice-aware reselection. In our view, the slice-aware reselection is very complicated and testability of this feature is extremely difficult, which makes the feature hardly deployable in the real fields. In 6G, we should seek for a better reselection, which must be less complex but fully functional.

Observation 13.	UEs tend to be concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion.

Observation 14.	Slice-aware cell reselection is too complex and its testability is bad.

Based on Observations 13 and 14, we propose:

Proposal 6	Study enhanced cell reselection for network to enable camping cell distribution efficiently and to support network slicing in a simpler but fully functional manner.

NTN mobility

In NTN scenario, the latency for the HO procedure could be huge due to the nature of the very large propagation delay. To be specific, when the UE receives a HO command from the serving cell, communication through the serving cell becomes impossible, and communication through the target cell can start after the connection setup procedure for the target cell is completed. Therefore, in NTN scenario, the very large propagation delay will cause huge service interruption time until the HO procedure is completed for both HO between TN and NTN and between NTN and NTN compared to TN scenario. To mitigate this inefficiency, in 5G NR, the conditional HO and RACH-less HO are introduced for the HO enhancement. Moreover, since the RSRP measurement results could be similar within a cell area in NTN scenario, the location-based handover and cell managements were introduced for NR NTN. In 6GR, the advanced HO mechanisms need to be considered from the early stage of 6GR.

Meanwhile, the location-based mechanism (e.g., location-based CHO/SMTC) introduced for NR NTN basically assumes that all the UEs knows the valid UE location (based on GNSS). However, in 6GR NTN, this assumption would be no longer valid to extend the applicable UE types for the NTN operation. Considering that the RSRP measurement-based operation would be still inefficient or infeasible in NTN scenario, some enhancement would be needed to cover the 6GR GNSS-less UE. In 6GR, the advanced radio measurement-independent mechanisms for the GNSS-less UE need to be considered from the early stage of 6GR.

In NR NTN, due to the satellite moving, even for the earth-fixed cell, the UE needs to perform the satellite switching for the same cell ID. In this case, the UE can receive SS/PBCH from the target satellite for the DL resynchronization and the UL resynchronization can be done based on the UE location and the location of the target satellite. However, in 6GR, in case of the GNSS-less UE, it would be necessary that the UEs perform PRACH transmission to the target satellite for the UL resynchronization during the satellite switching. In this case, huge number of UEs may need to transmit PRACHs to the target satellite. In case of earth-moving cell, a number of (RRC CONNECTED) UEs may need to performs the HO to the target cell associated with the target satellite. If the UE has the valid UE location information, RACH-less HO can be used, but the UL transmission for the HO confirmation could be excessive. If the UE has no valid UE location information, a number of UEs needs to transmit PRACH. It may cause the NW burden and increase satellite switching/HO failure and/or latency, especially when the bandwidth is narrow.

For the successful commercialization of the 6GR NTN, it is necessary to consider the enhanced throughput and reliability. Meanwhile, due to the satellite moving, the service interruption due to the satellite switching or HO will frequently occur. To be specific, until the completion of the satellite switching or HO, the ongoing service provision will be halt. Depending on the target scenario (e.g., voice call or emergency alert), the service continuity would be critical, so this kind of the service interruption need to be avoided. Alternatively, it can be considered that the GEO-based NTN operation is used, but it does not fit with the real-time services due to the huge propagation delay. In the real field, a number of satellite services consider the hybrid approach with a combination of GEO and LEO. To be specific, the GEO satellite is used for the service continuity and the LEO satellite is used for services that require relatively low latency and/or high throughput. In 6GR, it would be necessary to consider the multi-orbit scenario. In a similar approach, it can be considered that the multi-connection between TN and NTN. For the multi-connection between NTNs of different orbits or between TN and NTN, the relevant management mechanisms need to be carefully designed considering the NTN characteristics. For the enhanced throughput compared to 5G NR NTN, the multi-carrier operation (for both intra-satellite and inter-satellite) can be further considered in NTN scenario. According to the NTN characteristics, the large propagation delay, the limited EIRP at the satellite, and the limited number of active beams would need to be considered for the efficient multi-carrier management.

Observation 15.	Pain points of NR NTN mobility are significant service interruption, high-latency HO, simultaneous HO of large numbers of UEs, limited throughput/reliability and increased UE implementation complexity due to satellite mobility, long propagation delay, low link budget, and UE location info acquisition based on non-3GPP technology.

Proposal 7	Study mobility mechanisms that consider NTN-related aspects such as a) seamless TN-NTN/NTN-NTN mobility with low latency, b) GNSS-independent UE mobility management, c) mobility handling for large numbers of UEs, d) single/multi-cell operation based on multiple satellites (in different bands/orbits), e) multi-connection of TN and NTN.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Observation 1.	Each of NR mobility functionalities (e.g., RACH-less cell switch) were introduced with its own benefits. But, the eventual mobility performance optimization gain seems less promising with the fragmented optimizations for the diverged mobility schemes.

Observation 2.	The separate configuration for L3 MR and L2 MR can result in signaling redundancy because the same reference signals can be provided for L3 MR and L2 MR.

Observation 3.	Joint beam-level and cell-level measurement can improve mobility performance by leveraging the strengths of each approach.

Observation 4.	For L1 measurement activation/deactivation in 6GR mobility, the followings can be further studied for robust and smart activation/deactivation: 1) UE-based L1 measurement activation/deactivation; 2) smart L1 measurement activation/deactivation based on new 6GR measurement framework considering both beam-level and cell-level measurement; etc.

Observation 5.	Fragmented cell switch execution procedure may result in significant interruption due to e.g. RRC connection re-establishment followed by racing between L2 cell switch command (CSC) and L3 CSC.

Observation 6.	Network burden to reserve candidate radio resource for conditional mobility and network’s inability to fully control of UE mobility are obstacles to NR CHO commercialization.

Observation 7.	High robust mobility procedure (e.g., conditional cell switch) will be essential to support 6G sensing systems.

Observation 8.	Single reference based candidate configuration could make issues such as

- too complicated joint signaling for the entire CU and DU involved in cell switch preparation may be required; and

- marginal gain is expected due to less commonality of candidate configurations between different CUs.

Observation 9.	DG-based RACH-less cell switch can alleviate the network burden caused by radio resource reservation for CG-based RACH-less cell switch.

Observation 10.	In NR, RA-based TA acquisition can result in significant UE energy consumption when RA-based TA acquisition is frequently triggered by network. Furthermore, the lack of opportunities of UE-based TA measurement may trigger RA-based TA acquisition more frequently, potentially worsening UE energy consumption.

Observation 11.	Service continuity can be improved by further reducing L2 layer interruption during cell switch.

Observation 12.	Significant service interruption can occur due to RRC re-establishment based recovery from PCell failure in NR without dual connectivity.

Observation 13.	UEs tend to be concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion.

Observation 14.	Slice-aware cell reselection is too complex and its testability is bad.

Observation 15.	Pain points of NR NTN mobility are significant service interruption, high-latency HO, simultaneous HO of large numbers of UEs, limited throughput/reliability and increased UE implementation complexity due to satellite mobility, long propagation delay, low link budget, and UE location info acquisition based on non-3GPP technology.

Proposal 1.	6GR mobility framework supports all NR mobility functionalities from 6G day-1. NR LTM including Rel-20 NR mobility enhancements can be a starting point.

Proposal 2.	Study unified measurement and reporting framework including:

- single RRM configuration for reducing signaling redundancy;

- RRM procedure supporting beam-level, cell-level, and joint beam- and cell-level measurements;

- minimizing energy consumption of UE and NW by measurement/reporting overhead reduction (e.g., dynamic (de)activation of measurement/reporting, UE-based (de)activation of measurement/reporting).

Proposal 3.	Study unified cell switch execution procedure including:

- single cell switch command for all cell switch execution types (i.e., RACH-less or RACH-based, conditional or non-conditional);

- minimizing radio resource waste due to candidate radio resource reservation for conditional cell switch (e.g., dynamic control of conditional evaluation);

- how to provide network controllability of conditional cell switch

Proposal 4.	Study mobility functionalities with the following aspects:

- minimizing signaling redundancy of candidate configurations (e.g., multiple reference configurations)

- minimizing UE overhead due to early TA acquisition (e.g., expanding opportunities of UE-based TA measurements, multi-TA acquisition based on minimal UL transmission).

- further reducing L2 layer interruption during cell switch (e.g., HARQ continuation);

Proposal 5	Study PCell RLF recovery without interruption, e.g. without RRC re-establishment, in 6G carrier aggregation where the UE can utilize SCell to report PCell RLF and perform RLM.

Proposal 6	Study enhanced cell reselection for network to enable camping cell distribution efficiently and to support network slicing in a simpler but fully functional manner.

Proposal 7	Study mobility mechanisms that consider NTN-related aspects such as a) seamless TN-NTN/NTN-NTN mobility with low latency, b) GNSS-independent UE mobility management, c) mobility handling for large numbers of UEs, d) single/multi-cell operation based on multiple satellites (in different bands/orbits), e) multi-connection of TN and NTN.

Reference

[1]	RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	Reconfigurable Intelligent Surfaces are Most Attractive in mmWave Bands, Link: https://ma-mimo.ellintech.se/2025/10/30/reconfigurable-intelligent-surfaces-are-most-attractive-in-mmwave-bands/#respond