3GPP TSG-RAN WG2 Meeting #132                                                                  R2-2508406

Dallas, US, 17th-21st November 2025

Title: 	Consideration on modular RRC and RRC signalling improvements

Source: 	ZTE Corporation, Sanechips

Agenda item:	10.3.2.2

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, companies discussed the general aspects of RRC protocol design, regarding how to improve the RRC structure, RAN2 has made the following agreement.

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

In this document, we discussed the potential solutions for modular RRC design and our suggestions. Moreover, we also discussed the RRC signalling issues that we observed in 5G.

- Discussion

  - Modular RRC

    - Background

In 5G NR, the signalling structure of RRCReconfiguration message is similar to that of 4G, adopting a tree-like nested structure. In addition to the original two-level hierarchy of CellGroup and Cell, it further introduces two more levels: Carrier and BWP, resulting in an extremely complex structure of information elements. The typical structure of the 5G RRCReconfiguration message is shown in Figure 1.

Figure 1 Simplified structure of RRCReconfiguration in 5G

Regarding the NR RRC signalling structure, the issues raised by companies mainly include:

- Issue 1: Heavy tree-like structure: To reduce the signalling size during handover or resource reconfiguration, RRC reconfiguration signalling supports delta configuration (Optional, Need M). However, due to the tree-like hierarchical structure, even if the network intends to modify only one or two physical channel parameters, it might still need to carry the entire tree structure. The mandatory fields and their associated fields at each level contribute to the final increase in RRC signalling size. In addition, from network implementation point of view, a detailed analysis is required for whether and how to apply delta configuration to each IE, which significantly increases the implementation complexity. Furthermore, the IODT testing shows that, due to mismatch between UE and network implementations, delta configuration has higher probability of causing inter-operability issues (e.g. RRC re-establishment) in the field.

- Issue 2: Too many linkages between parameters. In 5G structure, there are numerous interdependencies exist among IEs. For instance, while CSI-MeasConfig is anchored at the Cell level root, the CSI-MeasConfig contains CSI-ReportConfig and the PUCCH resources in its CSI-ReportConfig need to be associated with the BWP-ID and the PUCCH resource ID within that BWP. Consequently, when the BWP configuration is updated or the PUCCH configuration under a BWP is modified, the corresponding CSI-MeasConfig also requires modification.

- Issue 3: Inflexible delta configuration during handover. During handover, to assist the target cell to generate delta configuration, the source cell can send the UE context to the target cell, However, the target cell may not always support all the functions or features that configured by the source. Sometimes, the target cell even cannot decode some IEs, so it is impossible for the target cell to switch-off those functions, because many fields are defined as SetupRelease structure and releasing the IE still requires signal explicit “release” indication. As a consequence, the target cell will be forced to trigger fullConfig which adversely affecting handover performance.

- Issue 4: One-size-fits-all design. Various device types are introduced in the later releases of 5G, for different UE types (e.g. RedCap, NCR-MT), their newly introduced specific parameters are often scattered across various branches (e.g., under PRACH, PUSCH, PDSCH). This results in the need for specific type of UE (e.g. RedCap) to still support the encoding, decoding, and storage of the entire RRC signalling structure during UE implementation, increasing the implementation complexity, especially for low-end devices.

Among above issues, based on our observation of 5G implementation, issue 1 and 2 are the most serious and require focus on resolution.

Observation 1	The 5G RRC signalling structure include the following main problems:

- Issue 1: Heavy tree-like structure which significantly increasing the complexity of delta configuration;

- Issue 2: Too many linkages between parameters which increasing the complexity of configuration modification;

- Issue 3: Inflexible delta configuration during handover procedure;

- Issue 4: One-size-fit-all design which increasing the implementation complexity of low-end devices.

    - Potential solutions for modular RRC

In this section, we provide three alternative solutions and give our analysis of the solutions:

- Option 1: Define separate signalling structures for the key verticals.

In this option, we directly define separate ASN.1 structure for RRC reconfiguration message for different key verticals, then, for a specific device type, it only needs to implement its own RRCReconfiguration structure, so that the implementation complexity can be reduced.

Figure 2 	Conceptual diagram of modular RRC Option 1

As we can see, this option only addresses the issue 4 in 2.1.1. From network implementation point of view, the network has to handle different kinds of devices. therefore, the network implementation complexity will be even higher than 5G. On the other hand, although this option seems to minimize the implementation effort of low-end device, for chipset vendor who wants to support multiple device types, the implementation effort will also be high because the code cannot be shared between different device types.

Observation 2	Define separate RRCReconfiguration structures for different device types leads to higher network implementation complexity.

More importantly, according to the SID [1], one design principle for 6G is scalable design for diverse device types.

  - Target scalable and forward compatible design for diverse device types.

With option 1, it is possible that the vendors that supporting eMBB will not implement IoT due to the non-scalable design, this may increase the risk of market fragmentation in future. So, when discussing the modular RRC design in RAN2, it is important to keep “scalable design” in mind, and make sure the design should not cause market fragmentation in future.

Proposal 1	The design of modular RRC must ensure that it does not cause market fragmentation.

- Option 2: Basic RRC module + feature/vertical based modules

In this Option, the RRC parameters that related to basic operation will be categorized into a basic RRC module. In addition, different feature or vertical cases can form different separate modules. The conceptual diagram is shown in below figure:

Figure 3 	Conceptual diagram of modular RRC Option 2

For Option 2, we observe the following potential problems or risks:

With this framework, the content of basic RRC module must be precise, because the parameters of basic RRC module are expected to be applicable to all different features or verticals. However, similar to 5G, we often introduce special device types in later releases, such as NCR/IAB, they are not like normal consumer devices, posing a risk that the parameters of basic RRC module may not be compatible with newly defined features or device types in the future. This could lead to poor scalability of the overall architecture.

Observation 3	It’s hard to define a precise basic RRC module, as special devices (e.g. NCR/IAB) introduced in later releases will be quite different from normal devices.

When new features or device types are introduced, new L1 parameters are often added simultaneously. Since L1 parameters are likely to be associated with BWPs or specific PHY channels (e.g. PUCCH, PUSCH), so, when defining new feature/vertical based module, how to indicate the associated BWP IDs or PHY channels? If we still have to reflects a hierarchical structure of cells, BWPs and channel info in module, it would further increase the size of the module configuration, which against the original desire.

Observation 4	In most cases, feature/vertical specific parameters need to be associated with specific BWP or physical channels, introducing the cell ID or BWP ID or Channel structure in module increases the size of module.

For each newly defined feature or device type, there should be a clear guideline for whether to introduce an independent module. For example, if feature#1 adds 50 specific parameters, while feature#2 only adds 5 specific parameters, is it worthwhile to define a separate module for feature#2?

When introducing new features or device types, the newly defined parameters may potentially be referenced by other modules in future, leading to parameter duplication across different modules. However, considering that different features or device types may be enabled simultaneously (e.g. eMBB+XR). For the same parameter that present in two modules, how to signal them to avoid ambiguity? This may increase the complexity unexpectedly.

Observation 5	It is hard to avoid zero-overlapping parameters between feature/vertical based modules. In this case, complexity/ambiguity appears when enabling multiple features/verticals simultaneously.

Moreover, since vertical specific parameters will be defined as separate RRC module, there is risk on market fragmentation, because companies may mainly focus on eMBB and RedCap device types, for features that introduced for special vertical (i.e. IoT device) may never be implemented. Or vice versa.

Observation 6	There is risk that vertical-based RRC module design may also lead to market fragmentation.

- Option 3: Function based RRC module.

In this option, we define RRC module based on functions, such as security control, measurement, mobility, radio bearer config, L1 configuration...etc. For the module which has a large number of parameters (e.g. L1 configuration), we can further define a set of sub-modules.

Figure 4 Conceptual diagram of modular RRC Option 3

Different from Option 2, the RRC module is defined based on different functions, so, it is clear that for a given feature, the corresponding parameter will only be defined in one RRC module. Inter-module association should be avoided or minimized at much as possible. In addition, when triggering RRC reconfiguration, the network can only include the RRC module for which the network wants to change the configuration, for other RRC module which are not signalled, the UE assumes the previous received configuration is still valid.

Proposal 2	To study function based RRC module design in 6GR and follow the below design principles:

- Unique Parameter Ownership: Each parameter must be owned by a single RRC module, no duplicated parameter in different RRC modules.

- Minimized Coupling: Avoid or minimize the association between different RRC modules. Each RRC module is self-contained.

- Granular configuration: Support one RRC message containing only a subset of RRC modules, for RRC modules that were configured but not signalled, the UE maintains the previous configuration;

In our assessment, ensuring RRC module independence will be a key challenge when defining the 2nd level modules for L1 configuration, we should consult RAN1 and RAN4 to explore the most suitable division.

Proposal 3	RAN2 should work with RAN1 and RAN4 to define the RRC modules for L1 related parameters, aiming to explore the optimal division that avoid or minimize the association between modules.

Moreover, with modular RRC design, we need to discuss the working procedure and the potential impact on our spec. For example, if L1 related parameters are formed into one big RRC module, whether it is up to RAN1 to define the ASN.1 structure and capture it in RAN1 spec, in RAN2 RRC spec we can just refer to their parent IEs? In our view, no matter how RRC modules are defined, the Uu RRC signalling design should always be determined by RAN2 experts and captured in one RRC spec. In the RRC spec, we can use separate sections to describe the ASN.1 structure of RRC modules.

Proposal 4	RAN2 is the leading group for defining and maintaining the ASN.1 structure for RRC modules. Different RRC modules can be described in separate sections of a single RRC spec.

  - Delta signalling enhancement

In addition to modular RRC, in this section, we discussed other aspects that related to delta configuration.

In 5G RRC, we use “Optional+Need M” to indicate the support of delta configuration. However, in different cases, the requirements for delta configuration are different, for example:

- Requirement 1: for non-handover case (i.e. intra-cell configuration change), the serving cell knows all the current configuration, so, it makes sense to only signal the intended modified parameters to the UE, for all other parameters that are not included in the RRC message, the UE is expected to maintain the configuration.

- Requirement 2: For handover case, the RRC message is generated by the target cell, considering the target cell may not support all the features that enabled by the source cell. It would be better if there is measure that allows the UE to release a feature when it is not signalled.

So, for a given parameter, the requirement may various based on which scenario is (e.g. handover or not).

Observation 7	The requirements for delta signalling may differ in different scenarios (e.g. in non-handover case, absent = maintain; in handover case, absent = release).

In 5G, the definition of Need code is fixed, for 6G, we should study a new type of Need Code which fits the different requirements in different scenarios.

Proposal 5	In 6GR, to study the Need Code that fits the different delta signalling requirements in different scenarios (handover or not).

  - Painful ToAddModList

ToAddModList and ToReleaseList structures are widely used in 5G RRC ASN.1, however, with the evolution of RRC spec, many lists have been extended because of one or two of following reasons:

- Reason 1: Increased number of entries;

- Reason 2: Newly introduced parameter added to each entry.

For reason 1, we usually define a new list named “ToAddModListSizeExt”, and the UE should concatenate the old list and new list into one. In this case, the entry of the new list refers to the same IE of the old list. However, sometimes, we define that old list can also release the entry that established by the new list, but sometimes not. This brings a lot of complexities to implementation.

For reason 2, we will define a new list named “ToAddModListExt”, and the new list only contain the newly introduced parameter(s). To make it work. The network must configure the same number of entries for the old list and the new list. Due to lack of ID/Index in the new list, when the network wants to release the new parameter, the network must release the entire entry. This also brings a lot of complexities in implementation.

Besides the implementation complexity, in RAN2, companies are continuously clarifying the UE/NW behaviour of handling the extended list, this because the extending methods bring many confusions.

Observation 8	In 5G, the methods for extending the toAddModList introduce significant implementation complexities and lead to ambiguities in UE and network behaviour.

For 6G RRC signalling design, we should study new solutions for extending list structures, one simple solution is to always define a new list that completely replace the previous one.

Proposal 6	In 6GR, to study new solutions for extending list structures (e.g. always introduce a new list that completely replaces the old one).

  - Error configuration handling

In last RAN2 meeting, there was proposal that UE can apply only part of RRC configuration (good part) and discard the wrong part, so that RRC re-establishment can be avoided. Firstly, we acknowledge the issue that observed by companies, however, from network perspective, the network generates the whole RRC configuration based on its own strategy or algorithm, if the UE only applies part of the configuration, it is possible the final configuration is not compatible with network expectation. And if the UE cannot inform the network about which part has been applied, it is impossible for the network to do follow-up reconfigurations.

Based on our KPI statistics, this kind of issue does not happen frequently, instead of keeping the connection for current UE, the more serious thing is to analyse the issue, find the “problematic” IE and fix it. In 5G, when the UE triggers RRC re-establishment, the UE only indicates “other” cause value, then it took a lot time in finding the problem. In 6G, we should study a solution that can assist the network in finding the problematic/wrong configuration. For example, if modular RRC is supported, at least the UE can indicate which RRC module that the UE cannot apply.

Observation 9	Rather than keeping the current UE connection, it is more important to find out which configuration causes the problem and resolve it completely.

Proposal 7	In 6GR, to study a solution that can assist the network to find the configuration which causes RRC reconfiguration failure.

- Conclusion

Based on the discussion above, we have following observations and proposals:

Observation 1	The 5G RRC signalling structure include the following main problems:

- Issue 1: Heavy tree-like structure which significantly increasing the complexity of delta configuration;

- Issue 2: Too many linkages between parameters which increasing the complexity of configuration modification;

- Issue 3: Inflexible delta configuration during handover procedure;

- Issue 4: One-size-fit-all design which increasing the implementation complexity of low-end devices.

Observation 2	Define separate RRCReconfiguration structures for different device types leads to higher network implementation complexity.

Observation 3	It’s hard to define a precise basic RRC module, as special devices (e.g. NCR/IAB) introduced in later releases will be quite different from normal devices.

Observation 4	In most cases, feature/vertical specific parameters need to be associated with specific BWP or physical channels, introducing the cell ID or BWP ID or Channel structure in module increases the size of module.

Observation 5	It is hard to avoid zero-overlapping parameters between feature/vertical based modules. In this case, complexity/ambiguity appears when enabling multiple features/verticals simultaneously.

Observation 6	There is risk that vertical-based RRC module design may also lead to market fragmentation.

Observation 7	The requirements for delta signalling may differ in different scenarios (e.g. in non-handover case, absent = maintain; in handover case, absent = release).

Observation 8	In 5G, the methods for extending the toAddModList introduce significant implementation complexities and lead to ambiguities in UE and network behaviour.

Observation 9	Rather than keeping the current UE connection, it is more important to find out which configuration causes the problem and resolve it completely.

Proposal 1	The design of modular RRC must ensure that it does not cause market fragmentation.

Proposal 2	To study function based RRC module design in 6GR and follow the below design principles:

- Unique Parameter Ownership: Each parameter must be owned by a single RRC module, no duplicated parameter in different RRC modules.

- Minimized Coupling: Avoid or minimize the association between different RRC modules. Each RRC module is self-contained.

- Granular configuration: Support one RRC message containing only a subset of RRC modules, for RRC modules that were configured but not signalled, the UE maintains the previous configuration;

Proposal 3	RAN2 should work with RAN1 and RAN4 to define the RRC modules for L1 related parameters, aiming to explore the optimal division that avoid or minimize the association between modules.

Proposal 4	RAN2 is the leading group for defining and maintaining the ASN.1 structure for RRC modules. Different RRC modules can be described in separate sections of a single RRC spec.

Proposal 5	In 6GR, to study the Need Code that fits the different delta signalling requirements in different scenarios (handover or not).

Proposal 6	In 6GR, to study new solutions for extending list structures (e.g. always introduce a new list that completely replaces the old one).

Proposal 7	In 6GR, to study a solution that can assist the network to find the configuration which causes RRC reconfiguration failure.

- Reference

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109, Sep 2025