3GPP TSG-RAN WG2 Meeting #132                                                       R2-2508417

Dallas, USA, Nov. 17th - 21st

Agenda item:	9.3.2

Source: 	ZTE Corporation

Title: 	Discussion on RRM measurement prediction

Document for:	Discussion and Decision

Introduction

In the Rel-20 AI-Mob WI [1], the following object is proposed:

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we discuss the LCM functionality management for RRM measurement prediction.

Discussion

Basis framework

In the AI-Phy BM, Option A and Option B are introduced. For Option A, the network provides full inference configuration to UE via CSI-ReportConfig, and the UE determines applicability and reports the applicability status per CSI-ReportConfigId via RRCReconfigurationComplete, RRCResumeComplete and UAI. For Option B, the network provides partial inference configuration (the sets of inference parameters) via OtherConfig, and the UE determines the applicability and reports the applicability status via RRCReconfigurationComplete and UAI, then the network can configure full inference configuration via CSI-ReportConfig.

For AI-Mob, in the Rel-19 SI, we have agreed to use AI-Phy BM agreements as baseline, so for applicability reporting in AI-Mob, we think both Option A and Option B shall be supported.

Proposal 1:	For applicability reporting in AI-Mob, both Option A and Option B mechanisms are supported.

- Option A: Network configures full inference configuration, and UE reports applicability status to the network via RRCReconfigurationComplete, RRCResumeComplete and UAI;

- Option B: Network configures partial inference configuration via OtherConfig, and UE reports applicability status to the network via RRCReconfigurationComplete and UAI, then the network provides full inference configuration based on the received applicability status.

Full inference configuration

In the AI-Phy BM, the existing CSI-ReportConfig in the CSI-MeasConfig is used to provide full inference configuration, and the UE determines that a CSI-ReportConfig is for inference if the configuration includes reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19. Similarly, AI-Mob can follow the same way. Considering we have agreed inference configuration for AI-Mob is based on RRM measurement framework in Rel-19 SI and the IE measConfig framework is used to provide measurement configuration in the currenet RRC spec, we think the MeasConfig framework can be considered as baseline for providing full inference configuration, and the UE can determine whether a measId is for inference configuration depending on the presence of inference related configuration (e.g. PW) in the associated MeasObjectNR or ReportConfigNR.

Observation 1: In AI-Phy BM, the UE is configured for inference using the existing CSI-MeasConfig framework, and the UE knows a CSI-ReportConfig is for inference when it includes reportQuantity-r19 set to special values (e.g. p-CRI-r19).

Proposal 2: The existing MeasConfig framework is used as a baseline for providing full inference configuration, the UE knows whether a MeasId is for inference configuration depending on the presence of inference related configuration in associated MeasObjectNR or ReportConfigNR.

And in the current RRC spec, the MeasObjectId associated with the MeasId indicates the measurement object on which the UE shall perform measurement. Similarly, we think the MeasObjectId associated with the measId can be used to indicate the predicted target for all the prediction scenarios (e.g. temporal domain, frequency-domain).

Proposal 3: Regarding full inference configuration in all prediction scenarios (e.g. temporal-domain, frequency-domain), the MeasObjectId that associated with the MeasId is used to indicate the predicted target.

Measured frequency:

For frequency domain prediction, we have agreed that the measured and predicted frequency can be considered as inference configuration. Considering only frequency domain prediction for co-located cases is supported and the UE does not know the co-located info, we think the network shall provide the measured frequency to the UE.

Regarding how to provide measured frequency, the current RRC spec has the following restriction:

#clause 5.5.2.1 of TS 38.331 The network applies the procedure as follows: //skip un-related part - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

With the restriction above, for applicability check, if the measured frequency is included into MeasObjectNR, the network needs to configure multiple MeasObjectNR for a given predicted frequency to provide different measured frequency. This breaks the rule that for a given SSB frequency, at most one MO can be configured for SSB based measurements. In the other hand, for performance monitoring, the network needs to configure the UE to perform actual measurement, in this case, the network needs to configure one MeasObjectNR without measured frequency, which also breaks the rule. Based on the analysis above, we think the measured frequency shall be included in the reportConfigNR. And, the measured frequency can be indicated via MeasObjectId.

Observation 2: The current RRC specification restricts that a given SSB frequency can be associated with at most one MO for SSB-based measurement. If measured frequency is included in MeasObjectNR, we need to break the rule in order to provide different measured frequencies for a given predicted frequency.

Proposal 4: Regarding full inference configuration with frequency-domain prediction, besides the predicted frequency, the measured frequency should also be configured by the network, this can be indicated by an additional MeasObjectId in reportConfigNR.

PW:

For temporal domain prediction case A, it is unclear how to provide PW configuration. In our understanding, the same logic of measured frequency for frequency domain prediction can be applied here, in order to provide multiple PW values and not break the rule that the at most one MO can be configured for a given SSB frequency, the PW configuration can be included in the ReportConfigNR. Also, in the AI-Phy BM, the nroftimeinstance-r19 (the number of future time instance) and TimeGap-r19 (time gap between two consecutive future time instances) are configured in the CSI report configuration. The PW configuration in AI-Mob can follow the same way, i.e. to be included in the reporting configuration. And the range of the PW can be further discussed in RAN2.

Proposal 5: For full Inference configuration, the PW configuration is introduced in ReportConfigNR, FFS the value range (to be determined in RAN2).

Skipping pattern:

In the R19 SI discussion, we have achieved the following conclusion:

• The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain - PW length (for temporal case A) - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted frequency (For inter-freq domain prediction) • For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

Based on the conclusions above, the skipping pattern refers to the SSB transmission timing. We think the skipping pattern is needed for case B, because the network may stop transmitting SSB at some SSB occasions for network energy saving, and the skipping pattern implies the SSB occasions that may not be transmitted by the network. With this info, the UE knows when to perform actual measurement.

Observation 3: Skipping pattern is needed, as it implies the SSB occasions that may not be transmitted by the network.

In the existing spec, the measObjectNR is used to indicate the frequency/time location of the SSB. So, we think the skipping pattern configuration is included in MeasObjectNR.

Proposal 6: For full inference configuration, the skipping pattern configuration is introduced in MeasObjectNR.

Associated ID

In the Rel-19 AI-Mob SI, the evaluation on generalization performance is performed, and the conclusions are as follows:

- For generalization over UE speeds, generalization performs well across all UE speeds in general;

- For generalization over frequency domain prediction, GC#2 always outperforms GC#1, and its prediction accuracy is close to the baseline case. Feeding the AI/ML model with the knowledge about the input & output frequency helps to improve prediction accuracy of GC#2

- For generalization over cell configurations for intra-frequency temporal domain case A in FR2 and case B in FR1, model is generalizable over cell configurations with different deployment scenarios (i.e., UMi and UMa)

Based on the simulation results in Rel-19 AI-Mob generalization evaluation, the generalization performs well.

Observation 4: In AI-Mob model generalization simulation, many factors are evaluated, including UE speed, cell configuration (i.e. Umi/Uma, ISD, BS antenna height, BS Tx power) and prediction direction (2G->4G or 4G->2G), the results show that generalization performs well.

For generalization issue, the associated id is introduced in AI-Phy BM, and is used to ensure the similar properties of DL beam / beam set (e.g. between training and inference):

RAN1 LS: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

For AI-Mob, only spatial domain prediction is for beam level prediction (i.e. use beam set A to predict beam set B), temporal domain prediction and frequency domain prediction are for cell level prediction. So, we think the associated id is only needed for spatial domain prediction. Considering spatial domain prediction is not supported in the UE-side model, no need to introduce associated ID in AI-Mob.

Observation 5: Associated id in AI-Phy BM is used to ensure similar properties of DL Tx beam/beam set (e.g. beam pattern) between training and inference phases. However, for AI-Mob, only spatial domain prediction is beam level prediction, but is not supported in the Rel-20.

Proposal 7: Do not introduce Associated ID in AI-Mob.

Applicable reporting options

- Option A

As mentioned above, the network provides one or more MeasId for full inference configuration, and the UE determines the applicability per MeasId. Similar to reporting applicability status per CSI-ReportConfigId in AI-Phy BM, for applicability status (applicable or inapplicable) reporting in AI-Mob, the UE indicates the applicability status per measId in the applicability reporting.

Proposal 8: For applicability reporting Option A, the UE indicates the applicability status (applicable or inapplicable) per measId in the applicability report.

- Option B:

For Option B, one critical issue is the content of the inference parameters set. In the AI-Phy BM, the inference parameters set include:

- Inference parameter set Id: used to identity an inference parameter set;

- Prediction object (resource for set A): used to indicate the prediction object;

- Inference related parameters (e.g. set B for BM-Case 1, nroftimeinstance-r19 for BM-Case 2).

AI-Mob can follow the same way. In the Rel-19 SI, we have agreed that:

The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

The set of inference parameters for Option B can include the inference parameters above, e.g. the PW length, skipping pattern and measured frequency.

Proposal 9: For applicability reporting Option B, the inference parameter set includes at least the following information:

  - Inference Configuration Set ID;

  - MeasObjectId for predicted frequency;

  - Prediction window (in case of temporal domain case A);

  - Skipping pattern (in case of temporal domain case B);

  - MeasObjectId for measured frequency (in case of frequency-domain prediction).

Upon receiving inference parameters set, the UE can determine the applicability and sends the applicability status per inference configuration set ID to the network.

Proposal 10: For applicability reporting Option B, the UE indicates the applicability status (applicable or inapplicable) per inference Configuration set ID in the applicability report.

When to activate inference configuration

In the AI-Phy BM, when the UE provides the inference configuration consisting of periodic CSI report configuration, the UE autonomously activates the applicable AI/ML functionalities upon reporting the applicability, i.e. the UE is ready for inference upon reporting the applicability.

TS 38.300-j00 clause 3.2: Activated AI/ML functionality: AI/ML functionality that is already enabled for performing inference. TS 38.300-j00 clause 22.2.3: When the inference configuration consists of periodic CSI report configuration, upon reporting the applicable functionalities, the UE autonomously activates the applicable AI/ML functionalities. When the inference configuration consists with semi-persistent CSI and/or aperiodic CSI report configuration, upon reporting the applicable AI/ML functionalities, applicable AI/ML functionality activation can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

Observation 6: In AI-phy BM, upon reporting applicability for full inference configurations with periodic CSI report configuration, the UE autonomously activates the applicable AI/ML functionalities, i.e. the UE is ready for performing inference.

For Rel-19 SI, we have two study goals:

- Study goal 1: To reduce measurement efforts;

- Study goal 2: To improve the handover performance.

For study goal 1, since the motivation is to reduce measurement efforts, it is expected to obtain the gain that measurement reduction and measurement gap reduction as early as possible. Based on applicability report, the network may modify the measurement gap (for frequency domain prediction) or stop transmitting SSB at some time instances (for temporal case B). Consequently, if the UE is not ready for performing inference, UE measurement will be impacted under the reduced gap or reduced transmitted SSB occasions, e.g. the measurement interval is extended. So, we think for study goal 1 (i.e. temporal domain case B and frequency domain prediction), the UE shall autonomously activate the applicable AI/ML functionality.

Observation 7: For the 1st study goal, the network may modify SSB transmission timing (for temporal case B) or measurement gap (for frequency domain prediction) based on received applicability report. If the UE is not ready for performing inference, the UE’s measurement may be impacted.

Proposal 11: Similar to AI-Phy BM, for temporal case B and frequency domain prediction, upon reporting the applicability, the UE activates applicable AI/ML functionality autonomously, i.e. the UE is ready for performing inference.

For temporal domain Case A, the typical use case is to predict when the handover will happen. If the UE performs inference when the UE is at the cell center (i.e. when the channel quality of the serving cell is good), the prediction results can not be used by the network to make a handover decision, which causes unnecessary GPU/CPU occupation and consumption. So, it is fine to activate the functionality when the UE is near cell edge.

Observation 8: For temporal case A, the typical use case is to predict when the handover will happen. So, to reduce the GPU/CPU occupation and consumption, it is fine to activate the functionality when the UE is near cell edge.

And, considering when to perform handover is up to NW implementation, we think the network shall control when to activate the AI/ML functionality in order to avoid unnecessary GPU/CPU occupation and consumption. To achieve this goal, the network can provide the inference configuration and a condition in advance (e.g. when the quality of the serving cell is good), and the UE activates the functionality and performs inference when the pre-configured condition is met. The pre-configured condition can be a measurement event or s-measure similar mechanism.

Observation 9: Since the network controls when to perform handover, it is more flexible and efficient if the network can also control when to activate AI/ML functionality defined for handover performance improvement.

Proposal 12: For temporal case A, the applicable functionality is activated when a pre-configured condition is met (e.g. s-Measure, measurement event).

Inference report

Regarding the content of the inference report, the following agreements are achieved:

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message - For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results). - For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

In the existing spec, the IE MeasResults covers the measured results. For temporal domain case A, the UE reports one or more instances of predicted measurement results within PW, we can introduce a separate list to report multiple future predicted results to the network.

And since when to perform measurement and the measurement interval are up to UE implementation, the network is unclear of the time info of each reported result. RAN2 can further discuss whether the UE needs to indicate the time info for each reported result.

Proposal 13:	For temporal case A, to introduce a separate list of predicted measurement results in MeasResults IE. RAN2 to discuss whether to also indicate the time info for each reported result.

In the existing measurement reporting framework, the measurement report includes the measurement results of each serving frequency, including:

- Cell results of each serving cell;

- Beam results of each serving cell, if configured by NW;

- Cell results of best neighbor cell on serving frequency, if configured by NW;

- Beam results of best neighbor cell on serving frequency, if configured by NW.

For inference report, the measurement report can follow the same way. For simplification, besides the prediction results of the prediction target, the measurement can include actual measurement results of serving frequencies.

Proposal 14:	As legacy, in the predicted measurement report, the UE can include the actual measurement results of other serving cells, best neighbor cell on serving frequency and beam level results if available.

Consideration on CSI-RS based AI-Mob

In the WID, there is no restriction for CSI-RS based AI-Mob. So, we are wondering whether CSI-RS based measurement are considered in AI-Mob.

In the existing RRM measurement framework, both SSB based measurement and CSI-RS based measurement are supported, and a single measurement framework (measConfig) is used to configure SSB based measurement and CSI-RS based measurement.

Observation 10: In the existing spec, both SSB based measurement and CSI-RS based measurement are supported, and are configured via the same measurement framework (i.e. MeasConfig).

Based on the analysis above, we do not see additional spec impact to support CSI-RS based measurement prediction. So, we think both SSB based and CSI-RS based measurement are considered in AI-Mob.

Proposal 15:	RAN2 to confirm that SSB based and CSI-RS based measurement prediction are considered in AI-Mob.

UE capability

Regarding applicability reporting, the following UE capabilities are introduced in the AI-Phy BM:

- aiml-BM-Case1-r19：indicate the support of BM-Case 1;

- aiml-BM-Case2-r19：indicate the support of BM-Case 2;

- applicabilityReportingCSI-r19：indicate the support of applicability reporting and/or its updates for Option A (full inference configuration).

- applicabilityReportingOther-r19：indicate the support of applicability reporting and/or its updates for Option B (partial inference configuration).

The UE capability for AI-Mob can follow the same way. Regarding temporal case A and temporal case B, although the same AI model can be used, the inference configuration, when to activate inference configuration and the content of inference report are different. So, we think it is needed to introduce UE capabilities for temporal case A and temporal case B, separately.

Proposal 16: Similar to AI-Phy BM, to introduce the following UE capabilities for RRM measurement prediction:

- UE capability for indicating the support of temporal domain case A measurement prediction;

- UE capability for indicating the support of temporal domain case B measurement prediction;

- UE capability for indicating the support of frequency domain measurement prediction;

- UE capability for indicating the support of Option A for applicability reporting;

- UE capability for indicating the support of Option B for applicability reporting.

Conclusion

In this contribution, we propose the following observations and proposal:

Observation 1: In AI-Phy BM, the UE is configured for inference using the existing CSI-MeasConfig framework, and the UE knows a CSI-ReportConfig is for inference when it includes reportQuantity-r19 set to special values (e.g. p-CRI-r19).

Observation 2: The current RRC specification restricts that a given SSB frequency can be associated with at most one MO for SSB-based measurement. If measured frequency is included in MeasObjectNR, we need to break the rule in order to provide different measured frequencies for a given predicted frequency.

Observation 3: Skipping pattern is needed, as it implies the SSB occasions that may not be transmitted by the network.

Observation 4: In AI-Mob model generalization simulation, many factors are evaluated, including UE speed, cell configuration (i.e. Umi/Uma, ISD, BS antenna height, BS Tx power) and prediction direction (2G->4G or 4G->2G), the results show that generalization performs well.

Observation 5: Associated id in AI-Phy BM is used to ensure similar properties of DL Tx beam/beam set (e.g. beam pattern) between training and inference phases. However, for AI-Mob, only spatial domain prediction is beam level prediction, but is not supported in the Rel-20.

Observation 6: In AI-phy BM, upon reporting applicability for full inference configurations with periodic CSI report configuration, the UE autonomously activates the applicable AI/ML functionalities, i.e. the UE is ready for performing inference.

Observation 7: For the 1st study goal, the network may modify SSB transmission timing (for temporal case B) or measurement gap (for frequency domain prediction) based on received applicability report. If the UE is not ready for performing inference, the UE’s measurement may be impacted.

Observation 8: For temporal case A, the typical use case is to predict when the handover will happen. So, to reduce the GPU/CPU occupation and consumption, it is fine to activate the functionality when the UE is near cell edge.

Observation 9: Since the network controls when to perform handover, it is more flexible and efficient if the network can also control when to activate AI/ML functionality defined for handover performance improvement.

Observation 10: In the existing spec, both SSB based measurement and CSI-RS based measurement are supported, and are configured via the same measurement framework (i.e. MeasConfig).

Basis framework:

Proposal 1:	For applicability reporting in AI-Mob, both Option A and Option B mechanisms are supported.

- Option A: Network configures full inference configuration, and UE reports applicability status to the network via RRCReconfigurationComplete, RRCResumeComplete and UAI;

- Option B: Network configures partial inference configuration via OtherConfig, and UE reports applicability status to the network via RRCReconfigurationComplete and UAI, then the network provides full inference configuration based on the received applicability status.

Full inference configuration

Proposal 2: The existing MeasConfig framework is used as a baseline for providing full inference configuration, the UE knows whether a MeasId is for inference configuration depending on the presence of inference related configuration in associated MeasObjectNR or ReportConfigNR.

Proposal 3: Regarding full inference configuration in all prediction scenarios (e.g. temporal-domain, frequency-domain), the MeasObjectId that associated with the MeasId is used to indicate the predicted target.

Proposal 4: Regarding full inference configuration with frequency-domain prediction, besides the predicted frequency, the measured frequency should also be configured by the network, this can be indicated by an additional MeasObjectId in reportConfigNR.

Proposal 5: For full Inference configuration, the PW configuration is introduced in ReportConfigNR, FFS the value range (to be determined in RAN2).

Proposal 6: For full inference configuration, the skipping pattern configuration is introduced in MeasObjectNR.

Proposal 7: Do not introduce Associated ID in AI-Mob.

Applicable reporting options

Proposal 8: For applicability reporting Option A, the UE indicates the applicability status (applicable or inapplicable) per measId in the applicability report.

Proposal 9: For applicability reporting Option B, the inference parameter set includes at least the following information:

  - Inference Configuration Set ID;

  - MeasObjectId for predicted frequency;

  - Prediction window (in case of temporal domain case A);

  - Skipping pattern (in case of temporal domain case B);

  - MeasObjectId for measured frequency (in case of frequency-domain prediction).

Proposal 10: For applicability reporting Option B, the UE indicates the applicability status (applicable or inapplicable) per inference Configuration set ID in the applicability report.

When to activate inference configuration

Proposal 11: Similar to AI-Phy BM, for temporal case B and frequency domain prediction, upon reporting the applicability, the UE activates applicable AI/ML functionality autonomously, i.e. the UE is ready for performing inference.

Proposal 12: For temporal case A, the applicable functionality is activated when a pre-configured condition is met (e.g. s-Measure, measurement event).

Inference report

Proposal 13:	For temporal case A, to introduce a separate list of predicted measurement results in MeasResults IE. RAN2 to discuss whether to also indicate the time info for each reported result.

Proposal 14:	As legacy, in the predicted measurement report, the UE can include the actual measurement results of other serving cells, best neighbor cell on serving frequency and beam level results if available.

Consideration on CSI-RS based AI-Mob

Proposal 15:	RAN2 to confirm that SSB based and CSI-RS based measurement prediction are considered in AI-Mob.

UE capability

Proposal 16: Similar to AI-Phy BM, to introduce the following UE capabilities for RRM measurement prediction:

- UE capability for indicating the support of temporal domain case A measurement prediction;

- UE capability for indicating the support of temporal domain case B measurement prediction;

- UE capability for indicating the support of frequency domain measurement prediction;

- UE capability for indicating the support of Option A for applicability reporting;

- UE capability for indicating the support of Option B for applicablility reporting.

Reference

[1] RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR