3GPP TSG-RAN WG2 Meeting #132	R2-2508428

# Dallas, USA, Nov. 17th-21st

Agenda item:	10.3.1.2 (FS_6G_Radio)

Source:	LG Electronics.

Title: 	Considerations on service-awareness in 6G

Document for:	Discussion and decision

1. Introduction

This document states our views on service-awareness on 6G.

2. Discussion

2.1 Packet prioritization within a QoS flow

In 5G NR, a QoS flow is mapped to a DRB according to the AS mapping rule. Packets belonging to a specific QoS flow are transmitted through the corresponding DRB and cannot be sent through more than one DRB. Since each DRB is configured based on defined QoS requirements, all packets carried by the same DRB are generally treated in the same manner.

However, in Immersive service (XR, AI), not all packets within a single QoS flow have equal importance. Differences in packet priority can significantly affect service quality, and treating all packets identically may lead to inefficiencies. The following examples illustrate why packet prioritization/differentiation within a single QoS flow is necessary.

Example 1 – Video frames in XR service

According to TR38.838, all video packets of an XR application can be transmitted through a single QoS flow. It means that the I-frame, the P-frame, and the B-frame can be transmitted on a single IP flow. Though they are transmitted on a single IP flow, the priority of each frame is different. Since the P-frame and the B-frame are encoded based on the I-frame, it can be assumed that the I-frame is more important than the P-frame and the B-frame. For example, if an I-frame is not successfully transmitted, B-frame and P-frame cannot be decoded in the receiver. Loss of I-frame is typically perceived as small video freeze or jump up to few seconds, and the B-frame or P-frame is of no use. Therefore, the I-frame should be prioritized over B-frame and P-frame within a single QoS flow for the video frame.

Example 2 – AI token-based transmission

One example of AI token-based transmission is video transmission. As mentioned above, all video packets can be transmitted through a single QoS flow. An additional characteristic of such traffic is error-tolerant, meaning that not all tokens composing a single video frame need to be transmitted perfectly. Considering this, even if a video frame is initially regarded as high priority, once a sufficient number of its tokens have been successfully delivered, the remaining tokens may be treated as low priority. In other words, the tokens within the same video frame may have different priority. In such cases, the token with high priority can be handled differently from the token with low priority. For example, it can be ensured that a sufficient number of high priority tokens are transmitted with guaranteed reliability.

Example 3 – TCP Packets and TCP ACK

The TCP packet is encapsulated into IP packet, and the IP packet is transmitted through PDCP. Thus, one type of PDCP SDU is an IP packet including TCP packet. The TCP ACK packet is feedback of received TCP packets, and transmitted in the opposite direction to the TCP packet. In other words, if TCP packet is transmitted in downlink, the TCP ACK packet is transmitted in uplink, and vice versa. The TCP ACK packet is also encapsulated into IP packet, and transmitted through PDCP. Thus, an IP packet including TCP ACK packet is also one type of PDCP SDU. To increase the TCP throughput, it is important to transmit TCP ACK as early as possible. However, the PDCP does not inspect the contents of the PDCP SDU (i.e. IP packet), and thus the PDCP does not differentiate TCP and TCP ACK packets. As the TCP ACK is one of main hurdle in maximizing TCP throughput, the TCP ACK packet should be prioritized over TCP packet within a single QoS flow.

Based on the above examples, we think that the prioritized/differentiated handling of the packets based on packet priority need to be studied to enhance performance, reduce service degradation, and meet diverse application requirements. We think that the prioritizing/differentiating the high priority packet within a single DRB can be achieved in the PDCP layer. That is, the PDCP entity maps a single DRB to multiple logical channels, and transmits packets on different logical channels according to the packet priority. In other way, the prioritizing/differentiating the high priority packet within QoS flow can be achieved by mapping a single QoS flow to multiple DRBs. That is, packets of a single QoS flow are transmitted on different DRBs according to the packet priority. Thus, we ask that RAN2 to study a prioritized/differentiated handling of the packets based on packet priority in a single QoS flow.

Proposal 1. Study a mechanism to prioritize/differentiate packets based on packet priority in a single QoS flow, e.g.

a. Mapping different DRBs for a single QoS flow.

b. Mapping different logical channels for a DRB.

2.1 Synchronized transmission of multi-modal traffic

According to TR 22.847, for multi-modal traffic in XR service, it is stated that the coordinated transmission of multi-modal data from multiple QoS flows is essential because the asynchronous transmission of multi-modal data degrades the XR user experience.

Table 5.1.6-2: Typical synchronization thresholds for immersive multi-modal VR applications synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other..

Here, the synchronization threshold is defined in TS 22.261 as the maximum tolerable delay between the two stimuli, sense, or objects. In other words, it can be interpreted as the maximum tolerable delay between data units from different QoS flows, since each stimuli, e.g., video, audio, or tactile data is associated with different QoS flows with different QoS flow requirements.

synchronization threshold: A synchronization threshold can be defined as the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous. NOTE 10: This definition is based on [41].

The QoS flow level support for multi-modal traffic (e.g., to apply the same QoS handling for associated QoS flows or to ensure same average transmission delay for data in associated QoS flows) may not be enough and packet level support for multi-modal traffic would be needed to ensure the synchronization between two data units, as defined in TR 22.847 and TS 22.261. This is because QoS flow level support may define which QoS flows should be transmitted together, but actually the more important point is to define which packets of each QoS flow need to be transmitted together for synchronization, i.e. two data units from different QoS flow need to be synchronized for transmission. If not, the multi-modal data would be transmitted in de-synchronized manner, resulting in unnecessary resource waste due to transmission of outdated data and performance degradation of XR service.

During the XR Rel-19 WI, RAN2 discussed to support synchronized transmission. However, the synchronization threshold was not specified due to concern about increasing UE complexity in identifying synchronized data units, and only the MMSID mechanism was introduced. The MMSID allows the network to obtain which QoS flows are associated with the same service from the 5GC when the QoS flow is established. Based on this information, the network attempts to enable synchronized transmission. However, in the logical channel prioritization procedure, the UE generates MAC PDUs based on logical channel priorities without considering synchronized data units. As a result, synchronized transmission for multi-modal traffic may not always be guaranteed, e.g., the data units for synchronized transmission cannot be transmitted within the synchronization threshold due to the current logical channel prioritization procedure.

The XR is considered one of the key applications in 6G and synchronized transmission of multi-modal data is closely tied to the overall user experience. To ensure this, RAN2 should study packet-level synchronization for the multi-modal traffic with inter-QoS flow dependencies, thereby meeting the required synchronization thresholds between different data units.

Proposal 2. Study the support of multi-modality with inter-QoS flow dependency on packet level.

2.3 Per-packet transmission mode

In 5G NR, three RLC operation modes are defined, i.e. AM (Acknowledged Mode), UM (Unacknowledged Mode), and TM (Transparent Mode). Among them, AM and UM are used for user data transmission, and TM is used for control signal transmission. These modes are inherited from UMTS and have been used for more than 25 years.

- AM is typically used for traffic requiring high reliability through retransmissions.

- UM is typically used for traffic requiring low latency without retransmissions.

When a DRB is setup, the transmission mode i.e. either AM or UM, is determined based on QoS requirements of a traffic flow. The transmission mode is maintained throughout the DRB lifetime, and the only way to change the transmission mode for the traffic flow is DRB release and addition. Once DRB is established, all the packets belonging to the DRB is transmitted with the same transmission mode.

However, as new traffics for XR and AI have emerged, we are wondering whether the current transmission mechanism, i.e., per DRB transmission mode, is appropriate to support this new traffic flow. It is identified that AI and XR-like traffic flow has a mixed characteristic within the same traffic flow. We think that this fixed per DRB transmission mode would be inefficient for such mixed‑type packet flows since required characteristic for each packet is different, as explained below.

- XR: I-frame requires high reliability while P/B-frame does not require high reliability.

- AI: inference data requires low latency, whereas training data does not require low latency.

Considering that both AI and XR are killer applications in 6G, we expect that AI and XR-like traffic would be dominant and transmission of such mixed‑type packet flows should be carefully considered and studied for better user experience.

One solution to support mixed-type packet flows is introducing per-packet transmission mode instead of per DRB transmission mode. In the per-packet transmission mode, whether the retransmission is required or not is determined per packet basis, and the need for ACK feedback is indicated to the receiver by in-band signaling. The receiver sends ACK feedback only when the packet requiring ACK feedback is received, and the transmitter retransmits the packet if the ACK feedback is not received until a timer expires. A simple example of per-packet transmission mode is shown in following figure.

Example of per-packet transmission mode operation

With this per-packet transmission mode, there is no need to differentiate between DRB types, i.e. AM DRB or UM DRB. A single DRB can serve both lossless and lossy transmission per packet basis.

As the per-packet transmission mode is suitable for mixed-type packet flows, we think it is worthwhile to reconsider the L2 transmission mode.

Proposal 3. Study per-packet transmission mode suitable for mixed type packet flow, e.g.,

a. Whether to perform retransmission is determined per packet basis

b. Single DRB supports both lossless and lossy transmission

## 2.4	Handling of the data burst

Immersive communication/XR enhanced with AI-assisted contextual awareness, e.g., XR or AI systems that adapt intelligently to the user’s environment, or conversational assistance, e.g., low-latency, AI-driven interactive services, represent a next-generation class of applications that integrate real-time sensor inputs, media streams e.g., video/audio/haptic, AI data, and cloud/edge AI inference. These emerging services place stringent requirements on data delivery, as they combine both high-throughput multimodal data transmission and low-latency AI-driven interactions with bounded end-to-end round-trip time.

XR and AI-powered contextual awareness or conversational support produce substantially more uplink traffic than traditional mobile services. This traffic can be bursty and span multiple modalities, including audio, video, and image capture for scene or environmental understanding, sensor readings, and user interaction signals. As a result, heterogeneous, delay-sensitive uplink bursts often occur irregularly in timing and vary widely in data volume.

Observation 1. Data bursts produced by XR/AI service have irregular periodicities and irregular data volume.

In 5G, two procedures—Buffer Status Report (BSR) and Data Status Report (DSR)—are used to indicate the data volume to the network. The BSR procedure provides the network with information about the amount of data volume in the UE side. Upon receiving this information, the network can determine how much data is waiting for transmission. The DSR procedure, in addition to reporting the data volume, also specifies the remaining time of the reported data. This allows the network to identify how much data is waiting for transmission in the UE buffer within a defined time duration.

However, the BSR and DSR procedures do not provide sufficient information for a data burst, because they only indicate the data volume currently present in the UE buffer. Consequently, even if the network receives BSR and DSR reports, it cannot determine the total data volume or the duration of the burst. From the network perspective, this lack of information makes it challenging to schedule uplink grants to the UE effectively, as the necessary knowledge is unavailable.

To more precisely schedule the uplink grant to the UE for a data burst, we think that two methods can be studied. The first method is that the UE informs the data burst related information for each data burst, e.g. data burst volume and data burst duration. Since this information can only be determined on the UE side, the network is difficult to allocate uplink grants effectively without such knowledge. The second method is for the UE to transmit BSRs more frequently during the data burst duration. Since the data volume waiting for transmission is dynamically changed during the data burst duration, it is important to report the real-time data volume changes to the network to adjust the scheduling of the uplink grant accordingly.

Proposal 4. For efficient uplink scheduling for a data burst, study a mechanism to provide precise data burst information, e.g., data volume and duration.

3. Conclusion

Based on the above discussions, we present the following proposals:

Proposal 1. Study a mechanism to prioritize/differentiate packets based on packet priority in a single QoS flow, e.g.

a. Mapping different DRBs for a single QoS flow.

b. Mapping different logical channels for a DRB.

Proposal 2. Study the support of multi-modality with inter-QoS flow dependency on packet level.

Proposal 3. Study per-packet transmission mode suitable for mixed type packet flow, e.g.,

a. Whether to perform retransmission is determined per packet basis

b. Single DRB supports both lossless and lossy transmission

Proposal 4. For efficient uplink scheduling for a data burst, study a mechanism to provide precise data burst information, e.g., data volume and duration.