3GPP TSG-RAN WG2 Meeting #132	R2-2508429

# Dallas, USA, Nov. 17th-21st

Agenda Item	: 8.2.2 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on open issues for AIoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on open issues for AIoT.

Discussion

  - (Issue 3-7) No data available case

In last RAN2 meeting, how to handle no data available case was discussed, and the following agreements are made.

Agreements 1. RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. 2. For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). 3. For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

In addition, after the POST131bis#017 email discussion, the rapporteur want to identify whether there is new case for no data available other than long writing operation. Thus, RAN2 need to check whether there is a case where NAS layer does not provide a response at all.

According to TS24.369, the NAS layer ignores a NAS message if the NAS message is that NAS message is too short or NAS message is unknown or unforeseen message type.

6.2 Message too short or too long 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3]. 6.2.2 Message too long Editor's note: Maximum AIoT NAS message length is FFS and to be defined by RAN3. 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

Observation 1. (Issue 3-7) In case of the NAS message error case, the NAS layer ignores/discards the NAS message, i.e., the NAS layer does not generate the response message at all.

As shown above, there is a case where the NAS layer does not respond to a message at all, i.e., the NAS message error case. In addition, as previously discussed in RAN2, there is case where the NAS layer cannot generate the NAS message due to NAS processing delay, i.e., the NAS processing delay case.

From the reader’s point of view, even if the D2R message with SDU length = 0 and the MDI = 0 is received, it is difficult to distinguish whether the device has no data due to the NAS message error or the NAS processing delay. In addition, without knowing this information, the reader unnecessarily schedules the uplink grant to the device even for the NAS message error case. Thus, the device should indicate why the SDU length = 0 is indicated in the R2D message.

In order to indicate the NAS message error case and NAS processing delay case, we think that there are three options as follows.

- Option 1. The estimated time when the NAS command response will be ready is included in the R2D message.

- Option 2. Introduce a new field (1bit) to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for NAS message error case.

- Option 3. If the SDU field is set to 0, the MDI field is set to 0 for the NAS message error case and the MDI field is set to 1 for the NAS processing delay case, i.e., revert to the previous agreement.

For Option 1, the device provides information on when the D2R message is ready. Using this information, the reader can schedule the D2R message on time. In this option, if the estimated time is set to zero, the reader identifies that the device has no data due to an error. Otherwise, if the estimated time is not zero, the reader schedules the D2R message on time. However, since this option requires an estimation of the time when the D2R message is ready, the feasibility of the estimation of the time is questionable due to the AIoT device's limited processing capacity.

The Option 2 and Option 3 are similar solutions. The only difference is whether to indicate the NAS message error and the NAS processing delay using the new field or the MDI field. However, during the [POST130][027][AIoT] email discussion, many companies had a concern to reuse the MDI field with the different purposes because the motivation for introducing the MDI field is to indicate whether the data in the MAC PDU is the last segment or not. In addition, for Option 2, one reserved bit in the R2D message format can be used for the new field. Thus, even if new 1bit field is introduced, there is no problem on the signaling overhead.

In our view, since it would be encouraged for the simple implementation that one single field should be used only for single purpose, we prefer to have option 2 to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for NAS message error case.

Proposal 1. (Issue 3-7) Introduce new field (1bit) to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for the NAS message error case.

According to C1-256631, it is agreed that the NAS layer discards the NAS message if integrity failure happens the NAS message, and the NAS layer indicates the integrity failure indication to the lower layer.

4.2.4 Integrity protection of AIoT NAS signalling messages Editor's note: Integrity protection of AIoT NAS signalling messages is FFS, pending available stage 2 requirements. In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers. In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIOTF shall discard the AIoT NAS message.

Then, RAN2 would need to discuss what is the AIoT device behavior when integrity failure indication is received from the upper layer. We think that there are two methods for the AIoT device behavior at reception of the integrity failure indication from the upper layer as follows.

- Method 1: The AIoT device notifies the reader that an integrity failure has occurred. The reader then reports the identification of the AIoT device where the integrity failure occurred to the AIoTF. This allows the AIoTF to identify the device where an integrity failure has occurred and perform the authentication procedure, e.g., inventory procedure, for that specific AIoT device.

- Method 2. The AIoT device does not provide any information to the network. The device then considers the ongoing procedure as failed and subsequently performs the re-access procedure to initiate the authentication process. In this method, the AIoTF determines that there is a problem with the ongoing procedure for the AIoT device if it does not receive a response to the NAS message.

In our view, both methods can handle the integrity failure case. However, it is a bit questionable whether the device notifies the reader that an integrity failure has occurred or not. This is because the integrity failure may be caused by an intruder. Thus, we slightly prefer the method 2.

Proposal 2. (Issue 3-7) At reception of the integrity failure indication from upper layer, the AIoT device considers that the on-going procedure is failed, and the device performs the re-access procedure.

  - (Issue 6-1) Handling inventory and command collision

In LS C1-256624, CT1 asks RAN2 view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens. In this LS, they expressed concern about a potential attack scenario in which a false AIoT paging message transmitted from a fake reader could interrupt ongoing valid paging or command procedures, potentially blocking valid inventory or command execution temporarily or continuously.

According to the current specification, the AIoT device considers that the newly received AIoT paging message is always valid. Then, the AIoT device stops the ongoing procedure regardless of whether the inventory/command procedure is completed or not, and the AIoT device delivers the paging ID to the upper layer. Thus, if we need to solve the concern by CT1, RAN2 need to redesign the AIoT paging procedure.

However, if RAN2 redesigns the AIoT paging procedure, it would have a significant impact on the specification, which is not desirable after completion of the WI. Furthermore, at RAN2#130, it was agreed that a parallel service request for an overlapping reader scenario can be addressed in Rel-20. Therefore, we believe it would be more appropriate to revisit the AIoT paging procedure design by considering the collision issue caused by a fake reader when discussing parallel service requests during Rel-20.

Proposal 3. (Issue 6-1) RAN2 should discuss the collision issue caused by fake reader when designing the AIoT paging message for supporting parallel service requests in Rel-20.

  - (Issue 6-5) Whether there is AS impact to support Temp ID

According to S2-2509655, three types of AIoT device identifiers are defined: AIoT Device Permanent Identifier, AIoT Device Temporary Identifier, and AIoT Filtering Information. However, how to identify the AIoT device identifier type has not yet been discussed in RAN2 and other WGs. Therefore, it may be necessary to determine which layer should indicate the device identifier type in order to reduce the processing time for its identification. For this, we can consider the AS layer indication method and the NAS layer indication method for indicating the device identifier type as follows.

- Method 1 – AS layer indication: Include the device identifier type in the AIoT paging message. When the AIoT device receives the AIoT paging message, it indicates the device identifier type to the upper layer when delivering the paging ID.

- Method 2 – NAS layer indication: Define the device identifier type within the paging ID itself. That is, the identification of the device identifier type relies on the NAS message.

In our view, the device identifier type indication is only needed when the NAS layer processes the paging ID, meaning it has no functional role within the AS layer. Based on this, it is preferable to define the device identifier type indication within the paging ID.

However, since it will be discussed in CT1 to define the device identifier type indication to the paging ID in the NAS layer in the next meeting. If the CT1 does not define the device identifier type indication in paging ID, RAN2 should then discuss how to define device identifier type indication in the AIoT paging message. Therefore, it is proposed that RAN2 defer further discussion until CT1 reaches a decision on whether to indicate the device identifier type in the paging ID or not.

Proposal 4. (Issue 6-5) RAN2 wait for the CT1 decision on whether to indicate the device identifier type in the paging ID or not.

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation 1. (Issue 3-7) In case of the NAS message error case, the NAS layer ignores/discards the NAS message, i.e., the NAS layer does not generate the response message at all.

Proposal 1. (Issue 3-7) Introduce new field (1bit) to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for the NAS message error case.

Proposal 2. (Issue 3-7) At reception of the integrity failure indication from upper layer, the AIoT device considers that the on-going procedure is failed, and the device performs the re-access procedure.

Proposal 3. (Issue 6-1) RAN2 should discuss the collision issue caused by fake reader when designing the AIoT paging message for supporting parallel service requests in Rel-20.

Proposal 4. (Issue 6-5) RAN2 wait for the CT1 decision on whether to indicate the device identifier type in the paging ID or not.