3GPP TSG-RAN WG2 Meeting #132	R2-2508431

# Dallas, USA, Nov. 17th-21st

Agenda Item	: 9.2.2 (Ambient_IoT_Solutions_Ph2)

Source	: LG Electronics Inc.

Title	: Discussion on Topology 2 for AIoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our view on the topology 2 in AIoT

Discussion

  - Essential NGAP IE required to generate AIoT paging message

In RAN2#131bis meeting, the following agreement is made, and there is a remaining FFS as follows.

Agreements 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information

Thus, we need to identify what information is required for the UE reader to provide AIoT information, and let’s discuss what information is needed for UE reader.

In Topology 2, the UE reader has to generate the AIoT paging message instead of the gNB. To do this, it is first necessary to determine the essential parameters required to generate the AIoT paging message. In Rel-19, the network reader receives the Inventory Request Transfer message from the AIoTF, which contains the following information:

- A-IoT Correlation Identifier IE

  - This IE contains only A-IoT Correlation Identifier IE. It is used to generate the transaction ID field.

- A-IoT Device Identification Requested IE:

  - This IE contains device identification for AIoT paging message, i.e., a single device, group devices, or all devices. It is used to set the Access Type field (CFA and CBRA) and the Paging ID Presence Indication field.

- Requested Service Area Information

  - This IE is used to select the network reader when the service request is received.

- Inventory Assistance Information

  - This IE contains the expected D2R message size, approximate number of target A-IoT devices, and time interval. It is used to set the number of access occasions and for D2R scheduling for Msg3.

- Follow on Command Indication

  - This IE is used to identify the presence of a command procedure after completion of the inventory procedure. It is used to allocate the AS ID during the random-access procedure.

As explained above, we consider the A-IoT Correlation Identifier IE, A-IoT Device Identification Requested IE, Inventory Assistance Information IE, and Follow on Command Indication IE to be essential for generating the AIoT paging message and D2R scheduling. Therefore, we propose that the network should provide these IEs to the UE reader to enable AIoT paging message generation.

Proposal 1. At least following NGAP IEs should be informed to generate the AIoT paging message from the network to the UE reader. FFS for other IEs.

- A-IoT Correlation Identifier IE;

- A-IoT Device Identification Requested IE, e.g., a single device, group devices, or all devices and device identifier;

- Inventory Assistance Information, e.g., expected D2R message size, approximate number of target A-IoT devices;

- Follow on Command Indication.

  - Indication for end of AIoT paging round

In the RAN2#131bis meeting, the following agreement was reached, with one remaining FFS as described below.

Agreements 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation.

In Rel-20, when the network configures AIoT radio resources for a UE reader, the UE reader utilizes these allocated resources to facilitate communication among the targeted AIoT devices. The UE reader manages the exchange of data between UE reader and AIoT devices within the configured AIoT radio resources. It means that the UE reader transmits the Access Trigger message by its decision, and only UE reader knows precisely when a AIoT paging round has been completed.

From the network’s perspective, there is no visibility into whether a AIoT paging round has finished within the allocated AIoT radio resources or not. Without this information, the network may encounter difficulties in optimizing AIoT radio resource usage, which could lead to inefficiencies. To enable more precise AIoT radio resource management, it would be beneficial for the network to receive explicit signaling from the UE reader indicating the completion of a AIoT paging round. Such an indication would allow the network to efficiently allocate the AIoT radio resources.

Proposal 2. The UE reader indicates the completion of a AIoT paging round to the network.

  - AIoT resource allocation

In RAN2#131bis meeting, how to allocate the AIoT radio resource between the UE reader and the AIoT devices was discussed, but it is not concluded yet. During the discussion, the following candidate two options were proposed.

- Option 1: The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- Option 2: The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

In our view, each option can be used for a different case. Option 1 can be used for the case where the gNB is not aware of the number of targeted devices for the inventory procedure, i.e., the inventory procedure is targeted to the unknown devices or all devices, and Option 2 can be used for the case where the gNB is aware of the number of targeted devices for the inventory procedure.

In addition, if the UE reader needs to perform the CFA procedure, Option 1 would cause the inefficiency. Since the CFA procedure is performed only one targeted AIoT device, the AIoT radio resources would be needed only short time. Thus, we suggest to introduce both options for AIoT resources allocation.

Proposal 3. The following AIoT resource allocation methods should be supported.

- The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

  - How to handle the allocated AIoT radio resources at RLF and HO

As stated in WID, RAN2 need to discuss how to handle the allocated AIoT radio resources for RLF and handover. In our understanding, since RLF or handover is associated with Uu interface, the allocated AIoT radio resources can be used even when the UE detects the RLF or UE performs the handover.

More specifically, for RLF case, if the UE reader detects the RLF, the UE reader initiates the RRC Connection Reestablishment. In this case, the allocated AIoT radio resources can be used until the RRC Connection Reestablishment is completed or failed. In addition, for handover case, even if the UE reader receives the handover command containing the AIoT radio resources for the target gNB from the source gNB, the UE reader can use the AIoT radio resources received from the source gNB until the handover is completed to the target gNB.

Thus, considering the above explanation, we propose that the allocated AIoT radio resources should be used before completion of the handover or the RRC Connection Reestablishment procedure.

Proposal 4. The allocated AIoT radio resources are considered as valid before completion of the handover or the RRC Connection Reestablishment procedure.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. At least following NGAP IEs should be informed to generate the AIoT paging message from the network to the UE reader. FFS for other IEs.

- A-IoT Correlation Identifier IE;

- A-IoT Device Identification Requested IE, e.g., a single device, group devices, or all devices and device identifier;

- Inventory Assistance Information, e.g., expected D2R message size, approximate number of target A-IoT devices;

- Follow on Command Indication.

Proposal 2. The UE reader indicates the completion of a AIoT paging round to the network.

Proposal 3. The following AIoT resource allocation methods should be supported.

- The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

Proposal 4. The allocated AIoT radio resources are considered as valid before completion of the handover or the RRC Connection Reestablishment procedure.