3GPP TSG-RAN2 #132	R2-2508433

# Dallas, USA, November 17th - 21st, 2025

Agenda Item:	8.7.2 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on RLC-H02 issue

Document for: 	Discussion and Decision

Introduction

This contribution discusses the below RLC open issue.

(RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

Discussion

In the last meeting, it was identified that when t-RxDiscard expires, RX_Next might be equal to or larger than RX_Highest_Status and RAN2 agreed to discuss solutions to handle this issue at the next meeting. It is ok to have a solution to handle the case RX_Next is larger than RX_Highest_Status, but we wonder whether the case RX_Next is equal to RX_Highest_Status is problematic.

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. <omitted> - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

The yellow condition in the above box is always satisfied before introducing t-RxDiscard since the STATUS PDU is triggered only when SN < RX_Highest_Status condition after receiving a poll is satisfied or t-Reassembly expires. This means that the STATUS PDU is constructed only when there is at least one SDU which needs to be indicated as NACK in the STATUS PDU before introducing t-RxDiscard. However, after introducing t-RxDiscard timer, the situation is changed. RX_Next is updated and the STATAUS PDU is triggered after expiry of t-RxDiscard and the discarded SDUs shall be indicated as ACK using the legacy STATUS PDU. In other words, the point is that the STATUS PDU should be triggered to indicate the discarded SDUs as ACK to the transmitting RLC entity regardless of whether there is at least one SDU which needs to be indicated as NACK in the STATUS PDU.

Observation 1. Before introducing t-RxDiscard, a STATUS PDU is transmitted only when there is at least one SDU which needs to be indicated as NACK in the STATUS PDU.

Observation 2. After introducing t-RxDiscard, a STATUS PDU should be transmitted to indicate discarded SDUs as ACK to the transmitting RLC entity regardless of whether there is at least one SDU which needs to be indicated as NACK in the STATUS PDU.

With the observation 2, we don’t think that the case RX_Next is equal to RX_Highest_Status is a problem since anyway RX_Highest_Status is to hold the highest possible value of the SN which can be indicated by "ACK_SN" in a STATUS PDU as shown below box and the discarded SDUs can be correctly indicated as ACK in the STATUS PDU after expiry of t-RxDiscard. Of course, in this case, RX_Next <= SN < RX_Highest_Status is satisfied and there is no NACK information in the STATUS PDU after expiry of t-RxDiscard, but ACK information for the discarded SDUs is correctly transmitted to the transmitting RLC entity and there is no problem.

c) RX_Highest_Status – Maximum STATUS transmit state variable This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

Observation 3. Even though RX_Next is equal to RX_Highest_Status after expiry of t-RxDiscard, ACK information for the discarded SDUs is correctly transmitted to the transmitting RLC entity and there is no problem.

Proposal 1. (RLC-H02) RAN2 confirm that the case RX_Next is equal to RX_Highest_Status is not a problem and no solution is needed for this case.

As shown below figure [1], however, the case RX_Next is larger than RX_Highest_Status after expiry of t-RxDiscard is a problem since the actual ACK_SN which can be indicated in the STATUS PDU, i.e., RX_Highest_Status, is lower than the RX_Next and the discarded SDUs cannot be correctly indicated as ACK in the STATUS PDU.

Based on the contributions in the last meeting, two options can be considered as a solution for this issue:

- Option 1. Set the ACK_SN to the RX_Next if STATUS PDU is triggered by the expity of t-RxDiscard;

- Option 2. Update RX_Highest_Status when t-RxDiscard expires and set the ACK_SN to the RX_Highest_Status.

Both options work. However, considering RAN2 agreement that existing STATUS PDU/procedure is baseline and reused, we think the option 2 is more aligned with the existing STATUS PDU/procedure. The option 1 needs to change basic principle of STATUS PDU/procedure and definition of RX_Next since ACK_SN in the STATUS PDU is based on RX_Next, but the option 2 only needs to change one simple action when t-RxDiscard expires, i.e., the option 2 doesn’t need to change basic principle of STATUS PDU/procedure since it still set ACK_SN in the STATUS PDU based on the RX_Highest_Status as in legacy. In addition, after transmitting the STATUS PDU, the option 1 still leave that RX_Next is larger than RX_Highest_Status and this problematic situation is not resolved, but the option 2 make RX_Highest_Status equal to or larger than RX_Next and this is more desirable from state variable management point of view as shown in below figure.

Option 1					  Option 2

Observation 4. Option 1 needs to change basic principle of STATUS PDU/procedure and definition of RX_Next which is not aligned with previous RAN2 agreement, but option 2 doesn’t change basic principle of STATUS PDU/procedure as RAN2 agreed before.

Proposal 2. (RLC-H02) When t-RxDiscard expires, update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received, and set the ACK_SN to the RX_Highest_Status in the STATUS PDU as in legacy (TP is given below).

Text proposal: ##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. Before introducing t-RxDiscard, a STATUS PDU is transmitted only when there is at least one SDU which needs to be indicated as NACK in the STATUS PDU.

Observation 2. After introducing t-RxDiscard, a STATUS PDU should be transmitted to indicate discarded SDUs as ACK to the transmitting RLC entity regardless of whether there is at least one SDU which needs to be indicated as NACK in the STATUS PDU.

Observation 3. Even though RX_Next is equal to RX_Highest_Status after expiry of t-RxDiscard, ACK information for the discarded SDUs is correctly transmitted to the transmitting RLC entity and there is no problem.

Observation 4. Option 1 needs to change basic principle of STATUS PDU/procedure and definition of RX_Next which is not aligned with previous RAN2 agreement, but option 2 doesn’t change basic principle of STATUS PDU/procedure as RAN2 agreed before.

Proposal 1. (RLC-H02) RAN2 confirm that the case RX_Next is equal to RX_Highest_Status is not a problem and no solution is needed for this case.

Proposal 2. (RLC-H02) When t-RxDiscard expires, update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received, and set the ACK_SN to the RX_Highest_Status in the STATUS PDU as in legacy (TP is given below).

Text proposal: ##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

- Reference

[1] R2-2507903 Discussion on the RLC STATUS PDU at t-RxDiscard expiry	Huawei, HiSilicon