3GPP TSG-RAN WG2 Meeting #132	R2-2508441

Dallas, USA, 17 – 21 November 2025	(Resubmission of R2-2507304)

Source:		NEC

Agenda item:		8.6.2	Corrections

Title:	Collision between PUSCH for early CSI and measurement gap

Document for:		Discussion and decision

- Introduction

In this paper, we point out that it is possible that the PUSCH for early CSI defined in RAN1 spec may collide with a measurement gap, and suggest to send LS to RAN1 to ask how to handle this kind of situation.

- Discussion

For early CSI report for LTM candidate cells, RAN1 has specified the PUSCH used by the UE to transmit the early CSI report for RACH-less LTM, CFRA based LTM and CBRA based LTM separately. According to the RAN1 spec, the PUSCH to transmit to the early CSI can be during the LTM procedure (e.g., for RACH-less LTM), or after completion of LTM procedure (e.g., for CBRA-based LTM)

The corresponding text in TS 38.214[1] is copied below:

### 5.2.4a CSI Reporting for LTM A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, ltm-CSI-ReportConfig, for a candidate cell. Each Reporting Setting ltm-CSI-ReportConfig is associated with either one or two Resource Settings <skip> For RACH-less LTM cell switch [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant [6, TS 38.213]. For RACH-based LTM cell switch using a contention-free random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the PUSCH scheduled by the RAR UL grant or MsgA PUSCH. For RACH-based LTM cell switch using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4 or Msg

According to Rel-18 LTM discussion, the Measurement gap can be applied upon execution of LTM procedure, and therefore the PUSCH defined by the RAN1 to transmit the CSI report to the candidate cell may collide with the measurement gap. However, according to TS 38.321, see below the corresponding text, during the activated measurement gap, the UE shall not perform transmission of CSI, and not transmit on UL-SCH except for Msg3 or the MSGA payload.

The corresponding text in TS 38.321[2] is copied below:

## 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a, 5.1.5, and 5.7. …

If this kind of collision happens, it is not clear whether the UE can transmit the early CSI report to the candidate cell by the PUSCH. For example, according to RAN1 spec, in case of CFRA-based LTM, the UE can transmit the CSI report to the candidate cell using the PUSCH for Msg A, but according to RAN2 spec, the UE can transmit the Msg A but cannot transmit CSI during measurement. For another example, for CBRA-based LTM, according to RAN1 spec, the UE transmit the CSI to the candidate cell using the first PUSCH resource corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission, but according to RAN2 spec, the UE cannot transmit the CSI using the corresponding PUSCH resource which overlaps with one measurement gap. This kind of contradiction may cause different implementations at the UE and network due to different understanding, and results in decoding failure at the network side.

Observation. There is contradiction in RAN1 spec and RAN2 spec on whether the UE can transmit CSI report to the candidate cell during the measurement gap or not.

Considering that there is conflict in the understanding of different spec, and considering that the early CSI report to the candidate cell is more critical compared with conventional CSI report, which targets for high data rate during the mobility procedure, we propose to send an LS to RAN1 to clarify whether the UE transmits the CSI report to the candidate cell if the PUSCH collides with measurement gap.

Proposal 1. Send LS to RAN1 to ask whether the UE transmits CSI report to the candidate cell using PUSCH during the measurement gap or not.

# 3. Conclusion

In this paper, we have the following proposal:

Observation. There is contradiction in RAN1 spec and RAN2 spec on whether the UE can transmit CSI report to the candidate cell during the measurement gap or not.

Proposal 1. Send LS to RAN1 to ask whether the UE transmits CSI report to the candidate cell using PUSCH during the measurement gap or not.

# Reference

[1] 38.214, v19.1.0, NR; Physical layer procedures for data.

[2] 38.321, v19.1.0, NR; Medium Access Control (MAC) protocol specification.

# Annex

3GPP TSG-RAN WG2 Meeting #131bis	R2-250xxxx

Dallas, USA, Nov. 17th-21st, 2025

Title:	LS on collision between PUSCH for early CSI and measurement gap

Response to:	-

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN WG2

To:	RAN WG1

Cc:	-

Contact Person:

Name:	TBD

E-mail Address:	TBD

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN2 has identified that it is possible that the PUSCH used for CSI to the candidate cell, defined in 5.2.4a of TS 38.214, collides with the measurement gap. But according to the current TS 38.321, CSI cannot be transmitted during the measurement gap.

# 2. Actions:

# To RAN1:

ACTION: RAN2 respectfully asks RAN1 to clarify whether the UE performs transmission of CSI to candidate cell using the PUSCH defined in TS38.214 during the measurement gap.

# 3. Date of Next RAN2 Meetings:

RAN2#133	9th February – 13rd February 2026		Göteborg, Sweden

RAN2#133bis	13rd April – 17th April 2026		TBD