3GPP TSG-RAN WG2 Meeting #132	R2-2508452

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.3.2.3

Source:	Apple

Title:	Views on 6G Spectrum Aggregation

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to 6GR SID (RP-252912), the control plane design is one of the RAN2-led objectives, which includes the aspects related to spectrum aggregation.

In RAN2#131bis meeting, RAN2 did some initial discussion on how to approach 6G NTN study to ensure the harmonized design and made the following plan.

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

This contribution gives our initial views on 6G spectrum aggregation from the following aspects:

- UL and DL spectrum efficiency

- Cross-CC initial access

- 6G CA.

# 6G Spectrum Aggregation Design

## Background – NR CC/Cell model

In NR, the UE specific serving cell/CC configuration can be summarized as follows:

- UE is configured with one or multiple serving cells, i.e. one PCell and optionally one or more SCells.

  - PCell is configured with one UL CC and one DL CC.

  - SCell can be configured with one DL CC and one UL CC, or only with one DL CC.

- For the serving cell configured with both UL CC and DL CC, the UL and DL CC belongs to the same band pair (NOTE: band pair is defined in RAN4 spec).

- If UE is configured with multiple serving cells, UE is required to support the simultaneous UL and DL transmission based on capabilities.

- For UL Tx switching, UE perform transmission via TDM across multiple UL CC through dynamic switching.

- One serving cell can be configure with one SUL CC, the serving cell contains 1 DL CC and two UL CCs (i.e. NUL and SUL), but the transmission over two UL CCs is in TDM.

- One serving cell can be configured with multiple PCI in MIMO mTRP mode. Except for the PCI of the serving cell, which is used for cell/connection management, all other additional PCIs are used only for data transmission.

In NR, from RRC configuration perspective, the CA configuration is based on the configuration of the serving cell.

Figure 1: NR CC/Cell model

## UL and DL spectrum efficiency

In NR, although the UL CC frequency and DL CC frequency of one serving cell can be explicitly indicated which can support 1 UL CC and 1 DL CC flexible association requirement, the general 3GPP requirements are that the uplink and downlink CC need to meet the frequency band pairing requirements.

Some new requirements to improve the spectrum efficiency (as follows) have emerged in 6G, which requires RAN2 to consider providing a flexible configuration method to associate the UL and DL CCs:

- UL CC and DL CC decoupling

- The number of UL CCs can exceed the number of DL CCs

Observation 1: The flexible and diverse spectrum usage requirements in 6G requires the RRC configuration to allow the flexible association between UL and DL CCs.

Due to potential new uplink and downlink configuration requirements in the future, 6G configuration design can support the following scenarios:

- Case 1: The associated UL CC and DL CC can come from the same or different bands

- Case 2: One UL CC is associated with two DL CCs, one DL CC is as reference CC, and another DL CC is for dedicated data transmission.

- Case 3: The number of UL CC > the number of DL CC

- Case 4: The number of DL CC < the number of UL CC

To support the above cases, we can keep the existing serving cell configuration model to provide the configuration, and assume one cell includes one UL CC and one DL CC, as follows:

- RRC configuration includes a set of DL CC configurations, and a set of UL CC configurations

- RRC configuration includes a set of serving cell configuration

- For each serving cell configuration, it can include one UL CC config and one DL CC config

The associated UL CC config and DL CC config only provide IDs from the UL config set and DL config set, respectively. The UE associates the corresponding configuration with the configuration set based on the ID.

Figure 2: Example of the RRC configuration to support the flexible UL and DL association

Proposal 1: 6G RRC configuration should support the flexible UL and DL association under single-CC serving cell configuration model.

## Cross-CC Initial Access

In field deployments, operators may deploy multiple CCs in the same area for different purposes. A typical multi-CC deployment is an overlapping deployment of coverage frequency layer and capacity frequency layer:

- The coverage frequency layer is in lower frequency band and used for IDLE/INACTIVE UE to camp on.

- The capacity frequency layer is in higher frequency band and mainly used to for UE dedicated transmission.

- NOTE: The cells on capacity layer have larger bandwidth and smaller coverage than the cells on coverage layer.

Figure 3: Example of the coverage layer + capacity layer multi-CC deployments

In this deployment, from UE point of view, UEs are expected to camp on cells on the coverage layer to avoid frequency cell reselection and performs the data transmission via the cells on the capacity layer to have the better transmission service. From network point of view, all IDLE/INACTIVE UEs camping on the same frequency layer can help network reduce the number of CC used to paging transmission, thereby saving network resources and energy used for paging.

Observation 2: In the multiple CC deployment, from both NW and UE perspective, it is desirable for UE in IDLE/INACTIVE state to camp on the coverage layer and to perform the UE dedicated data transmission on the capacity layer.

According to current NR design, the IDLE/INACTIVE UE performs the initial access on the camping cell. Assuming all IDLE/INACTIVE UEs camp on coverage frequency layer (F1), if network would like to provide the service to CONNECTED UE via capacity layer (i.e. F2), UE will first perform the initial access procedure and establish the RRC connection on F1 and then network provides the configuration to 1) handover the UE from F1 cell to F2 cell or 2) add F2 as SCell, as shown in Figure 4 .

Throughout the entire procedure from initial access to switching CC for connected data transmission, the following issues can be identified:

- The initial access congestion is likely to occur in the coverage layer, as all initial access procedures are performed there.

- The procedure of switching to the cell on capacity layer for UE dedicated transmission has a long delay and requires high signaling overhead.

Figure 4: NR procedures for adding Cell#2 (on capacity layer) as a serving cell for data transmission

Observation 3: According to NR design, if all UEs in IDLE/INACTIVE state camp on coverage layer and then switches to capacity layer for UE dedicated transmission after entering the CONNECTED state, it will cause the following two issues: 1) initial access congestion on coverage layer, and 2) long delay and high signaling overhead to switch UE to capacity layer.

In 6G design, to address the above issues in the "coverage layer + capacity layer" multi-CC deployment, RAN2 should study the cross-CC initial access procedure to shorten the latency of starting the capacity layer to provide services to UEs, as shown in Figure 5. That is, a UE not in CONNECTED state can camp on the coverage CC and when entering CONNECTED state, switch to the capacity CC during the initial access procedure.

Figure 5: Illustration of the UE operation during the cross-CC initial access procedure

Figure 6:  3 Phase of the cross-CC initial access procedure

For 6G cross-CC initial access design, RAN2 study on the UE and network operation can focus on the three phases as shown in figure 6:

- Phase 1: Before UE initial access

  - From UE perspective

    - UE camps on the cell on coverage layer, and perform all the UE operation in IDLE/INACTIVE mode on this cell, e.g. SIB, Paging, measurement, mobility, etc.

  - From network perspective

    - Network pages the UEs over the coverage layer

    - On capacity layer, if there is no UE working there, network can set it to energy saving mode.

- Phase 2: During UE initial access

  - From UE perspective

    - Acquire the initial access related configuration over capacity layer

    - Select the frequency (F2) for initial access and connected data transmission purpose

    - The access control needs to consider both coverage layer and capacity layer info

    - FFS when to switch from coverage layer to capacity layer.

  - From network perspective

    - Transmit RS to help UE perform the measurement on frequency on capacity layer

    - Support the initial access and access control over capacity layer.

- Phase 3: After initial access and starts the UE dedicated transmission.

  - From UE perspective

    - Keep UE connection over capacity layer.

  - From network perspective

    - Provide the service over capacity layer.

Proposal 2: RAN2 6G study should support the cross-CC initial access procedure, and study the UE and network operation in three phases:

- Phase 1: Camp on coverage layer before UE initial access procedure

- Phase 2: During cross-CC initial access procedure

- Phase 3: Keep connection on capacity layer after UE initial access procedure and entering CONNECTED state.

Proposal 3: Before UE initial access, UE camps on single CC for paging, SIB acquisition, and cell reselection evaluation.

Proposal 4: During the cross-CC initial access procedure, RAN2 study on the following directions:

- CC selection for initial access and UE transmission in CONNECTED state

- Potential impact on the access control and paging

- Method to provide the system information related to the cross-CC initial access

- Timing to perform the CC switching during the initial access, e.g. before initiating access, during initial access (at which step?), or immediately after the initial access.

## 6G CA

Regarding the spectrum aggregation in CONNECTED state, according to the following RANP#109 agreements (RP-252909), 6G RAN will focus on MRSS+CA deployment.

- The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

In NR CA, PCell is typically deployed on coverage layer and undertakes many special functions in CA connection, e.g. management and recovery of RRC connection, and transmission of important control signaling, always activated. The main functions supported on PCell and SCell are briefly summarized in the following Table.

Table. The functions supported on PCell and SCell in NR CA

In MRSS+ CA deployment, on coverage layer, the frequency resources are shared between 5G RAT and 6G RAT, and there are not many resources available for UE dedicated data transmission.

For the case that PCell is configured on coverage layer, PCell can only be used to provide the coverage and basic connection management (e.g. link/connection recovery), and the SCell/SCC on the capacity layer should play a more important role in connection management related to data transmission.

Observation 4: In MRSS and CA deployment, if PCell is on coverage layer, the UE connection and data transmission is mainly on SCell and some PCell function in NR should be considered offloading to SCell, e.g. RACH for SR purpose.

For the case that PCell is configured on capacity layer and SCell is configured on coverage layer, PCell has the smaller coverage than SCell. so that it is very likely that PCell has radio problem but SCell ​​works well. If NR CA design is followed, UE will initiate RRC connection re-establishment procedure, which will seriously affect the current UE RRC connection.

Figure 7: Scenario and Proposal for PCell Recovery via SCell

Observation 5: In MRSS and CA deployment, if PCell is on capacity layer, due to coverage issues, recovery scenarios are more frequent than NR, and the PCell recovery over coverage layer (SCell) should be considered.

Since the CA situation in MRSS+CA deployment is different from the traditional NR CA deployment and is more dependent on SCell/SCC, it is necessary to study whether SCC/SCell can support more connection management functions.

Proposal 5: RAN2 study on 6G CA should consider the following two directions:

- Support more functions on SCell (e.g. RLM, RA-SR)

- PCell failure recovery over SCell .

# Conclusions

This contribution presents our views on 6G spectrum aggregation from RAN2 perspective. We have the following proposals:

<UL and DL spectrum efficiency>

Observation 1: The flexible and diverse spectrum usage requirements in 6G requires the RRC configuration to allow the flexible association between UL and DL CCs.

Proposal 1: 6G RRC configuration should support the flexible UL and DL association under single-CC serving cell configuration model.

<Cross-CC initial access>

Observation 2: In the multiple CC deployment, from both NW and UE perspective, it is desirable for UE in IDLE/INACTIVE state to camp on the coverage layer and to perform the UE dedicated data transmission on the capacity layer.

Observation 3: According to NR design, if all UEs in IDLE/INACTIVE state camp on coverage layer and then switches to capacity layer for UE dedicated transmission after entering the CONNECTED state, it will cause the following two issues: 1) initial access congestion on coverage layer, and 2) long delay and high signaling overhead to switch UE to capacity layer.

Proposal 2: RAN2 6G study should support the cross-CC initial access procedure, and study the UE and network operation in three phases:

- Phase 1: Camp on coverage layer before UE initial access procedure

- Phase 2: During cross-CC initial access procedure

- Phase 3: Keep connection on capacity layer after UE initial access procedure and entering CONNECTED state.

Proposal 3: Before UE initial access, UE camps on single CC for paging, SIB acquisition, and cell reselection evaluation.

Proposal 4: During the cross-CC initial access procedure, RAN2 study on the following directions:

- CC selection for initial access and UE transmission in CONNECTED state

- Potential impact on the access control and paging

- Method to provide the system information related to the cross-CC initial access

- Timing to perform the CC switching during the initial access, e.g. before initiating access, during initial access (at which step?), or immediately after the initial access.

<6G CA>

Observation 4: In MRSS and CA deployment, if PCell is on coverage layer, the UE connection and data transmission is mainly on SCell and some PCell function in NR should be considered offloading to SCell, e.g. RACH for SR purpose.

Observation 5: In MRSS and CA deployment, if PCell is on capacity layer, due to coverage issues, recovery scenarios are more frequent than NR, and the PCell recovery over coverage layer (SCell) should be considered.

Proposal 5: RAN2 study on 6G CA should consider the following two directions:

- Support more functions on SCell (e.g. RLM, RA-SR).

- PCell failure recovery over SCell .

# References

- RP-252912	Study on 6G Radio