# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508453
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	10.3.3.2

Source:	Apple

Title:	Views on 6G Energy Efficiency

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# 1 Introduction

The SID of 6G Radio (SID RP-251881) was agreed in RAN#108 [1]. 6G energy efficiency study is one of the most important SID objectives involving all RAN WGs:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device.

Energy efficiency was initially discussed in RAN2#131b [2], and it was agreed to strive for joint design of NW and UE power saving and to identify which 5G features can be considered and improved in 6G.

Design goal 1. Strive to develop solutions that consider network energy efficiency and UE power saving

- For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we share our view on study direction of 6G Network and UE energy efficiency, including:

- 5G learnt lessons and 6G design principle

- Views on directions of 6G UE energy efficiency

  - 5G UE PS features inherited to 6G

  - Study directions of 6G UE Power Saving

- Views on directions of 6G Network energy efficiency

  - 5G NES features inherited to 6G

  - Study directions of 6G NES

# 2 Discussion

Energy efficiency study is always a hot topic in 5G, which includes UE power-saving mechanisms and Network Energy Saving (NES) features. We think it is important to first analyze the lessons learnt in 5G and achieve consensus on design principles to avoid these lessons in 6G study.

## 2.1 5G learnt lessons and 6G design principle

### 2.1.1: Lessons learnt in 5G UE power saving (UPS) study

UE power-saving mechanisms were introduced from Release-15. As illustrated in Figure. 1, a lot of features have been specified by Rel-19.

Figure.1 All the UE power saving features specified in 5G

However, most of them are not commercially deployed. We think the three lessons learnt include:

- Network vendors do not have incentive to deploy 5G UE power saving features which benefit only to the UE.

In 5G, UE power saving is usually regarded to be mutual exclusive with network energy saving. Few UE power saving features can bring benefit to network but just add network complexity and may degrade system performance.

- As few UE power saving features are 5G day-1 design, backward compatibility issues lead to low device penetration in the market.

Please note that although BWP and RRC_INACTIVE mode are Rel-15 features, they are optional features which were hardly deployed in Rel-15.

- Multiple duplicated UE power saving features are specified for the same purpose.

For example, both PDCCH skipping and Search Space Group Switching (SSGS) are targeted to reduce PDCCH monitoring. As another example, both PEI and LP-WUS are targeted to reduce paging monitoring.

Observation 1: The three lessons learnt in 5G UE power saving (UPS) study include:

- Few UPS features can benefit to both NW and UE. NW vendors do not have incentive to deploy them.

- Few UPS features are 5G day-1 design. Backward compatibility issues lead to low device penetration.

- Multiple duplicated UPS features for the same purpose (e.g. PDCCH skipping and SSGS).

### 2.1.2: Lessons learnt in 5G Network Energy Saving (NES) study

5G NES started in Rel-18 and is further enhanced in Rel-19. Figure. 2 summarizes all the specified NES features.

Figure.2 All the Network Energy Saving (NES) features specified in 5G

However, there were a lot of complains about 5G NES study. As 5G UPS, we think the three lessons learnt include:

- Limited consideration on how NES features jointly work with UE power saving features.

Similarly, most 5G NES designs are regarded to be mutual exclusive with 5G UPS. For example, OD-SSB leads to less frequent transmission of essential synchronized signal. It increases complication and difficulty for UE because UE depends on SSB to maintain AGC and time/frequency synchronization. One exception is Cell DTX/DRX which essentially allows more sleeping time common for both NW and UE. However, 5G only supports partial alignment between Cell DTX/DRX and UE CDRX (i.e. the UE's CDRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa). We think it is not sufficient and 6G needs to introduce better alignment mechanism.

- NES was introduced in late stage of 5G. Only limited NES gain due to backward compatibility consideration.

One typical example is paging adaptation which requires NW to configure two paging resources for legacy and new UEs and send two sets of paging messages. It goes against the intention of NES. 6G should not be bound by such legacy constraints, allowing for the exploration of cleaner and more energy-efficient techniques.

- Duplicated signaling design for similar function.

One typical example is OD-SSB and SSB adaptation. Both can dynamically adapt the periodicity of SSB transmission in SCell but different adaptation signaling are specified (MAC-CE for OD-SSB and DCI for SSB adaptation).

Observation 2: The three lessons learnt in 5G Network Energy Saving (NES) study include:

- Limited consideration on how NES features jointly work with UE power saving features.

- NES was introduced in late stage of 5G. Limited NES gain due to backward compatibility consideration.

- Duplicated signaling design for similar function (e.g. MAC-CE for OD-SSB and DCI for SSB adaptation).

### 2.1.3: 6G design principle to address 5G learnt lessons

When we put Observation 1 and Observation 2 together, we can see that the three lessons in 5G are quite aligned:

- The first lesson requires 6G to design joint energy efficiency solution with the assumption that NES feature and UPS feature are default enabled together. This ensures 6G design can achieve mutual benefit to both NW and UE.

- The second lesson requires 6G to design day-1 energy efficiency solution in native way to avoid compatibility issue. Please note that the compatibility issue includes both backward compatibility with 5G and compatibility across different UE types.

- The third lesson requires 6G to design efficient signaling to enable / disable NES feature or UPS feature or both. And RAN2 should strive for a single solution for same / similar energy saving function.

Thus, we propose the following 6G energy efficiency design principles common to both NES and UPS.

Proposal 1: 6G energy efficiency design follows the below design principles common to both Network Energy Saving (NES) and UE Power Saving (UPS):

- To achieve mutual benefit to both NW and UE, design joint solutions with the assumption that NES feature and UPS feature are default enabled together.

- To avoid compatibility issue, design day-1 solutions common to all UE types without degradation of both user experience/QoS and system performance.

- To ease deployment, design efficient signalling to enable/disable NES feature or UPS feature or both.

## 2.2 Views on directions of 6G UE energy efficiency

### 2.2.1: 5G UE PS features inherited to 6G

As illustrated in Table. 1, different UE PS solutions were introduced in different releases. It is important to first identify whether one or some of the 5G designs can be inherited to 6G.

In Table.1, we summarize our analysis on our view on which 5G UE PS features can be inherited and enhanced in 6G from RAN2 perspective. According to the Table, we prefer to first agree inheriting the following RAN2-led 5G UPS features as starting point.

- CDRX operation

- SCell activation/deactivation

- PS UAI

- eDRX operation

- PEI/LP-WUS and paging subgroup-based paging reception

- Measurement relaxation/offloading

- Inactive RRC state.

Proposal 2: RAN2 assume that 6G UE power saving design can inherit the following RAN2 led 5G UE PS features as starting point:

- CDRX operation; SCell activation/deactivation, PS UAI, eDRX operation, WUS and subgroup-based paging, measurement relaxation/offloading.

- Whether LP-WUS is mandatory to be supported is not discussed/decided in RAN2.

| UPS feature | RRC state | Area | 5G Design | Inherit to 6G? | Leading WG |
| --- | --- | --- | --- | --- | --- |
| UE C-DRX Operation | Connected | Time domain | 1. Dual DRX group in CA 2. DCP based DRX control 3. LP-WUS specific DRX control | Yes. Need to further consider 1) Unified CDRX framework 2) Traffic based control | RAN2 |
| SCell activation/deactivation | Connected | Frequency domain | 1. MAC CE based activation/deactivation | Yes. | RAN2 |
| UAI | Connected | UE situation | 1. UE preference ONCC/BW/MIMO/Scheduling offset/Traffic pattern/OH/RRC state/QoS | Yes. Need to further consider how to let network enable this feature and respect UE preference. | RAN2 |
| BWP switching | Connected | Frequency domain | 1. RRC controlled, DCI based, Timer based BWP switching 2. BWP specific PHY config | Yes. Depend on RAN1 6G BWP design. | RAN1 |
| Cross-carrier scheduling | Connected | PDCCH | 1. Support the PDCCH on PCell/SCell to schedule PDSCH/PUSCH on other serving cells | Yes. | RAN1 |
| PDCCH skipping/SSSG | Connected | PDCCH | 1. Skip PDCCH in DRX active time by DCI 2. Adjust SearchSpace in DRX active time by DCI | Yes. Let RAN1 decide whether to support one or both. | RAN1 |
| BFD/RLM Relaxation | Connected | Measurement | 1. Relax measurement for BFD/RLM when serving quality is good | FFS The benefit is limited. | RAN4 |
| I-DRX operation | Idle/inactive | Time domain | 1. Support eDRX | Yes | RAN2 |
| Paging reception | Idle/inactive | Time domain | 1. Subgrouping and PEI triggered paging reception 2. Subgrouping and LPWUS triggered paging reception 3. Cross-slot paging reception 4. TRS occasion for synchronization in SIB17 | Yes for 1,2,3. 1) Unified paging reception framework to cover 1 and 2. 2) TRS is up to RAN1 | RAN2 on UE paging reception direction |
| Measurement relaxation | Idle/inactive | Measurement | 1. Relax neighbor measurement in low mobility and/or in not cell edge condition | Yes. Need to further consider 1) consider serving cell relaxation 2) merge with LPWUS based measurement optimization | RAN2/4 |
| LP-WUS based measurement relaxation/offloading | Idle/inactive | Time domain | 1. Relax serving/neighbor measurement on MR when serving quality is good. 2. Offload serving measurement from MR to LR when serving quality is much better. | Yes. Need to further consider 2) merge with non-LPWUS based measurement relaxation | RAN2/4 |
| RRC INACTIVE state | RRC State | RRC State | 1. In this state, UE with CN/NAS connection can suspend AS link and perform UE based mobility when there is no data activity. | Yes. Rely on 6G RRC state design | RAN2 |

Table. 1: Analysis on which 5G UE PS features can be inherited to 6G

### 2.2.2: Study directions of 6G UE Power Saving

In this section, we discuss the specific 6G UE PS feature study directions on top of 5G design as baseline.

#### 2.2.2.1 6G C-DRX operation

As illustrated in Table.1, we need to consider the simplification and improvement on top of 5G CDRX design from the following two aspects:

- Unified C-DRX operation

5G NR introduced multiple CDRX operations in different releases to achieve the same goal with the different UE capabilities, which is redundant.

  - For basic CDRX operation, UE always wakes up in DRX onduration duration.

  - For DCP based CDRX operation, UE wakes up in DRX onduration duration based on the DCP indication. If DCP is not detected, UE can wake up in the associated DRX onduration directly (based on configuration).

  - For LPWUS based CDRX operation, therea re two options

  - In Option 1-1, the design is almost identical to the DCP design, except for WUS signalling.

  - In Option 1-2, UE only wakes up upon receiving the LPWUS indication, and the potential wake occasion is not restricted by the legacy DRX cycle and DRX onduration timer configuration.

In 6G, we can design the unified WUS based CDRX framework to cover all 5G CDRX operations, and NR LP-WUS based CDRX operation in Option1-2 can be the starting point.

The key points of the 6G WUS based CDRX framework (based on LP-WUS CDRX option 1-2) are as follows, and are illustrated in Figure. 3.

  - NW configures the WUS config in the periodical way.

  - In WUS config, the TDM pattern is mandatory provided, but the WUS signal config can be absent or present.

  - UE follows the WUS TDM pattern to determine potential wake up occasion.

  - If UE is outside DRX active time.

  - If WUS signal config is absent, UE always starts DRX onduration Timer and enters DRX active time from the associate wake up occassion.

  - If WUS signal config is present, UE monitors WUS signaling (e.g. DCP, LP-WUS), and UE follows the detected WUS to start DRX onduration Timer and enter DRX active time.

  - When UE is in DRX active time

  - UE starts DRX Inactivity Timer/HARQ RTT Timer/Retransmission Timer according to the transmission (NOTE: it’s the same operation applicable in all 5G DRX operations.)

  - NOTE: The other event to trigger UE enters DRX active time (e.g. SR, RACH) can be kept.

Figure. 3 Unified 6G CDRX framework

Observation 3: One unified CDRX framework can be designed to cover all NR CDRX operations.

- Traffic based CDRX control

In NR, the CDRX configuration and operation are the same regardless of which traffic being transmitted and whether it’s only for padding transmission.

Regarding the DRX inactivity timer, the use case is when the transmission may trigger data activity over a subsequent time period. For CONNECTED UE may have no data transmission but transmit some control plane signaling (e.g. periodically measurement reporting which is used for network load balance purpose) which will not lead to the subsequent data activity. In such case, where the transmission will not lead to the subsequent data activity, starting the DRX inactivity timer is meaningless and not good for UE power.

When uplink skipping feature is disabled, the DRX inactivity timer still needs to be enabled and keeps UE in DRX active time when only padding data is being transmitted in uplink. This design is against the UE PS principle and has no other significance. In 6G we need to avoid this situation in CDRX design.

Observation 4: Starting the DRX inactivity timer regardless of the transmission situation is neither beneficial nor meaningful to the terminal.

#### 2.2.2.2 6G paging

NR introduces two WUS based paging features. In R17, PEI based paging mechanism was introduced, where PEI was used as the WUS signal and the design is based on the PDCCH DCI. In R19, LP-WUS based paging mechanism was introduced, where LP-WUS was used as the WUS signal and the design is based on new signal.

Because the paging frameworks based on PEI and LP-WUS are very similar and are mainly used for different UE device types (different HW requirements), in 6G design, we should also consider the unified subgroup and WUS based paging mechanism, and the study on WUS signal design can be left to RAN1.

Observation 5: One unified WUS and subgroup based paging reception framework can be designed to cover all NR paging operations.

#### 2.2.2.3 6G measurement relaxation and offloading

NR introduces neighbor measurement relaxation and LP-WUS based serving/neighbor measurement relaxation/offloading. Even though they are applied for the different UE device types (equipped with LR or not), we can see the similarity and further unified design needs to be considered.

In 6G IDLE/INACTIVE measurement framework, we can consider three measurement modes in general and the mode switching is based on the low mobility or not-cell-edge condition.

  - Normal measurement mode: UE performs the serving/neighbor measurement according to normal requirement.

  - Measurement relaxation mode: UE performs the serving/neighbor measurement with longer measurement interval when certain condition is met, e.g. UE is not at cell edge or UE is in low mobility condition.

  - Measurement offloading mode: UE doesnot perform the neighbor measurement, and serving measurement is only performed on LR not on MR.

Observation 6: A unified measurement mode design can cover power-saving solutions for NR measurements and simplify UE operation.

#### 2.2.2.4 Summary of starting point of 6G UE PS study directions

Based on the discussion in previous sub-sections, we propose:

Proposal 3: As starting point, RAN2 study the following directions of 6G UE PS design

- For C-DRX operation, consider the unified WUS-based CDRX framework and the different DRX timer operations based on different transmission conditions.

- For UE paging operation, consider the unified WUS-based paging reception framework.

- For measurement, consider the unified measurement models (i.e. normal, relaxation, offloading) to cover the UE measurement operations, regardless of whether LP-WUS is supported.

## 2.3 Views on RAN2 directions of 6G Network energy efficiency

### 2.3.1: 5G NES features inherited to 6G

As illustrated in Table. 2, different NES solutions were introduced in Rel-18 and Rel-19. It is important to first identify whether one or some of the 5G designs can be inherited to 6G.

In Table. 2, we summarize our analysis on the restriction of all 5G NES features, and which can be inherited to 6G. According to the Table, we prefer to first agree inheriting the following three RAN2-led 5G NES features as starting point:

- Cell DTX/DRX

- paging adaptation

- NES specific mobility enhancement

Proposal 4: RAN2 assume that 6G NES design can inherit the following RAN2 led 5G NES features as starting point:

- Cell DTX/DRX: 5G design as baseline at least for RRC_CONNECTED state.

- Paging adaptation: Support non-uniform/bundled PF/PO but solution needs further discussion.

- NES specific mobility enhancement: 5G NES CHO design as baseline.

Meanwhile, although it also makes sense to inherit some NES features leading by RAN1/RAN4 (e.g. RACH adaptation, SSB-less SCell, OD-SSB and spatial/power adaptation), we think it is better to first evaluate these features in the leading WG. Otherwise, it may lead to duplicated discussion and possible misalignment with other WGs. Thus, we propose:

Proposal 5: RAN2 assume that following 5G NES features need to be first discussed in other WGs:

- RACH adaptation, SSB-less SCell, OD-SSB and SSB adaptation, OD-SIB1, spatial/power adaptation.

| NES feature | Use case | Main restriction(s) in 5G | Whether inherit to 6G? | Leading WG |
| --- | --- | --- | --- | --- |
| Cell DTX/DRX | Allow gNB sleeping under lower system traffic loading. | 1. Only support partial alignment with UE CDRX. 2. User experience / service QoS will be degraded. 3. Limited NES gain as common signal is not impacted by Cell DTX/DRX. | Yes: 5G design as starting point at least for CONNECTED state. Whether applicable for IDLE/INACTIVE state needs to wait RAN1 initial access design. | RAN2 |
| NES state dependent CHO | Encourage offloading UE to another cell by relaxing CHO threshold when source cell enters NES state. | 1. Only CHO is considered (i.e. cell reselection and LTM are not considered) 2. NES state of neighbor cell is not considered. | Yes: 5G CHO design as starting point. Can further consider NES state of neighbor cell and other mobility types. | RAN2 |
| Inter-band SSB-less SCell | Extend Rel-15 intra-band SSB-less SCell to inter-band co-located CA | Not support the scenario of non-co-located CA | Yes for at least intra-band/inter-band SSB-less SCell in co-located CA. | RAN4 |
| Spatial and/or power adaptation | All or subset of antenna elements associated to a logical antenna port is dynamically disabled/enabled |  | Yes, but depend on 6G MIMO design. | RAN1 |
| Legacy UE barring | Bar legacy UE to maximize NES gain | May lead to coverage hole | No: NES should be targeted for 6G day-1 design | RAN2 |
| Paging adaptation | Enable non-uniform/bunded PF/PO for longer gNB sleeping time | 1. Limited NES gain due to backward compatibility (sending two sets of paging) 2. Not flexible (only extend N to 32). | Yes: Support flexible non-uniform/bunded PF/PO from day-1. | RAN2 |
| RACH adaptation | Activate an additional RACH resource when system loading is high | Limited NES gain due to backward compatibility (two sets of RACH resource) | Yes, but detailed solution depends on 6G RACH baseline | RAN1 |
| OD-SSB | Dynamic activation of OD-SSB for SCell activation latency reduction | Only support NW triggered solution. | Yes, but the adaptation signaling should be unified. Also need to wait RAN1 initial access design. | RAN1 |
| SSB adaptation | Dynamic adaptation between different SSB periodicity | Different adaptation signaling for the similar purpose (MAC-CE for OD-SSB, DCI for SSB adaptation) |  | RAN1 |
| OD-SIB1 | Decouple coverage carrier (with SIB1) and capacity carriers (with OD-SIB1) | Limited NES gain because SSB is still transmitted in capacity carriers. | Need to jointly consider with SSB design | RAN1 |

Table. 2: Analysis on which 5G NES features can be inherited to 6G

### 2.3.2: Study directions of 6G NES

In this section, we discuss the specific 6G NES study directions on top of 5G design as baseline.

#### 2.3.2.1 Study directions of 6G Cell DTX/DRX

As illustrated in Table.2, we think there are 3 restriction of 5G Celll DTX/DRX design from RAN2 perspectives. 6G design needs to well address them.

Restricion#1:5G Cell DTX/DRX only supports partial alignment with UE CDRX

We think the alignment between Cell DTX/DRX and UE CDRX is the key to achieve mutual benefit to both NW and UE. In 5G, it was also discussed in Rel-18, but only partial alignment (i.e. the UE's CDRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa) is supported. It is captured in TS 38.300 [3]:

15.4.2.3	Cell DTX/DRX

Omit unrelated part

Once the gNB recognizes there is an emergency call or public safety related service, the network should ensure that there is no impact to that service (e.g. it may release or deactivate cell DTX/DRX configuration). The network should also ensure that there is at least partial overlapping between UE's connected mode DRX on-duration and cell DTX/DRX active duration, i.e. the UE's connected mode DRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa.

Figure. 4 Issue of 5G partial alignment mechanism

The main reasoning of only supporting partial alignment is due to some UE vendors’ concern on degradation of UE QoS (e.g. restriction on UE CDRX configuration). However, the partial alignment can only ensure there is no timing offset drifting between Cell DTX/DRX and UE CDRX, but it may result in NW energy inefficiency as illustrated in Figure. 4. In Case 2, although it satisfies the partial alignment requirement, gNB can’t sleep although Cell DTX/DRX is enabled. Thus, we think 6G should introduce an enhanced alignment mechanism to achieve energy efficiency to both NW and UE.

Observation 7: 5G Cell DTX/DRX only supports partial alignment, which can only ensure there is no timing offset drifting between Cell DTX/DRX and UE CDRX, but it may result in NW energy inefficiency. 6G should improve it.

Restricion#2: User experience / service QoS will be degraded under Cell DTX/DRX

5G Cell DTX/DRX doesn’t well consider user experience or service QoS, which are further reflected in the following two restrictions:

- Active duration of Cell DTX/DRX can’t be extended for UE’s pending transmission (i.e. Cell DTX/DRX doesn’t specify mechanism similar to inactivity timer of UE CDRX).

- UL/DL initial transmission (even for high priority traffic) is not allowed during inactive duration of Cell DTX/DRX.

During Rel-18, UE vendors raised concern on the risk of degradation of service QoS. However, no solution was specified. The main reason is because Cell DTX/DRX is an enhancement feature in 5G and thereby can be de-configured when the NW identify high priority traffic is pending. In 6G, we think this argument is not valid anymore because Cell DTX/DRX is targeted for a day-1 feature. Thus, to provide incentive for UE vendor to deploy Cell DTX/DRX, we think it is important to study UE service QoS enhancement under Cell DTX/DRX.

Observation 8: 5G Cell DTX/DRX doesn’t well consider user experience or service QoS because it was regarded as an enhancement and can be de-configured when the NW identify high priority traffic is pending. 6G should guarantee user experience / QoS to provide incentive for UE vendor to deploy Cell DTX/DRX.

Restricion#3: Limited NES gain as common signal is not impacted by Cell DTX/DRX

In 5G, transmission of common signals (including SSB, SIB, paging and RACH) is not impacted by Cell DTX/DRX due to backward compatibility consideration. It means that gNB must keep waking up during inactive duration of Cell DTX/DRX to monitor these common signals, which significantly limits NES gain.

In 6G, as Cell DTX/DRX is day-1 feature, it is a good opportunity to maximize NES gain by at least restricting transmission of common signals. Our initial consideration is that 6G SSB transmission may not be impacted by Cell DTX/DRX because SSB is important for the UE to maintain AGC and time/frequency synchronization. On the other hand, it should be possible to restrict paging and RACH during inactive duration of Cell DTX/DRX. However, since common signal highly depends on initial access design in RAN1, we prefer to first wait RAN1 progress on initial access design.

Observation 9: Although NES gain of Cell DTX/DRX highly depends on whether/how common signals can be transmitted during inactive duration, RAN2 needs to first wait RAN1 progress on initial access design.

#### 2.3.2.2 Study directions of 6G paging adaptation

According to Table.2, we think 5G paging adaptation has two restrictions from RAN2 perspectives. 6G design needs to well address them.

Restricion#1: Limited NES gain due to backward compatibility (sending two sets of paging)

Due to backward compatibility, NW needs to always configure two sets of paging resource for legacy UE and new UEs respectively when 5G paging adaptation is enabled. Correspondingly, NW also needs to send two sets of paging messages for legacy UE and new UEs respectively. It significantly limits its NES gain.

6G paging adaptation should be targeted for day-1 feature and should not be bound by such legacy constraints, allowing for the exploration of cleaner and more energy-efficient techniques.

Restricion#2: The 5G solution is not flexible (only extending N and Ns with one new value)

As illustrated in Figure.5, 5G paging adaptation studied two solutions to achieve non-uniform/bundled PF:

- Option a): bundle PFs by revising the paging formula in TS 38.304.

- Option b): concentrate all POs in one PF by extending the values of N and increasing POs per PF.

The conclusion is that both solutions can work but more companies had concern on option a) because it will lead to major specification impacts (i.e. introduce a new formula in TS 38.304). Thus, option b) was finally adopted. However, due to concern of backward compatibility, N is only extended to 32 and Ns is only extended to 8.

It is obvious that 5G paging adaptation solution can’t be directly reused in 6G because it is not flexible enough to handle 6G use cases. 6G paging adaptation should design a flexible solution to enable / disable non-uniform/bunded PF/PO distribution from day-1.

Observation 10: 5G paging adaptation solution has limited NES gain due to backward compatibility and it can’t flexibly enable PF bundling for all 6G use cases.

Figure. 5 Two studied solutions of 5G paging adaptation to achieve non-uniform PF

#### 2.3.2.3 Study directions of NES specific mobility enhancement

In Rel-18, NES specific CHO was introduced. Its basic idea is to encourage UE offloading to another cell by relaxing CHO threshold when source cell enters NES state (upon reception of the “NES CHO” bit in DCI 2-9). Otherwise, the UE may either encounter RLF when the source cell is turned off, or the UE’s service QoS is degraded when the source cell enables Cell DTX/DRX.

We think that it makes sense to extend the idea in three directions for 6G:

- Direction#1: Extend the NES specific CHO to other HO type (e.g. LTM)

- Direction#2: Consider NES state of both source cell and neighbour cell

In Rel-18, some solutions were proposed to consider the NES state of neighbour cell in CHO. For example, the neighbour cell(s) in NES state may be deprioritized during CHO condition evaluation. However, it was not adopted because it is not clear how the UE can know the NES state of neighbour cells. Considering more dynamic signalling will be introduced in 6G to support Inter Cell Beam Management (ICBM), we think that 6G can reconsider this direction.

Observation 11: 6G can consider extend 5G NES CHO to other HO type and consider NES state of neighbour cells.

- Direction#3: Enhance cell reselection by considering NES state of both source cell and neighbour cell

In Rel-18, NES specific cell reselection enhancement was discussed in study item and captured in TR 38.864 [4]:

### 6.5.1 Cell selection/reselection Omit unrelated part The NW should be able to configure NES-capable UEs to prioritize/down-prioritize a specific NES cell or NES cells on a specific frequency. It is left to the WI phase whether the existing mechanism for cell (re)selection is sufficient according to the NES techniques specified.

However, it was not approved in normative phase due to limited TU. Meanwhile, please note that such cell reselection enhancement was adopted in Rel-19 OD-SIB1. As captured in TS 38.304 [5], NES-specific reselection priority and NES-specific excluded cell list were introduced to prioritize/deprioritize a NES frequency/cell.

#### 5.2.4.1 Reselection priorities handling Omit unrelated part If dedicated frequency priority parameters (odsib1-CellReselectionPriority, odsib1-CellReselectionSubPriority) are provided in system information, the UE supporting OD-SIB1 ignores the cellReselectionPriority and cellReselectionSubPriority in the system information and applies the dedicated ones to determine frequency prioritization. If intraFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores intraFreqExcludedCellList and does not consider the cell(s) (if any) in intraFreqODSIB1-ExcludedCellList as candidates for cell reselection. If interFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores interFreqExcludedCellList and does not consider the cell(s) (if any) in interFreqODSIB1-ExcludedCellList as candidates for cell reselection.

Thus, we think 6G can study both HO and Cell reselection enhancement considering NES state of both serving and neighbour cells.

Observation 11: Cell reselection enhancement was adopted in Rel-19 OD-SIB1, where NES-specific reselection priority and excluded cell list were introduced to prioritize/deprioritize a NES frequency/cell.

#### 2.3.2.4 Summary of starting point of 6G NES study directions

Based on the discussion in previous sub-sections, we propose:

Proposal 6: As starting point, RAN2 study the following directions of 6G NES design

- Cell DTX/DRX: a) enhance alignment mechanism between Cell DTX/DRX and UE CDRX; b) User experience/ service QoS enhancement under Cell DTX/DRX.

- Paging adaptation: design a flexible solution to enable / disable non-uniform/bunded PF/PO distribution from day-1.

- NES specific mobility enhancement: HO and Cell reselection enhancement considering NES state of both serving and neighbour cells.

# 3 Conclusion

In this contribution, we share our views on RAN2 study directions of 6G Network and UE power efficiency. Our observation and proposals are summarized in one page as follows:

Design principle of Network and UE power energy efficiency

Observation 1: The three lessons learnt in 5G UE power saving (UPS) study include:

- Few UPS features can benefit to both NW and UE. NW vendors do not have incentive to deploy them.

- Few UPS features are 5G day-1 design. Backward compatibility issues lead to low device penetration.

- Multiple duplicated UPS features for the same purpose (e.g. PDCCH skipping and SSGS).

Observation 2: The three lessons learnt in 5G Network Energy Saving (NES) study include:

- Limited consideration on how NES features jointly work with UE power saving features.

- NES was introduced in late stage of 5G. Limited NES gain due to backward compatibility consideration.

- Duplicated signaling design for similar function (e.g. MAC-CE for OD-SSB and DCI for SSB adaptation).

Proposal 1: 6G energy efficiency design follows the below design principles common to both Network Energy Saving (NES) and UE Power Saving (UPS):

- To achieve mutual benefit to both NW and UE, design joint solutions with the assumption that NES feature and UPS feature are default enabled together.

- To avoid compatibility issue, design day-1 solutions common to all UE types without degradation of both user experience/QoS and system performance.

- To ease deployment, design efficient signalling to enable/disable NES feature or UPS feature or both.

Views on RAN2 directions of 6G UE energy efficiency

Proposal 2: RAN2 assume that 6G UE power saving design can inherit the following RAN2 led 5G UE PS features as starting point:

- CDRX operation; SCell activation/deactivation, PS UAI, eDRX operation, WUS and subgroup-based paging, measurement relaxation/offloading.

- Whether LP-WUS is mandatory to be supported is not discussed/decided in RAN2.

Proposal 3: As starting point, RAN2 study the following directions of 6G UE PS design

- For C-DRX operation, consider the unified WUS-based CDRX framework and the different DRX timer operations based on different transmission conditions.

- For UE paging operation, consider the unified WUS-based paging reception framework.

- For measurement, consider the unified measurement models (i.e. normal, relaxation, offloading) to cover the UE measurement operations, regardless of whether LP-WUS is supported

Views on RAN2 directions of 6G Network energy efficiency

Proposal 4: RAN2 assume that 6G NES design can inherit the following RAN2 led 5G NES features as starting point:

- Cell DTX/DRX: 5G design as baseline at least for RRC_CONNECTED state.

- Paging adaptation: Support non-uniform/bundled PF/PO but solution needs further discussion.

- NES specific mobility enhancement: 5G NES CHO design as baseline.

Proposal 5: RAN2 assume that following 5G NES features need to be first discussed in other WGs:

- RACH adaptation, SSB-less SCell, OD-SSB and SSB adaptation, OD-SIB1, spatial/power adaptation

Proposal 6: As starting point, RAN2 study the following directions of 6G NES design

- Cell DTX/DRX: a) enhance alignment mechanism between Cell DTX/DRX and UE CDRX; b) User experience / service QoS enhancement under Cell DTX/DRX.

- Paging adaptation: design a flexible solution to enable / disable non-uniform/bunded PF/PO distribution from day-1.

- NES specific mobility enhancement: HO and Cell reselection enhancement considering NES state of both serving and neighbour cells.

# 4 References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.

[2] RAN2#131b, Chair Notes.

[3] TS 38.300-v18.6.0, NR and NG-RAN Overall Description; Stage 2, 2025-06.

[4] TR 38.864-v1.0.0, Study on network energy savings for NR (Release 18), 2022-12.

[5] TS 38.304-v19.0.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 19), 2025-09.