# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508457
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	8.1.2

Source:	Apple

Title:	Remaining issues on functionality-based LCM (including A104/S050/ N079)

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 AI/ML for air interface. And in RAN2#131b [2], some essential corrections were discussed. In this contribution, we discuss the following open issues related to LCM procedure of AI/ML based beam management:

- Periodic CSI report changing from ‘not applicable’ to ‘applicable’

- Discussion of important RILs

  - S050: UAI retransmission after mobility

  - A104: Simplification of procedure text on applicability reporting

  - N079: Empty list of data collection candidate configurations

# 2 Discussion

## 2.1 Periodic CSI report changing from ‘not applicable’ to ‘applicable’

This issue was discussed in RAN2#131b [2], but no conclusion can be made:

Periodic CSI report changing from ‘not applicable’ to ‘applicable’ R2-2507295 On the remaining issue and RILs Z001 Z002 for LCM of UE side model ZTE Corporation discussion Rel-19 NR_AIML_air-Core Proposal 2: When the previously configured inference configuration with periodic CSI reporting changes from "inapplicable" to "applicable", and the applicability report is transmitted via the RRCReconfigurationComplete , UE shall not activate this inference configuration automatically. The handling should be consistent with that when the report is sent via UAI. - Oppo thinks that we can just rely on UAI - Xiaomi thinks that we should only send the UAI not RRCreconfiguraiton complete as this just confuses the situations. Qualcomm thinsk that the UE should use RRC complete if it is available. - - Interdigital thinks we can capture this Understanding is: when the configuration becomes from inapplicable to applicable the UE sends an UAI. It is expected that the network can reconfigure the UE. Noted

According to above discussion Note, we think that the root cause of the divergence is due to the highlighted part of RAN2#131 agreement [1].

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

RAN2 made the above agreement before RAN1 provide input on when RRC layer submits the inference configuration to lower layer. In R1-2508122, RAN1 has provided the feedback to adopt option 2 (i.e. RRC layer only submits the applicable inference configuration to lower layer):

From RAN1 perspective, Option 2 can resolve the issue. - No additional RAN 1 impact was identified for Option 2.

With such RAN1 input, we think the part b) of RAN2#131 agreement will lead to the following issues:

- It will lead to different UE behaviors for different inference configurations which are reported as applicable in same RRCReconfigurationComplete message. It increases unnecessary UE complexity.

      - For the inference configurations received in immediate RRCReconfiguration, the UE will submit them in PHY layer and autonomously activate them.

      - For the inference configurations received in previous RRCReconfiguration, the UE will not submit them in PHY layer.

- The above behavior 1)-b) is not aligned with RAN1 input in R1-2508122.

Observation 1: Reporting applicability status change in RRCReconfiguration leads to below issue:

- It will lead to different UE behaviors for different inference configurations which are reported as applicable in same RRCReconfigurationComplete message. It increases unnecessary UE complexity.

- For the inference configurations received in immediate RRCReconfiguration, the UE will submit them in PHY layer and autonomously activate them.

- For the inference configurations received in previous RRCReconfiguration, the UE will not submit them in PHY layer.

- The above behavior 1)-b) is not aligned with RAN1 input in R1-2508122.

Furthermore, we think that the reporting applicability status change in RRCReconfigurationComplete is a corner case. It happens only when applicability status changes at the same time of RRCReconfiguration reception. Thus, it is not necessary to specify the UE behavior.

Observation 2: The reporting applicability status change in RRCReconfigurationComplete is a corner case. It happens only when applicability status changes at the same time of RRCReconfiguration reception. Thus, it is not necessary to specify the UE behaviour.

Given these issues, we think the simplest solution is to revert the RAN2#131 agreement, i.e. the UE reports applicability status change only via UAI.

Proposal 1a: Revert RAN2#131 agreement: RRCReconfigurationComplete shall only include applicability/inapplicability status for all inference configurations included in the immediately preceding RRCReconfiguration message.

Proposal 1b: For a CSI configuration which was reported as non-applicable, the UE can only use UAI to update the applicability status change and then needs to wait NW to send a followed RRCReconfiguration with inference configuration.

During offline discussion, there is another option that the non-applicable periodic configuration is not reported to the lower layer and the UE autonomously releases it. We disagree with solution. It is totally conflicted with the following RAN2#130 agreement:

Agreements

1.	When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

2.	The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

3.	The UE shall report when CSI-ReportConfig becomes not applicable

And some other agreements were on top of this agreement. So, we think cost of reverting the RAN2#130 agreements is significant and should not be pursued.

Proposal 2: As agreed in RAN2#130, when UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

## Discussion of important RIL

In this section, we discuss some important RIL(s) raised by companies in ASN.1 review.

### 2.3.1 S050: UAI retransmission after mobility

This RIL is described as follows:

[Description]: If the UE has send a UAI and within 1 second a handover has occurred or if the UE has send a UAI and a CHO or LTM cell switch has occurred, UE is required to resend the UAI. This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility For applicability reporting, based on the RRC specification, UE will send the new applicability after handover in RRCReconfigurationComplete. Thus, there is no need to retransmit the UAI. [Proposed Change]: Add suggested check: 2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup: 3> if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2: 4> if the UE assistance information was not initiated to report assistance information about the applicability of configurations subject to the applicability determination procedure: 5> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information; 5> start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration; 4> start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

First, the following justification (i.e. UAI retransmission interrupts ongoing mobility) is not correct.

This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility

In legacy RRC spec, we specify the UAI retransmission within 1s HO execution because the source cell may receive latest UAI after it initiated handover preparation due to time ambiguity of inter-node exchange, which results in reception of outdated UAI by target cell.

Observation 3: In legacy RRC spec, UAI retransmission within 1s HO execution is specified because the source cell may receive latest UAI after it initiated handover preparation due to time ambiguity of exchange inter-node message, which results in reception of outdated UAI by target cell.

It is correct that UE will report applicability after handover via RRCReconfigurationComplete and it may lead to duplicated applicability reporting reception by target cell. However, we think the issue of the proposed change is that the generated UAI may contain both applicability report and other UE assistance information. Then, this change will mistakenly forbid the UE to retransmit UAI with other UE assistance information.

Observation 4: The issue of the proposed change in S050 is that the generated UAI may contain both applicability report and other UE assistance information. Then, this change will mistakenly forbid the UE to retransmit UAI with other UE assistance information.

Since the bad consequence is just duplicated applicability reporting (rather than interrupt ongoing mobility), we tend to think nothing is broken and thereby prefer to not optimize this case. So, we propose:

Proposal 3 (RIL S050): Suggest to PropReject S050. No specification changes to optimize the UAI retransmission with applicability reporting within 1 second HO execution and after CHO/LTM execution.

### 2.3.2 A104: Simplification of procedure text on applicability reporting

This RIL is described as follows:

[Description]: In existing applicability reporting procedure text (see below example text from of Section 5.3.5.3): - including csi-InferencePrediction is used to describle inference configuration of CSI prediction. - including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19 is used to describle inference configuration of beam management. The description of beam management is complex. However, according to current IE structure of predictionConfiguration-r19, configurationForChannelPredictio is now only used for inference configuration of beam management. So, the description of beam management inference can be simplified to “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19 configurationForChannelPrediction”. ===========related copy from Section 5.3.5.3======================================= 2> if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or [RIL]: A104, AIML 2> if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or 2> if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or [RIL]: A104, AIML 2> if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation): 3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): [RIL]: A104, AIML 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status[RIL]: V100, AIML of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; [Comments]: ZTE: Agree to have this modification which make the text procedure more readable and aligned the description method for both AIML CSI-Prediction and AIML based BM [WI CR rapporteur-v031]: We agree that the procedural text could be simplified as suggested. However, since the ASN.1 structure is still under discussion in N021/H003, we suggest to come back to A104 after resolving N021/H003.

Please note that RAN2 has concluded N021/H003/A105, and the below ASN.1 changes are correspondingly made:

predictionConfiguration-r19 CHOICE { csi-InferencePrediction-r19 ENUMERATED {true}, configurationForBM-PredictionAndDataCollection-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredictedRS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... }, configurationForBM-Monitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceForRS-PAI-r19 ENUMERATED {n1, n2, n3, n4, n5, n6, n7, n8}OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R ... }, configurationForCSI-Monitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, timeInstanceForSGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } }

So, we think the A104 can be agreed with modification to new IE name of BM inference configuration (i.e. from configurationForChannelPrediction to configurationForBM-PredictionAndDataCollection-r19). And we provide the TP in Appendix.

Proposal 4 (RIL A104): Agree A104 with modification to new IE name of BM inference configuration: the procedure description of beam management inference in Section 5.3.5.3/5.3.13.4/5.7.4.3 can be simplified to “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19configurationForBM-PredictionAndDataCollection-r19.”. The TP can be found in Appendix.

### 2.3.3 N079: Empty list of data collection candidate configurations

The issue is:

[Description]: In the current CR of TS 38.331, when UE receives DataCollectionPreferenceConfig, without any data collection candidate configurations (that is dataCollectionCandidateConfigToAddModList is empty), then UE can use the dataCollectionRequest=true flag. However, it is not clear that how to do setup with an empty list. DataCollectionPreference-r19 ::= SEQUENCE { dataCollectionRequest-r19 ENUMERATED {true} OPTIONAL, dataCollectionPreferredConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19 OPTIONAL, dataCollectionStopConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19 OPTIONAL, ... } There are several alternatives to resolve this. Option 1: use a separate flag in otherConfig, independent of DataCollectionPreferenceConfig to enable dataCollectionRequest flag if UE is preferred to be configured to start data collection without having any candidate configurations. However, this would require changes in many places in the specification. Option 2: use a separate flag in otherConfig, within DataColletionPreferenceConfig, that enables dataCollectionRequest flag to be true. This would require changes in the ASN.1 and in the procedure of 5.7.4.3.

We don’t think any spec change is needed due to the following reasons:

- Current RRC allows to have empty list if clarified in field description. One example on can be found below:

MBSMulticastConfiguration-r18-IEs ::= SEQUENCE {

mbs-SessionInfoListMulticast-r18      MBS-SessionInfoListMulticast-r18           OPTIONAL,   -- Need R

mbs-NeighbourCellList-r18             MBS-NeighbourCellList-r1                    OPTIONAL,   -- Need S

drx-ConfigPTM-List-r18                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r18                  PDSCH-ConfigBroadcast-r17                        OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r18        MTCH-SSB-MappingWindowList-r17                OPTIONAL,   -- Need R

thresholdMBS-List-r18                 SEQUENCE (SIZE (1..maxNrofThresholdMBS-r18)) OF ThresholdMBS-r18     OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                          OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                           OPTIONAL

}

MBSMulticastConfiguration field descriptions

mbs-NeighbourCellList List of neighbour cells providing one or more MBS multicast services for RRC_INACTIVE that are provided by the current cell. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS multicast services signalled in mbs-SessionInfoListMulticast in the MBSMulticastConfiguration message are not provided in any neighbour cell. When a non-empty mbs-NeighbourCellList is signalled, the current serving cell does not provide information about MBS multicast services of a neighbour cell that is not included in mbs-NeighbourCellList, i.e., the UE cannot determine the presence or absence of an MBS multicast service of a neighbour cell that is absent. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS multicast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS multicast service in neighbouring cells based on the absence of this field.

- dataCollectionPreferredConfigurationList-r19 is optional. If NW don’t provide any candidate configuration, it can just make this field absent.

Observation 5: Current RRC allows empty list if clarified in field description.

Thus, we propose:

Proposal 5 (RIL N079): Suggest to PropReject N079 because current RRC allows empty list for SetupRelease if clarified in field description and dataCollectionPreferredConfigurationList-r19 is optional.

# 3 Conclusion

In this contribution, we discuss the remaining open issues and some important RILs of Rel-19 AI/ML for PHY. Our observations are:

Observation 1: Reporting applicability status change in RRCReconfiguration leads to below issue:

- It will lead to different UE behaviors for different inference configurations which are reported as applicable in same RRCReconfigurationComplete message. It increases unnecessary UE complexity.

- For the inference configurations received in immediate RRCReconfiguration, the UE will submit them in PHY layer and autonomously activate them.

- For the inference configurations received in previous RRCReconfiguration, the UE will not submit them in PHY layer.

- The above behavior 1)-b) is not aligned with RAN1 input in R1-2508122.

Observation 2: The reporting applicability status change in RRCReconfigurationComplete is a corner case. It happens only when applicability status changes at the same time of RRCReconfiguration reception. Thus, it is not necessary to specify the UE behaviour.

Observation 3: In legacy RRC spec, UAI retransmission within 1s HO execution is specified because the source cell may receive latest UAI after it initiated handover preparation due to time ambiguity of exchange inter-node message, which results in reception of outdated UAI by target cell.

Observation 4: The issue of the proposed change in S050 is that the generated UAI may contain both applicability report and other UE assistance information. Then, this change will mistakenly forbid the UE to retransmit UAI with other UE assistance information.

Observation 5: Current RRC allows empty list if clarified in field description.

Based on Observations, we have the following proposals:

Proposal 1a: Revert RAN2#131 agreement: RRCReconfigurationComplete shall only include applicability/inapplicability status for all inference configurations included in the immediately preceding RRCReconfiguration message.

Proposal 1b: For a CSI configuration which was reported as non-applicable, the UE can only use UAI to update the applicability status change and then needs to wait NW to send a followed RRCReconfiguration with inference configuration.

Proposal 2: As agreed in RAN2#130, when UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

Proposal 3 (RIL S050): Suggest to PropReject S050. No specification changes to optimize the UAI retransmission with applicability reporting within 1 second HO execution and after CHO/LTM execution.

Proposal 4 (RIL A104): Agree A104 with modification to new IE name of BM inference configuration: the procedure description of beam management inference in Section 5.3.5.3/5.3.13.4/5.7.4.3 can be simplified to “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19configurationForBM-PredictionAndDataCollection-r19.”. The TP can be found in Appendix.

Proposal 5 (RIL N079): Suggest to PropReject N079 because current RRC allows empty list if clarified in field description and dataCollectionPreferredConfigurationList-r19 is optional.

# 4 References

[1] RAN2#131, Chair Notes.

[2] RAN2#131b, Chair Notes.

# Appendix (TP of Proposal 4)

Section 5.3.5.3

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19; or

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

Section 5.3.13.4

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19:

3>	for each such serving cell, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19:

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

Section 5.7.4.3

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including configurationForBM-PredictionAndDataCollection-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed: [RIL]: N034 AIML

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForBM-PredictionAndDataCollection-r19, for which the applicability status has changed

6>	include an entry in the applicabilityReportConfigIdList, and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;