3GPP TSG-RAN WG2 Meeting #132                                                             R2-2508481

Dallas, USA，17th –21th November, 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Remaining open issues for NES

Document for:	Discussion and Decision

1	Introduction

During last meeting, the following two issues were discssed but not concluded. In this contribution, we would like to further disuccss about these two issues and provide corresponding proposals.

- Issue 1: impact of od-ssb-PositionsInBurst on ssb-ToMeasure

- Issue 2: SSB adaptation for deactivated SCell

2	Discussion

2.1 Issue1: impact of od-ssb-PositionsInBurst on ssb-ToMeasure

According to the specification, the granularity of od-ssb-PositionsInBurst is per od-ssb-Config and it refers to the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks. While the granularity of ssb-ToMeasure is per MO and it refers to the set of SS blocks to be measured within the SMTC measurement duration.

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, UE applies the value ssb-PositionsInBurst provided in ServingCellConfigCommon.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS block index 0, the second bit corresponds to SS block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS block is not to be measured while value 1 indicates that the corresponding SS block is to be measured (see TS 38.215 [9]). When the field is not configured the IAB-MT measures on all SS blocks. Regardless of the value of this field, SS blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

According to [1], for OD-SSB, there is 1-1 mappting between OD-SSB configuration and SMTC. UE should set up SMTC for serving cell measurement based on the activated OD-SSB configuration. Thus, since od-ssb-PositionsInBurst is configured per OD-SSB configuration, if od-ssb-PositionsInBurst is changed, the corresponding activated OD-SSB configuration is changed, resulting in the change of SMTC accordingly. In this case, if the applied ssb-ToMeasure keeps the same for all the OD-SSB configurations as the granularity of this parameter is per MO, then a misalignment may occur between the OD-SSB transmission pattern and the OD-SSB measurement pattern, e.g., two inidicated bitmaps are not aligned resulting in measurement result lost or unnecessary measurement. Similar issue also exists for SSB adaptation.

Observation 1: Different granularity of od-ssb-PositionsInBurst (per OD-SSB configuration) and ssb-ToMeasure (per MO) may result in misalignment between OD-SSB transmission pattern and OD-SSB measurement pattern.

There was a proposal from some companies last meeting to clarify the od-ssb-PositionsInBurst is configured as a subset of ssb-ToMeasure within the associated MeasObjectNR [2]. However, considering there are at most 16 candidated values of od-ssb-PositionsInBurst and the choice structure allows short, medium and long bitmap, it seems not possible to ensure all the configured od-ssb-PositionsInBurst is a subset of ssb-ToMeasure, unless strict restriction on NW configuration is introduced, e.g., ssb-ToMeasure can only be configured as long bitmap and od-ssb-PositionsInBurst associated with any OD-SSB configuration is configured as a subset of ssb-ToMeasure.

Observation 2: It seems not possible to ensure all the configured od-ssb-PositionsInBurst is a subset of ssb-ToMeasure considering there are at most 16 candidated values of od-ssb-PositionsInBurst and the choice structure allows short, medium and long bitmap.

Actually, the key point of this mismatch is the different configuration granularity. And the most straightforward solution is to configure the od-ssb-PositionsInBurst and the ssb-ToMeasure with the same granularity. There are two candidate solutions.

- Alt1: configure ssb-ToMeasure per OD-SSB configuration.

- Alt2: configure ssb-ToMeasure per SMTC

Both Alt1 and Alt2 can work, considering we didn’t agree to configure SMTC within OD-SSB configuration, it is more suitable to adopt Alt2. In addition, Alt2 can also solve the misalignment between SSB transmission pattern and SSB measurement pattern for SSB adaptation, while Alt1 is OD-SSB specific solution. Thus, we suggest RAN2 to agree to configure ssb-ToMeasure per SMTC. a TP is provided as below.

### 6.3.2	Radio resource control information elements

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]],

[[

smtc5list-r19                       SSB-MTC5List-r19                                                OPTIONAL,   -- Need R

smtc6list-r19                       SSB-MTC6List-r19                                                OPTIONAL,   -- Need R

smtc7-SSBAdapt-r19                  SSB-MTC-SSBAdapt-r19                                      OPTIONAL,   -- Cond IntraFreqConnected

neighSCellMeasSkipping-r19          ENUMERATED { enabled }                                          OPTIONAL    -- Cond SCellMO

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC5-r19

SSB-MTC6List-r19::=                 SEQUENCE (SIZE(1..16)) OF SSB-MTCx-r19

SSB-MTC-SSBAdapt-r19 ::=            SEQUENCE (SIZE(1..2)) OF SSB-MTCx-r19

SSB-MTCx-r19 ::=             SEQUENCE {

ssb-ToMeasure-r19                 SSB-ToMeasure,

SMTC-r19 						 SSB-MTC

}

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need S

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Cond NeighbourCell

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17                                        OPTIONAL   -- Need R

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

Proposal 1: RAN2 agree to configure ssb-ToMeasure per SMTC to solve the misalignment between SSB transmission pattern and SSB measurement pattern for both DO-SSB and SSB adaptation.

2.2 Issue2: SSB adaptation for deactivated SCell

In last meeting, we discussed whether SSB adaptation can be supported for deactivated SCell or not but without any conclusion. We checked the RAN1/RAN4 status, actually there was no discussion in RAN1 on the applied scenario for SSB adaptation, but in RAN4, the following agreement was achieved last meeting. Even there is no defined requirement, it does not mean SSB adaptation can not be supported for deactivated SCell at least from standard perspective, that’s why they have the note to clarify the agreement is not intended to impact the discussion in other WGs. Actually, in legacy, we have quite many examples where there is no RAN4 defined requirement, but supported in other WGs, e.g, 32 CC CA and 4 Rx MIMO.

Agreement (on Thursday): RAN4 not to define requirement for DCI based SSB adaption for deactivated SCell measurement and SCell activation. - Note: this RAN4 agreement does not intend to impact the discussion and decision in other WGs.

In our understanding, there is no motivation to limit the applied scenario for SSB adaptation. Even without any change on our spec, SSB adaptation can be supported for deactivated SCell. Some may argue that the blocking issue is UE shall not monitor PDCCH for deactivated SCell, thus SSB adaptation indicated by DCI cannot apply to deactivated SCell [3].

According to the current MAC specification, when SCell is deactivated, UE shall not monitor the PDCCH on/for the SCell, however we think “PDCCH for the SCell” refers to cross carrier scheduling, e.g., when SCell is deactivated, UE shall not monitor the PDCCH that cross schedules the deactivated SCell. But DCI 2-9 can be carried via PCell, for which regardless of the activation/deactivation status of SCell, UE needs to always monitor. In addition, DCI 2-9 is a group common signalling and can indicate the SSB adaptation status for more than one SCell.

1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Thus, we think RAN2 can confirm from RAN2 perspective, SSB adaptaion can be supported for deactivated SCell via DCI-2-9 carried by PCell or other activated SCell.

Proposal 2: From RAN2 perspective, SSB adaptation can be supported for deactivated SCell via DCI 2-9 carried by PCell or other activated SCell. No specification change is needed.

3. Conclusions

Based on the above discussion, we propose following observations and proposals:

Observation 1: Different granularity of od-ssb-PositionsInBurst (per OD-SSB configuration) and ssb-ToMeasure (per MO) may result in misalignment between OD-SSB transmission pattern and OD-SSB measurement pattern.

Observation 2: It seems not possible to ensure all the configured od-ssb-PositionsInBurst is a subset of ssb-ToMeasure considering there are at most 16 candidated values of od-ssb-PositionsInBurst and the choice structure allows short, medium and long bitmap.

Proposal 1: RAN2 agree to configure ssb-ToMeasure per SMTC to solve the misalignment between SSB transmission pattern and SSB measurement pattern for both DO-SSB and SSB adaptation.

Proposal 2: From RAN2 perspective, SSB adaptation can be supported for deactivated SCell via DCI 2-9 carried by PCell or other activated SCell. No specification change is needed.

- Reference

[1] RAN2#131bis meeting minutes

[2] R2-2506817 [C184]Impact of od-ssb-PositionsInBurst on ssb-ToMeasure, CATT

[3] R2-2506936 [H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues, Huawei