3GPP TSG-RAN WG2 #132	R2-2508504

Dallas, USA, Nov. 17th - 21st

Source:                    	DENSO CORPORATION

Title:  	RRC corrections for Rel-19 Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2		RRC corrections

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for past meeting RAN2#130[2], the following agreements are made for S&F operation:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

This paper attempts to investigate about remaining consideration on S&F operation based on the above objectives and agreements.

- Discussion

In RAN2#130 meeting, it was agreed that the time information for switching to S&F mode is indicated by using the agreed time information for switching to normal mode, and the switching direction is determined by operation mode indication. Then, in the current running CR, the transition time information is defined as t-ModeSwitching-r19. We would like to raise a concern about coexistence of t-ModeSwitching-r19 and t-Service.

From legacy UE point of view, starting S&F mode equals to NTN service stop, since non-S&F capable UE is barred when the satellite is in S&F mode. Therefore, to inform that the legacy UE will not be able to use the NTN service, the satellite may need to set S&F mode starting time to t-Service in normal mode.

Observation 1: Satellite may need to set same value of t-ModeSwitching-r19 to t-Service in normal mode, since starting S&F mode equals to NTN service stop from legacy UE point of view

In that case, Rel-19 (S&F capable) UE may not be able to distinguish whether NTN service will actually stop or not, since a value is set in t-Service. Furthermore, according to the current RRC specification (clause 5.3.3.23), there would be a case where the UE “shall” perform leaving RRC_CONNECTED upon expiry of t-Service.

Observation 2: There is a case where the UE “shall” perform leaving RRC_CONNECTED upon expiry of t-Service, according to the current RRC specification

To avoid this issue, we would like to propose to add NOTE like the following TP to clarify that S&F capable UE can ignore t-Service if the value of t-Service is same as t-ModeSwitching-r19 in normal mode.

5.3.3.23 UE actions upon detecting discontinuous coverage In discontinuous coverage scenario, upon expiry of t-Service or being out of the current serving cell coverage, the UE shall: 1> if timer T310 is running: 2> stop timer T310, and perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'. NOTE: The UE capable of Store and Forward operation may not apply the above discontinuous coverage handling if sf-OperationMode-r19 in previously received system information is absent and the value of t-ModeSwitching-r19 equals to t-Service.

Proposal 1: RAN2 to add NOTE to clarify that S&F capable UE can ignore t-Service, if the value of t-Service is same as t-ModeSwitching-r19 in normal mode

- Summary and proposal

This paper discussed about remaining consideration on S&F operation and proposed the following:

Observation 1: Satellite may need to set same value of t-ModeSwitching-r19 to t-Service in normal mode, since starting S&F mode equals to NTN service stop from legacy UE point of view

Observation 2: There is a case where the UE “shall” perform leaving RRC_CONNECTED upon expiry of t-Service, according to the current RRC specification

Proposal 1: RAN2 to add NOTE to clarify that S&F capable UE can ignore t-Service, if the value of t-Service is same as t-ModeSwitching-r19 in normal mode

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2504673, “Report from Break-out session on NR-NTN and IoT-NTN”