3GPP TSG-RAN WG2 Meeting #132                                                R2-2508505

Dallas, USA, Nov. 17th - 21st 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining MAC Issues

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post131bis][215][MIMOevo] CR for TS 38.321 (Samsung), there are still some remaining MAC issues, In this paper we would like to share our views on these remaining issues.

- Discussion

In this chapter, we first discuss PUCCH release upon TAT expiry, then share our views on the UE initiated CSI report processing during the measurement Gap.

2.1 PUCCH Release upon TAT Expiry

In the last meeting, there was a hot discussion on PUCCH release upon TAT expiry at the SCell with the corresponding PUSCH CG configured, and in the post email discussion, there is still no consensus. However, during the last meeting discussion, companies share the understanding that the PUCCH resource for the UEIBM can also be used for the HARQ-ACK or other CSI reporting.

During the CB session, companies prefer to focus on the case that “the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI)”. However, the UE is unable to determine whether the corresponding PUCCH resource ID is only associated to the type-1 CG PUSCH of mode B.

During the follow-up discussion, companies also expressed concern about the flexibility of HARQ-ACK/NACK for dynamic DL scheduling and the implementation complexity if they choose to release the PUCCH resource. Thus we think for this case, the UE should not release the PUCCH resource.

Proposal 1: For Mode-B UEI reporting, if the PUSCH resource of a UEI report configuration points to a SCell whose sTAG's TAT has expired, the UE should not release the corresponding PUCCH resource.

If the proposal 1 is agreed, the second issue is about the UE’s behaviour, there are also 2 options:

- Option 1: UE does not transmit UEIRI on the PUCCH if UE initiated CSI report is triggered.

- Option 2: The UE transmits UEIRI on the PUCCH if UE initiated CSI report is triggered.

For this issue, some companies have concern to transmit the UEIRI from the power saving perspective. However, we think it’s quite a corner case, and thus would not introduce much impact on the power consumption. Thus it can be left to the UE implementation to transmit it or not.

Proposal 1a: If proposal 1 is agreed, it can be left to the UE implementation to transmit UEIRI on the PUCCH or not. No spec change is needed.

2.2 UEIRI in the Measurement Gap

In the post-meeting email discussion, another remaining issue concerns the transmission of the UEIRI in the measurement Gap and thus prefer to make it clear in the following specification:

TP on TS 38.321 ## 5.14 Handling of measurement gaps During an activated measurement gap that has not been cancelled (as specified in clause 10.6 in TS 38.213 [6]), the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: - not perform the transmission of HARQ feedback, SR, andCSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

However, from the perspective of protocol conciseness, we don’t think such kind of clarification is necessary. According to the current spec, the UE would not perform the CSI reporting in the measurement Gap, obviously, the UE would also not transmit the UEIRI to the network because the UEIRI is one step of the UE initiated CSI report. Even without modifying the specification, there is no room for misunderstanding.

Proposal 2: On the UE initiated CSI report processing during the measurement Gap, no spec change is needed.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For Mode-B UEI reporting, if the PUSCH resource of a UEI report configuration points to a SCell whose sTAG's TAT has expired, the UE should not release the corresponding PUCCH resource.

Proposal 1a: If proposal 1 is agreed, it can be left to the UE implementation to transmit UEIRI on the PUCCH or not. No spec change is needed.

Proposal 2: On the UE initiated CSI report processing during the measurement Gap, no spec change is needed.