3GPP TSG-RAN WG2 Meeting #132                                                R2-2508506

Dallas, USA, Nov. 17th - 21st 2025

Source:        ZTE Corporation

Title:           [S017][H402] and RRC other Remaining Issues

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

In the post email discussion [Post131bis][216][MIMOevo] CR for TS 38.331 (Ericsson), there are still some remaining issues that are need further discussed, furthermore, RAN1 updated part of the UEIBM high layer parameters table as in [1].

In this paper, we’d like to share our views on these remaining issues.

- Discussion

In this chapter, we first discuss the aperiodic trigger state issue as proposed in the RIL[S017], and then share our views on the newly added parameters, which are also related to the RIL[H402].

2.1 [S017] on Aperiodic Trigger State

In the last meeting, there is a modification to the aperiodic trigger state in the MAC specification to clarify that the aperiodic trigger state MAC CE can also be applied to the UE initiated mode-A CSI report. Meanwhile in the [S107] it proposed with the similar intention. However, companies have different views for the modification part in the post email discussion.

As some companies have pointed out, according to the 38.214, for the UE initiated mode-A CSI report, when the UE received DCI that includes the CSI trigger state associated with the CSI-ReportConfig in the CSI request field, the UE would send the UE initiated CSI report, which is the main difference from the legacy aperiodic CSI report.

##### 5.2.1.5.4 UE Initiated reporting The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or

Furthermore, some companies also mentioned that a trigger state can only be associated with either aperiodic CSI reporting or UE-initiated CSI reporting as in the RAN1 spec:

##### 5.2.1.5.4 UE Initiated reporting The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType.

Based on the above, the modification in Annex TP1 can be adopted to the RRC spec by reference to the 38.214 as in the proposal 1.

Proposal 1: [S017]Adopt the modification in Annex TP1 for the CSI-AperiodicTriggerStateList description.

2.2 [H402] on PUCCH/PUSCH Resource

Another issue is about the number of the PUCCH and PUSCH resource for the UE initiated CSI report. According to the latest RAN1 higher layer parameter list in [1], the UE could be configured with a list of the PUCCH and PUSCH Resource as in the Table 1:

Table 1: Updated PUCCH/PUSCH Resource

| configuredPUSCHResourceListOfModeB-r19 | This parameter is used to inform provide the list of Type-1 CG PUSCH resources for second channel in mode-B, each of the Type-1 CG PUSCH resources corresponds to a BWP | List of {configuredGrantConfigIndex, UL bwp-Id, serving cell index} |
| --- | --- | --- |
| PUCCHResourceList-r19 | This parameter is used to configure the list of periodic PUCCH resources for first PUCCH carrying UEIRI, each of the periodic PUCCH resources corresponds to a BWP - to request dynamically scheduled PUSCH to carry UE-initated/event-driven beam report for mode-A - to notify Type-1 CG PUSCH to carry UE-initated/event-driven beam report for mode-B. | List of {periodicityAndOffset, PUCCH-ResourceID, BWP-Id, pucchCell-r19} |

This issue was also proposed in the [H402] as below:

| RIL Id | WI | Class | Title | Status |
| --- | --- | --- | --- | --- |
| H402 | MIMO | 2 | PUCCH/PUSCH resources can only be configured in one BWP | PropReject |

However, the modification in the H402 is not enough and there are also some open issues to be confirmed. For the PUSCH resource configuration, we have agreed to remove the serving cell index based on the RAN1 following agreement:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

Now in the parameter list table, it’s “List of {configuredGrantConfigIndex, UL bwp-Id, serving cell index}” it seems that the PUSCH resources in the PUSCH list can be configured on the different cells, which is conflict with the above agreement, thus we think from the Asn.1 coding aspect it should be “List of {configuredGrantConfigIndex, UL bwp-Id} on the same cell.

Proposal 2: RAN2 to confirm that the PUSCH resources in the PUSCH list should be on the same cell, i.e. the CC where the corresponding CSI report configuration is configured.

If this proposal 2 is agreed, the “maxNrofBWPs” can be taken as the maximum value for the list size.

Proposal 2a: If the proposal 2 is agreed, take the maxNrofBWPs as list size of the PUSCH resource.

For the PUCCH resource, in the latest RAN1 parameter list table, it’s “List of {periodicityAndOffset, PUCCH-ResourceID, BWP-Id, pucchCell-r19}”. However, it’s still unclear on the maximum value of such kinds of the PUCCH resource. This should be further confirmed with RAN1.

Proposal 3: Send an LS to confirm with RAN1 on list size of the PUCCH resource.

Except the undefined maximum PUCCH resource value, the Asn.1 coding TP based on the latest parameter table was also provided in the Annex TP2.

Proposal 4: Agree the TP2 in the Annex for the PUSCH/PUCCH Resource configuration.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: [S017]Adopt the modification in Annex TP1 for the CSI-AperiodicTriggerStateList description.

Proposal 2: RAN2 to confirm that the PUSCH resources in the PUSCH list should be on the same cell, i.e. the CC where the corresponding CSI report configuration is configured.

Proposal 2a: If the proposal 2 is agreed, take the maxNrofBWPs as list size of the PUSCH resource.

Proposal 3: Send an LS to confirm with RAN1 on list size of the PUCCH resource.

Proposal 4: Agree the TP2 in the Annex for the PUSCH/PUCCH Resource configuration.

- Reference

[1]R1-2508205  LS on Rel-19 higher layers parameters list Post RAN1#122bis (contact: Ericsson)

- Annex

TP1:

#### – CSI-AperiodicTriggerStateList The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of CSI trigger states for aperiodic and/or a mode-A UE-initiated CSI report. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). For the aperiodic CSI report, upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state. For the mode-A UE-initiated CSI report, upon reception of the value associated with a trigger state associated with the corresponding CSI-ReportConfig, the UE sends the CSI report as in TS 38.214 [19], clause 5.2.1.5.4.

TP2

CSI-ReportUE-Initiated-r19 ::=               SEQUENCE {

eventTypeUE-Initiated-r19                    CHOICE {

event1-r19                             SEQUENCE {

event1Threshold-r19                     RSRP-Range,

...

},

event2-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

conditionFulfillmentIndicator-r19      ENUMERATED {enabled}     OPTIONAL,  -- Need R

...

},

event7-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

valueOfQ-r19                           INTEGER (1..8),

conditionFulfillmentIndicator-r19      ENUMERATED {enabled}    OPTIONAL,    -- Need R

...

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                NULL,

modeB-r19                                SEQUENCE {

pusch-ResourceOfModeB-r19                SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUSCH-ResourceOfModeB-r19,

minimumPucch-PuschOffset-r19 ENUMERATED {sym0, sym1, sym2, sym4, sym8, sym16, sym32, sym64, sym128, sym256, sym512}

}

},

nrofReportedRS-UE-Initiated-r19       ENUMERATED {n1, n2, n3, n4},

tci-ServCellIndex-r19                    ServCellIndex             OPTIONAL,    -- Need R

currentBeamReport-r19                    ENUMERATED {enabled}      OPTIONAL,    -- Need R

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19                   INTEGER (2..16),

eventDetectionTimeWindow-r19             ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

}                          OPTIONAL,    -- Need R

pucch-ResourceList-r19                SEQUENCE (SIZE (1..maxNrofPUCCHResource)) OF PUCCH-Resource-r19

…

}

PUSCH-ResourceOfModeB-r19  ::=                      SEQUENCE {

configuredGrantConfigIndex-r19           ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                            BWP-Id

}

PUCCH-Resource-r19                       SEQUENCE {

periodicityAndOffset                     CHOICE {

sym2                                     NULL,

sym6or7                                  NULL,

sl1                                      NULL,            -- Recurs in every slot

sl2                                      INTEGER (0..1),

sl4                                      INTEGER (0..3),

sl5                                      INTEGER (0..4),

sl8                                      INTEGER (0..7),

sl10                                     INTEGER (0..9),

sl16                                     INTEGER (0..15),

sl20                                     INTEGER (0..19),

sl40                                     INTEGER (0..39),

sl80                                     INTEGER (0..79),

sl160                                    INTEGER (0..159),

sl320                                    INTEGER (0..319),

sl640                                    INTEGER (0..639)

},

resource                                 PUCCH-ResourceId,

ul-BWP-Id-r19                            BWP-Id,

pucch-Cell-r19                           ENUMERATED {spCell, pucch-Scell}

}