3GPP TSG-RAN2#132	R2-2508525

Dallas, US, 2025-11-17 - 2025-11-21

Agenda Item:	10.3.3.3 Others

Source:	Ericsson

Title:	Other common user and control plane issues

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses other aspects which impact both user plane and control plane and which were not discussed in other contributions.

# 2	Discussion

## 2.1      System Information Security

Integrity protection of system information (SI) (e.g. signature in MIB/SIB) is a potential new security requirement to consider for 6G. However, before introducing SI integrity protection it's important to understand the potential attacks that are possible with unprotected SI. For example, an attack that results in compromise of user data is more important to protect against and may justify a more complex solution than an attack that results in a temporary denial of service. Security requirements also depend on the type of network, for example users in private network may have stricter security requirements than users in a public network. Another aspect to consider is the recent rise of post-quantum cryptography (PQC) which requires significantly longer signatures and hence may make it difficult to include signatures in SI.

A related area is confidentiality protection (encryption) of system information. Encryption is used today in 5G for positioning SIBs (posSIB) to ensure that only authorized users have access to the contents. The encryption is carried out by the LMF and the encrypted SIBs are sent to the UEs transparently via the gNB. For 6G, a generalized encryption mechanism could be considered that can be used also for other types of SIBs.

As SA3 has the security expertise they should be the main responsible for the SI security study but it’s important that RAN2 is involved as RAN2 is responsible for the overall SI design and will need to provide input on e.g. what parts of SI that can be considered sensitive.

Note that the SA study on 6G requirements [4] includes a requirement on False Base Station (FBS) detection so at least SI integrity protection is something that is already considered by SA3.

System information security should be studied jointly with SA3 with SA3 being the main responsible.

## 2.2      Reliability, Robustness and Resiliency

Addressing use cases beyond best effort requires that the connectivity is always available, performance is stable and resilient to disturbances and failures. The objective is that the communication is dependable from a user perspective that service providers can provide such a guarantee.

Stable performance

To ensure stable performance, standards should incorporate tools that enable fine-grained scheduling decisions and allow the scheduler to maintain clear visibility into the various queues – such as data volumes, delay budgets and other relevant factors. Accordingly, UE reporting formats should be harmonized and unified to prevent the need for multiple overlapping features and report types. Furthermore, for better queue management, 6G should natively support rate control mechanisms such as L4S as described in our companion contribution [2].

6G needs to support stable user plane performance considering fine-grained scheduling decisions, scheduler visibility of data volumes, delay budgets, etc.

Mobility

The mobility framework of 6G needs to be robust enough to cope with (sudden) varying radio conditions but also cope with potential outages of different components of the operators' networks. Conditional handover is one mechanism which can address (sudden) varying radio conditions but is not designed to address outages of various network components (e.g. power outage impacting some components in the operators network), see [5].

The 6G mobility framework need to consider resiliency not only to cope with varying radio conditions, but also with sudden outages of different network components.

Failsafe connections

Another shortcoming of NR is that the PCell is in large the single point of failure for the UE, meaning that even if the UE has an SCell, the UE triggers RLF if the PCell goes down. The PCell and SCell differentiation was logical and made sense in Rel-10 when Carrier Aggregation was introduced as at that time there was only one connection (i.e. the PCell connection) and secondary connections (i.e. SCells were added) and those secondary connections were deemed less important. However, as long as the UE has one workable connection with the network, the UE should continue using this connection, see also [3].

If all connections between the UE and the network disappears, the 6GR should also allow for a seamless recovery of the connection

Resilient connections must be supported in 6G which do not rely on a single point of failure like the PCell in NR, and if the connection is broken there should be ways to recover the connection seamlessly.

Observability

Some new use cases have very stringent reliability and QoS requirements which operators have to ensure that their networks fulfil. It is not enough that there are features in place in their network which can ensure it, they also need to be able to monitor to have it black-on-white that the performance requirements are fulfilled, and they need to be able to show this to service providers to which the operator has SLAs.

We already have some L2 measurements defined in NR which allows this. However, one piece of information which is missing to achieve needed level of observability is end-to-end latency, i.e. packet latency from the ingress point in the UPF to the egress in the UEs PDCP layer and vice versa. RAN2 should study ways to monitor the end-to-end latency.

Observability in terms of end-to-end packet latency in the 3GPP network is not sufficient in NR and needs enhancements.

RAN2 to study enhanced Reliability, Robustness and Resiliency for 6G, specifically:

- User plane procedures allowing for stable user plane performance, including instantaneous monitoring of the performance. This should be addressed in UP-discussions.

- Mobility procedures which can cope with sudden unavailability of the radio link as well as sudden unavailability of various network components. This should be addressed in the mobility-discussions.

- Resilient connections and seamless recovery at connection failure. To be addressed in control plane-discussions.

- Observability in terms of end-to-end packet latency. This should be addressed in UP-discussions

## 2.3      IoT

The 6G SI has as an objective to “target scalable and forward compatible design for diverse device types”. Support for 6G Massive Communication (IoT) was endorsed for FR1 in RAN#109. All 6G UEs, including the low-end devices, shall support a common mandatory baseline functionality. Unlike in previous generations, the first requirement for 6G Massive Communication is to avoid having to specify an even-lower-end device in a later release since that is known to create backwards compatibility issues and unnecessary complexity.

It is easier to scale up functionality in a later release than to scale down as experience from earlier radio generations has shown.

6G should support massive communication (IoT) from initial release.

A challenge to address in 6G is to ensure the longevity and coexistence required by massive IoT applications. To name a few, key industrial sectors, such as automotive, smart utilities, and critical infrastructure monitoring, operate on cycles that demand long term solutions and support, at least for a decade. Securing these use-cases calls for a massive IoT framework integrated into 6G from initial phase.

A stable, forward compatible 6G massive IoT solution accelerates commercial adoption.

A key aspect at this early stage is to define the mandatory baseline functionality. This shall be aligned with the general 6G functionality, so that low-end 6G UEs are supported inherently.

The procedures and signalling such as initial access, handover etc., should be designed to support low-end UEs inherently.

In RAN#109, it was agreed that the minimum number of UE receive/transmit antenna is 1 for the lowest tier 6G device. Consequently, the discussion on minimum requirements for low-end 6G devices has already started in RAN1 (e.g., half duplex operation, peak data rate, minimum bandwidth, etc.). From RAN2 perspective, a starting point in this discussion can be TS 22.261 which defines 5G requirements specified for low-end devices. In the following, we consider some of those requirements that are essential to enable 6G Massive IoT Communication:

- Scheduling framework common for all 6G devices for initial access resources and radio channel configuration considering potential coverage enhancements and reduced bandwidths.

- Relaxed control plane and user plane timing relationships to account for low processing capabilities and increased latency for infrequent small packets.

- Increased UE energy efficiency targeting long battery life considering relaxed measurements and simplified procedures.

There is a clear objective to harmonize and create a common design for all 6G devices. In our view, the best way to ensure a common design is to have a common discussion which implies the requirements above should be addressed in common with the discussions on CP, UP, mobility, etc.

The essential requirements below for 6G Massive Communication (IoT) should be addressed in common include:

    - Scheduling framework for initial access resources and radio channel configuration considering coverage enhancements and reduced bandwidths.

    - Relaxed CP and UP timing relationships due to low processing capabilities and increased latency.

    - Increased energy efficiency targeting long battery lifetime considering relaxed measurements and procedures.

Lastly, although these essential requirements are specific to massive IoT communication, separate solutions should not be introduced for IoT and low-end UEs but one should strive to keep 6G as a single RAT addressing a wide range of use cases and verticals. Therefore, any feature required to fulfil IoT requirements, e.g. WUR and UE PS features for extremely long battery life, small data optimizations, and coverage enhancements, should be generally available to any 6G UE unless motivated (e.g., high-end eMBB UEs need not support 1 Rx and HD-FDD).

Separate solutions for IoT should be avoided and features required for IoT requirement fulfilment (e.g. WUR & UE PS features for long battery life, small data optimizations, and coverage enhancements) should be generally available to any UE unless motivated (e.g. use of 1Rx and HD-FDD in high-end UEs).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	6G needs to support stable user plane performance considering fine-grained scheduling decisions, scheduler visibility of data volumes, delay budgets, etc.

Observation 2	The 6G mobility framework need to consider resiliency not only to cope with varying radio conditions, but also with sudden outages of different network components.

Observation 3	Resilient connections must be supported in 6G which do not rely on a single point of failure like the PCell in NR, and if the connection is broken there should be ways to recover the connection seamlessly.

Observation 4	Observability in terms of end-to-end packet latency in the 3GPP network is not sufficient in NR and needs enhancements.

Observation 5	It is easier to scale up functionality in a later release than to scale down as experience from earlier radio generations has shown.

Observation 6	A stable, forward compatible 6G massive IoT solution accelerates commercial adoption.

Based on the discussion in the previous sections we propose the following:

Proposal 1	System information security should be studied jointly with SA3 with SA3 being the main responsible.

Proposal 2	RAN2 to study enhanced Reliability, Robustness and Resiliency for 6G, specifically:

-	User plane procedures allowing for stable user plane performance, including instantaneous monitoring of the performance. This should be addressed in UP-discussions.

-	Mobility procedures which can cope with sudden unavailability of the radio link as well as sudden unavailability of various network components. This should be addressed in the mobility-discussions.

-	Resilient connections and seamless recovery at connection failure. To be addressed in control plane-discussions.

-	Observability in terms of end-to-end packet latency. This should be addressed in UP-discussions

Proposal 3	6G should support massive communication (IoT) from initial release.

Proposal 4	The procedures and signalling such as initial access, handover etc., should be designed to support low-end UEs inherently.

Proposal 5	The essential requirements below for 6G Massive Communication (IoT) should be addressed in common include:

i.	Scheduling framework for initial access resources and radio channel configuration considering coverage enhancements and reduced bandwidths.

ii.	Relaxed CP and UP timing relationships due to low processing capabilities and increased latency.

iii.	Increased energy efficiency targeting long battery lifetime considering relaxed measurements and procedures.

Proposal 6	Separate solutions for IoT should be avoided and features required for IoT requirement fulfilment (e.g. WUR & UE PS features for long battery life, small data optimizations, and coverage enhancements) should be generally available to any UE unless motivated (e.g. use of 1Rx and HD-FDD in high-end UEs).

# 4	Reference

RP-251881, New SID: Study on 6G Radio

R2-2508472, User Plane Function Assessment in 6GR, Ericsson

R2-2508579, Procedures for 6G Access and Spectrum Aggregation, Ericsson

TR 22.870, Study on 6G Use Cases and Service Requirements

R2-2508721, Aspects on 6G mobility and measurements