3GPP TSG RAN WG2 #132			R2-2508528

Dallas, US, 17th-21st November, 2025

Source:	NTT DOCOMO, INC.

Title:	Views on 6G TN/NTN integration

Agenda Item:	10.2.2

Document for: 	Discussion and Decision

- Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. In this contribution, we provide our views on 6G TN/NTN integration.

10.2.2 – TN/NTN integration

Including contributions on what are the minimal essential set of requirements for NTN, what are the critical functions for day-1, what may be left for later.  Identify the set of functionalities that should be considered in common design with TN and how to move forward with TN/NTN integration in the study phase.

- Discussion

In 5G NR, NTN was introduced in Rel-17, but not from Day 1, which may result in difficulty implementing NTN features globally. Based on this lesson learned from NR NTN, “Unified design between TN and NTN” is the key word for 6GR to achieve easier NTN support by UE and advantage against non-3GPP NTN system.

As discussed in our RAN1’s tdoc [2], our feeling of NR NTN is that it is difficult to say, unfortunately, that the achievable data rate is sufficiently high to be used comfortably at normal times. Basically, coverage overlaps between TN and NTN may be one of potential directions for emergency cases. Having said that, if the achievable data rate is quite low as in 5G NR, then NTN will not be used comfortably at normal times in the overlapped area, which implies that development for TN/NTN overlap will not be successful from business perspective. Therefore, “Meaningful role in 6G cellular NW” is necessary to make 3GPP NTN system deployed in real world. At least several Mbps/UE and potentially more than 10 Mbps/UE for both DL and UL are desirable to be used comfortably at normal times by users, instead of kbps level or < 1 Mbps in the max assumptions.

2.1	Essential features for Day1 NTN

At the previous RAN2 meeting, the chair’s guidance was made, to identify what are the essential features for 6G day1 NTN and which features can be common between TN/NTN.

We provide our views on each feature that has been introduced since Rel-17, considering whether it is necessary or highly important factor for 6G Day 1 deployment of NTN, such as significantly reducing signaling overhead and aligning with our commercial interests mentioned above. Note that we listed the features irrespective of leading WG, however, we believe that it should be decided in the WGs that led the corresponding NR features whether the features are essential and common with TN.

Rel-17 features

- Pre-compensation for frequency doppler, timing advance (RAN1)

Essential to overcome excessive propagation delay and satellite movement, including broadcasting satellite ephemeris information.

- HARQ enhancement, scheduling offset (RAN1)

Essential to accommodate larger propagation delay.

- RRC connected mode mobility (RAN2)

Essential to support mobility for NTN, by network providing neighboring cell's satellite ephemeris needed to access the target serving NTN cell in the handover command.

- Location/time-based CHO (RAN2)

Essential to reduce signalling overhead. Whether and how to design this feature will be based on the design of 6G TN mobility.

- Multiple SMTCs per carrier (RAN2)

Almost essential for multiple satellite scenarios. Without this mechanism, UE will not be able to measure all neighboring cells belonging to different satellites.

Rel-18 features

- Uplink coverage enhancement (RAN1)

Essential from our perspective as an operator for coverage enhancement features in general. This includes support for repetition of PRACH, Msg3 and PUCCH for Msg4 HARQ-ACK in the RACH procedure.

- NTN-TN and NTN-NTN mobility enhancement ​(RAN2)

Almost essential to avoid UEs performing unnecessary cell search in areas where NTN/TN-cells are not available. We assume TN/NTN coexistence scenarios with difference bands are typical cases.

- Service continuity enhancement (RAN2)

RACH-less handover for NTN and application of location/time-based CHO to earth-moving cell can be considered as essential feature while it is necessary to align with 6G TN design.

- NW verified UE location (RAN1)

From our perspective, we cannot say this feature is essential in our commercial network. However, we can acknowledge there is some need in certain areas and countries, so we are open to discussion.

Rel-19 features

- DL coverage enhancements (RAN1)

SSB periodicity extension is a very important feature, not only due to satellite beam/power capacity, but also from the perspective of NW and UE power savings.

SIB PDCCH/PDSCH and Msg4 repetition can be natively supported in terms of coverage enhancement that will be beneficial for TN.

- UL capacity enhancements (RAN1)

Basically, any enhancements that can achieve higher data rate are welcome from our perspective as explained while other measures except for OCC can also be considered.

- Multicast/Broadcast Service (RAN2)

NOT essential for Day 1 NTN deployment since we cannot see any urgent or strong need for it.

- Regenerative payload support (RAN3)

Essential in our view to support gNB on-board that can achieve lower latency and higher throughput, as we believe NTN should offer competitive performance as explained.

In summary, we would like to propose the following:

Proposal 1:

- Each WG should decide whether to support NTN-related features in 6G Day 1, which were led by the WG in NR.

- From RAN2 perspective, the following features can be considered essential for 6G NTN:

  - RRC CONNECTED/IDLE mobility between NTN/NTN and NTN/TN:

    - Satellite ephemeris information should be provided

  - Multiple SMTCs per carrier

  - Location/time-based CHO and RACH-less HO, following a TN mobility design in 6G

  - Support of regenerative payload

2.2	Common design

For 6G, functionalities should ideally share a common design between TN and NTN. This means a function should be applicable to both network types. NW can configure the function for both TN and NTN cells, and the UE can perform the operation without regard to whether the cell is TN or NTN. Having such a commonality is expected to ease UE and especially gNB implementation of functions that are applicable to NTN and expand NTN commercial deployment. In our view, the aforementioned features that use satellite ephemeris information cannot be common with TN design since such information is not beneficial for TN in general. We think that features extended for NTN in NR that are particularly beneficial for TN are worth designing with a common design. From that perspective, coverage enhancement that operators generally take care of and SSB periodicity extension for NES are notable examples.

- Cell Selection: The default SSB periodicity was extended to 160 ms in NR Rel-19 NTN. 6G TN also considers such an extension for NES.

- Coverage Enhancement: Repetition of transmission/reception during RACH may not need separate designs for NTN/TN, which are beneficial for both scenarios.

Proposal 2:

- From RAN2 perspective, the following can be designed in a common way with TN

  - Cell search/selection process especially in terms of assumed SSB periodicity

  - Coverage enhancement: repetition of transmission/reception at least during RACH procedure

- Conclusion

In this contribution, we provided the following observations and proposals of our views on 6G TN/NTN integration.

Proposal 1:

- Each WG should decide whether to support NTN-related features in 6G Day 1, which were led by the WG in NR.

- From RAN2 perspective, the following features can be considered essential for 6G NTN:

  - RRC CONNECTED/IDLE mobility between NTN/NTN and NTN/TN:

    - Satellite ephemeris information should be provided

  - Multiple SMTCs per carrier

  - Location/time-based CHO and RACH-less HO, following a TN mobility design in 6G

  - Support of regenerative payload

Proposal 2:

- From RAN2 perspective, the following can be designed in a common way with TN

  - Cell search/selection procedure especially in terms of assumed SSB periodicity

  - Coverage enhancement: repetition of transmission/reception at least during RACH procedure

References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

- R1-2507814, “Discussion on overview of 6GR air interface”, NTT DOCOMO