3GPP TSG-RAN WG2 Meeting #132	R2-2508539

Dallas, U.S, Nov 17-21, 2025

Agenda item:	9.2.2

Source:	Sony

Title:	Discussion on Topology 2 for Ambient IoT

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 the following study item for Ambient IoT Ph2 was agreed [1].

2. Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify RRC-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1

For objective 2, Deployment Scenario 2 with Topology 2 with intermediate UE as Reader, the following activities for RAN2 have been outlined.

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

And for this upcoming meeting RAN2 should only consider DO-DTT and DT. (DO-A to be treated after March 2026).

At RAN2#131bis [5] the following agreements were made.

Agreements

- 1.  gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this paper, we discuss aspects related to Uu procedures, Invenory reports during RLF and Resource handling for the UE reader in Topology 2.

# 2 Discussion on Topology2

During the Rel-19 study phase [2], also Topology 2 was discussed the following high-level architecture was defined, .Figure 1

Figure 1, architecture topology 2

Figure 6.4.2-1 Logical system architecture for topology 2

In this topology the gNB is able to support an AIoT-enabled intermediate UE in topology 2.

## 2.1 Uu procedure (RRC based solution)

At the last meeting RAN2#131bis it was confirmed that the RRC based solution is to be used for the gNB to provide information to the UE reader.

It has been agreed that the gNB uses RRC dedicated signaling to provide the UE reader with relevant information to handle communication with the AIoT device, but the exact content needs further discussion.

Observation 1: It has been agreed that the gNB uses RRC dedicated signaling to provide the UE reader with relevant information to handle communication with the AIoT device, but the exact content needs further discussion.

The RRC based solution defined during the study [2] has the following protocol structure, see Figure 2.

Figure 2, Figure 6.4.2.1.1-1: RRC based solution of Topology 2

The following basic procedure steps were also defined using the A-IoT enabled intermediate UE as a AIoT reader.

Figure 3, Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

1a.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB1b.	The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources.

1c/2a	RRC communication with the A-IoT enabled UE takes place.

NOTE 1: RRC based communication is only depicted schematically, details subject to RAN2.

2.	The A-IoT-enabled gNB sends an Inventory response message to the A-IoT CN.

NOTE 2: In step 2, the A-IoT-enabled gNB may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s).

3.	The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s).

4a/4b.	After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled gNB may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result.

NOTE 3: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

NOTE 4: Step 4a/4b between A-IoT-enable UE and A-IoT-enabled gNB subject to RAN2.

The steps above is only high level as pointed out in Note 1 above. The signaling details between the gNB and AIoT enabled UE needs to be specified.

The specification needs to consider everything from finding suitable UE Reader to configure the selected UE Readers with relevant configuration parameters to be used when communicating with AIoT devices, e.g., resources to be used.  Also the gNB needs to convey relevant information from the CoreNetwork (CN), like Inventory or Command messages.

In the study (TR 38.769) it was concluded that it is very much up to RAN3 and SA2 on how to find and select suitable UE readers.

Proposal 1: Wait for progress in RAN3 and SA2 for UE reader selection in topology 2 before RAN2 may investigate if there may be any RAN2 impact.

### CN related information:

Information that would be relevant to convey to the reader has been concluded by SA2 in [6] [23.360] for topology 1 in Rel-19.

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests.

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN:

a)	A-IoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of A-IoT devices based on AF request.

c)	Size of the Inventory Response message from the A-IoT Device.

d)	Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN as specified in clause 5.9.

For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN:

e)	Size of the Command Response message from the A-IoT Device.

Bullet c) is determined by AIOTF based on the A-IoT Device ID length for the A-IoT Devices that are expected to respond to Inventory.

If not provided by the AF, bullet d) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the A-IoT service provider represented by an AF and the operator.

The assistance information is used by the NG-RAN for performing service operations, e.g., radio resource allocation by using bullets b), c) and e), A-IoT Device responses aggregation by using bullet d).

For topology 2 and since there should be no impact in the AIoT device itself, it seems logical that the same information is provided to the UE based reader. What can be further discussed is if all information is transparently conveyed by the gNB, or some information may be handled or altered by the gNB. E.g. it could be approximately number of devices that the gNB may distribute over multiple UE readers.

Proposal 2: Further study if Assistance information provided by CN intended for the topology 2 UE based Reader is transparently conveyed via the gNB or the base station may handle or modify some of the information.

## 2.2 Resource handling

### Resource allocation

Based on agreements that the gNB is using RRC dedicated signaling to convey resources to be used for AIoT interface transmissions, it is still open what exact information that will be provided, how it is triggered e.g., whether some assistance information can be relevant to be used etc.

In the study [2], the following conclusions where made related to resource allocation request.

For the radio resources allocation request, following alternatives are studied:

- Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

It was concluded in the study to further study how the resources are to be requested, and two alternatives where outlined, where the first is coming from the UE reader, and the second is coming from the CN.

Currently it is not clear if a certain resource request is even needed or it would be enough to get a service request and then up to gNB to handle that service request and allocate suitable resources.

Proposal 3: Based on proposed alternatives for CN- based on UE reader-based resource request RAN2 to study these alternatives and the option where it is handled by gNB based on related service request.

### Resource scheduling

Further it needs to be discussed how the resource scheduling will be done.

The initial options may be that resources are either scheduled by the base station, or the UE reader is allocated with a set of resources and handles the scheduling freely. This would correspond to mode 1 and mode 2 related to Sidelink Relays.

Further discussion is needed on the resource scheduling alternatives, in terms of which entity is responsible.

Proposal 4: Discuss alternatives for Resource scheduling can be similar as to Sidelink Relay, where either the gNB handles the resource scheduling or the UE reader autonomously handles the scheduling.

### Resource validity

The validity of the resource configuration provided to the UE reader is relevant to discuss including the following aspects.

- Timer based or explicit release

- Temporary out of connection

- During RLF

- During HO

In the study item phase, it was concluded that radio resources are not valid across multiple cells [2, ch 6.3.6], but further study is needed for resources that are semi-statically allocated for 4 different scenarios, including all RRC states (CONNECTED, INACTIVE and IDLE). The IDLE scenario is down prioritized in this release.

For the INACTIVE case two options are discussed, whether resources are explicitly released or released by a timer.

For the CONNECTED case, two scenarios are depicted where the second scenario support “temporary out of connection. In any of the cases, and similar as for Inactive case, the resources are either explicitly released or released by a timer.

For any of the cases, RRC INACTIVE or RRC CONNECTED it seems logical to support both explicit release as well as timer-based release.

Proposal 5: Both explicit release and timer-based release seems logical to support for a UE reader in any of RRC_CONNECTED or RRC_INACTIVE state.

### Interference

The allocation of resources needs also to be carefully understood. E.g. to avoid inference between multiple UE readers or AIoT enabled UEs, if they are in the vicinity of each other.Several AIoT-enabled UEs may be configured as intermediate node readers to communicate with AIoT devices. It is expected that the NW/gNB assign certain resources to these AIoT-enabled UEs for communication with the AIoT devices. In a dynamic area where multiple AIoT-enabled UEs are in the vicinity of each other and the number of AIoT devices may vary, a lack of scheduling management may result in severe interference and collisions between the AIoT-enabled UEs. In our view, it is necessary to study solutions where the assignment of the physical resources are performed such that it avoids interference among AIoT enabled UEs.

Proposal 6:	RAN2 to study solutions on how to perform resource allocations for AIoT-enabled UEs to avoid potential interference between UE readers that are in vicinity of each other

### Mobility

At the last RAN2 meeting it was agreed that the allocated resources only are valid in the serving cell.

This would mean that there is no need to support any mobility in terms of Handover procedure. For a configured UE reader, the allocated resources would not be valid anymore after a Handover has been completed.

One can discuss whether the resource allocation is revoked at time for when the Handover procedure is initiated by the serving gNB or when it is terminated (i.e. when the UE sends "Handover complete” the RRCREconfigurationComplete to the new base station.

The handover procedure starts with serving gNB sending Handover Request to target gNB, which after some admission control, provides a reply including a RRCReconfiguration message intended for the UE. The serving gNB forwards this to the UE which then starts performing the handover to the target NB, where the procedure is then completed when the UE send “the RRCREconfigurationComplete to the new base station.

Since the AIoT activities for a UE reader would not be very time-critical, it would seem reasonable that the AIoT activities are terminated or suspended when a Handover procedure is initiated.

On the other hand, it may be left to reader implementation whether AIoT activities can be commenced until the UE has moved successfully to the new cell. There may be little rational for why a reader would conduct AIoT activities during an ongoing Handover, but from a resource validity point of view, it may be beneficial to not delete the configured AIoT resources in case the Handover fails. If that is the case, the UE may simply stay on serving cell and uses configured resources.

Observation 2: Based on agreement that configured resources are only valid in serving cell, mobility can be assumed not to be supported.

Proposal 7: In case the UE reader performs a Handover to a new cell, the configured resources are valid until the handover procedure is completed and the UE is connected to the new cell.

## 2.3 Inventory reports during RLF

A-IoT enabled UE may encounter RLF and perform re-establishment either on the same cell or a different cell. One potential problem during RLF could be the pending RRC Inventory Report messages and whether A-IOT-enabled UE should store these messages during RLF and send them when the connection has been established or discard these messages. The solution depends on if upper layers can recover or account for the missing inventory reports or not. We think RAN2 should discuss this scenario.

Proposal 8:	RAN2 to discuss the handling of pending inventory reports during A-IoT enabled UE RLF.

# 3 Conclusion

In this contribution we have made the following observations and proposals.

Observation 1: It has been agreed that the gNB uses RRC dedicated signaling to provide the UE reader with relevant information to handle communication with the AIoT device, but the exact content needs further discussion.

Proposal 1: Wait for progress in RAN3 and SA2 for UE reader selection in topology 2 before RAN2 may investigate if there may be any RAN2 impact.

Proposal 2: Further study if Assistance information provided by CN intended for the topology 2 UE based Reader is transparently conveyed via the gNB or the base station may handle or modify some of the information

Proposal 3: Based on proposed alternatives for CN- based on UE reader-based resource request RAN2 to study these alternatives and the option where it is handled by gNB based on related service request

Proposal 4: Discuss alternatives for Resource scheduling can be similar as to Sidelink Relay, where either the gNB handles the resource scheduling or the UE reader autonomously handles the scheduling.

Proposal 5: Both explicit release and timer-based release seems logical to support for a UE reader in any of RRC_CONNECTED or RRC_INACTIVE state.

Proposal 6:	RAN2 to study solutions on how to perform resource allocations for AIoT-enabled UEs to avoid potential interference between UE readers that are in vicinity of each other

Observation 2: Based on agreement that configured resources are only valid in serving cell, mobility can be assumed not to be supported.

Proposal 7: In case the UE reader performs a Handover to a new cell, the configured resources are valid until the handover procedure is completed and the UE is connected to the new cell.

Proposal 8:	RAN2 to discuss the handling of pending inventory reports during A-IoT enabled UE RLF.

# 4 References

- . RP-252894)New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2, (Ambient_IoT_Solutions_Ph2, leading WG: RAN1; REL-20; WID: RP-252894)

- TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19

- S2-2507761, Architectural Assumptions

- TR 23.700-13, V19.0.0 (2025-03), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

- Minutes RAN2#131bis

- TS 23.060 ver 19