3GPP TSG RAN WG2 #132	R2-2508549

Dallas, USA,  Nov. 17th - 21st, 2025

Agenda item:	8.8.2

Source: 	ITRI

Title: 	Serving cell SMTC and reference location in System Information

Document for:	Discussion

1	Introduction

In RAN2#131bis the issues raised in [A200] and [H251] are postponed.

In [A200] it is proposed to allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

In [H251] it is proposed to modify the field description of smtc in SIB2 to specify the smtc is for serving cell measurements when smtc5list is also configured and broadcast by a NTN cell.

From the specification descriptions of current running CR and the perspective of backward compatibility this contribution discusses whether the smtc in SIB2 need to be associated with a reference location.

2	Discussion

As specified in TS 38.300 [1], network can configure up to 7 SMTCs for UE in RRC_IDLE/RRC_INACTIVE, including the SMTC of the serving cell. Wherein the total number of configurable SMTCs offsets across smtc4list and smtc5list is 6 [2]. If the network configures the UE with more than 4 SMTCs in RRC_IDLE/RRC_INACTIVE, the UE selects up to 4 SMTCs based on location and only considers them for measurements. [2]

It could be understood as that only if smtc5list is included in SIB2 then the UE supporting downlink coverage enhancements would perform SMTC selection based on location.

Observation 1: Only when smtc5list is included in SIB2 then a NTN UE capable of DL_CE would perform location-based SMTC selection.

SIB2 field descriptions [3]

refLocList Indicates a reference location associated to an SMTC configuration in smtc5list in an NTN (quasi-)Earth fixed cell. If present, it includes the same number of entries as smtc5list. The first entry in this list corresponds to the first entry in smtc5list, the second entry corresponds to the seccond entry in smtc5list, and so on.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The network only includes SMTC offsets in smtc5list for (quasi-)Earth fixed cells. The total number of configurable SMTC offsets across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

As shown in the table above, according to the field description [3], if refLocList is present, the refLocList includes the same number of entries as smtc5list. Each entry of refLocList is one to one corresponds to an entry of smtc5list in the same order. That is, each entry of smtc5list is associated with a reference location in refLocList-r19.

Observation 2: Each entry of smtc5list is associated with a reference location in the refLocList-r19 that could support location-based SMTC selection.

However, the field description of smtc in SIB2 specifies a frequency specific configuration rather than a cell specific configuration, the smtc in SIB2 is not necessarily associated with the serving cell measurements. It is not clear whether and how a UE could take account of smtc in SIB2 for location-based SMTC selection.

Observation 3: The current field description is not clear whether the smtc in SIB2 is used for serving cell measurements. It is not clear whether and how a UE could take account of smtc in SIB2 for location-based SMTC selection.

For NTN, UE can adjust the actual offset values. The offset value of the smtc in SIB2 is evaluated based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms and UE can adjust the actual offset based on the actual propagation delay difference between the serving cell and the neighbour cells.

Considering NTN UE in RRC_IDLE/RRC_INACTIVE would adjust the offset values according to the PCI and the associated ntn-Config of ntn-NeighCellConfigList in SIB19, and each entry of smtc4list and smtc5list may further indicate the PCIs that follow the SMTC. It is feasible to consider smtc in SIB2 is configured based on the ntn-Config associated with the serving cell.

Observation 4: UE can base on smtc in SIB2 to adjust the offset values associated with the PCIs in ntn-NeighCellConfigList and in the entries of smtc4list and smtc5list. It is feasible to consider that the smtc in SIB2 is configured based on the ntn-Config associated with the serving cell.

Therefore, it is proposed to clarify the field description to associate smtc in SIB2 with the reference location of the serving cell.

Proposal 1: For NTN, it is proposed to associate smtc in SIB2 with the reference location of the serving cell.

3	Conclusion

This contribution discusses whether the smtc in SIB2 need to be associated with a reference location, observations and the proposal is provided as in the following:

Observation 1: Only when smtc5list is included in SIB2, a NTN UE capable of DL_CE would perform location-based SMTC selection.

Observation 2: Each entry of smtc5list is associated with a reference location in the refLocList-r19 that could support location-based SMTC selection.

Observation 3: The current field description is not clear whether the smtc in SIB2 is used for the measurements of the serving cell. It is not clear whether and how a UE could take account of smtc in SIB2 for location-based SMTC selection.

Observation 4: UE can base on smtc in SIB2 to adjust the offset values associated with the PCIs in ntn-NeighCellConfigList and in the entries of smtc4list and smtc5list. It is feasible to consider that the smtc in SIB2 is configured based on the ntn-Config associated with the serving cell.

Proposal 1: For NTN, it is proposed to associate smtc in SIB2 with the reference location of the serving cell.

4	References

[1] R2-2507779	Miscellaneous Stage 2 corrections for NR NTN phase 3	THALES (Rapporteur)	CR	Rel-19	38.300	19.0.0	NR_NTN_Ph3-Core, Prague, Czech Republic.

[2] R2-2507703	Report from session on NES, NR NTN and IoT NTN	Session chair (ZTE)	Report,  Prague, Czech Republic.

[3] R2-2507946	Prague, Czech Republic	Ericsson		CR	Rel-19	38.331	19.0.0	NR_NTN_Ph3-Core, Prague, Czech Republic.