3GPP TSG-RAN WG2 Meeting #132		         R2-2508550

Dallas, USA, 17th - 21st, November 2025

Agenda Item:	10.4

Source: 	ITRI, Acer Incorporated

Title:  	Discussions on the RLM/RLF for 6G mobility

Document for:	Discussion

Introduction

During the RAN#108 meeting, the study on 6G radio was approved [1]. This paper presents the perspective on the RLM/RLF topic to be addressed under the following agenda item:

“Mobility framework and targets, Measurement Framework, Intra-RAT Connected Mobility (e.g. L3, CHO, LTM, RLM/RLF), Cell Selection and Reselection, Inter-RAT.”

Discussion

In 5G NR systems, 3GPP has specified two complementary procedures: RLM and RLF handling to maintain reliable connectivity. These mechanisms work in tandem to ensure service quality and recovery from connectivity issues.

Radio Link Monitoring Framework

The RLM procedure serves as the foundation for link quality assessment. During normal operation, the UE continuously monitors the quality of configured reference signals on its active BWP. This monitoring process involves measuring signal quality metrics and comparing them against predefined thresholds.

The 5G NR RLM framework has been designed with sufficient flexibility to support multiple failure scenarios. Notably, it encompasses both BFD for addressing beam-specific issues in millimeter-wave deployments, and traditional RLF handling for managing overall connection failures. This dual capability is particularly important in 5G networks that utilize beamforming techniques, where the failure of a specific beam does not necessarily indicate complete link failure.

Radio Link Failure Recovery Process

When the UE determines that radio link failure has occurred—typically after a series of consecutive quality threshold violations—it initiates a structured recovery procedure. The process begins with the declaration of RLF, at which point the UE immediately starts the RRC re-establishment timer T311. During the T311 timer period, the UE performs the cell selection procedure to identify a suitable cell for re-establishment. Once a suitable cell is identified, the UE sends an RRC re-establishment request message. If the network can retrieve the UE context, the connection could be re-establishment.

However, if the UE cannot complete re-establishment—either due to the absence of suitable cells or unsuccessful re-establishment attempts—it must transition to RRC_IDLE state. This fallback mechanism ensures that the UE returns to a known state and can subsequently attempt to establish a fresh connection through normal procedures.

Implications for 6G Development

Given the proven effectiveness of the 5G NR RLF procedure in maintaining connection reliability, we consider this framework essential as a baseline for 6G system design. The fundamental principles of continuous monitoring, threshold-based failure detection, and structured recovery procedures have demonstrated their value across diverse deployment scenarios. Therefore, preserving the core RLM and RLF concepts while extending them to address 6G-specific challenges represents a pragmatic approach to ensuring robust connectivity in next-generation wireless systems.

Proposal 1. 5G RLF handling should be adopted as a baseline for 6G.

In 5G NR networks, the PCell serves as the anchor point for the UE's RRC connection and carries critical control signaling. When an RLF occurs on the PCell in traditional single-cell configurations or carrier aggregation deployments, the consequences can be severe. Since the PCell is responsible for maintaining the RRC connection state, its failure necessitates a complete RRC re-establishment procedure to restore connectivity.

This conventional recovery approach, while functionally reliable, introduces substantial service interruption that can significantly impact user experience. The re-establishment process involves multiple steps: RLF declaration, cell selection during the T311 timer window, transmission of the RRC re-establishment request, network authentication, and reconfiguration of radio bearers. Throughout this entire sequence, user plane data transmission is suspended, resulting in interruption durations that can range from hundreds of milliseconds to several seconds depending on network conditions.

Enhanced Recovery Through Dual Connectivity Architecture

The introduction of dual connectivity in 5G NR provides a fundamentally more resilient approach to handling PCell failures. In dual connectivity configurations, the UE simultaneously maintains connections to two cell groups: the MCG anchored by the PCell, and the SCG anchored by the PSCell. This architectural diversity creates a critical redundancy path that can be leveraged during failure scenarios.

When a PCell failure occurs in dual connectivity mode, the UE has an alternative communication channel available through the SCG. Instead of immediately initiating RRC re-establishment—which would disrupt the entire connection—the UE can report the PCell failure to the network via the SCG link using dedicated failure information messages. This reporting mechanism allows the network to maintain awareness of the UE's connection state and take appropriate recovery actions while keeping the SCG connection active.

Baseline for 6G Network Resilience

Given the substantial improvements in service continuity and connection reliability demonstrated by the existing PCell fast failure recovery mechanism, we consider this approach an essential baseline for 6G system design. The fundamental principle of leveraging connection diversity to enable seamless recovery without full re-establishment.

The dual connectivity-based fast recovery framework provides a proven foundation that can be extended to support 6G's multi-connectivity architectures, such as multi-RAT dual connectivity or coordinated multi-point connectivity scenarios.

Proposal 2: 5G PCell fast failure recovery should be considered as a baseline for 6G.

Given that Day-1 6G deployments are expected to prioritize CA configurations due to infrastructure maturity and cost-effectiveness, we propose enhancing CA resilience through an SCell-based fast PCell recovery mechanism.

Proposed Recovery Mechanism

Our proposed mechanism adapts the proven dual connectivity fast recovery concept to CA scenarios. Upon detecting an RLF on the PCell, the UE evaluates the status of configured SCells. If one or more SCells maintain adequate link quality, the UE transmits a PCell failure information message through a surviving SCell instead of initiating conventional RRC re-establishment.

This failure report may contain critical diagnostic information including the failure cause, neighboring cell measurements, or last known PCell conditions. By leveraging the SCell as an emergency communication channel, the network could receive timely notification while the UE's connection context remains valid and the remaining CA configuration stays active.

Network Recovery Actions and Benefits

Upon receiving the failure report, the network can orchestrate tailored recovery actions. Recovery options could include: initiating controlled handover to a different cell or release the radio connection. These actions can be delivered through the functional SCell, with the entire recovery process typically completing within 100-300 milliseconds compared to 500+ milliseconds for traditional re-establishment.

Impact on 6G Network Reliability

By leveraging SCell-based failure reporting, the network can deliver reliable recovery commands that significantly reduce service interruption time and improve overall CA availability. We therefore consider this SCell-based fast PCell recovery mechanism could be an enhancement for 6G carrier aggregation frameworks.

Proposal 3: Study PCell failure recovery without interruption in 6G CA, where the UE utilizes an SCell to report PCell RLF and enables fast recovery without RRC re-establishment.

Conclusion

Proposal 1. 5G RLF handling should be adopted as a baseline for 6G.

Proposal 2: 5G PCell fast failure recovery should be considered as a baseline for 6G.

Proposal 3: Study PCell failure recovery without interruption in 6G CA, where the UE utilizes an SCell to report PCell RLF and enables fast recovery without RRC re-establishment.

References

[1] RP-251881, New SID: Study on 6G Radio, June 2025.