3GPP TSG-RAN WG2#132											   R2-2508554

Dallas, USA, 17 – 21 November 2025

Agenda Item:	9.7.2

Source: 	NEC

Title:	        Voice over GSO based on UP-CIOT

Document for:	 Discussion

# Introduction

RAN2#131bis was the first meeting we discuss the topic voice over GSO, majority of companies in RAN2 prefer the UP solution (because of the higher impacts on RAN2 specs of the CP solution), and further SA2 also down-selected to UP solution as in S2-2509293.

Assuming with UP solution, there are some aspects request further study in RAN2, which we will share our initial opinion:

- Handle voice packets of different sizes

- Support new SPS periodicities like 120ms and 240ms

# Discussion

  - Support variable voice packet size

Supporting variable voice packet size is asked in the LS from SA2 in S2-2507636, and RAN2’s answer in reply LS is copied as in blew:

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? RAN2 reply to Question 3: Scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

We can see that the different packet sizes are caused by transition between voice packet and silence packet, also for voice packet, packet size vary because of ROHC state changes.

For voice packet and silence packet transition, two SPSs of different sizes may be configured and dynamically activated/deactivated by DCI based on reported BSR or received MAC PDU with padding. For example, when SPS of large size is used for voice packet, once silence periodicity starts, the network may know packet change based on the received padding. In this case, the network may activate SPS of small size with long periodicity and deactivate SPS of large size with short periodicity. We might consider minor enhancement to make the activation of one SPS and deactivation of another SPS more efficient. But it should be doable.

For ROHC state change, the network can know variation of data volume based on reported BSR only, then the network dynamically schedules UE based on SR/BSR. Alternatively, network configure SPS of the maximum size. using DG, Huge RTT latency may make the packet delay budget cannot be fulfilled at all.

Reference to TS 23.203 [4], for VoLTE, the IMS signaling has QCI 5 with Non-GBR resource type, while the voice data has QCI 1 with GBR resource type. Both have 100ms packet delay budget.

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) | GBR | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 5 (NOTE 3) | Non-GBR | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |

For Voice over GSO, Normative service requirements and KPIs on IMS voice call using GSO satellite access is captured in TS 22.261, as shown in Table 1 below. Where we did not find packet delay budge information.

Table 1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |  |

Packet delay Budget would be the key if dynamic scheduling is acceptable at all. We either ask SA4 this requirement or wait for their work.

If SPS is used, overscheduling cannot be avoided. Consequently, the gain brought by ROHC would disappear due to the limitation on scheduling.  It is still under discussion in SA2 whether we will go with UP IP solution or UP non-IP solution, it might end up without ROHC.

Proposal 1: RAN2 down prioritize handling of variable packet size during talk spurt resulting from ROHC, unless 1) input from SA4 regarding packet delay budget requirements is received and DG is possible 2) SA2 selects UP IP solutions and will introduce ROHC in either AS or NAS.

Proposal 2: RAN2 supports two SPS where networks can dynamically switch to support different packet size transition between talk spurt and science period

In LTE, SPS periodicity is configured by RRC signaling and SPS related time and frequency domain resource is activated/deactivated by DCI. In NR, configured grants of two types are introduced, i.e. CG type 1 and CG type 2. UL grant for CG type 1 is configured by RRC signaling. While UL grant for CG type 2 is activated/deactivated by DCI, which is similar to SPS of LTE. Then, for voice service, if CG type 1 is used, when talk spurt periodicity switches to silence periodicity or silence periodicity switches to talk spurt periodicity, RRC signaling for switching two SPS resources can result in larger latency compared to L1 signaling. Besides, introducing one new type has a big impact on specification. Therefore, we prefer to use SPS in LTE, i.e. SPS periodicity is configured by RRC signaling and time/frequency domain resources can be activated/deactivated by DCI.

Proposal 3: SPS in LTE can be used for voice transmission, i.e. SPS periodicity is configured by RRC signaling and time/frequency domain resource is activated/deactivated by DCI.

Considering voice packet and silence packet have different periodicities, in order to accommodate them, two SPSs with the different periodicities can be configured to the UE and can be dynamically switched as mentioned in proposal 2. DCI can be used for this purpose. For example, when one SPS is activated by DCI, it also means the previously activated SPS is deactivated, which may reduce PDCCH signaling overhead and timely activate/deactivate SPS resource.

Proposal 4: When two SPS resources for UL or DL are configured, a joint SPS activation/deactivation by DCI can be introduced to activate one SPS resource and deactivate the other SPS resource simultaneously.

  - New SPS periodicities

For VoLTE, the packet interval of talk spurt is 20ms and the packet interval of silent period is 160ms. In case of IMS voice call over GSO, SA4 also considers defining bundling packets intervals of talk spurt like 120ms or 240ms, they did not mention whether 160ms of silent period would be changed.

In legacy NB-IoT, only UL SPS for BSR is supported, with periodicity divided by 10240ms as below.

SR-SPS-BSR-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

As mentioned in SA4 LS [5], for 120ms and 240ms bundling periods that are not divided by 10240ms, RAN2 reply “RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues”

In our understanding, new periodicities have to be introduced to support voice over GSO, regardless of what the value will be defined, which likely will be multiple time of 20ms, and cannot divide 10240.

Based on current SPS formular, upon H-SFN warp around, the period will be kept as the same. So there would be no issue of mismatch between the SPS resource and packet arrival time. On the other hand, company mentioned that upon HFN wrap around, the SFN value where SPS locates in the new H-SFN cycle will be different from SFN value where SPS resource located in the previous H-SFN cycle. But this is not a problem rather a intention to keep the periodicity.  The potential collision can be handled in RAN1.

Proposal 5: introduce new periodicities (FFS on values) for both Downlink and Uplink to support voice over GSO,

Proposal 6: RAN2 send LS to RAN1 and kindly ask RAN1 work on collision between SPS occasions with other channels.

# Conclusion

In summary, we would like to share our view on Voice over GSO as follows:

Proposal 1: RAN2 down prioritize handling of variable packet size during talk spurt resulting from ROHC, unless 1) input from SA4 regarding packet delay budget requirements is received and DG is possible 2) SA2 selects UP IP solutions and will introduce ROHC in either AS or NAS.

Proposal 2: RAN2 supports two SPS where networks can dynamically switch to support different packet size transition between talk spurt and science period

Proposal 3: SPS in LTE can be used for voice transmission, i.e. SPS periodicity is configured by RRC signaling and time/frequency domain resource is activated/deactivated by DCI.

Proposal 4: When two SPS resources for UL or DL are configured, a joint SPS activation/deactivation by DCI can be introduced to activate one SPS resource and deactivate the other SPS resource simultaneously.

Proposal 5: introduce new periodicities (FFS on values) for both Downlink and Uplink to support voice over GSO.

Proposal 6: RAN2 send LS to RAN1 and kindly ask RAN1 work on collision between SPS occasions with other channels.