3GPP TSG-RAN WG2 Meeting #132	R2-2508578

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.3.1

Source:	Ericsson

Title:	Discussion on data transfer and model transfer in 6G

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we share our views on data transfer and model transfer in 6G.

# 2	Discussion

## 2.1 Data transfer

The concept of data transfer in the 6G RAN SI includes the transfer of various types of data from the UE to the network [1]. In this section we focus on data transfer over the air interface for the purpose of AI/ML data collection, sensing, and observability.

### 2.1.1 Framework

For AI/ML, several use cases have been introduced in NR Rel-19 and Rel-20 (e.g. beam management and mobility), to support AI/ML models at the network side (gNB) or at the UE side. There are associated data collection procedures for both these cases, namely a network-side data collection procedure (i.e. data collection for training of network-side models) and a UE-side data collection procedure (i.e. data collection for training of UE-side models). Accordingly, we assume that in 6G there will also be network-side and UE-side data collection.

For AI/ML network-side data collection, the model training may be handled at the RAN (xNB), where the xNB is the data consumer of the collected data from the UE to the network. Since the xNB is the data consumer, a requirement for 6G is that the xNB (rather than another network node) is the termination point for this kind of data.

In case of network-side data collection, the xNB can be the consumer of the collected data, e.g. if the AI/ML model training is handled at the xNB.

In the case that the training for the network-side model is implemented at OAM, the OAM should be of course involved. In this case, the xNB is still the termination point for the data collection report from the UE to the network, but the xNB needs to transfer the collected data to the OAM according to configuration received from the OAM. However, data transfer in the network (i.e., northbound from the RAN) is up to RAN3 discussions.

Data transfer from RAN to other network entities is up to RAN3 discussions.

Regarding sensing, RAN2 has agreed to study sensing from RAN2#133bis. Accordingly, once the RAN2 aspects of sensing are settled, RAN2 can assess whether the agreements for AI/ML data transfer could be applied to transfer of sensing data.

The discussion on data transfer for sensing needs to be postponed until sensing reaches sufficient maturity in 6G discussions.

Another key aspect is how the support for observability fits into the data collection framework. In 6G, the need for observability (i.e., the tools to monitor, analyse and ensure the network performance) is stronger than in any previous generation of mobile networks. Like in earlier generations, it is important that deployment-related data is collected to be used for understanding the shortcomings of the deployment, etc. At the same time, 6G will cover new services and business models beyond eMBB. It is crucial that operators can collect information in a way that service planning and delivery can be automated. For example, by understanding where a service can be delivered and the required functionality, or by assessing whether the corresponding conditions are fulfilled (e.g., user plane KPIs, location or coverage of the UE, etc.). Thus, it is not only important to collect the relevant information, but also to be able to correlate the different elements.

The fundamental enabler for supporting such observability functionalities over the air interface are data collection procedures. As captured in Observation 1, for data collected by the UE for the purpose of NW-side data collection, it is important to have a termination point in xNB, since the data can be consumed at the xNB, or transferred from the xNB to the OAM. We foresee the same approach for the data collected for observability purposes, i.e. those data that in NR are conveyed in MDT. Namely, in 6G networks, the data collection for observability purposes could leverage the framework for data collection for AI/ML model training.

A common framework in the air interface is designed for collecting data from the UE for the purpose of network-side AI/ML data collection and for network observability, where the data is terminated in the xNB.

### 2.1.2 Data types

RAN2#131bis agreed the following:

For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

Accordingly, some proposed example data assumptions for data collection in some AI use cases and MDT are summarized in Table 1. For example, in beam management (BM), data size of one data collection for L1-RSRP assumes a single sample per UE for BM Case 1 (spatial prediction) and 10 samples per UE within 400 ms for BM Case 2 (temporal prediction). In AI mobility, the data size of one data collection for L3-RSRP considers the assumption of 25 samples per UE and 33 measured cells per sample. Channel matrix is used in the calculation for CSI prediction [2]. However, if using precoding matrix, data size in one data collection will be much smaller.

Table 1: Data assumptions for data transfer in 6G

| Data type | Applicable use cases | Data collection | Data size | Reporting frequency [2] | QoS requirement |
| --- | --- | --- | --- | --- | --- |
| CSI | CSI prediction | UE-side data collection | ~1.5 Mb in 440 ms (Note 1) | Weeks/months | Relaxed latency |
| L1-RSRP (beam-level) | beam management | NW-side data collection | 582 bits for BM Case 1, and 5.82 kb in 400 ms for BM Case 2 (Note 2) | Weeks/months | Relaxed latency |
|  |  | UE-side data collection | 582 bits for BM Case 1, and 5.82 kb in 400 ms for BM Case 2 (Note 2) | Weeks/months | Relaxed latency |
| L3-RSRP (cell-level, beam-level) | AI mobility (e.g. RRM measurement prediction) | NW-side data collection | ~6 kb in 2 sec for FR1 and ~23 kb in 1 sec for FR2 (Note 3) | Weeks/months | Relaxed latency |
|  |  | UE-side data collection | ~6 kb in 2 sec for FR1 and ~23 kb in 1 sec for FR2 (Note 3) | Weeks/months | Relaxed latency |
|  | logged MDT | NW-side data collection | Single report: 462 bits in 320 ms Total report: ~240 kb in 2.8 min (Note 4) | Weeks/months | Relaxed latency |
| L3-RSRQ, L3-SINR (cell-level, beam-level) | logged MDT | NW-side data collection | Single report: 924 bits in 320 ms Total report: ~480 kb in 2.8 min (Note 4) | Weeks/months | Relaxed latency |
| Location Info | logged MDT | NW-side data collection | Single report: 208 bits in 320 ms Total report: ~108 kb in 2.8 min (Note 4) | Weeks/months | Relaxed latency |
| Time | logged MDT | NW-side data collection | Single report: 13-61 bits in 320 ms Total report: ~6.8 kb in 2.8 min (Note 4) | Weeks/months | Relaxed latency |
| Note 1: Assume channel matrix with float 32 for target CSI, and 10 CSI-RS observation instances as input to predict one future CSI instance. CSI-RS period is 40 ms. [2] Note 2: Assume 7 bits for L1-RSRP of the strongest beam and 4 bits for L1-RSRP of the remaining beams, and (Set A, Set B) = (128, 16). In BM Case 1, 582 bits per sample [2]. In BM Case 2, 10 samples in one data collection and CSI-RS period is 40 ms. Note 3: Assume 25 samples from one UE in one data collection, where one sample includes L3-RSRP values for 33 measured cells. In FR1 case, one cell-level L3-RSRP for each measured cell. In FR2 case, one cell-level L3-RSRP and three beam-level L3-RSRP per measured cell. 7 bits for each L3-RSRP. Sample period is 80 ms in FR1 and 40 ms in FR2. Note 4: Assume 520 samples from one UE in one xNB in one data collection, 320 ms as sample period (i.e. report period) and one absolute time information (48 bits) for these 520 samples, where one sample includes - One serving cell: one cell-level L3-RSRP (7 bits), one cell-level L3-RSRQ (7 bits), one cell-level SINR (7 bits), one beam-level L3-RSRP (7 bits), one beam-level L3-RSRQ (7 bits), one beam-level L3-SINR (7 bits) - 64 neighbour cells: one cell-level L3-RSRP (7 bits), one cell-level L3-RSRQ (7 bits), one cell-level SINR (7 bits) - One location information (208 bits) and one relative time information (13 bits) Note 5: Relaxed latency means e.g. minutes, hours, days or no latency requirement Note 6: Assistance information (e.g. cell identity, beam index, carrier frequency, PLMN) is not counted. |  |  |  |  |  |

Consider data assumptions in Table 1 for study of data transfer and data collection in 6G.

### 2.1.3 Procedures

The data collection radio configuration from RAN to UE is determined by the RAN for AI/ML use cases and network observability in 5G, as the RAN makes the decision for measurement resource allocation no matter whether it is network-side data collection or UE-side data collection for AI/ML, or another use case.

Additionally, as discussed in Section 2.1.1, RAN can be the data consumer, i.e. the so-called gNB-centric data collection in 5G. In this case, the RAN is also the initiator of the data collection configuration. This case can still be assumed in 6G. Furthermore, in the case of e.g. model training handled by a network node different than the RAN node that collects the data, the OAM may request the RAN to initiate this data collection configuration for centralized data collection management purpose.

Regarding UE-side data collection, in 5G the UE can indicate its preference to initiate data collection by sending a data collection start request or its preferred candidate radio configurations for data collection (out of pre-configured candidates) to RAN. Based on this, the RAN may decide to provide a data collection configuration to the UE. Similarly to 5G, it can be assumed that also in 6G the UE can provide its preference to initiate data collection.

Data collection radio configuration transmitted from RAN to UE is decided by RAN, where data collection may also be initiated upon request from another network entity (e.g., the OAM) or from the UE.

For observability purpose, the collected data could be consumed by the RAN immediately after being collected. On the other hand, data transfer for AI/ML data collection in 6G has relaxed latency requirement for the given examples in Table 1, so to reduce signalling overhead it should be supported that the UE logs data and sends it in bulk.

A request/response mechanism was used in 5G for data collection report to request “one-by-one” the various samples of data collected by the UE. This method requires redundant signalling exchange between network and UE and moreover introduces unnecessary transmission delay. Therefore, it is proposed to study ways for efficiency improvement and overhead reduction in data collection report of 6G.

Support reporting of logged data collected for network-side purposes, and study request/response mechanism for the data collection report.

In some use cases (e.g. AI mobility, AI positioning), it is possible for the UE to perform measurements in the RRC_IDLE mode and RRC_INACTIVE mode for data collection purpose. Although data collection in RRC_IDLE/INACTIVE might be difficult or not be possible in some use cases and it is yet unclear which RRC states will exist in 6G, RAN2 should study ways to enable data collection in different RRC states, in order to reduce the impact on UE and NW side (as it is for example in the Rel-19 data collection framework, which might have significant impacts on the UE and NW conventional operations). We also note that depending on the specific use case, data collection during mobility should be also studied in the context of 6G.

Study data collection in all RRC states and data collection during mobility in 6G.

Related to the UE-side data collection, similar to the discussion we have in 5G, the data are collected by the UE for the purpose of training a UE-side model, and it is not clear whether this data can be really of use for the network optimization. There may be cases where the same data measured for UE-side purposes is also relevant to NW-side purposes, so it may be meaningful for the RAN to have visibility of it without the UE performing the same measurements twice. However, in general, it is not clear whether all the data collected by the UE can be really standardized or whether it is needed to standardize it. Consequently, RAN2 should consider ways to enable partial or no visibility of UE-side data at the RAN.

Study how to enable partial or no visibility at the RAN of the data collected by the UE for the purpose of UE-side data collection.

## 2.2 AI/ML model transfer

Unlike 5G, the proliferation of AI/ML use cases and the expected wider deployment of AI/ML in real world devices might significantly increase the overhead in the 3GPP network of traffic related to AI/ML model transfers from the UE-side training server to the UE. Even though the UE-side models may remain proprietary and the inherent content or structure of the model may remain unknown to the network, i.e. not standardized, it is important for the network to be in control of any AI/ML model transfer/delivery procedure. This means that it should be possible for the MNO to control the model transfer process, such as being able to allow/not allow the model transfer as well as managing the model transfer traffic, in order to ensure, for example, the QoS of user traffic and limit the network overload.

Study ways to enable a controllable model transfer/delivery procedure. It should be possible for the network to control the model transfer/delivery from the UE-side training server to the UE.

## 2.3 Cooperation with other WGs

In 5G it was identified that many of the data collection aspects are not in the scope of RAN2 and thus the involvement of several other working groups (e.g. RAN3, SA2, SA5, SA3) was needed for achieving complete and meaningful standardized solutions. We expect a similar situation to occur in the 6G study for data collection and model transfer, and in our view, it should be ensured that the studied solutions are consistent across the entire network, i.e. across different working groups, and that it is clear what aspects should be studied by which group. In fact, this is the spirit of the LS that SA sent to SA2, SA5, SA3, SA6, TSG RAN, RAN2 and RAN3 in [3].

Moreover, RAN2#131bis already agreed to study some aspects that are largely outside of RAN2’s scope:

“1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). ”

Thus, RAN2 should identify at an early stage which aspects require the involvement of other working groups and notify the relevant groups.

RAN2 to identify which data collection and model transfer aspects pertain to the scope of other working groups and coordinate with the relevant groups at an early stage of the 6G study.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In case of network-side data collection, the xNB can be the consumer of the collected data, e.g. if the AI/ML model training is handled at the xNB.

Observation 2	Data transfer from RAN to other network entities is up to RAN3 discussions.

Observation 3	The discussion on data transfer for sensing needs to be postponed until sensing reaches sufficient maturity in 6G discussions.

Observation 4	Data collection radio configuration transmitted from RAN to UE is decided by RAN, where data collection may also be initiated upon request from another network entity (e.g., the OAM) or from the UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A common framework in the air interface is designed for collecting data from the UE for the purpose of network-side AI/ML data collection and for network observability, where the data is terminated in the xNB.

Proposal 2	Consider data assumptions in Table 1 for study of data transfer and data collection in 6G.

Proposal 3	Support reporting of logged data collected for network-side purposes, and study request/response mechanism for the data collection report.

Proposal 4	Study data collection in all RRC states and data collection during mobility in 6G.

Proposal 5	Study how to enable partial or no visibility at the RAN of the data collected by the UE for the purpose of UE-side data collection.

Proposal 6	Study ways to enable a controllable model transfer/delivery procedure. It should be possible for the network to control the model transfer/delivery from the UE-side training server to the UE.

Proposal 7	RAN2 to identify which data collection and model transfer aspects pertain to the scope of other working groups and coordinate with the relevant groups at an early stage of the 6G study.

# 4	References

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, June 9-13, 2025.

R2-2311720, Reply LS on Data Collection Requirements and Assumptions, RAN2#124, Nov 13-14, 2023.

R2-2506761 LS on Guidance on 6G data related work tasks (source: SA).