3GPP TSG-RAN2#132	Tdoc R2-2508579

Dallas, USA, 2025-11-17 - 2025-11-21

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	Procedures for 6G Access and Spectrum Aggregation

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on the procedures for accessing the 6G Radio Interface, including paging, system information and random access procedures. Also aspects related to spectrum aggregation in 6GR are addressed. We analyse problems observed in previous generations and suggest high level principles to avoid those in 6GR.

# 2	Discussion

## 2.1	Common Transmission and Reception Occasions and Network Energy Saving

Improved network energy efficiency is an important ambition of the 6G design. Common signal and channel transmissions, such as SSB, Paging, System Information broadcast, and receptions such as Random Access, can have significant impact on network energy consumption. One important means to achieve this is to reduce the frequency of those transmissions in DL and receptions in UL, when necessary, e.g., to increase the SSB periodicity from 20 milliseconds in NR to 160 milliseconds in 6GR.

It is also important to coordinate those transmissions/receptions in a way that allows base station hardware to benefit from deep sleep. When their occasions are spread in time in an uncoordinated manner, the network will be unable to enter deeper sleep states, as it is frequently interrupted by wake-up events. As transition to and from deep sleep state requires some time, sufficiently long periods with no DL transmissions (or UL monitoring) at all are needed to efficiently leverage the deep sleep energy saving potential. As shown in TR 38.864 [1], notable energy savings can be achieved when the periodicity and timing of these transmissions and receptions are aligned, allowing them to occur closer in time and creating longer idle periods in between.

NR Rel-19 introduced several mechanisms to enable more condensed timelines, such as on-demand SSBs, on-demand SIB1, condensed Paging Occasions, dynamic activation of Random-Access occasions based on (expected) traffic load. However, since backwards compatibility for legacy UEs had to be considered, the benefits that a network can achieve with Rel-19 network energy saving features are limited. Examples include legacy SSB transmissions prior Rel-19 and system information scheduling, which currently prevent the network from reaching its full energy-saving potential.

To fully realize this potential in 6G, it is essential that all such transmissions and receptions be addressed in a coordinated manner, enabling energy-efficient configurations during low-load conditions while maintaining the ability to dynamically adapt to high-performance demands when needed.

Enabling clustered scheduling of common control transmissions/receptions (SSB, paging, system information, and random access) to achieve sparsity across the network can significantly improve network energy efficiency.

## 2.2	Paging

The length of DL transmission-free periods is ultimately limited by the length of the SSB period (e.g., 160 milliseconds), since SSBs constitute a DL signal that cannot be avoided. To allow as long DL transmission-free periods as possible, other tasks preventing deep sleep should to the extent possible be gathered close to the SSB transmissions. This applies to paging among other network procedures. To this end, network energy saving can be achieved by allowing the network to configure the paging occasions after synchronization signal broadcasts (SSBs) in clusters instead of distributing them uniformly over time as defined in legacy, provided that the number of POs remains unchanged. This approach enhances energy efficiency without negatively affecting paging latency or capacity.

This possibility of configuring POs in clusters was introduced in release 19 Network Energy Saving (NES) WI. This was achieved by tweaking the paging occasion algorithm in NR which was designed based on the principle of uniform distribution, i.e. the value range for parameter ns max is increased from 4 to 8 and minimum N is decreased from T/16 to T/32. A solution that would allow a non-uniform clustered configuration of PFs/POs closer to SSBs to facilitate longer deep sleep periods for the network and UE is also preferred in 6G.

Study mechanisms to enable (non-)uniform PO configurations where POs can be clustered in conjunction with SSB transmissions.

## 2.3	System information

As discussed in section 2.1 above, an important aspect to achieve energy savings is to have the periodicity and timing of transmissions and receptions aligned in order to get longer idle periods in between. In NR, the broadcast of the other system information (SIB2 and above) is done in different SI messages which are transmitted in SI windows with the same size. These SI windows are consecutive and in each SI window only a single SI message (i.e. with the same content) can be transmitted.

To achieve the energy savings in 6GR it should be possible to schedule different common control transmissions/receptions, including SIB1, SI messages and Paging Occasions, within a short time period after the SSB transmissions. There is then a need to have a more flexible scheduling of the SI messages in 6GR to allow this. This would enable both network energy savings and a reduced SI acquisition time for the UE, and thereby possibly also the corresponding UE energy consumption.

The system information scheduling mechanism in NR prevents network energy savings and an efficient resource utilization for broadcast of system information.

6GR should support that all System Information messages are broadcasted together with other common control transmissions/receptions within short time-periods that are clustered with the SSB transmissions.

Different alternatives for not broadcasting SI messages or SIB1 have been introduced in NR, in Rel-15 and Rel-19 respectively. These are referred to as on-demand System Information (for SI messages) and on-demand SIB1, and are considered as possible ways for network energy savings also in 6GR.

The case where the SI messages are broadcasted should be considered the baseline, even if on-demand SI is supported. When evaluating on-demand system information, the possible benefits should therefore be weighed against the complexity. It should also be considered that if on-demand system information (for other SI or for SIB1) is to be supported, it is only useful if it is implemented by UEs already from the first release.

If on-demand system information (other SI or SIB1) is to be supported, it is only useful if implemented by UEs already from the first release.

System information in NR is divided into two parts, minimum system information and other system information. The former consists of the MIB and SIB1 and the latter consists of the other SIBs in NR. The minimum system information, mainly SIB1, includes information for the UE to camp in a cell, to perform access to the cell (including Unified Access Control information) and information for the UE to acquire the other system information in the cell, i.e. the SI scheduling information.

With all this information in SIB1 the message can however become very large. This is not just an issue related to that there is a risk that the maximum size of the SIB1 message could be reached in some cases, but also that it leads to issues with detection performance and resource utilization. The resource utilization is especially a problem when there is a need for beam sweeping in many directions and with multiple repetitions.

Large SIB1 messages need to be transmitted over multiple symbols (e.g. 12) in NR. This is a problem especially when there is a need for beam-sweeping in many directions and multiple repetitions.

A question is then whether all the contents that is includede in SIB1 in NR is needed equally fast by the UE, or if some parts can be transmitted separately. As an example, the access related information, including Unified Access Control (UAC), may not need to be transmitted together with the information for camping in the cell. This is how the information was divided between SIB1 and SIB2 in LTE. We therefore think that it should be studied if some of the information that is in SIB1 in NR could be transmitted separately in 6GR, which would allow the network to use a longer periodicity for that part.

Study whether some of the information in NR SIB1 can be transmitted separately in 6GR, to allow the network to use a longer periodicity for that part.

## 2.4	Random access

The core principle of the random-access procedure in 6GR is that it should support all 6G UEs from the very beginning. In NR, issues emerged when lower-end devices (e.g., RedCap) introduced after the baseline UE in the first release (Rel-15) required additional mechanisms, such as MSG1 indication. This revealed the need for improvements to better accommodate diverse UEs. In 6GR, the aim is to ensure that the random-access procedure works seamlessly for all UEs and features from day one.

It is beneficial if 6GR supports a common random-access procedure for all UEs (including lowest-complexity UEs).

2-step RA has been introduced in NR, and it can reduce 1 RTT compared to 4-step RA. It has been argued that 2-step RA is useful to reduce delay.

- For large cells, there is a need to configure guard period/band, e.g., MSGA-PUSCH must have gaps to cater to unknown TA. In addition, there is a configurable offset between preamble and MSGA-PUSCH, and it takes value of 1 slot to 32 slots; there must be resources reserved for MSGA-PUSCH. These lead to capacity loss and gNB receiver complexity, and 2-step RA has therefore not been implemented nor deployed to date.

- Some argue 2-step RA may be useful in small cells which do not need TA adjustment and so there is no capacity loss due to the need to configure guard period, but then it is not clear why PRACH, which was designed to derive timing advance, is needed in MSGA.

It was argued that 2-step RA is useful for operation in the unlicensed spectrum, in which case when a UE tries to access the spectrum, there is a chance of collision and thus delay. However, it is not clear yet whether unlicensed spectrum operation (which has not seen any business success) would be supported from day 1. The other use case could be NTN, in which the propagation delay of one Round Trip is large. For NTN, the UE would however perform TA pre-compensation whereby there is no need of MSGA-PRACH to derive the timing.

To summarize, 2-step RA in NR suffers from spectrum inefficiency for large cells with a need of TA adjustment. For small cells without TA adjustment, it is unclear if MSGA-PRACH is needed. On the other hand, it is clear 4-step RA should be the baseline random access type in 6GR.

4-step RA can be the baseline for 6GR study, while further discussion on the use case of 2-step RA is needed.

In NR, both CBRA and CFRA are supported. CBRA is used, among other things, for initial access when the UE is first powered up. CFRA is used in cases where the network knows that the UE would perform random access right after or within a short time after receiving the configuration, e.g., PDCCH order or handover command. Since a reserved preamble is used for the UE, CFRA would not result in preamble collision, and thus give a short random-access delay. In addition, the preamble will not be locked/reserved for a too long time and leading to resource waste. Thus, we believe both types should be considered in the study.

Both CBRA and CFRA should be considered for 6GR study.

The need to have a larger MSG3 size in the cell edge to support a long I-RNTI is discussed in [3]. With all these in mind, we propose that:

Study a common random-access procedure for all UEs (including lowest-complexity UEs), with a baseline assumption of 4-step RA, CBRA/CFRA and a larger MSG3 size in the cell edge.

## 2.5 	Spectrum aggregation

In RAN2#131bis the following was agreed:

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

In this section we discuss further the areas of improvement for spectrum aggregation. Among others, we see the following areas of improvement for carrier aggregation in 6GR:

- Avoid scheduling dependencies between carriers and strict DL/UL scheduling timelines. The strict time lines in NR between PDSCH and PUCCH and the Downlink Assignment Index (DAI) linking the UL and DL scheduling across carriers became very complex with the introduction of carrier aggregation with multiple numerologies, and combined with advanced MIMO schemes and different UEs supporting different ACK/NACK timing it resulted in complex restrictions as to when HARQ feedback is scheduled in UL, leading to reduced scheduling flexibility and reduced performance. In 6GR, solutions for asynchronous feedback should be studied to overcome this. In [2], we have studied the effects of carrying UCI and HARQ feedback on PUSCH by simulation and the results show improved end to end performance when considering the protocol effects.

Carrying UCI and HARQ feedback on PUSCH can remove scheduling dependencies between carriers and strict DL/UL scheduling timelines and improve end to end performance.

- Reduced setup latency for adding additional carriers. Traffic is often bursty and a challenge in 5G has been to setup and activate additional SCells in time before the buffer is emptied to ensure good user performance and effective load balancing between deployed carriers. This would require the network to identify the relevant carrier to transmit the data on and activate those carriers quickly after the UE’s state transition to RRC Connected, see Figure 1.

Figure 1: Illustration of carrier identification

For identifying the relevant candidate carriers to be used, features like Early measurement reporting were standardized in NR. However, a UE’s dwelling time in energy saving state (RRC Idle mode or RRC Inactive state) is much shorter (under 25 seconds on an average) than the time the UE can take to perform inter-frequency measurements (up to a minute per frequency) as per RAN4 requirements while being in energy saving state. Thus, RAN2 needs to study together with RAN4 in identifying the candidate carriers to be used for the UE upon state transition. A topic that can be further looked into in this regard is the reusage of the measurements across RRC states so that the cell detection and measurement requirements could be minimized. Also, providing cell reselection related measurements like intra-frequency measurements on the carrier where the UE performs state transition could be studied to aid the network sided AI models to identify the relevant carrier candidates.

Figure 2: Simulation results showing the benefits of fast spectrum aggregation activation for a cell edge UE. The plots with CA activated after 5ms of RA procedure show significant improvement in object bitrate compared to other CA activation delays. This motivates for faster CA activation. The results also show the impact of poor UL when the object size is larger. When object size is 10 MB, the performance of PCell @ 800 MHz and PCell @ 7 GHz without CA are quite similar due to poor UL on 7 GHz (plots are generated for UEs on 7GHz UL coverage border with RSRP=-110dBm). This motivates for the decoupling of UL carrier and DL carrier.

Once a candidate carrier has been identified, there is a need to activate those carriers early enough to be able to use them before the primary carrier becomes a bottleneck. As can be seen from the plots shown above, activating SCells quickly gives gains in bitrates for all object sizes. These results show the object bit rate of subsequent downloads, meaning that the TCP connection is kept open between subsequent downloads so that there is no slow start. Features like on demand-synchronization signal based SCell activation have been looked into in Rel-19 and such features show potential to enable faster addition carrier activation opportunities. RAN2 should study how on demand synchronization signals can be used in the 6G context to speed up the carrier activation and/or achieve faster UL and DL decoupling. Further, for certain deployments, even when the UE is configured with multiple carriers to transmit/receive data, the UE could use the current serving carrier’s synchronization signal as the QCL source. This would also enable faster carrier activation and RAN2 should also study such features.

- Flexible combining of UL/DL carriers to support increased coverage of high band carriers, which are often limited by the uplink. The best carrier for the downlink may not be the best carrier for the uplink. Being able to flexibly combine a high carrier downlink with a low carrier uplink would allow increasing the coverage of high band carriers.

Figure 3: Illustration of UL/DL decoupling.

Referring to Figure 3 above, UE#2 in the High Band (HB) range extension area is expected to camp on the Low Band carrier while in idle/inactive state, where it is in good UL and DL coverage. Camping on a single carrier maximizes the UE battery life. Upon state transition to connected state, the same features discussed in the paragraph above for fast carrier activation can also be used here for quick identification and activation of additional HB carrier for DL. The main benefits of UL/DL decoupling are experienced in connected state for data transmission after DRB activation. As shown in the simulation results in the paragraph above on fast carrier activation, the DL performance on the HB carrier is limited by the poor UL on the cell border (7GHz@RSRP=-110 dBm). This is due to that TCP acknowledgements are not delivered fast enough on the UL to trigger transmission of new TCP segments on the DL.

For a UE in connected state configured with UL/DL decoupling, timing and pathloss reference (e.g. SSB) is beneficial on the LB carrier to support UL transmissions. In addition, PDCCH can be configured on the LB carrier for scheduling the LB PUSCH, thus avoiding the need for cross-carrier scheduling, which is beneficial especially when LB and HB are using different numerologies.

Figure 4: DL reference and channel allocation for UL/DL decoupling

- Improved resiliency. In 5G CA, the PCell became a single point of failure, as RLF declared on the PCell caused the UE to trigger RRC re-establishment, regardless of the status of SCells. In later NR releases several resilience features were introduced mainly on top of the DC architecture, PDCP duplication, SCG failure recovery, MCG failure recovery, resulting in several solutions for the same issue and added complexity. In 6GR, we design for good resilience from the start. One possible approach is to abandon the hierarchical signaling structure that was used in NR RRC, where information elements were grouped into different cell types, PCell and SCell, later extended to SpCell, PSCell and SCell, imposing limitations to the supported configurability depending on the cell type. In [4], we propose an alternative flat RRC structure for 6GR, in which a UE is configured with lists of serving frequencies or component carriers, physical channels, reference signals, and where each element in the lists may refer to an element in another list to describe the properties, e.g. an element in the reference signal list may point to an element in the component carrier list to indicate on which component carrier the UE can expect the reference signal. Such a signaling structure would not restrict for which component carriers RLM could be configured, and would thus enable configurations where resilience can be improved by letting the UE declare RLF only when radio link problem is observed on all configured component carriers.

Figure 5: Illustration of improved resiliency by RLM enhancement.

RAN2 to study solutions for fast CA and UL/DL decoupling setup, e.g.:
a) measurements for identifying candidate carriers for the UE upon state transition,
b) on-demand synchronization signal for fast carrier activation,
c) information that can aid the UE in fast carrier activation, e.g. QCL source information.

RAN2 to study solutions for increased CA resiliency, e.g. by enhancing RLM so that RLF is declared only when radio link problem is observed on all configured carriers.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	Enabling clustered scheduling of common control transmissions/receptions (SSB, paging, system information, and random access) to achieve sparsity across the network can significantly improve network energy efficiency.

Observation 2	The system information scheduling mechanism in NR prevents network energy savings and an efficient resource utilization for broadcast of system information.

Observation 3	If on-demand system information (other SI or SIB1) is to be supported, it is only useful if implemented by UEs already from the first release.

Observation 4	Large SIB1 messages need to be transmitted over multiple symbols (e.g. 12) in NR. This is a problem especially when there is a need for beam-sweeping in many directions and multiple repetitions.

Observation 5	It is beneficial if 6GR supports a common random-access procedure for all UEs (including lowest-complexity UEs).

Observation 6	4-step RA can be the baseline for 6GR study, while further discussion on the use case of 2-step RA is needed.

Observation 7	Both CBRA and CFRA should be considered for 6GR study.

Observation 8	Carrying UCI and HARQ feedback on PUSCH can remove scheduling dependencies between carriers and strict DL/UL scheduling timelines and improve end to end performance.

We propose

Proposal 1	Study mechanisms to enable (non-)uniform PO configurations where POs can be clustered in conjunction with SSB transmissions.

Proposal 2	6GR should support that all System Information messages are broadcasted together with other common control transmissions/receptions within short time-periods that are clustered with the SSB transmissions.

Proposal 3	Study whether some of the information in NR SIB1 can be transmitted separately in 6GR, to allow the network to use a longer periodicity for that part.

Proposal 4	Study a common random-access procedure for all UEs (including lowest-complexity UEs), with a baseline assumption of 4-step RA, CBRA/CFRA and a larger MSG3 size in the cell edge.

Proposal 5	RAN2 to study solutions for fast CA and UL/DL decoupling setup, e.g.: a) measurements for identifying candidate carriers for the UE upon state transition, b) on-demand synchronization signal for fast carrier activation, c) information that can aid the UE in fast carrier activation, e.g. QCL source information.

Proposal 6	RAN2 to study solutions for increased CA resiliency, e.g. by enhancing RLM so that RLF is declared only when radio link problem is observed on all configured carriers.

# 4	References

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR

R2-2508615, UL HARQ- and CSI feedback in 6G, Ericsson

R2-2508311, RRC Inactive state in 6G, Ericsson

R2-2508614, RRC ASN.1 structure for 6G, Ericsson