3GPP TSG-RAN WG2 Meeting #132	R2-2508581

Dallas, USA,  Nov. 17th - 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on data collection and data transfer

Agenda Item:	10.3.3.1

Document for:	Discussion and Decisions

- Introduction

During the RAN#108 meeting, the new SID on “Study on 6G Radio” was approved, and it was further revised in RAN #109 meeting[1]. The new SID describes the objective of the work on AI/ML for 6GR and RAN as below, in which the study on AI/ML framework needs to be led by RAN2.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Besides, the SID also put the data transfer as one of the objectives in the 6GR interface protocol architecture and procedure design as below,

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In parallel, the RAN level study on the requirements of new and existing services was conducted in RAN #109 meeting, and following services are approved to be captured in TR38.914 section 5.4[2].

- Mobile broadband

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory services

This paper discusses the AI/ML framework for 6GR and the data transfer for existing and new services.

- Data collection and data transfer

  - Data collection framework

At the RAN2 #131bis meeting, following agreements was made on the standardized data collection framework[3].

Agreements

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

For sensing-related use cases, the radio resource used for sensing signal transmission as well as the data collection overhead may coexist with ongoing communication services. Unlike AI/ML data collection, sensing requires dedicated physical resources for signal generation, reception, and reporting. Therefore, it is necessary to explicitly confirm that the resource consumption for sensing conduction and sensing data collection shall be minimized so that it does not degrade the communication KPIs (e.g., user-plane throughput, latency, power saving performance). This ensures that the sensing function can be introduced without compromising the primary role of 6G networks as communication systems.

Proposal 1: RAN2 to confirm the requirement of sensing use cases/services that the resource allocated for sensing procedure and data collection should have minimum impact on the communication services.

The termination point group of data collection includes the entity produces the data, the entity consumes it, and where the standardized data collection is anchored. Since different use cases such as sensing, AI/ML, SON, MDT, and QoE may involve different producer–consumer chains, the selection of termination points directly impacts protocol design, security scope, data visibility, and management ownership. Studying the set of candidate termination-point grouping (e.g., UE-RAN-RAN, UE-RAN-CN, UE-RAN-OAM, etc.) allows RAN2 to identify which combinations need standard support, which can reuse existing procedures, and which may require new signalling. This prevents fragmented solutions across use cases and enables a unified framework for standardized data collection. Therefore, we can consider following candidate termination point combinations (i.e. data producer-data consumer-data collection point) for standardized data collection:

- UE-RAN-RAN (e.g., gNB-centric NW-side data collection in 5GA AIML)

- UE-RAN-CN

- UE-RAN-OAM (e.g., OAM-centric NW-side data collection in 5GA AIML)

- UE-CN-CN(e.g., UE-side data collection for case 1a/1b in 5GA AIML)

- UE-CN-OAM

- RAN-CN-CN

- RAN-CN-OAM

- RAN-OAM-OAM

For the 6th – 8th bullet, these are not RAN2 scope, so we propose following.

Proposal 2: RAN2 to support the study of following candidate termination point combinations (i.e. data producer-data consumer-data collection point) for standardized data collection:

- UE-RAN-RAN

- UE-RAN-CN

- UE-RAN-OAM

- UE-CN-CN

- UE-CN-OAM

  - Data transfer

Clarification on the service data and non-service data

At the RAN2 #131bis meeting, following conclusion was made on the data type analysis [3].

For next meeting companies can consider at least the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

  - For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting and total data size

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

In the current discussion on standardized data transfer, different types of data are being considered under the same framework, although their characteristics and transfer requirements are fundamentally different. Therefore, it is necessary for RAN2 to first clarify whether the ongoing discussion targets service data, non-service data, or both, so that the subsequent requirement analysis and protocol impact can be bounded accordingly. Service data refers to the original or transformed content that is directly consumed by the end user or application (e.g., video, image, text), whereas non-service data refers to information generated inside the network to support or optimize the service (e.g., measurement results, sensing outputs, AI/ML models or datasets). Mixing these two categories without clear distinction may lead to ambiguity in aspects such as ownership, protection scope, QoS treatment, transfer frequency, and data volume expectation.

Proposal 3: RAN2 to clarify the discussion about the data transfer for service data and non-service data:

- Service data: The source data or the transformed source data of the service itself, such as video, image and text.

- Non-service data: The data generated in the NW to facilitate the specific service, such as measurement result, sensing result, AI/ML related model or dataset.

Future 6G services such as AI/ML- and sensing-based applications will impose diverse and stringent data transmission demands beyond the legacy solutions. These services require flexible QoS, ultra-high throughput, and ultra-low latency for both user/data and control/signaling exchanges among the functions. 6G may also need to support data transfer across multiple endpoints, e.g. UE–RAN, UE–CN, UE-UE, and RAN–RAN, to enable diverse use cases like AI/ML model updates or real-time sensing data fusion. Meeting these needs calls for a rethinking of 5G(A) protocol and procedure design, ensuring dynamic adaptability to different AI/ML and sensing requirements while maintaining reliability and spectrum efficiency.

Observation 1: Flexible QoS, high throughput, low latency and transmission between a broader set of endpoints such as UE to RAN, UE to CN, UE to UE, and RAN to RAN are proposed for the new services data.

The control and user plane mechanisms in 5G(A), designed for legacy services, may not meet the massive data and model exchange demands of 6G new services. Current SRBs restrict RRC message size, while DRBs remain bound to UPF-based PDU sessions, restricting direct UE–gNB data transfer.

Observation 2: Existing control plane and user plane solutions in 5G(A) may be inadequate for transferring the large volumes of both service and non-service data that will be required in 6G.

The traffic pattern and characteristics of new 6G services should be carefully studied before defining interface protocols and procedures. The study may start from XR (immersive communication) as the baseline, given its strong linkage to AI/ML and sensing and the availability of prior analysis on traffic behavior. Based on XR findings, it can be extended to AI/ML, sensing, and other new services following RAN plenary decisions and/or by coordinating with SA1 to better understand the data transfer requirements, ensuring protocol and procedure design reflects real traffic features.

Proposal 4: The traffic pattern and/or characteristics of new services data should be studied thoroughly before defining the necessary interface protocol and procedure

- The new services may start from XR as the baseline/starting point, and include AI/ML, Sensing, and other new services according to RANP finding.

- RAN2 should initiate a study and/or coordinate with SA1 to better understand the data transfer requirements for/of emerging use cases/services.

A unified data transfer framework for difference services offers clear benefits in simplifying architecture and improving efficiency across diverse 6G services. However, the study should also take into account the existing legacy services, which already rely on MDT- and positioning-based mechanisms for data reporting and management. These legacy solutions are well established and widely deployed, providing a proven foundation in terms of triggering logic, measurement control, and data handling. Therefore, MDT and positioning can serve as the practical baseline for new services like AI/ML and Sensing, while the evolution toward a unified framework could be further investigated by considering the traffic pattern and/or characteristics of new identified services, such as a new data plane which enables new service data to be transmitted through dedicated bearers, minimizing interference with legacy traffic and improving efficiency.

Proposal 5: RAN2 should study the unified data transfer framework for both the legacy service data and new identified service data.

- The MDT-based and/or positioning-based solution should be assumed as the baseline/starting point.

- If data transfer requirements allow, to support a new data-plane for data transfer for new services (e.g. AI/ML, sensing, etc) to make data transfer more efficiently.

As data transfer becomes central to AI/ML and sensing services, 6G should ensure operator-controlled data management with user consent and visibility. Operators must be able to initiate, monitor, and terminate data transfers while guaranteeing user awareness and control over how data is collected and used. To achieve this, mechanisms such as mutual authentication, data encryption, and policy-based access control should be studied to protect data integrity and confidentiality. This approach will enable secure and transparent AI/ML operations, ensure regulatory compliance, and strengthen user trust in AI/ML- and sensing-based 6G services.

Proposal 6: RAN2 should study the operator-controlled service data management (i.e. initiate, monitor, and termination) with UE consent and visibility.

- Inter-WG issues

In 5G(A), RAN3 and SA working groups were involved in discussions on data visibility, controllability, and security. However, the resulting solutions were compromised and fragmented, such as the MAC CE-based NCC delivery for inter-CU LTM and UE data collection for AI/ML.

In 6G, these challenges will persist and become even more critical. The volume, diversity, and sensitivity of exchanged data will continue to grow, not only for new services but also for broader network optimization, automation, such as further mobility enhancement. This evolution requires standardized, end-to-end (E2E) mechanisms to guarantee visibility, controllability, security, and efficiency of data transmission. Achieving this will demand tight coordination among RAN and SA working groups to ensure architectural consistency and security assurance.

Proposal 7: RAN2 shall coordinate with RAN3 and SA WGs on following issues:

- SA1: The data transfer requirements of emerging use cases like sensing.

- RAN3, SA2, SA5: The QoS requirements for new services and E2E solution for data transfer and/or data collection, new plane and/or RAN-CN interface.

- RAN3, SA3, SA5: The security and UE consent of model and data transfer and/or data collection solutions, e.g. MAC CE encryption and integrity protection.

- Conclusion

In this contribution, we have the following observations and proposals,

Data collection

Proposal 1: RAN2 to confirm the requirement of sensing use cases/services that the resource allocated for sensing procedure and data collection should have minimum impact on the communication services.

Proposal 2: RAN2 to support the study of following candidate termination point combinations (i.e. data producer-data consumer-data collection point) for standardized data collection:

- UE-RAN-RAN

- UE-RAN-CN

- UE-RAN-OAM

- UE-CN-CN

- UE-CN-OAM

Data transfer

Proposal 3: RAN2 to clarify the discussion about the data transfer for service data and non-service data:

- Service data: The source data or the transformed source data of the service itself, such as video, image and text.

- Non-service data: The data generated in the NW to facilitate the specific service, such as measurement result, sensing result, AI/ML related model or dataset.

Observation 1: Flexible QoS, high throughput, low latency and transmission between a broader set of endpoints such as UE to RAN, UE to CN, UE to UE, and RAN to RAN are proposed for the new services data.

Observation 2: Existing control plane and user plane solutions in 5G(A) may be inadequate for transferring the large volumes of both service and non-service data that will be required in 6G.

Proposal 4: The traffic pattern and/or characteristics of new services data should be studied thoroughly before defining the necessary interface protocol and procedure

- The new services may start from XR as the baseline/starting point, and include AI/ML, Sensing, and other new services according to RANP finding.

- RAN2 should initiate a study and/or coordinate with SA1 to better understand the data transfer requirements for/of emerging use cases/services.

Proposal 5: RAN2 should study the unified data transfer framework for both the legacy service data and new identified service data.

- The MDT-based and/or positioning-based solution should be assumed as the baseline/starting point.

- If data transfer requirements allow, to support a new data-plane for data transfer for new services (e.g. AI/ML, sensing, etc) to make data transfer more efficiently.

Proposal 6: RAN2 should study the operator-controlled service data management (i.e. initiate, monitor, and termination) with UE consent and visibility.

Inter-WG issues

Proposal 7: RAN2 shall coordinate with RAN3 and SA WGs on following issues:

- SA1: The data transfer requirements of emerging use cases like sensing.

- RAN3, SA2, SA5: The QoS requirements for new services and E2E solution for data transfer and/or data collection, new plane and/or RAN-CN interface.

- RAN3, SA3, SA5: The security and UE consent of model and data transfer and/or data collection solutions, e.g. MAC CE encryption and integrity protection.

References

- NTT DOCOMO, CMCC, AT&T, Vodafone, RP-252912, “Revised SID: Study on 6G Radio,” Sep. 2025.

- CMCC (moderator), RP-252960, “pCR for 38.914 on Requirements of new and existing services for 6G” Sep.2025.

- RAN2 Chair (InterDigital), “RAN2 #131bis Chair Notes”, Oct., 2025.