3GPP TSG-RAN WG2 Meeting #132	R2-2508585

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.7.2

Source:	ZTE Corporation, Sanechips

Title:	XR Remaining Issues

Document for:	Discussion, Agreement

# Introduction

In this contribution, we will discuss some remaining issues on XR and present our views.

# Discussion

After RAN2#131 meeting, the following open issue is proposed[1]:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N001] RX_Next in clause 7.1 The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires. [Rapp]: It is true, and we need to update it. Your poropsal is one way, which may miss some information for legacy case. Another way is to capture like: if t-RxDiscard is configured, xxx. Otherwise, xxxx. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-N01 |

In RAN2#131bis, the following agreement has been made:

- (RLC-N01) We will clarify RX_Next state variable update when t-RxDiscard expires. FFS an exact wording and where to do the change (e.g. procedure or variable definition) (offline)

We understand this issue is important for RX window advance(e.g. to avoid the RX window stalling), and should be captured accurately. But for the case that t-RxDiscard is not configured or does not expire, the legacy procedure is accurate and should be kept.

When t-RxDiscard is configured or expires, the receiving RLC entity will not wait for the obsolete RLC SDUs, it has already captured by “the first RLC SDU not delivered to the upper layers, but still waited for”. The only thing need to do is to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

Proposal 1(RLC-N01): keep the description for RX_Next – Receive state variable, except to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

After RAN2#131 meeting, the following option issue is proposed[1]:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. [Rapp]: This issue was discussed in RAN2#130 based on Apple’s proposal. The agreement includes the ACK/NACK/Discard cases. I understand the motivation in Apple’s contribution is that in the case where a NACK has just been received, there is no need for polling since the status is already known, and the retransmission can be performed directly. While the current Note in the specification also states “may stop and reset,” so I think this should not be a major issue. But considering it has not been extensively discussed during RAN2#130, let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |

In RAN2#131bis, the following agreement have been made:

- (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence.

- (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 (offline)

For this issue, we have the same feeling as rapporteur that t-PollRetransmit should be stopped when all RLC SDUs are already acknowledged, because t-PollRetransmit is used for triggering POLL and RLC STATUS report. When the transmitting RLC entity knows the status of all RLC SDUs,  POLL and RLC STATUS report need not be triggered.

But for “indicated as discarded from upper layer” case, there are the following sub-cases:

- sub-case 1: when stopReTxDiscardedSDU is configured and indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU and if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers, the RLC STATUS report is still necessary for TX window advance(e.g. to avoid TX window stalling)

- sub-case 2: when stopReTxDiscardedSDU is not configured and indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU and if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers, the RLC STATUS report is still necessary to trigger the RLC SDU or RLC SDU segment re-transmission or TX window advance.

Observation 1: Even if indicated from upper layer (e.g. PDCP) to discard a RLC SDU, POLL and RLC STATUS report is still necessary for TX window advance.

Proposal 2(RLC-N02): Update the NOTE in clause 5.3.3.3 as follows:

NOTE:	If stopReTxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged, the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

After RAN2#131 meeting, the following option issue is proposed[1]:

| Huawei, HiSilicon | In section 5.3.4, the content when STATUS PDU is triggered by other triggering conditions are reused. However, when t-RxDiscard expires, RX_Next is updated to RX_Next_Discard_Trigger and after the update, RX_Next might be larger than RX_Highest_Status. Hence, the following condition does not hold anymore. - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: Hence, when the status report is triggered by t-RxDiscard, a straightforward solution would be just to set the ACK_SN to the updated RX_Next. Then, the SDUs before RX_Next are indicated as ACK The TP can be follows: When constructing a STATUS PDU not triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== When constructing a STATUS PDU triggered by the expity of t-RxDiscard, the AM RLC entity shall: - set the ACK_SN as the updated RX_Next | RLC-H02 |
| --- | --- | --- |

In RAN2#131bis, the following agreement have been made:

- (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

For this issue, we have the same understanding with Huawei, we can keep the existing UE behavior unchanged and add the case for expiry of t-RxDiscard.

Proposal 3: For constructing a RLC STATUS PDU, keep the existing UE behavior unchanged and add the case for expiry of t-RxDiscard. E.g. use the TP provided by Huawei.

# Conclusion

Based on the discussion we propose the following:

Proposal 1(RLC-N01): keep the description for RX_Next – Receive state variable, except to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

Observation 1: Even if indicated from upper layer (e.g. PDCP) to discard a RLC SDU, POLL and RLC STATUS report is still necessary for TX window advance.

Proposal 2(RLC-N02): Update the NOTE in clause 5.3.3.3 as follows:

NOTE:	If stopReTxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged, the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Proposal 3: For constructing a RLC STATUS PDU, keep the existing UE behavior unchanged and add the case for expiry of t-RxDiscard. E.g. use the TP provided by Huawei.

# Reference

- R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp

- R2_131bis XR and LTE Broadcast notes (Dawid)_EOM