3GPP TSG-RAN WG2 Meeting #132                                                                                R2-2508591

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.3.2.3

Source:	HONOR

Title:	Discussion on Random Access for 6GR

Document for:	Discussion and Decision

- Introduction

6GR study item has been approved in the RAN plenary #108 meeting and revised in RAN plenary #109 meeting [1], from which, the objective of initial access for 6GR is,

(2) Physical Layer structure for 6GR, …… h) Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

In this contribution, we will discuss the RA procedure with the consideration of 6G scenarios, UE types/features and energy saving.

- Discussion

The NR system supports 4-step RA procedure and 2- step RA procedure. 2-step RA procedure is introduced in Rel-16, which is used to reduce the delay for RA when the UE is under good coverage. The MsgA contains preamble and PUSCH payload, it requires higher link quality and has restriction for cell edge users to use. Meanwhile, 2-step RA has a higher probability of collision and may fall back to 4-step RA due to PUSCH payload transmission failure. Therefore, 4-step RA cannot completely be replaced by 2-step RA, which should be a supplementary technology. Compared with 5G systems, 6G systems may provide richer experiences, such as extreme low-latency requirements and massive communications. Therefore, the access delay and small signaling overhead of 2-step RA may be more suitable for 6G systems. Considering some defects of 2-step RA, the 2-step RA of 6G system may not be a simple 2-step RA translation of 5G, but an enhancement and evolution to provide fast access. 4-step RA procedure can support a larger coverage area, has a relatively mature conflict handling mechanism, a relatively simple process, and high reliability. But it may cause large signalling overhead and delay in the dense link scenarios.

Therefore, both the 4-step RA procedure and 2-step RA procedure can be as a baseline in the 6G Day 1 and can be studied for 6G scenarios.

Furthermore, the NR system supports both Contention-Based Random Access (CBRA) and Contention-Free Random Access (CFRA). CBRA allows multiple UEs to attempt network access simultaneously using the same preamble, which may result in collisions. In contrast, CFRA assigns dedicated preambles to UEs by the network, effectively avoiding collisions and improving access reliability. However, reserving dedicated resources leads to low resource utilization efficiency. CBRA is suitable for massive access scenarios, while CFRA can be used for reducing interruption latency for HO and BFR. Considering the respective advantages and limitations of CBRA and CFRA, both mechanisms can serve as baseline for 6GR.

Proposal 1: The 4-step/2-step CFRA/CBRA procedure of 5G NR can be as baseline for 6GR.

In 5G NR, the 4-step RA procedure was introduced in Release 15, followed by the 2-step RA in Release 16. From Release 17 onward, various enhancements to the RA procedure have been introduced, including support for RedCap, network slicing, MSG3 repetition, and MSG1 repetition.

The 6G systems will support an extensive range of device types, from low-power (e.g., LPWA and IoT) devices to high-performance terminals (e.g., CPE and premium smartphones). These devices exhibit significant disparities in capabilities, power constraints, and QoS requirements, which motivates the classification of devices into distinct types. This direction has been explicitly recognized in 6G Study Items.

In this context, it is essential that specific device capabilities and features can be efficiently identified and differentiated during the RA procedure. This will enable the network to dynamically assign appropriate radio resources and access parameters, thereby improving resource utilization, increasing access success rate, and enhancing the overall service experience. Therefore, the design of the RA procedure should incorporate flexible support for diverse device types and features to meet the complex and varied access requirements of 6G networks. However, the RACH partition in 5G is complexity. 6G can study a simpler solution to support diverse UE types and features.

Proposal 2: Study how to support the RA procedure for diverse UE types and features.

In the RAN1#122 meeting, the following agreement was raised：

Agreement • Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s).

It is important to consider energy saving in 6G Day 1. The network monitors PRACH at each random access occasion, which causes a lot of energy consumption for the network. In the 5G NR, RACH adaptation was introduced to save network energy. For the RACH adaptation, the network can configure legacy RACH configuration with a long period and activate additional RACH resource when RA conflict occurs. Due to the co-existence of legacy UEs without RACH adaptation capability, network needs to configure two sets of RACH resources for legacy UE and NES UE and notifies the NES UEs when additional RACH resources is activated, which brings additional signaling overhead to the network. Thus, 5G's network energy saving RACH adaptation technology still has significant room for improvement.

Though long-period RACH resources decrease NW energy, the energy consumption of UE will be increased especially when lots of UEs attempt RA. For example, UE attempts RA more frequently and access latency may be increased due to conflict. Therefore, it is beneficial to study RACH adaptation to support energy saving both for network and UE in Day 1.

Proposal 3: Study the RACH adaptation to support joint energy saving for UE and network.

The 5G RA procedure can be triggered by several events, which can be summarized as follows:

- Connection Management: Whether it is initial access from RRC_IDLE, RRC Connection Re-establishment, or RRC Connection Resume, the RA procedure provides fundamental support. These events ensure that the UE can efficiently connect to the network in various scenarios.

- UL Synchronization: When data arrives, positioning is required, or TAG synchronization is needed, the RA procedure ensures time alignment between the UE and the network in these scenarios.

- Resource Request: The UE can trigger the RA procedure to request uplink resources from the network when there are no available PUCCH resources for SR, or SR failure.

- Cell Switch: When the UE needs to switch between different cells, the RA procedure can achieve UL synchronization, resource allocation, and connection recovery for the target cell, thereby enhancing the user experience in mobility scenarios.

- System Information Request: The RA procedure can be initiated to request other system information or on-demand SIB1 from the network, enabling the UE to obtain necessary configuration.

- Others: Additional scenarios include beam failure recovery, consistent UL LBT failure, and SDT in RRC_INACTIVE, all of which depend on the RA procedure.

These triggering events cover core scenarios including connection establishment, synchronization, resource allocation, handover, and recovery, thereby ensuring efficient and reliable interaction between the UE and the network. Similar to 5G, even more features (e.g., SDT) may be introduced in the later phase,  the RA design should support the basic functions in Day 1 and accommodate the additional features. Thus, RAN2 recommends supporting basic NR RA-triggering events in 6G Day 1, e.g., connection management, UL synchronization, resource request, cell switch, system information request, BFR, and LBT.

Proposal 4: The RA design needs to consider basic NR RA-triggering events (e.g., connection management, UL synchronization, resource request, cell switch, system information request, BFR, and LBT).

While the 5G NR On-demand SSB (OD-SSB) mechanism offers network energy saving, there are still the following limitations. Firstly, OD-SSB triggering is network-controlled via MAC-CE, and UEs are limited to passive monitoring based on pre-configured parameters, without the ability to actively request OD-SSB transmission. Secondly, the mechanism is only defined for UEs in RRC_CONNECTED state and relies on Primary Cell (PCell) transmission control signaling.

RAN1 has raised the scenario that always-on SSB with large periodicity and on-demand SSB with short periodicity [2] and many companies supported the UE to request on-demand SSB in RRC_IDLE state and RRC_INACTIVE state. Therefore, it is needed to introduce UL-WUS to request the OD-SSB and find out what could be used as the UL-WUS. RA as a basic uplink request mechanism could be used to carry the OD-SSB request, so it is recommended to study RA to support the OD-SSB.

Proposal 5: RAN2 to study whether and how to use the RA procedure for UE-triggered OD-SSB request.

In the RAN2#131bis meeting, the following agreement was raised：

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

6G may introduce new scenarios/deployment. For example, single cell with multi-carriers scenarios (SCMC) as a unified multi-carriers design technology, can utilize fragmented spectrum to improve cell capacity and increase data transfer rate to meet the coverage and data transmission requirements of 6G network. In this case, always-on signal and SIB1 may only be sent on the anchor carrier (carrier1), as Figure 1 shows.

Figure 1: SCMC scenario

If introducing SCMC as described above in 6G, RA procedure will be supported at least in anchor-carrier for initial access in SCMC. For SCMC, when lots of UEs attempt to access in a carrier of SCMC, the network would like to offload a part of UEs to other carrier (non-anchor carrier) to improve the UE access success rate, that is “RA offloading”, as Figure 2 shows. Therefore, RAN2 needs to consider the impacts resulted from the new scenarios/deployment like SCMC.

Figure 2: RA offloading in SCMC

Proposal 6: RAN2 to discuss the impacts on RA design for the new scenarios/deployment (e.g., SCMC).

- Conclusions

In this contribution, we discussed RA procedure and made the following proposals:

Proposal 1: The 4-step/2-step CFRA/CBRA procedure of 5G NR can be as baseline for 6GR.

Proposal 2: Study how to support the RA procedure for diverse UE types and features.

Proposal 3: Study the RACH adaptation to support joint energy saving for UE and network.

Proposal 4: The RA design needs to consider basic NR RA-triggering events (e.g., connection management, UL synchronization, resource request, cell switch, system information request, BFR, and LBT).

Proposal 5: RAN2 to study whether and how to use the RA procedure for UE-triggered OD-SSB request.

Proposal 6: RAN2 to discuss the impacts on RA design for the new scenarios/deployment (e.g., SCMC).

- References

- RP-252912

- R1-2508187