3GPP RAN WG2 Meeting #132    	                                                                                R2-2508593

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.2.2

Source:	HONOR

Title:	Discussion on NTN for 6GR

Document for:	Discussion and Decision

- Introduction

For the study of 6GR, a harmonized 6G Radio design for TN and NTN is one of the objectives in the SID and involves work from RAN2.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

During RAN2#131-bis meeting, RAN2 discuss the initial thinking of TN/NTN integration and come to the agreements as following.

- Agreement: For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we analyse the features introduced in 5G NTN case by case and divide the NTN specific features into two categories as studied from Day1 and considered in the later phase.

- Discussion

2.1 Deployment Scenarios

Overview

In the initial phase of 5G NTN study, RAN studied deployment scenarios for the non-terrestrial networks (NTN) and identify the specific scenario and deployment for study. And the detailed study was performed in RAN groups. For example, RAN2 discuss different mobility solutions for earth-fixed cell and quasi-earth-moving cell. Since RAN plenary is still working on the deployment scenarios and requirements for 6G including the aspects for NTN. Thus, we can take the 5G as the baseline and further discuss the impacts based on the identified NTN deployment scenarios from RAN plenary.

Proposal 1: RAN2 to take 5G assumption (e.g., multi-orbit, different types of feeder link, etc.) as the baseline and discuss the difference between 5G and 6G NTN deployment scenarios until the guidance from RAN plenary.

Beam Hopping

NTN deployment constraints such as payload power limitation, large satellite foot-print and limited feeder link bandwidth are identified as the in Release 19. To enhance the system level downlink coverage to cope with the issue brought by the limited energy in NTN, the extended SSB periodicity is introduced in 5G. However, from the perspective of UE, the extension of the SSB/SIB1 default period in the later phase greatly affects the UE experience. For example, measurement on the turned-off beam is unnecessary and would result in larger UE power consumption.

Proposal 2: RAN2 to consider the beam hopping for NTN from Day1.

Support of different UE types

For NR NTN, the coverage is assumed as continuous. During the study of IoT NTN, discontinuous coverage is assumed since the sparse satellite constellation deployment specific for the NB-IoT UEs. The difference between the assumption results from the basic design for the NB IoT which is the RAT is different from the legacy UE and NB-IoT UE. To support the IoT NTN, IoT specific enhancement like S&F Satellite operation is introduced. Besides, enhancement for the HD-FDD RedCap UE is introduced in Release 19. For 6GR, even the discussion of the different UE types was already initiated in RAN1, there is still no conclusion for the UE types. Thus, it is hard to conclude whether different RAT would be deployed for different UE types as in 5G. We propose to delay the discussions related to UE types for NTN specific enhancement until we figure out the basic design related to UE type.

Proposal 3: Delay the NTN-specific discussion related to UE types (e.g., similar to enhancement introduced in 5G discontinuous coverage scenario).

GNSS resilient operation

During the former phase of 5G, we assumed the UE could obtain its location based on GNSS. In Rel-20, RAN1 started the study of the GNSS resilient NR-NTN operation to decouple the dependency of the external third-party system. For 6G, since RAN plenary is still working on the study on 6G Scenarios and Requirements, we could delay the discussion related to GNSS resilient until the guidance from RAN plenary. Thus, discussions for features related to positioning (e.g., Network verified UE location, positioning with single satellite) could be delayed and only be initiated if found necessary.

Proposal 4: Delay the NTN-specific discussion related to positioning until guidance about GNSS resilient operation from RAN plenary.

2.2 NTN specific features

Mobility enhancement

During the 5G, multiple mobility enhancements were introduced considering the characteristic of NTN for both RRC_CONNECTED UEs and RRC_INACTIVE/RRC_IDLE UEs. Since mobility is the basic function for the cellular network, we need to identify the essential function to enable the service continuity among NTN nodes and between NTN and TN.

Considering the variation of signalling strength is relatively small, location-based and time-based conditions are introduced for cell reselection and handover. For example, UE based mobility could be triggered based on the relative distance between the UE and satellite. Especially for RRC_CONNECTED UEs, location-based and time-based CHO could avoid signalling storm between NW and UEs since the UE could triggered the handover based on the pre-configurations when the event related to the location/time is met. The location-based and time-based mechanism could greatly improve the performance for the UE triggered mobility.

Observation 1: In NTN, location-based and time-based conditions is the essential function for UE triggered mobility (e.g., reselection and handover)

In 5G, multiple SS/PBCH block measurement window and timing (SMTC) could be adjustment along with the change of relative distance between UE and the satellite. For the connected UEs, NW could adjust the SMTC based on the assistance information reported by UE. While for the and RRC_INACTIVE/RRC_IDLE UEs, UE could perform the modification based on its location and the satellite related information. This is more important when the difference of UE-gNB RTT is bigger due to larger difference of orbit altitude. Even though, RAN1 is still discussing the reference signal and RAN4 is working on the general design of measurement gap, we could assume the window for measurement needs to be enhanced compared to TN.

Observation 2: In NTN, the window for measurement needs to be enhanced due to the difference of orbit altitude.

Referring to the enhancement of TN-NTN mobility in 5G, the most important enhancement is to sharing the information between TN and NTN, e.g. TN/NTN cell specific information broadcasted by SIB. UE could perform the measurement and reselection between TN and NTN based on the broadcast information. For 6G, NTN is a key enabler to extend the coverage of TN network. To facilitate better use of NTN, the unified design for TN/NTN should consider the TN-NTN mobility from day 1.

Observation 3: For 6G unified TN/NTN design, TN-NTN mobility is important to be considered in Day1.

To avoid signalling storm, RACH-less HO is introduced in 5G by avoiding the RACH procedure during the handover which is mainly applied in the limited scenario. Satellite switch with-resync is introduced to enable the UE to synchronize with the target satellite without handover only applied to quasi-Earth fixed scenario. These enhancements aim to improve the mobility performance but with more specification impacts or only applied in the limited scenario. We could consider similar enhancement in the later phase.

Observation 4: RACH-less HO and Satellite switch with-resync could be considered in the later phase.

UP enhancement

HARQ disable/enable mechanism is introduced in 5G to avoid the due to the transmission delay caused by the large UE-gNB RTT. The impact is more severe when the altitude is relatively high (e.g., GSO), while it is limited when the satellite is deployed in the LEO. In addition, it is more reasonable that the UE would perform data transmission via LEO instead of GEO. What’s more, this enhancement has fewer specification impacts and could be studied based on the fundamental framework for TN.

Observation 5: HARQ disable/enable could be considered in the later phase.

Based on the above observations, we divide the NTN specific features into two categories as essential one which should be studied from Day1 and important ones which could be considered in the later phase.

Proposal 5: RAN2 to consider the following essential features for unified TN/NTN from Day1: Time-based/location-based UE triggered mobility, measurement window adjustment, TN-NTN mobility.

Proposal 6: RAN2 could consider the following NTN specific enhancement in the later phase: HARQ disable/enable, RACH-less HO and Satellite switch with-resync.

- Conclusions

In this contribution, we discussed NTN related issues and made the following observations and proposals:

Observation 1: In NTN, location-based and time-based conditions is the essential function for UE triggered mobility (e.g., reselection and handover)

Observation 2: In NTN, the window for measurement needs to be enhanced due to the difference of orbit altitude.

Observation 3: For 6G unified TN/NTN design, TN-NTN mobility is important to be considered in Day1.

Observation 4: RACH-less HO and Satellite switch with-resync could be considered in the later phase.

Observation 5: HARQ disable/enable could be considered in the later phase.

Proposal 1: RAN2 to take 5G assumption (e.g., multi-orbit, different types of feeder link, etc.) as the baseline and discuss the difference between 5G and 6G NTN deployment scenarios until the guidance from RAN plenary.

Proposal 2: RAN2 to consider the beam hopping for NTN from Day1.

Proposal 3: Delay the NTN-specific discussion related to UE types (e.g., similar to enhancement introduced in 5G discontinuous coverage scenario).

Proposal 4: Delay the NTN-specific discussion related to positioning until guidance about GNSS resilient operation from RAN plenary.

Proposal 5: RAN2 to consider the following essential features for unified TN/NTN from Day1: Time-based/location-based UE triggered mobility, measurement window adjustment, TN-NTN mobility.

Proposal 6: RAN2 could consider the following NTN specific enhancement in the later phase: HARQ disable/enable, RACH-less HO and Satellite switch with-resync.

- References

- RP-252912 Revised SID: Study on 6G Radio