3GPP RAN WG2 Meeting #132                                                                                    R2-2508599

Dallas, US, Nov. 17th – 21st, 2025

Agenda Item:	9.2.2

Source:	HONOR

Title:	Discussion on Topology 2 for AIoT

Document for:	Discussion and Decision

- Introduction

In the RAN2#131-bis meeting [1], A-IoT Topology 2 (T2) was discussed and following agreements have been made:

Agreements

- gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

- gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation.

- The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this contribution, we provide our views on the issues related to A-IoT T2.

- Discussion

  - A-IoT Radio Resource Allocation and Validity

A-IoT Resource Allocation Modes

As outlined in RAN2#127 agreement [2], the network (e.g., gNB) is in control of resources to be used by A-IoT air interface. Based on the current specification [3],  three baseline assumptions for A-IoT resource allocation can be considered: dynamic grant (e.g., DCI via PDCCH indicating resources in every transmission time interval), semi-statical/persistent (e.g., uplink configured grant type 1/2 where RRC provides period, time offset and frequency resource configurations with or without activation signaling) and the shared resource allocation (e.g., Sidelink Mode 2 with a shared resource pool).

RAN2#127-bis [4] has agreed that only dedicated resource configuration instead of shared resource pool should be provided to the UE reader via dedicated signaling, and R20 WID [5] revealed the dedicated signaling is RRC signaling. Therefore, our understanding is that RAN2 shall consider the configured grant (CG)-like resource allocation to a specific UE reader.

R20 A-IoT WID

The work is subject to the following constraints:

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

Referring to the configured grant scheduling introduced in Rel-16, we believe that at least these three configured grant-like A-IoT resource allocation methods could be studied in T2:

- Type 1: The RRC signaling includes all resource parameters for A-IoT communication, such as period, time offset, frequency domain resources, and possible modulation and coding methods for R2D/D2R transmission. It becomes effective immediately upon the UE’s correct reception of the A-IoT resources and could be released upon the Re-configuration message.

- Type 2: The RRC signaling encompasses all A-IoT communication resource parameters (e.g., frequency, cycle). Activation or deactivation of the configuration is controlled by a subsequent message (e.g., DCI, MAC CE). These resources remain valid until an explicit deactivation message is received.

- Type 3: The RRC signaling initially provides partial A-IoT communication resource parameters (e.g., frequency domain). The remaining configuration (e.g., time domain) is added by a subsequent message (e.g., DCI, MAC CE, RRC signaling) and UE actives the resource accordingly. The subsequent message can be used to configure, update (modify), or release the A-IoT resources.

Proposal 1: For A-IoT communications, RAN2 could study the following configured grant-like resource allocation methods:

- Type 1: The RRC signaling contains all A-IoT resource parameters and takes effect immediately upon correct reception by the UE.

- Type 2: The RRC signaling includes all A-IoT resource parameters, which are activated by a subsequent message and remain valid until deactivation.

- Type 3: The RRC signaling initially provides partial A-IoT resource parameters, with the remaining configured later.

Information Obtained by the gNB for A-IoT resource allocation

RAN2#127 agreement has indicated that the resource allocation is the gNB’s responsibility. According to the SA2’s discussion [6], the following assistance information provided by AIOTF is visible to gNB, which could also be helpful for gNB to estimate and configure resource for A-IoT air interface between UE reader and A-IoT devices in T2. For example, the gNB will allocate resource according to service type or the number of target devices, e.g., more resources will be allocated for full inventory or a large number of devices. Moreover, gNB could consider the message size for (expected or maximum) D2R and R2D transmissions as an important factor for resource configuration, especially when the upper layer packet size is not fixed or the packet sizes for different A-IoT services (e.g., inventory or read/write command) differ.

TS 23.369 clause 5.4

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests.

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN:

a) A-IoT service type (e.g. Inventory, Command).

b) Optionally, approximate number of A-IoT devices based on AF request.

c) Size of the Inventory Response message from the A-IoT Device.

d) Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN as specified in clause 5.9.

For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN:

e) Size of the Command Response message from the A-IoT Device.

In addition to AIOTF, UE reader can also serve as an assistance provider to enhance the A-IoT resource allocation. For example, if the previously configured resources become invalid (e.g., due to a timer expiration) while the A-IoT procedure is still ongoing, UE reader may request notify the gNB and request additional resources. At this moment, UE reader can provide assistance information to gNB for A-IoT resource reallocation, which may include the information related to the service operation, such as the number of received device responses, any buffered A-IoT data (if the UE is required to aggregate data), estimated resource usage duration and so on.

Moreover, when UE reader switches to a new gNB as it moves, the A-IoT task context (e.g., A-IoT correlation identifier, Inventory/Command assistance information, received A-IoT responses, and A-IoT resource configuration) should be transferred from the source gNB to the new gNB (e.g., via Handover Request), which can be used as a reference for new gNB to allocate A-IoT resources for the remaining A-IoT procedure.

Proposal 2: gNB can allocate/update A-IoT resources for UE reader based on the assistance information provided by CN, UE reader and/or other gNB.

Specifically, the UE could provide the location related information for the initial/subsequent resource allocation. Typically, a gNB’s coverage area is much larger than that of UE reader. Ideally, a gNB should select only one UE reader within a single or multiple adjacent A-IoT area. This enables UE readers located far apart under the same gNB to reuse the same A-IoT time-frequency resources. Thus, the gNB can consider the A-IoT resource reuse based on the proximity of UE readers, thereby enhancing the system capacity within the gNB. Specifically, UE can provide its detailed location information (e.g., the A-IoT area ID where it is located) to assist the gNB in A-IoT resource configuration; and the gNB can provide a predefined A-IoT resource configuration related to specific A-IoT serving area for UE selection via SIB or dedicated signaling (e.g., A-IoT service request).

Proposal 3: UE readers’ location related information could be provided as the assistance information for A-IoT resource allocation.

A-IoT Resource Allocation Granularity

During the study phase, many options of resource allocation granularity were proposed as follows.

- per UE reader: For example, gNB may preconfigure a part of A-IoT resources after UE is authenticated to be a UE reader based on the UE capability (e.g., supported inventory frequency). The A-IoT resource type may be periodic or continuous (i.e., remain valid until released), and UE can utilize these resources for any A-IoT operation.

- per service request: For example, when an Inventory Request arrives at the gNB side, it should allocate resources for the whole A-IoT procedure (e.g., for inventory & command procedures if a follow on command indication is carried). The A-IoT resource type may be periodic, continuous or one-shot, and UE reader can schedule the specific resource by itself.

- Other options, such as per A-IoT paging (round), per R2D/D2R transmission (pair), per access round, and for some port of service request (e.g., for a fixed time, or a fixed number of transmissions).

In our view, overly fine-grained resource granularity may lead to high Uu signaling overhead between UE reader and gNB. Specially, gNB can only configure A-IoT resource via RRC messages, naturally along with an obvious delay cost, which will prolong the overall A-IoT operation duration if finer resource allocation is supported. This will also increase the A-IoT devices’ monitoring time and power consumption.

Moreover, compared with “per service request” granularity, allocating resources per UE reader may result in the resource overallocation/waste since gNB has no knowledge of the arrival time and UL A-IoT data size of a specific A-IoT service. Therefore, it is suggested to support “per service request” resource granularity as baseline.

Proposal 4: It’s suggested to support “per service request” resource granularity as baseline.

A-IoT Resource Allocation related Signaling

During the Rel-19 study item phase, there were two main alternatives proposed for resource allocation on A-IoT air interface in T2. As captured in TR 38.769 [7], gNB may allocate a set of A-IoT radio resource to the UE reader based on the service request from CN or request from UE reader.

For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN. NOTE 2: The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

In our view, there exists the following A-IoT resource allocation related signaling:

- A-IoT resource allocation message (gNB -> UE): From a functional perspective, A-IoT resource allocation message may be used to add/modification/release A-IoT resource configuration. Considering that gNB may allocate resources based on the UE’s request, it is necessary to introduce a new RRC message to support the A-IoT resource allocation. Moreover, other A-IoT related messages may also include the configuration of A-IoT radio resources, such as Inventory Request that is generally the first A-IoT related message sent to UE reader.

- A-IoT resource request message (UE -> gNB): UE reader requests the A-IoT radio resources, potentially for resource reconfiguration (e.g., request more resources or releasing resources). The A-IoT resource request message may include possible UE assistance information (e.g., number of received device responses, refer to the information from UE reader mentioned above) for the gNB to reference.

- A-IoT resource ACK message (UE -> gNB): UE reader confirms the successful reception of configured A-IoT resources. In 5G NR semi-persistent scheduling, UE typically responds to gNB via RRCReconfigurationComplete (CG type 1) or MAC CE (for UL grant activation validation, CG typ2 2) when obtaining the activated UL grant. In our view, it is needed for UE reader to respond as legacy because gNB may reconfigure the A-IoT resources when an RLF has occurred, but UE may not be aware of this and continue using the outdated resources during the RLF.

- A-IoT grant request message (UE->gNB): UE reader requests the A-IoT grant in the Uu interface, for example, when it has uplink A-IoT data to send and no currently available UL A-IoT grant exists. From the UE’s perspective, a BSR-like procedure may be required because the A-IoT traffic initiated by CN may be transparent to gNB and gNB is not aware of actual traffic situation in the UE reader.

Proposal 5: At least the following A-IoT resource allocation related signaling between UE-reader and gNB should be considered and specified:

- A-IoT resource allocation message (gNB -> UE)

- A-IoT resource request message (UE -> gNB)

- A-IoT resource ACK message (UE -> gNB)

- A-IoT grant request message (UE->gNB)

Resource Validity for RLF case

R20 A-IoT WID has agreed RAN2 will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). In RAN2#128 discussion, for UE reader performing A-IoT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation, two options of resource validity are proposed for down-selection in the WI phrase.

Reader UE performing A-IoT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation. (downselection between the following options will be performed in the WI phase) - Option 1: Resources remain valid at the reader for a time period (FFS whether the same time period as in RRC_CONNECTED options or a different time period. FFS whether the time period is related to the ongoing A-IoT operation). - Option 2: Resources are temporarily invalid at the reader during the “temporary out of connection” condition but may become valid again if the UE recovers from the condition.

In our view, we believe that the choice of option may depend on whether the UE needs to respond the previously received A-IoT service results to the gNB after recovering from RLF to the RRC-connected state. For example, if the UE is expected to send the A-IoT data that was received from A-IoT devices but not transmitted to gNB after re-establishing the connection, it would be beneficial for the UE to continue the A-IoT operation during RLF. However, if the UE discards the previous data upon RLF, then setting an additional resource validity duration for the RLF case (Option 1) would also be meaningless.

Proposal 6：RAN2 to discuss whether the UE could send the received A-IoT data after recovering from RLF.

In the legacy RRC re-establishment procedure, the UE will start T311 after transmitting the RRCReestablishmentRequest. If the UE receives an RRCReestablishment or RRCReestablishmentReject before T311 expires, T311 will stop. If the T311 expires, the UE will transition to the RRC_IDLE state. Currently, the T311 value options are: 1000ms, 3000ms, 5000ms, 10000ms, 15000ms, 20000ms, 30000ms, with the default value of 30000ms [8]. The time required for the RRC re-establishment process may be long even compared to the entire A-IoT procedure, and UE can continue the A-IoT operation during the RLF to avoid the waste of resources and time, particularly in scenarios where RLF occurs repeatedly (e.g., the channel quality of the serving cell is poor). Therefore, we prefer Option 1 for resource validity in the case of RLF.

Proposal 7: Support Option 1 for resource validity during RLF case.

With regard to how the UE reader determines the validity of configured A-IoT radio resources upon RLF occurs, we illustrate the following options of invalidity condition(s) based on the legacy procedure of UE detecting RLF and initiating the RRC connection re-establishment:

- Case 1: UE reader terminates the validity of resources upon detecting radio problem, i.e., upon T310 starting.

- Case 2: UE reader terminates the validity of resources when RLF is detected, i.e., upon T310 expiring.

- Case 3: UE reader terminates the validity of resources upon initiating the RRC re-establishment procedure, i.e., upon T311 starting.

- Case 4: UE reader terminates the validity of resources upon going back to idle (no suitable cell is found), i.e., upon T311 expiring.

- Case 5: UE reader terminates the validity of resources upon T311 stopping but a different cell is selected.

- Case 6: UE reader terminates the validity of resources upon sending the RRCestablishmentRequest message, i.e., upon T301 starting.

- Case 7: UE reader terminates the validity of resources upon T301 expiring and going back to idle.

- Case 8: UE reader terminates the validity of resources upon receiving the configured resources from a new cell.

Given that RAN2#128 has indicated that resources remain valid only in the same cell for which they were configured, we believe that case 3 (i.e., upon T311 starting) can be considered as one of resource invalidity condition(s). This is because, after the UE initiates the RRC re-establishment and cell reselection, it may select a new cell in which the previously configured resources are no longer valid. Moreover, during the cell reselection process, the UE needs to perform extensive measurements, which may affect the A-IoT operations.

Proposal 8: Support “T311 starts” as one of resource invalidity condition(s) upon RLF occurring.

  - UE reader selection

During the study phase, it has agreed that for RRC based architecture, gNB may be involved in reader selection but the information shared between gNB and CN is up to RAN3/SA2 (in RAN2#128 meeting). In our opinion, the information may include:

- Candidate or final UE readers selected by AIOTF [9];

- One or more UE reader ID(s) or the target area information provided by AF for A-IoT reader selection [9];

From gNB’s perspective, in addition to the information from CN/AF, the gNB is the entity closest to UE and can access more detailed UE-related information (e.g., radio resource allocation, UE’s RRC state, UE’s location, and ongoing services with high priority), enabling the gNB to accurately select UE readers.

- UE’s RRC state: Though UE shall be in the RRC-CONNECTED state (including RLF) during A-IoT operation due to the constraint defined in R20 WID, UEs in any RRC state can still be considered as candidate readers. When the UE readers in RRC-IDLE/INACTIVE states are requested to initiate the A-IoT service request, NW could send the paging message to trigger the transition of their RRC states.

- UE’s location: The gNB/CN may be not aware of the exact location of UEs in RRC-IDLE/INACTIVE states that are listed as candidate readers. It would be beneficial for gNB/CN to include the area information (e.g., requested A-IoT area list provided by AIOTF) in the paging message, allowing UEs that are authorized to act as reader in these areas to initiate random access (e.g., if UE is configured with serving A-IoT areas during authentication).

Proposal 9: UEs in any RRC state can be considered as candidate readers. NW could send the paging message to trigger the RRC state transition of requested UE readers.

Proposal 10: gNB/CN can include the information related to the requested A-IoT area list in the paging message.

- Conclusions

In this contribution, issues related to A-IoT T2 have been discussed and made the following proposals:

Proposal 1: For A-IoT communications, RAN2 could study the following configured grant-like resource allocation methods:

- Type 1: The RRC signaling contains all A-IoT resource parameters and takes effect immediately upon correct reception by the UE.

- Type 2: The RRC signaling includes all A-IoT resource parameters, which are activated by a subsequent message and remain valid until deactivation.

- Type 3: The RRC signaling initially provides partial A-IoT resource parameters, with the remaining configured later.

Proposal 2: gNB can allocate/update A-IoT resources for UE reader based on the assistance information provided by CN, UE reader and/or other gNB.

Proposal 3: UE readers’ location related information could be provided as the assistance information for A-IoT resource allocation.

Proposal 4: It’s suggested to support “per service request” resource granularity as baseline.

Proposal 5: At least the following A-IoT resource allocation related signalings between UE-reader and gNB should be considered and specified:

- A-IoT resource allocation message (gNB -> UE)

- A-IoT resource request message (UE -> gNB)

- A-IoT resource ACK message (UE -> gNB)

- A-IoT grant request message (UE->gNB)

Proposal 6: RAN2 to discuss whether the UE could send the received A-IoT data after recovering from RLF.

Proposal 7: Support Option 1 for resource validity during RLF case.

Proposal 8: Support “T311 starts” as one of resource invalidity condition(s) upon RLF occurring.

Proposal 9: UEs in any RRC state can be considered as candidate readers. NW could send the paging message to trigger the RRC state transition of requested UE readers.

Proposal 10: gNB/CN can include the information related to the requested A-IoT area list in the paging message.

- References

- RAN2#131-bis Chairman notes

- RAN2#127 Chairman notes

- TS 38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- RAN2#127-bis Chairman notes

- RP-251885

- TS 23.369, “Architecture support for Ambient power-enabled Internet of Things; Stage 2”, 3GPP

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

- TS 38.311, “NR; Radio Resource Control (RRC); Protocol specification”, 3GPP

- TR 23700-13, “Study on Architecture support of Ambient power-enabled Internet of Things”, 3GPP