3GPP TSG RAN WG2 Meeting #132	R2-2508609

Dallas, USA     17-21 November 2025

Agenda Item:	10.3.2.2 RRC Structure and (re) configuration

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Reducing RRC signalling overhead

Document for:	Discussion & Decision

# Introduction

The SI for 6G Radio (6GR) approved in RAN#109 [1], has among the RAN2-led objectives the following:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we discuss modularization of RRC Configuration for reducing signalling overhead, and modular update of configuration.

# Background and Motivation

In current NR deployments, RRCReconfiguration sent during handover is typically monolithic, meaning a large set of dedicated parameters is delivered to the UE at each handover, after initial Setup (first RRCReconfiguration) or upon RRC resume/reestablishment.

In practice, many parameters are identical or near-identical across adjacent cells within a layer (e.g., SCell composition and carrier characteristics, PDCP/RLC/MAC defaults, measurement/reporting configurations, mobility timers). Nevertheless, the UE receives the full set again at every handover.

Moreover, the RRC Reconfiguration procedure results in success or failure. In case of a failure indication, the network is unaware of the success or failure of the configured functionalities, which in some scenarios could succeed independent of another functionality.

This raises two questions:

- Is it necessary to deliver all parameters, when establishing connection in each cell, or can the signalling be reduced by reusing unchanged parameters with explicit scope?

- Can the RRC configuration be modularized to enable independent diagnosis and reporting of faults?

# Modularization of RRC Configuration

The monolithic RRC configuration means that if the UE cannot apply some required part of the configuration, the UE initiates RRC connection re-establishment, signaling (reestablishmentCause = reconfigurationFailure). There are no partial-apply behavior and no efficient mechanism for the UE to indicate to the gNB which part of the configuration failed to apply, so the network has limited insight into what went wrong and may repeat the same failure.

The only workable option is to revert back to the configuration that was there when RRCReconfiguration was sent. However, if the RRCReconfiguration was the first RRCReconfiguration after sending RRCSetup, then there are no DRBs that are established. Therefore, in this situation there is no working fallback connection for the UE.

Observation 1: Monolithic RRC configuration and no partial success of configuration can potentially lead to a deadlock, where the network configuration provided can fail repeatedly.

Proposal 1: RAN2 shall consider designing modules of configuration, so that

- The modules that are successfully configured can be retained, and only failed modules are reconfigured.

- An identifier can be included in failure message to indicate where the failure occurred

# Shared and cell-specific configurations:

Large portion of configurations many parameters are identical or near identical across adjacent cells within a layer (e.g., SCell composition and carrier characteristics, PDCP/RLC/MAC defaults, measurement/reporting baselines, mobility timers).

We note that, with the current mechanism, the gNB may provide RRCReconfiguration upon handover or resuming RRC connection in a new cell. This results in transmission of a large parameter set even when the target cell shares the same frequency layer and similar network policies as the source cell.

Of course, there are also parameters that are cell and UE-specific. Modularizing such parameters, such that only these parameters are provided, as long as there is a common understanding of parameters provided to the UE, could be a way forward in reducing message size, on reconfigurations and handover.

Observation 2: Despite RRC reconfiguration performed after every handover or after every RRCReestablishment, there are common parameters can be reused across cells.

Proposal 2: RAN2 shall consider RRC configuration with validity area scope. Unchanged RRC configuration shall be carried over from source to target cell.

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Monolithic RRC configuration and no partial success of configuration can potentially lead to a deadlock, where the network configuration provided can fail repeatedly.

Observation 2: Despite RRC reconfiguration performed after every handover or after every RRCReestablishment, there is a good number of parameters that can be reused across cells.

Based on above observations, we propose the following:

Proposal 1: RAN2 shall consider designing modules of configuration, so that

- The modules that are successfully configured can be retained, and only failed modules are reconfigured.

- An identifier can be included in failure message to indicate where the failure occurred

Proposal 2: RAN2 shall consider RRC configuration with validity area scope. Unchanged RRC configuration shall be carried over from source to target cell.

# References

| [1] | RP-252912, "Revised SID: Study on 6G Radio, RAN#109," 3GPP, Beijing, China, 2025. |
| --- | --- |