3GPP TSG-RAN2 #132	R2-2508610

# Dallas, TX, USA, Nov 17 – 21, 2025

Agenda item:	10.2.1 (FS_6G_Radio)

Source:	LG Electronics

Title: 	6G UE Capability Framework

Document for:	Discussion and decision

1.	Introduction

This document states our views on UE capability framework in 6G based on the last agreements on 6G capability discussion:

- Agreements on UE capabilities for next meeting For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them. Identify what actions to be triggered with other WGs. Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities. Understand if there is impact to other WGs. Companies can focus on the practical IODT problems in the field. Possible ways to address them.

2.	Discussion

2.1	Enhanced UE capability filtering

In NR, the network-initiated enquiry with filtering has been the practical way to obtain just the capability elements needed for the current serving cell. Without this mechanism, a 6G UE capability framework would either require the network to keep a complete “full UE capability” at all times or force the UE to retransmit full UE capability information whenever the UE is attached; it must cause bulk signaling, uplink segmentation, and network interface/storage burden.

Today in NR, a gNB can send UE Capability Enquiry with band filtering and common filtering (e.g., NR-only) so the UE composes capability information limited to those bands/CG/RAT. The gNB then configures CA/DC and MIMO layers based on that filtered capability report. While this NR filtering is coarse-grained because the filtering needs to be more fine-grained for specific band combinations (BC) or exclude unused BC linkages, we believe it was the right direction for distributed capability report what is operation-able on the serving cell, without a full UE capability.

However, UE capability report relies on enquiry with coarse filtering information (e.g., RAT, band, cell group) but the largest contributor to message size is the BC capabilities with Feature Set/Feature Set Combination linkages that must be for CA and DC operation. Even when the network requests only a small set of bands, the UE often composes massive BC capability information because current filters do not let the network pinpoint which BCs are actually relevant to near-future operation. This leads to sizable UE Capability Information, and duplicated capability band or RF parameters that the cell may not immediately use.

For example, in NR, the BC payload grows rapidly with each additional filtered band. If the network filters to bands {1, 2, 3}, the UE typically needs to compose 4 pairs of band combination {1-2}, {1-3}, {2-3}, and {1-2-3}. If {1, 2, 3, 4} are filtered, the UE now faces to compose 11 pairs of band combination {1-2}, {1-3}, {1-4}, {2-3}, {2-4}, {3-4}, {1-2-3}, {1-2-4}, {1-3-4}, {2-3-4}, and {1-2-3-4}. This expansion quickly dominates message size because pairs increase exponentially whenever filtered band added. While NR allows certain indications to omit fallback BC capabilities, that is not much useful in practice because detailed capability information often varies within supported BC capability (e.g., different UL MIMO layers or BW classes across combinations). When per-BC capability truly differs, the UE cannot rely on fallback indication, as a result, the UE must include the full BC capabilities.

Observation 1: Band combination capability information for CA/DC dominates capability information size, but current filtering (RAT/band/cell-group) are too coarse. Per-BC filtering is needed to avoid composing unused BCs on the current serving cell.

Given above, RAN2 is asked to study a per band combination filtering mechanism for reduced capability report, enabling the network to explicitly indicate which BCs are of interest, and allowing the UE to omit BC capability reporting accordingly. The aim is to cut BC-related payload.

Then we believe there is a clear benefit; with per-BC filtering, a serving cell about to consider, {1-2} for CA can ask only for {1-2} only instead of every BC implied by bands filtering {1, 2, 3, 4}. The UE then composes just those BC {1-2} entry.

Proposal 1: RAN2 to study defining per band combination filtering for UE capability retrieval including only particular band combination capabilities to reduce band combination related size.

In addition to Proposal 1, we suggest to consider the filtered BC capability in detail.

When the network first requests capability for band {1, 2}, the UE composes band and feature-set information for those bands. If the network later requests band {1, 2, 3}, the UE re-composes the band 1 and band 2 with related feature sets and their linkages, even though the network already received exactly same capability information. While NR uses identifiers for feature sets, feature-set combinations, and BCs, these IDs are typically tied to the report instance and not guaranteed reusable across other capability enquiries. The result is duplication, larger messages, and wasted processing on both sides.

Observation 2: When capability enquiries including same band filtering which was required before, the UEs should re-compose identical band/feature-set information because feature set IDs and feature set combination IDs are report-instance scoped.

Thus, RAN2 is asked to study an index-based linkage and delta reporting framework that allows a UE to reuse identifiers for previously reported band/feature-set information across successive capability enquiries, reporting only the deltas (e.g., newly added BC capabilities, modified linkage) if possible. The main purpose is robust reuse of UE capability information so that both the network and the UE can avoid re-composition and retransmission of identical UE capabilities.

To address this, RAN2 should coordinate with other WGs (e.g., RAN3 for inter-node exchange and SA2/CT1 for CN storage/UCMF interactions) to define the lifetime of capability indices and whether capability information is retained across vendor boundaries.

Proposal 2: RAN2 to study an index-based linkage and delta reporting for previously reported feature set information to avoid duplication and support incremental updates. This allows the UE to reuse the reported feature set IDs and feature set combination IDs not only for identical band, but also for other bands when composing UE capability reports.

Next, we suggest to consider more efficient way to report the filtered BC capabilities.

Networks requests many sets of BC capability upfront to prepare for CA/DC operation. However, practically, just some parts of BC capabilities are used and much of that rest BC capabilities are never used before the stored capability is discarded. This causes large upfront RRC signaling with limited operational value. If capability could be reported just-in-time, when pre-provision conditions indicate that CA/DC is likely to be required, the signaling exchange would be lighter, more relevant, and better aligned with the network’s near-future operation.

Observation 3: The network may request wide BC capability information upfront for future CA/DC configuration, but much of it is never used before the stored capability is discarded.

Thus, RAN2 is asked to study a UE-initiated, pre-provision condition-based reporting procedure for band combination capability, enabled by the network pre-provided conditions with existing measurement events. Upon configuration of the conditions, the UE continuously evaluates the conditions as one of measurement behavior. When the condition is satisfied, the UE sends a targeted BC capability report without waiting for an explicit network enquiry.

Working example can be described below:

Step 1. The UE accesses a cell and establishes RRC connection.

Step 2. The UE receives UECapabilityEnquiry with band filtering {band 1, band 2}; no BC capability is requested.

Step 3. The UE composes band 1 and 2 capability and defines linked feature sets (e.g., FS IDs #1/#2); no BC capability is composed.

Step 4. The UE sends UECapabilityInformation containing the above band 1 and band 2.

Step 5. The network configures measurements including Event A3, and indicates: when the event A3 is satisfied, report BC capability for {BC 1,2} together. (Serving: band 1; neighbor tied to A3: band 2.)

Step 6. The UE evaluates the event A3.

Step 7. Upon the event A3 satisfaction, the UE sends MeasurementReport and also sends UECapabilityInformation including BC capability for {BC 1,2}, with necessary identity linking to previously signaled bands/feature sets.

Step 8. The network uses the freshly received {BC 1,2} capability to configure DC.

Proposal 3: RAN2 to study a UE initiated band combination reporting mechanism without explicit network enquiry, enabled by the network pre-provided conditions.

2.2	Minimal mandatory capability set

In LTE, the UE category was introduced as a compact way to convey baseline UE capabilities (e.g., peak DL/UL, modulation, MIMO layers, buffer sizes), enabling predictable configuration with minimal UE capability signaling. That single-number, throughput-centric approach was not suited to support NR’s multi-dimensional features (i.e., FR1/FR2, diverse band combinations, many optional features across layers), so NR moved to fully explicit capability signaling.

However, while explicit signaling maximizes flexibility, it also repeats baseline capability information that can be shared across broad device types (e.g., eMBB, XR, wearable/RedCap, NTN), driving up massive UE Capability Information message size.

Observation 4: LTE’s throughput-based UE category was unsuitable to support NR’s multi-dimensional features, so NR adopted fully explicit capability signaling. However, this flexibility causes repeated transmission of baseline UE capabilities ineffectively for broad device types (e.g., eMBB, XR, wearable/RedCap, NTN), results in massive UE Capability Information size.

Thus, we suggest RAN2 is invited to study a UE type-based minimal mandatory capability set that is assumed without explicit reporting once the UE declares its type whereas non-baseline capability information would continue to be signaled explicitly as deltas.

For the study, RAN2 should first discuss define a capability set of each UE types (e.g., eMBB, XR, wearable/RedCap, NTN) capturing clear baseline commonalities. Per each UE type, defined capability set must be able to reflect cross-vendor including essential PHY and L2/L3 parameters. The principles should be coordinated with RAN1, RAN4, and SA2.

Proposal 4: To reduce capability message size, RAN2 to study a UE type-based minimal mandatory capability set that is assumed without capability reporting, with additional capabilities signaled as deltas.

2.3	Dynamic UE capability change

In NR, several situations considered as temporary or dynamic capability restriction on a UE’s actual capability: due to overheating/power-saving modes, MUSIM operation, and need-for-gap. Looking ahead to 6GR, additional cases will likely become more frequent or fundamental, such as dual registration across 5G and 6G and device form-factor changes (e.g., folded/unfolded mode may be likely to affect practical MIMO layers, antenna ports, or bandwidth).

Today these temporary capability changes are typically conveyed via UE Assistance Information (UAI). By this design, UAI is advisory and based on the network acceptance; the UAI is treated procedurally closer to a RRC re-configuration preference than to a change in the UE’s capability. This principle creates some systemic issues:

- Trust between the network and the UE

When the UE reports a change to the UE capability bounds, that report should be treated as truth, not a negotiable preference. However, UAI can be ignored or deferred by the network, this leads to repeated reconfiguration attempts and degraded user experience.

- Fragmented UAI fields

Each different issue (overheating, MUSIM, power saving, need-for-gap) defines their own UAI field information and separated UE behaviors. As a result, network reactions are inconsistent across vendors/operators, and the UE must rely on per-feature heuristics rather than a single, predictable capability update principle.

Observation 5: In NR, temporary capability change requests (overheating, power-saving, MUSIM, need-for-gap) are conveyed via UAI, which is advisory and based on the network acceptance; UAI can be ignored or deferred by the network even though the UE reports a change to the UE capability bounds. Moreover, UAI is fragmented on a per-use-case basis, leading to inconsistent network behaviour.

These weaknesses are amplified in 6GR. Dual registration may cause hard mutual exclusions on RF capability; form-factor transitions may directly change feasible antenna/MIMO states. Treating such events as mere “preferences” is procedurally mismatched because they are UE capability changes in essence.

Given above reasons, RAN2 is asked to study how capability updates can be conveyed via a unified capability signaling, i.e., using a capability container for UE capability report rather than via UAI. The goal is to align meaning and procedure: dynamic/temporary capability changes are treated as UE capability update.

Proposal 5: RAN2 to study UE capability update via the same RRC signaling (e.g., UE Capability Information in NR) for dynamic/temporary capability changes instead of relying on UAI.

2.4	RACS enhancement

Next, we suggest to consider Radio Access Capabilities Signaling (RACS) optimization together with dynamic capability.

In NR, existing ID-based optimizations (i.e. RACS) introduced to cut repeated capability signaling by mapping an identifier to a stored “full UE capability”. RACS uses an identifier mapped to a full capability information either a manufacturer-assigned ‘pre-defined ID’ or a ‘network-assigned ID’ created after the UE sends its capability to the network.

However, in practice, RACS effectiveness is so limited due to below reasons:

- Maintenance burden

With manufacturer-assigned IDs, the network must continuously maintain the global mapping (across releases, vendors, and product variants) and ensure it is available at every node where the UE may attach; otherwise, the network falls back to a full UE capability report. With the network-assigned IDs, the network may only assign an ID to the UE after receiving a full UE capability information at least once, which means the first capability exchange would be always huge.

- No reuse for partial/delta/dynamic capability

RACS maps a single ID to one UE capability information. The RACS doesn’t define reuse for partial, filtered, or incremental reports, nor delta signalling for temporary capability changes.

- Fragile management

When capability information expires from the network or is newly required due to UE mobility resulting in the network vendor change, the UE often has to re-transmit identical capabilities (bands, FS/FSC, BC entries). These operational realities mean UEs often recompose and resend large portions of UE Capability Information. As a result, the intended capability signaling savings of RACS are inconsistent, and networks frequently revert to full or wide filtered capability enquiries, reintroducing duplication

Observation 6: In NR, RACS mainly optimizes when a full UE capability information is reported and trusted; it lacks standardized for distributed capability reports and dynamic capability updates. After mobility, vendor change, or cache loss, the networks may revert to full or wide re-enquiry and UEs re-sends identical bands/FS/FSC/BC information.

Given above reasons, in 6G, device diversity and more frequent dynamic capability changes will amplify these pain points. What is missing is a lightweight, network-managed capability identifier that both sides can safely reuse across long time period.

Thus, we suggest to study such a capability ID based framework to implement more efficient ID-based optimizations. The focus is to specify how to reuse already reported capability information can be referenced consistently and safely without re-transmission based on RACS enhancement.

Working example can be described below:

Step 1. The UE accesses a cell and establishes RRC connection.

Step 2. The UE receives UECapabilityEnquiry with band filtering {band 1, band 2};

Step 3. The UE composes band 1 and 2 capability, related band combination capability and defines linked feature sets (e.g., FS IDs #1/#2), and feature set combination.

Step 4. The UE sends UECapabilityInformation containing the above band 1 and band 2.

Step 5. The network provides a network assigned ID#1 representing the previous UE capability information for band 1 and band 2.

Step 6. The UE determines to report UE capability change due to overheating problem.

Step 7. The UE sends UECapabilityInformation including reduced CC information to resolve the overheating problem.

Step 8. The network provides a network assigned ID#2 representing the last UE capability information for the overheating resolution.

Step 9. The UE confirms that the overheating problem is resolved.

Step 10. The UE sends UECapabilityInformation including the network assigned ID#1 instead of composing band 1 and 2 capability, related band combination capability.

Proposal 6: RAN2 to study UE capability ID approach as RACS enhancements, enabling safe reuse of multiple previously reported UE capabilities. When the UE needs to report/update UE capabilities, the UE can use one of IDs assigned by the network if the ID is suitable to represent the target capability information.

Last but not least, we suggest studying delta capability reporting in relation to the UE capability ID approach, to support more flexible capability ID usage.

Under the current capability framework, even small changes to a network assigned ID, representing one of the previous capability reports, the UE should re-compose all related capability containers. If UE can send delta information based on the network assigned ID, the UE capability ID approach can be expended wider use, and allowing the UE to send delta-payload only.

We believe that the delta capability reporting can bring maximum benefits when applying to the dynamic capability change.

Proposal 7: RAN2 to study delta capability reporting based on the UE capability ID approach for dynamic capability update/change cases, in order to maximize its benefits.

3. Conclusion

Based on the above discussions, we present the following proposals:

Observation 1: Band combination capability information for CA/DC dominates capability information size, but current filtering (RAT/band/cell-group) are too coarse. Per-BC filtering is needed to avoid composing unused BCs on the current serving cell.

Observation 2: When capability enquiries including same band filtering which was required before, the UEs should re-compose identical band/feature-set information because feature set IDs and feature set combination IDs are report-instance scoped.

Observation 3: The network may request wide BC capability information upfront for future CA/DC configuration, but much of it is never used before the stored capability is discarded.

Observation 4: LTE’s throughput-based UE category was unsuitable to support NR’s multi-dimensional features, so NR adopted fully explicit capability signaling. However, this flexibility causes repeated transmission of baseline UE capabilities ineffectively for broad device types (e.g., eMBB, XR, wearable/RedCap, NTN), results in massive UE Capability Information size.

Observation 5: In NR, temporary capability change requests (overheating, power-saving, MUSIM, need-for-gap) are conveyed via UAI, which is advisory and based on the network acceptance; UAI can be ignored or deferred by the network even though the UE reports a change to the UE capability bounds. Moreover, UAI is fragmented on a per-use-case basis, leading to inconsistent network behaviour.

Observation 6: In NR, RACS mainly optimizes when a full UE capability information is reported and trusted; it lacks standardized for distributed capability reports and dynamic capability updates. After mobility, vendor change, or cache loss, the networks may revert to full or wide re-enquiry and UEs re-sends identical bands/FS/FSC/BC information.

Proposal 1: RAN2 to study defining per band combination filtering for UE capability retrieval including only particular band combination capabilities to reduce band combination related size.

Proposal 2: RAN2 to study an index-based linkage and delta reporting for previously reported feature set information to avoid duplication and support incremental updates. This allows the UE to reuse the reported feature set IDs and feature set combination IDs not only for identical band, but also for other bands when composing UE capability reports.

Proposal 3: RAN2 to study a UE initiated band combination reporting mechanism without explicit network enquiry, enabled by the network pre-provided conditions.

Proposal 4: To reduce capability message size, RAN2 to study a UE type-based minimal mandatory capability set that is assumed without capability reporting, with additional capabilities signaled as deltas.

Proposal 5: RAN2 to study UE capability update via the same RRC signaling (e.g., UE Capability Information in NR) for dynamic/temporary capability changes instead of relying on UAI.

Proposal 6: RAN2 to study UE capability ID approach as RACS enhancements, enabling safe reuse of multiple previously reported UE capabilities. When the UE needs to report/update UE capabilities, the UE can use one of IDs assigned by the network if the ID is suitable to represent the target capability information.

Proposal 7: RAN2 to study delta capability reporting based on the UE capability ID approach for dynamic capability update/change cases, in order to maximize its benefits.